3GPP TSG RAN WG1 #100bis			                                   					       												        R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.6

Source:	Moderator (AT&T)

Title:	Summary of Email Approval [100b-e-NR-UEFeatures-eMIMO-03]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email approval [100b-e-NR-UEFeatures-eMIMO-03]  during RAN1 #100bis-e. According to the Chairman’s Notes:

[100b-e-NR-UEFeatures-eMIMO-03] Email discussion/approval till 4/24 – Ralf (ATT), (16-3 family) - Decide how many new FGs to add as a result of separating basic from optional components in 16-3a “Regular eType-II” - Decide how many new FGs to add as a result of separating basic from optional components in 16-3b “Port selection eType-II”

The following was discussed and agreed during RAN1 #100bis-e within the scope of [100b-e-NR-UEFeatures-eMIMO-03]  “Email discussion/approval” Error: Reference source not found.

The following will be removed from the final document, however, in the meantime, please take note of this guidance of the RAN1 MCC technical officer:

W.r.t the naming convention, the following suggestion […] may be helpful to keep the previous company’s name (only the most recent one) in the filename, so that we can easily tell which previous version this is based on, and may solve the issue when there are crossing emails. e.g. something like the following: 5_Incoming_Liaison_Statements/Summary-1_v1-LG 5_Incoming_Liaison_Statements/Summary-1_v2-LG-CATT 5_Incoming_Liaison_Statements/Summary-1_v2-LG-vivo 5_Incoming_Liaison_Statements/Summary-1_v3-CATT-HWHiSi

# Summary of Email Approval [100b-e-NR-UEFeatures-eMIMO-03]

The following is the proposal in Error: Reference source not found for discussion in this email approval:

FL Proposal 3: (16-3 family)

- Decide how many new FGs to add as a result of separating basic from optional components in 16-3a “Regular eType-II”

- Decide how many new FGs to add as a result of separating basic from optional components in 16-3b “Port selection eType-II”

## Regular eType-II

The following are the alternatives in Error: Reference source not found for discussion in this email approval:

Alt. 1:

| 16-3a | Regular eType-II | Basic components: - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II - 8 parameter combinations (FFS: Value of L per the number of antenna ports) - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with value R=1 - Rank restriction - FFS: UCI omission Optional components - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with R=2 - Rank 1 to 4 Support of rank 3,4 - CBSR - FFS: The maximum number of configured aperiodic CSI Report Settings - FFS: Support of mixed codebook types | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-3a-1 | Frequency unit size for eType-II | For regular eType-II: Support of PMI sub-bands with N3>19; [Support of PMI sub-bands with R=2 and N3 <=19] | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3a-2 | Rank for eType-II | Support of rank 3,4 for regular eType-II | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3a-3 | CBSR for eType-II | Support for CBSR for regular eType-II | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3a-4 | FFS: AP-CSI reports for eType-II | FFS: The maximum number of configured aperiodic CSI Report Settings | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3a-5 | FFS: Mixed codebook type for eType-II | FFS: Support of mixed codebook types | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |

Alt. 2:

| 16-3a | Regular eType-II | Basic components: - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II - 8 Support of parameter combinations (FFS: Value of L per the number of antenna ports) 1-6 - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with value R=1 N3<=19 - Rank restriction 1 and 2 - FFS: UCI omission Optional components - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with R=2 - Rank 1 to 4 Support of rank 3,4 - CBSR - FFS: The maximum number of configured aperiodic CSI Report Settings - FFS: Support of mixed codebook types | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-3a-1 | CSI-RS and number of PMI subbands for eType-II | For regular eType-II: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources across all CCs simultaneously, total # of Tx ports across all CCs simultaneously, Max # of PMI subbands N3}, where N3>=19 | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3a-2 | Rank for eType-II | Support of rank 3,4 for regular eType-II | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3a-3 | CBSR for eType-II | Support of CBSR for regular eType II | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3a-4 | Codebook parameter combination 7-8 for eType II | Support of codebook parameter combinations 7-8 | 16-3a, TBD |  |  |  |  |  |  |  |  |  |
| 16-3a-5 | FFS: Mixed codebook type for eType-II | FFS: Support of mixed codebook types | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |

Alt. 3:

| 16-3a | Regular eType-II | Basic components: - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II - 8 parameter combinations (FFS: Value of L per the number of antenna ports) - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with value R=1 - Rank restriction - FFS: UCI omission - Support of rank 1,2 Optional components - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with R=2 - Rank 1 to 4 Support of rank 3,4 - CBSR - FFS: The maximum number of configured aperiodic CSI Report Settings - FFS: Support of mixed codebook types | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-3a-1 | Frequency unit size for eType-II | For regular eType-II: Support of PMI sub-bands with N3>19; [Support of PMI sub-bands with R=2 and N3 <=19] | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3a-2 | Rank for eType-II | Support of rank 3,4 for regular eType-II | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3a-3 | CBSR for eType-II | (1) Whether UE supports CBSR (2) If UE supports CBSR, whether UE supports 4 values of restriction, i.e. amplitudeSubsetRestriction as in 38.214 | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3a-4 | FFS: AP-CSI reports for eType-II | FFS: The maximum number of configured aperiodic CSI Report Settings | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3a-5 | FFS: Mixed codebook type for eType-II | FFS: Support of mixed codebook types | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3a-6 | UCI omission | Support of UCI omission | 16-3a, TBD |  |  |  |  |  |  |  |  |  |

Alt. 4: Replace existing FG 16-3a with the following FG:

| 16-3a | Regular eType-II | CSI-RS capability: A list of supported combinations, each combination is of {Max # of Tx ports in one resource, max # of resources and total # of Tx ports} to support regular eType II Support of parameter combinations {support L=6, not support L=6} Number of PMI subbands {R=1, R=1-2} Jointly reported with component 1, i.e., a list of supported combinations, each combination is of { Max # of Tx ports in one resource, max # of resources and total # of Tx ports, R=1 or R=1-2} Supported rank: {1-2, 1-4} Support amplitude subset restriction level {no amplitude subset restriction, support amplitude subset restriction} | TBD | YES | N/A | Regular eType-II codebook is not supported. | FFS: Per band or Per band per BC or per band | N/A | N/A |  | Component 1 (same to FG2-36/40/41/43): Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component 2 (already agreed in RAN1#99): candidate values {‘support L=6’, ‘not support L=6’} Component 3: candidate values {‘R=1’, ‘R=1-2’} Component 4 (already agreed in RAN1#99): candidate values {‘RI={1,2}’, ‘RI={1,2,3,4}’} Component 5 (already agreed in RAN1 #98b): candidate values { ‘no amplitude subset restriction’, ‘support amplitude subset restriction’} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

According to the feedback in [1], Alt. 1 has twice as many supporting companies as all other alternatives. However, it seems Alt. 1, Alt. 2, and Alt. 3 are not complete proposals due to the FFS for 16-3a-4 and 16-3a-5. Furthermore, there’s divergence as to whether new FGs are needed, or simply candidate component values. Companies are invited to address these topics and make constructive suggestions on how to finalize the number of rows/FGs for a new baseline. At the least, we should try to complete the proposals, so we don’t have to add new rows/FGs after RAN1 #100bis-e.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We prefer the following FG design. However, as long as UE has flexibility to report its capability, we are fine the number of FGs, i.e. merging the FGs FG1: Basic FG that includes the triplets and the features that UE has to support conditioned on UE supports Rel-16 Type 2 FG2: R=2 FG3: L=6, i.e. parameter setting 7/8 FG4: RI=3/4 FG5: CBSR. At least for soft amplitude part. FFS: whether hard amplitude is part of FG1 The following we are open for further discussion FG6: FFS: AP-CSI, i.e. whether UE needs to increase its maximum value range for FR2 FG7: FFS: Concurrent codebookTypes, this can be a separate FG not depending on Rel-16 Type II or Type II port selection FG8: UCI omission especially the part that divides part 2 into 3 groups. |

The highlighted parts below of the reference FG description in [2] were discussed on April 20, 2020 during the eMIMO GTW conference call of rAN1 #100bis-3.

| 16-3a | Regular eType-II | Basic components: - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II including PMI sub-band size (FFS: Support of PMI sub-bands with R=2 as new FG instead) - ALT 1-1) 8 parameter combinations (FFS: Value of L per the number of antenna ports) ALT 1-2) Support of parameter combinations 1-6 - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with value R=1 - Support Rank restriction of rank 1,2 - FFS: UCI omission [based on CSI group definition] - FFS: CBSR with hard amplitude restriction Optional components - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with R=2 - Rank 1 to 4 Support of rank 3,4 - ALT 1) CBSR with soft amplitude restriction (capture consequence if not supported  hard amplitude restriction is supported) - ALT 2) CBSR - ALT 3) soft amplitude restriction - FFS: The maximum number of configured aperiodic CSI Report Settings for all codebook types (ALT 1 new 16-x ALT 2 handle by type or BC: more ports in FR1 than FR2) - FFS: new Support of mixed codebook types (new FG) - optional parameter combinations (see Alt. 1-1/1-2) | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

As discussed during the conference call, let’s continue the discussion on the highlighted items. Note that there was consensus (no agreement) to separate basic and optional components in FG 16-3a. The below is in this context.

Signalling load from including PMI sub-band size into existing triplet in component (1) also taking into account type (FFS: per band or per band per BC) versus new FG for support of PMI sub-bands with R=2

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We prefer to have separate FG to indicate whether UE supports R=2. Note that it was already agreed that it is optional for UE to support R=2 for N3>19, the remaining question is whether it is optional for UE to support R=2 for N3<=19 |
| ZTE | We support to report the quadruplet {Max # of Tx ports in one resource, Max # of resources, total # of Tx ports, maximum # of PMI subbands per CQI subband}. It can be reported in the basic FG 16-3a as Huawei suggested, which means if this quadruplet is reported, UE supports eType II CSI. We think it is beneficial for NW to make full use of UE’s resource to calculate CSI. A UE may support R=2 for 16 ports but R=1 for 32 ports. If maximum # of PMI subbands per CQI subband is reported separately, UE has to consider the worst case and report R=1 as the maximum number. This would cause UE to under report its capability for eType II CSI processing, and NW would lose the performance gain of R=2 for 16 ports. Regarding signaling load, compared to what we have in Rel-15, the only change is to add one bit to indicate maximum # of PMI subbands per CQI subband in each quadruplet, which is not large compared to the total number of bits. In addition, RAN2 can figure out further details to save overhead. For example, if maximum # of PMI subbands per CQI subband is not reported, the default value is 1. It’s up to RAN2 to optimize the signaling. |
| CATT | We prefer to have separate FG. Regarding signaling overhead, it is not just one more bit to indicate maximum #of PMI subband per CQI subband. UE has to report more quadruplets. E.g., for a UE supporting 32 ports, UE needs to report one more quadruplet to indicate that it supports 16 ports with R=2. |
| Qualcomm | First, to help make a progress, we may first agree on R=1 in basic feature and R=2 as optional. Whether R is reported separately reported or together with CSI-RS capability is a secondary issue. Second, in principle, we support to report the quadruplet {Max # of Tx ports in one resource, Max # of resources, total # of Tx ports, maximum # of PMI subbands per CQI subband}. R=2 introduces two new additional impacts: 1) number of PMI subbands great than legacy spec, and 2) 2 PMIs in one CQI subband. These two impacts add on the computation complexity and memory to process CSI calculation, so it is natural for UE to support max 2 resources if R=1 and max 1 resource if R=2. If report R=2 separately, it would mean that UE has to support same CSI-RS capabilities for R=1 and R=2, so that the UE may fall back to report R=1 only. Reporting R=2 jointly with CSI-RS capabilities would encourage UE to report R=2 by taking complexity into account. There could be some ways to reduce overhead. We could report triplet in basic feature for R=1 only, and add a new row for reporting triplet for R=2 as an option. In this way, the overhead for default feature is same to having separate FG of R=2. Moreover, RAN2 could define a common set for the quadruplet/triplet and use index to refer one of them for R=1/2. Handling the overhead is more upto RAN2 design, RAN1 should concentrate on providing proper design. |
| Intel | In our view the UE capability signaling overhead can be adjusted by the UE. E.g. UE can indicate 7 quadruplets or 2 quadruplets. So, we are OK to support R=2 indication in quadruplet (basic component 1), good tradeoff between optimization of UE capabilities according to processing power and signaling overhead. |
| vivo | R=2 for N3>19 is optional, which has been agreed and in our view it is also optional for N3<=19. Our preference is separate FG |
| MotM/Lenovo | R=2 roughly doubles the computational complexity and hence should be reported jointly with CSI-RS capabilities. OK to include R in existing triplet in basic component 1. Any signaling optimization can be left to RAN2. |
| LGE | We prefer to include maximum # of PMI SBs per CQI SB into the triplet as a basic component. From a NW perspective of guaranteeing PMI accuracy and the performance, it is favorable to support R=2 with almost the same amount of overhead for FD basis indication to fully utilize the UE capabilities. |
| Nokia/NSB | In principle it is ok to distinguish resource capabilities for R=1 and R=2 to avoid underreporting. However, basic components should only list mandatory capabilities for a UE supporting regular eType2. A quadruplet with R=2 is not a basic component if R=2 is optional. One solution can be to list the triplets for R=1 in basic component 1 and the triplets for R=2 in optional component 1. To reduce the overhead the maximum total number of triplets can be the same as for regular Type2, i.e. 16, including the variants with R=1 and R=2. However, overhead optimization should be discussed in RAN2. RAN1 should focus on providing a proper design which is consistent with agreements. |
| DOCOMO | We prefer to have a separate FG to indicate R. As per current agreement R=2 is optional since N3>19 is optional. Further, for N3<=19, whether R=2 is optional is not yet decided. Hence, as per our current understanding, reporting value of R within the triplet is not necessary and may unnecessarily increase the reporting overhead since there can be multiple triplets reported by the UE |
| Huawei | R=2 will double the computational complexity. So we prefer to jointly report in basic component 1. |
| Fraunhofer | OK to include it in basic component 1. |

Alt. 1-1 versus Alt. 1-2, what’s basic and optional (see new optional component (8))

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | It is hard to differentiate basic and optional, below is our understanding For a component that is either “support” or “not support”, UE is mandatory to support if it is basic component of a FG, i.e. there will be no additional capability related signalling. For a component associated with value range, even if the component is categorized as basic component for a FG, UE can report any value in the value range or not reporting the corresponding component provided that the FG itself is optional. |
| ZTE | We think Alt 1-2, which is already agreed, is sufficient for the report of supported parameter combinations. Then optional component 8 should be the support of parameter combination 7 and 8. |
| CATT | According to past RAN1 agreement, combination 7 and 8 shall be optional feature. |
| Qualcomm | We support Alt1-2 as it was previous agreement in RAN1 #99. Agreement For Rel.16 Type II codebook/CSI, the support for parameter combinations 1, 2, 3, 4, 5, and 6 is mandatory, while the support for parameter combinations 7 and 8 is optional. |
| Intel | Considering the agreement, parameter combinations 7 and 8 shall be optional while parameter combinations 1-6 shall be mandatory (Alt. 1-2). Since parameter combinations 7 and 8 are supported only with 32 CSI-RS port, there is no need to indicate L value per number of ports. |
| vivo | Combination 7 and 8 are optional |
| MotM/Lenovo | As per the RAN1 agreement, parameter combinations with L=6, i.e., combinations 7 and 8, should be optional |
| LGE | Support Alt1-2 according to the agreement in RAN1#99 |
| Nokia/NSB | Alt 1-2. It was already agreed that parameter combinations 1 to 6 are mandatory w/o capability signalling. The optional parameter combinations 7-8 can be in optional component 8 with 1 bit capability [Agreement in RAN1#98b-Chongqing: Mandatory support for L=2, 4 • Supported without additional UE capability signalling Agreement in RAN1#99-Reno: For Rel.16 Type II codebook/CSI, the support for parameter combinations 1, 2, 3, 4, 5, and 6 is mandatory, while the support for parameter combinations 7 and 8 is optional. Note: Mandatory support for L=2,4 was agreed in RAN1#98bis] |
| DOCOMO | Adhering to RAN1 agreement, parameter combination 7 and 8 should be optional |
| Huawei | Alt 1-2 seems to be aligned with agreement better. |
| Fraunhofer | Combinations 7 and 8 should be optional and 1-6 should be mandatory. |

Updated component (4): Support of rank 1,2

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | This is agreed to be basic component for Rel-16 Type II, i.e. mandatory to support for UE that supports Rel-16 Type II |
| ZTE | We support this change. |
| CATT | We support to explicitly state that rank 1, 2 are supported as basic feature. But we need a clarification on whether rank restriction shall be supported as basic feature. |
| Qualcomm | We support this change as it was previous agreement Agreement For Rel.16 Type II codebook/CSI, - The support for maximum rank of 3 is optional - The support for maximum rank of 4 is optional |
| Intel | OK |
| vivo | Ok |
| MotM/Lenovo | We support this change |
| LGE | Support |
| Nokia/NSB | Ok as it was already agreed as basic component w/o capability signalling [Agreement in RAN1#98b-Chongqing: Mandatory support for maximum rank of 1 and 2 • Supported without additional UE capability signalling] |
| DOCOMO | We support this |
| Huawei | It shall be basic component. We suggest to remove it from the list. |
| Fraunhofer | Support. |

A lot of companies asked for clarification of basic component (5): UCI omission, “based on CSI group definition” was proposed as clarification but still not deemed sufficient. Proponents of this basic feature are asked to clarify what they mean with this basic capability

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Our understanding is that unlike Rel-15 Type II, there is no wideband PMI reporting for Rel-16 Type II which was clarified in the last RAN1#100 meeting. And For Rel-15 Type II sub-band PMI reporting, it is either completely omitted or transmitted. However, in Rel-16, the enhancement for subband PMI reporting is to break it into 3 groups while omitting some group(s) can makes the CSI reporting not even useful, in our view. This becomes a new UE behavior compared to Rel-15 Type II CSI reporting |
| ZTE | In our understanding, Rel-15 comprises three groups of Part 2 CSI (see Table 5.2.3-1 in 214), i.e., - Group 0: wideband part-2 CSI, - Group 1: subband part-2 CSI for even subbands, - Group 2: subband part-2 CSI for odd subbands. Rel-16 also comprise three groups of Part 2 CSI. Although the three groups contain different CSI parameters, the omission order and rule for the three groups in Rel-16 is same as Rel-15. Hence we think essentially the Rel-16 UCI omission procedure is not different from Rel-15. Further, we have agreed CSI will not be re-calculated if UCI omission happens in previous RAN1 meetings. CSI parameters are mapped to the UCI sequence based on order of the three groups. UE just omits the low-priority bits if needed. The design of UCI mapping and omission has guaranteed that UE will not re-calculate CSI. Therefore, same as Rel-15, we can either remove UCI omission in the list, or clarify it is a basic component. |
| Intel | In our view we can remove UCI omission from the list with the understanding that this feature is mandatory supported, so we can avoid unnecessary discussion on the details. |
| MotM/Lenovo | We agree with ZTE and Intel; no CSI recalculation is needed to support UCI omission so it incurs no additional complexity at the UE. We prefer it is removed from the list. |
| LGE | We slightly prefer to include UCI omission into a basic component. As in Rel-15 CSI reporting in PUSCH, the principle of omission is quite similar to that of Rel-16 which does not require re-calculation of CSI and this operation can be regarded as inherent to support Rel-16 Type II CSI codebook. |
| Nokia/NSB | We can remove UCI omission from the list as it is clearly an integral functionality of the design of eType II CSI feedback. UCI omission is not an additional component but the result of UCI mapping when a mismatch between PMI size and PUSCH capacity occurs. |
| DOCOMO | UCI omission feature should be mandatory supported, and we are also fine with removing that from the list |
| Huawei | Same with DCM. It is basic and shall be removed from the list. |
| Fraunhofer | UCI omission is a part of the eType-II codebook. Without UCI omission, eType-II reporting may not work. We are fine to remove UCI omission from the list. |

CBSR with hard/soft amplitude restriction (FFS in basic component (6), Alt. 1, Alt. 2, Alt. 3 in optional components …)

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We need a separate FG for CBSR anyway, at least for soft amplitude. Since this stage of the discussion is mainly for the discussion of the number of FG(s), we can leave the discussion of whether CBSR with hard amplitude is basic component later. |
| ZTE | Alt 1) and Alt 3) are the same in our view. To us, at least we need Alt 1) or Alt 3) as one optional FG. Whether this FG can be extended to include Alt 2) can be further discussed. |
| CATT | We have clear agreement in RAN1#98bis that hard amplitude restriction is mandatorily supported. We just need to capture CBSR with hard amplitude restriction in basic feature. Agreement For amplitude restriction mechanism: - Alt 0. Analogous to Rel.15 Type II (SD beam group restriction + per coefficient hard amplitude restriction) - Four beam groups are selected via higher-layer configured bitmap B1 - For each spatial beam in each of the four beam groups, hard restriction (maximum amplitude of 0 or 1) is applied to any of the coefficients associated with the beam (the restriction is applied for both polarizations of the beam). This maximum amplitude restriction is higher-layer configured with four bitmaps - Alt 3A from RAN1#98 (soft with sum-power-ratio constraint), simplified to for each where denotes the number of NZCs associated with . - The value of is configured from the Rel.15 2-bit amplitude restriction table - The number of beam-groups is the same as Rel.15 Type II CBSR Support Alt0 as mandatory and Alt3A (described above) as optional analogous to Rel.15 Type II codebook. - Cf. Notes in UE capability agreement in RAN1#98bis regarding “mandatory” and “optional” |
| Qualcomm | We think at least “soft amplitude restriction” should be a separate row as an optional feature (due to previous agreement). We are open to either Alt1/2/3. |
| Intel | UE feature list should be aligned with the RAN1 agreement (Alt 1 or Alt 3). |
| vivo | Per RAN1 agreement hard amplitude restriction is mandatory thus can be part of basic feature. Soft amplitude restriction is optional |
| MotM/Lenovo | As per the CBSR agreement, hard amplitude restriction is mandatory, whereas soft amplitude restriction should be optional |
| LGE | We support Alt1 or Alt3 in optional FG. |
| Nokia/NSB | We can list it in the basic components as “CBSR without support for amplitudeSubsetRestriction”, w/o capability signalling. In the optional component we can add “CBSR with support for amplitudeSubsetRestriction” |
| DOCOMO | Adhering to RAN1 agreement, we support for mandatory hard amplitude restriction. Soft amplitude restriction should be an optional feature |
| Huawei | hard amplitude restriction is mandatory |
| Fraunhofer | Soft amplitude restriction should be an optional component, while hard amplitude restriction is mandatory and should be listed in the basic components. |

Optional component (6), how to enable different maximum number of configured aperiodic CSI Report Settings for FR1 and FR2: new FG? FRx differentiation, per BC signalling …)>

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Per BC is okay for us as of now |
| ZTE | To make it per band or per BC can solve the concern for FR2. |
| CATT | In our view, it can be reported per band. |
| Qualcomm | In our view, it should be per-band only and should be a new FG. We don’t agree to have it for FR2 as eType II/Type II are not typically used because there are not many ports in FR2. This feature is related to FG2-35 (increasing the max value of component 2), which is reported per-band, so making it per-band is sufficient. Allowing per-BC may include band combo of FR1 + FR2. So, this should be per-band. |
| Intel | Per band reporting is OK. |
| vivo | Agree with QC |
| MotM/Lenovo | Agree with QC |
| LGE | Support per-band reporting |
| Nokia/NSB | In our view, this should be a separate FG and per-band as it is not restricted to a particular codebook, |
| Huawei | New FG. Per band is fine. |
| Fraunhofer | Per band reporting is fine. |

Is a new FG “Support of mixed codebook types” needed?

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We are open for further discussion. We slightly prefer to have this FG |
| CATT | We prefer to have this FG. |
| Qualcomm | We think this new FG is essential. The motivation is to restrict CSI-RS capabilities in the case of concurrent codebooks with mixed types, otherwise UE has to underreport the capability of each codebook. As it applies to concurrency of any codebook types, Type I, II eType II, etc, it should be a separate FG. Further details can be discussed via email discussion in this week or week after. |
| Intel | We support this FG. |
| vivo | We see the motivation, details can be FFS |
| MotM/Lenovo | We see the merit of this FG. Further discussion is needed though |
| LGE | While we are open to discuss, the concern is the use case for concurrent CBs and the corresponding benefits compared to the typical NW implementation where a specific CB type is switched to report CSI depending on the NW payload and scheduling among the configured CBs |
| Nokia/NSB | We support this new capability in a separate FG. |
| DOCOMO | Although we are open to discuss, one concern of mixed CB type reporting is that it would increase overhead of UE capability signaling, especially for the case NW only implements type I single panel codebook and UE implements multiple CSI codebook types. |
| Huawei | We support this new FG. |

## Port selection eType-II

The following are the alternatives in Error: Reference source not found for discussion in this email approval:

Alt. 1:

| 16-3b | Port selection eType-II | Basic components: - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II - 6 parameter combinations (combos with L=6 don’t apply) (FFS: Value of L per the number of antenna ports) - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with value R=1 - Rank restriction - FFS: UCI omission Optional components: - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with R=2 - Rank 1 to 4 Support of rank 3,4 - FFS: The maximum number of configured aperiodic CSI Report Settings - FFS: Support of mixed codebook types | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-3b-1 | Frequency unit size for port selection eType-II | For port selection eType-II: Support of PMI sub-bands with N3>19; [Support of PMI sub-bands with R=2 and N3 <=19] | 16-3b, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3b-2 | Rank for port selection eType-II | Support of rank 3,4 for port selection eType-II | 16-3b, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3b-3 | FFS: AP-CSI reports for port selection eType-II | FFS: The maximum number of configured aperiodic CSI Report Settings | 16-3b, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3b-4 | FFS: Mixed codebook type for port selection eType-II | FFS: Support of mixed codebook types | 16-3b, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |

Alt. 2:

| 16-3b | Port selection eType-II | Basic components: - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II - 6 parameter combinations (combos with L=6 don’t apply) (FFS: Value of L per the number of antenna ports) - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with value R=1 N3<=19 - Rank restriction - FFS: UCI omission Optional components: - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with R=2 - Rank 1 to 4 Support of rank 3,4 - FFS: The maximum number of configured aperiodic CSI Report Settings - FFS: Support of mixed codebook types | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-3b-1 | CSI-RS and number of PMI subbands for port selection eType-II | For port selection eType-II: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources across all CCs simultaneously, total # of Tx ports across all CCs simultaneously, Max # of PMI subbands N3}, where N3>=19 | 16-3b, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3b-2 | Rank for port selection eType-II | Support of rank 3,4 for port selection eType-II | 16-3b, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3b-4 | FFS: Mixed codebook type for port selection eType-II | FFS: Support of mixed codebook types | 16-3b, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3c | Number of AP-CSI report settings per BWP | Support up to 8 configured aperiodic CSI report setting per BWP | TBD |  |  |  |  |  |  |  |  |  |

Alt. 3:

| 16-3b | Port selection eType-II | Basic components: - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II - 6 parameter combinations (combos with L=6 don’t apply) (FFS: Value of L per the number of antenna ports) - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with value R=1 - Rank restriction - FFS: UCI omission - Support of rank 1,2 Optional components: - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with R=2 - Rank 1 to 4 Support of rank 3,4 - FFS: The maximum number of configured aperiodic CSI Report Settings - FFS: Support of mixed codebook types | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-3b-1 | Frequency unit size for port selection eType-II | For port selection eType-II: Support of PMI sub-bands with N3>19; [Support of PMI sub-bands with R=2 and N3 <=19] | 16-3b, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3b-2 | Rank for port selection eType-II | Support of rank 3,4 for port selection eType-II | 16-3b, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3b-3 | FFS: AP-CSI reports for port selection eType-II | FFS: The maximum number of configured aperiodic CSI Report Settings | 16-3b, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3b-4 | FFS: Mixed codebook type for port selection eType-II | FFS: Support of mixed codebook types | 16-3b, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3b-5 | UCI omission | Support of UCI omission | 16-3b, TBD |  |  |  |  |  |  |  |  |  |

Alt. 4: Replace existing FG 16-3a with the following FG:

| 16-3b | Port selection eType-II | CSI-RS capability: A list of supported combinations, each combination is of {Max # of Tx ports in one resource, max # of resources and total # of Tx ports} to support eType II port-selection Number of PMI subbands {R=1, R=1-2} Jointly reported with component 1, i.e., a list of supported combinations, each combination is of { Max # of Tx ports in one resource, max # of resources and total # of Tx ports, R=1 or R=1-2} Supported rank: {1-2, 1-4} | TBD | YES | N/A | eType-II port-selection codebook is not supported. | FFS: Per band or Per band per BC or per band | N/A | N/A |  | Component 1 (same to FG2-36/40/41/43): Maximum size of the list is 16. The candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component 2: candidate values {‘R=1’, ‘R=1-2’} Component 3 (already agreed in RAN1#99): candidate values {‘RI={1,2}’, ‘RI={1,2,3,4}’} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

According to the feedback in [1], Alt. 1 has twice as many supporting companies as all other alternatives. However, it seems Alt. 1, Alt. 2, and Alt. 3 are not complete proposals due to the FFS for 16-3b-4 and 16-3b-5. Furthermore, there’s divergence as to whether new FGs are needed, or simply candidate component values. Companies are invited to address these topics and make constructive suggestions on how to finalize the number of rows/FGs for a new baseline. At the least, we should try to complete the proposals, so we don’t have to add new rows/FGs after RAN1 #100bis-e.

The issues and positions are also very aligned between regular eType-II and port selection eType-II. It may be beneficial to first stabilize the former and then discuss the latter.

Can we agree Alt. 1 as new baseline/reference for single-DCI based multi-TRP? Further details such as component description, candidate values, type, xDD/FRx differentiation can be part of a second round email discussion.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We prefer the following FG design. However, as long as UE has flexibility to report its capability, we are fine the number of FGs, i.e. merging the FGs FG1: Basic FG that includes the triplets and the features that UE has to support conditioned on UE supports Rel-16 Type 2 port selection FG2: R=2 FG3: RI=3/4 The following we are open for further discussion FG4: FFS: AP-CSI, i.e. whether UE needs to increase its maximum value range for FR2 FG5: FFS: Concurrent codebookTypes, this can be a separate FG not depending on Rel-16 Type II or Type II port selection FG6: FFS: UCI omission especially the part that divides part 2 into 3 groups. |
| ZTE | We think this set of FGs can adopt a same structure as regular eType II CSI. The details can be settled once the FGs for regular eType II is stable. |
| CATT | We agree ZTE’s view that FG for PS eType II can reuse the same structure as regular eTypeII. |
| Qualcomm | We can finalize this part after eType II is stable. |
| Intel | Same view as ZTE and CATT. |
| MotM/Lenovo | Should be considered only after eType II is finalized |
| Nokia/NSB | We understand the question is whether the structure of port selection eType II can follow that of the regular eType II, where applicable, and this is fine for us. |
| Huawei | Same feeling. No discussion for this one until eType II is stable. |
| Fraunhofer | Same view than CATT. |

# Conclusions

…

# References

- R1-2001868, Summary on UE features for eMIMO, Moderator (AT&T)

- R1-2001484, RAN1 UE features list for Rel-16 NR after RAN1#100-E, AT&T & NTT DOCOMO