3GPP TSG RAN WG1 #100bis			                                   					       												        R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.9

Source:	Moderator (AT&T)

Title:	Summary of Email Approval [100b-e-NR-UEFeatures-Mobility-02]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email approval [100b-e-NR-UEFeatures-Mobility-02] during RAN1 #100bis-e. According to the Chairman’s Notes:

[100b-e-NR-UEFeatures-Mobility-02] Email discussion/approval proposal 2 and proposal 5 (high priority item) in R1-2001870 by 4/24 – Ralf (ATT)

The following was discussed and agreed during RAN1 #100bis-e within the scope of [100b-e-NR-UEFeatures-Mobility-02] “Email discussion/approval proposal 1 (high priority item) in R1-2001870” Error: Reference source not found.

The following will be removed from the final document, however, in the meantime, please take note of this guidance of the RAN1 MCC technical officer:

W.r.t the naming convention, the following suggestion […] may be helpful to keep the previous company’s name (only the most recent one) in the filename, so that we can easily tell which previous version this is based on, and may solve the issue when there are crossing emails. e.g. something like the following: 5_Incoming_Liaison_Statements/Summary-1_v1-LG 5_Incoming_Liaison_Statements/Summary-1_v2-LG-CATT 5_Incoming_Liaison_Statements/Summary-1_v2-LG-vivo 5_Incoming_Liaison_Statements/Summary-1_v3-CATT-HWHiSi

# Summary of Email Approval [100b-e-NR-UEFeatures-Mobility-02]

The following is the proposal in Error: Reference source not found for approval in this email discussion:

FL Proposal 2 (21-2):

| 21-2 | UE power sharing for DAPS HO | ALT 1) Indicates support of dynamic UL power sharing during DAPS-HO operation. ALT 2) Indicates support of UL power sharing mode during DAPS-HO operation. ALT 3) Indicates support of UL power sharing modes during DAPS-HO operation. ALT 4) … | DAPS (Note: RAN2 feature) | Yes | N/A | ALT 1) The UE is only able to perform semi-static power allocation for source and target cell, or to drop the transmission to the source. ALT 2) The UE is only able to to drop the transmission to the source. | [Per BC] for inter-frequency case, Per Band for intra-frequency case | No | N/A | N/A |  | ALT 1) Optional with capability signalling ALT 2) Optional with capability signalling {values: Semi-static-mode1, Semi-static-mode2, Dynamic} ALT 3) Candidates are - no power sharing mode, - no power sharing mode, and semi-static mode 1 - no power sharing mode, semi-static mode 1, and semi-static mode 2 - no power sharing mode, semi-static mode 1, and dynamic mode no power sharing mode, semi-static mode1, semi-static mode 2, and dynamic mode ALT 4) … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- High priority

  - Conclude the type

  - Conclude the component description

  - Conclude the consequence if the feature is not supported by the UE

  - Conclude candidate component values, if any

Companies are asked to provide their views and comments in the following tables.

Regarding the type:

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | The type should be per BC, as the feature should be applicable for intra-frequency (per band) as well as inter-frequency (per BC). |

Regarding the component description:

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | The description should be such that it can indicate all the power sharing modes that the UE support. If we support Alt 2, where the capability only down selects from one of the power sharing modes, it is not clear it this means UE cannot support more than 1 power sharing mode or not. Therefore, the description is quite ambiguous. For Alt 1 description, it is clear the capability is only for indicating dynamic power sharing mode. This would be better than Alt 2 as the capability is clear on which power sharing mode are being optionally supported. For this it should be made clear that no power sharing and semi-static power sharing mode 1 and 2 should be supported by default. Alt 3 is our preference as it provide the flexibility for the UE to support a meaningful combination of power sharing mode, while be crystal clear on which power sharing modes are being supported by the UE. |

Regarding the consequence if the feature is not supported by the UE:

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | The description should be reflected based on the description agreed. If Alt 1 is agreed, then it should be clarified that UEs not supporting this feature should still support no power sharing mode, semi-static power sharing mode 1 and 2. If Alt 3 is agreed, then the consequence should be that UE only support UL DAPS without any power sharing between source and target cell. |

Regarding candidate component values:

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | Our preference is Alt 3 and explicitly list the supported combinations. - no power sharing mode, - no power sharing mode, and semi-static mode 1 - no power sharing mode, semi-static mode 1, and semi-static mode 2 - no power sharing mode, semi-static mode 1, and dynamic mode - no power sharing mode, semi-static mode1, semi-static mode 2, and dynamic mode |

…

# Conclusions

…

# References

- R1-2001870, Summary on UE features for NR mobility enhancements, Moderator (AT&T)