3GPP TSG RAN WG1 #100bis-e			R1-2002846

e-Meeting, April 20th – 30th, 2020

Title:	Reply LS on open PUR issues for NB-IoT/eMTC

Response to:	R2-2001816 / R1-2001517

Release:	Release 16

Work Item:	NB_IOTenh3-Core, LTE_eMTC5-Core

Source:	RAN1

To:	RAN2

Cc:	    -

Contact Person:

#### Name:	Johan Bergman

E-mail Address:	johan (dot) bergman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN1 thanks RAN2 for the LS on open PUR issues for NB-IoT/eMTC. The LS (R1-2001517) included the following questions to RAN1:

- Confirm whether the L1 adjustment on the (N)PUSCH repetition number is not intended to update the higher layer (i.e. RRC) configuration but to be used instead of the configuration provided by higher layers.

- Clarify whether the L1 adjustment on the (N)PUSCH repetition number is intended to apply only to the next upcoming PUR UL transmission or for all future PUR UL transmissions, and whether PHY layer would store the adjustment.

- Clarify how the L1 adjustment on the (N)PUSCH repetition number is updated/released and whether RRC layer can update or release the adjustment after the L1 adjustment has been previously received.

RAN1 considers that the answers to the questions above are covered through the following response:

- The L1 adjustment on the (N)PUSCH repetition number is intended to apply for future PUR UL transmissions until a new L1 adjustment or RRC reconfiguration is received, i.e. the UE uses the information from the most recently received L1 adjustment or RRC (re)configuration.

- The decision on whether the L1 adjustment on the (N)PUSCH repetition number is intended to update the higher layer (i.e. RRC) configuration or to be used instead of the configuration provided by higher layers can be made in RAN2, and then RAN1 will update the RAN1 specifications in accordance with the RAN2 decision if needed.

2. Actions:

To RAN2

ACTION:

RAN1 respectfully asks RAN2 to take the above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #101e	2020-05-25 to 2020-06-05	e-Meeting

TSG-RAN WG2 Meeting #102	2020-08-24 to 2020-08-28	Toulouse, FR