3GPP TSG RAN WG1 Meeting #100bis-e                                                                  R1-2002953

E-meeting, April 20 - April 30, 2020

Title:	Draft reply LS on DCP

Response to:	R1-2001507 (R2-2002201)

Release:	Rel-16

Work Item:	NR_UE_pow_sav-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

#### Name:	Xiaolei TIE

E-mail Address:	tiexiaolei@huawei.com

Attachments: -

1. Overall Description:

RAN1 would like to thank RAN2 for the LS to ask RAN1’s preference on the following Option 1 and Option 2 for the two flags on CSI/SRS reporting when DCI format 2_6 is configured.

Option 1:

ps-TransmitPeriodicCSI = TRUE: Report all types of periodic CSI, including L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

ps-TransmitPeriodicL1-RSRP = TRUE: Only report L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

In this option, the two flags cannot both be set to TRUE and it is not possible to control the UE only to report periodic CSI apart from L1-RSRP.

Option 2:

ps-TransmitPeriodicCSI = TRUE: Report all types of periodic CSI apart from L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

ps-TransmitPeriodicL1-RSRP = TRUE: Only report L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

In this option, the two flags are independent and it is possible to control the UE only to report periodic CSI apart from L1-RSRP.

RAN1 discussed the two options in RAN1#100-e, and achieved the agreement that Option 2 is preferred:

Agreements:

- P-CSI and L1-RSRP reports are independently configured and to allow UE only to report periodic CSI apart from L1-RSRP.

2. Actions:

To RAN2:

ACTION: RAN1 kindly asks RAN2 to take the above information into account for power saving configuration signalling.

3. Date of Next TSG-RAN1 Meetings:

TSG-RAN WG1 Meeting #101-e			25 May – 5 June, Electronic

TSG-RAN WG1 Meeting #102			24 – 28 August 2020, Toulouse, France