3GPP TSG-RAN WG1 Meeting #100bis-e 	R1-2003068

E-meeting, 20th – 30th, April,  2020

Title:	Reply  LS on DCP

Response to:	R1-2001507 (R2-2002201)

Release:	Rel-16

Study Item:	NR_UE_pow_sav-Core

Source:	RAN1

To:	RAN2

CC:

Contact Person:

Name:	Fang-Chen Cheng

E-mail Address:	fcc@catt.cn

1. Overall Description:

RAN1 would like to thank RAN2 on the LS on DCP regarding the MAC-PHY interactions for DCP (DCI with CRC scrambled by PS-RNTI) monitoring and the start of drx-onDurationTimer.  RAN1 had discussed the MAC-PHY interaction for DCP with the following agreements in physical layer procedure of DCI format 2_6 detection.

L1 procedure of DCI format 2_6 detection

- Successful decoding of DCI format 2_6

  - L1 sends a positive indication to MAC when the value of wakeup indication bit is “1”

  - L1 sends a negative indication to MAC when the value of wakeup indication bit is “0”

- Miss-detection - all CRC checks fails on DCI format 2_6

  - L1 does not send any indication to MAC

- All invalid monitoring occasions – following legacy behavior to wake up

  - L1 sends a positive indication to MAC

Text proposal of CR to TS38.213 of the above physical layer procedure were also agreed.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take into account consideration of RAN1’s replies.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #101e    	25th  May -  5th  June. 2020     e-Meeting

TSG-RAN WG1 Meeting #102   	24th  -  28th  August. 2020     Toulouse, France