3GPP TSG RAN WG1 Meeting #100bis-e                    	                  	R1-2003069

e-Meeting, April 20th – 30th, 2020

Title:	LS on criterion of pathloss measurement failure for power control of SRS for positioning

Response to:

Release:	Rel-16

Work Item:	NR_POS

Source:	RAN WG1

To:	RAN WG4

Cc:

Contact Person:

Name:	Xiaotao Ren

E-mail Address:	renxiaotao@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 had discussed the issue of criterion for inaccurate pathloss measurement for power control of SRS for positioning. RAN1 agreed that the pathloss reference for SRS for positioning may be a SSB from the serving cell, a SSB from a non-serving cell, a CSI-RS resource from the serving cell, a DL-PRS resource from a TRP of the serving cell, or a DL-PRS resource from a TRP of a non-serving cell.

According to the descriptions in current specification 38.213 as shown below, if the UE determines that the UE is not able to accurately measure PLb,f,cqd, the UE calculates PLb,f,cqd using a RS resource obtained from the SSB of the serving cell that the UE uses to obtain MIB.

--------------------------------------------Start of related Descriptions in 38.213 g10------------------------------------- 7.3.1 UE behaviour ----------------------------------------------------- Uncorrelated part omitted ----------------------------------------------------- If a UE transmits SRS based on a configuration by IE SRS-Positioning-Config on active UL BWP b of carrier f of serving cell c, the UE determines the SRS transmission power PSRS,b,f,ci,qs in SRS transmission occasion i as [dBm] where, - PO,SRS,b,f,cqs and αO,SRS,b,f,cqs are provided by p0 and alpha respectively, for active UL BWP b of carrier f of serving cell c, and SRS resource set qs is indicated by SRS-ResourceSetId from SRS-ResourceSet, and - PLb,f,cqd is a downlink pathloss estimate in dB calculated by the UE, as described in Clause 7.1.1 in case of an active DL BWP of a serving cell c, using RS resource indexed qd in a serving or non-serving cell for SRS resource set qs [6, TS 38.214]. A configuration for RS resource index qd associated with SRS resource set qs is provided by pathlossReferenceRS - if a ssb-Index is provided, referenceSignalPower is provided by ss-PBCH-BlockPower - if a dl-PRS-ResourceId is provided, referenceSignalPower is provided by dl-PRS-ResourcePower If the UE determines that the UE is not able to accurately measure PLb,f,cqd, the UE calculates PLb,f,cqd using a RS resource obtained from the SS/PBCH block of the serving cell that the UE uses to obtain MIB The UE indicates a capability for a number of pathloss estimates that the UE can simultaneously maintain. ----------------------------------------------------- Uncorrelated part omitted ----------------------------------------------------- -------------------------------------------End of related Descriptions in 38.213 g10---------------------------------------

RAN1 had identified possible ambiguity in the above descriptions, specifically the meaning of “the UE is not able to accurately measure PLb,f,cqd”. It is RAN1 understanding that the criterion of inaccurate pathloss measurement is within the scope of RAN4’s work.

2. Actions:

To RAN4:

ACTION: 	RAN1 kindly requests RAN4 to take the above information into consideration. RAN1 kindly requests RAN4 to define the criterion for determining whether or not a UE can measure the pathloss accurately. Any feedback on the defined criterion for inaccurate pathloss measurement would help RAN1 to clarify the meaning of “the UE is not able to accurately measure PLb,f,cqd” in TS 38.213.

3. Date of Next TSG-RAN1 Meetings:

TSG-RAN1 Meeting #101-e	25th May – 5th June 2020 	e-meeting

TSG-RAN1 Meeting #102	24th – 28th August 2020 	         Toulouse, FR