3GPP TSG RAN WG1 #100bis-e			R1-2003073

e-Meeting, 20th – 30th April, 2020

Agenda item:	7.2.11

Source: 	AT&T, NTT DOCOMO, INC.

Title: 	RAN1 UE features list for Rel-16 NR after RAN1#100bis-E

Document for:	Information

- Introduction

This contribution includes Rel-16 NR RAN1 UE features list based on the agreements made at RAN1#100bis-e meeting.

- NR_2step_RACH

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9. NR_2step_RACH | 9-1 | Basic channel structure and procedure of 2-step RACH | - RACH type selection for CBRA according to SSB-based RSRP threshold - msgA PRACH resource configuration including separately configured ROs not applicable to 4-step RO configuration and fully shared ROs but different preamble sequences partitioning with 4-step RO preamble sequences configuration - msgA PUSCH resource (DMRS included) and waveform determination for 2-step CBRA - Supporting up to two msgA PUSCH configurations in an UL BWP - Supporting a separate or common transform precoder configuration for 2-step CBRA, when 2-step CBRA and 4-step CBRA co-exist - Supporting intra-slot frequency hopping for msgA PUSCH transmission when NR Rel-15 waveform is used - Validation of MsgA PRACH and PUSCH - Mapping between preamble of MsgA PRACH and PUSCH occasion with DMRS resource of MsgA PUSCH - msgB monitoring and decoding for 2-step CBRA - (for UE in any RRC state) monitoring msgB PDCCH with CRC masked by msgB-RNTI in Type-1 CSS set, and decoding multi-cast msgB PDSCH carrying SuccessRAR, FallbackRAR and BI - (for RRC connected UE only) monitoring msgB PDCCH with CRC masked by C-RNTI in USS set, and decoding the unicast PDSCH carrying absolute TA MAC CE - PUCCH transmission for HARQ-ACK feedback to a msgB - Power control for msgA PRACH, msgA PUSCH and PUCCH carrying HARQ-ACK feedback to msgB | TBD | Yes | N/A | UE cannot initiate a 2-step RACH process, and thus would not be expected understand the 2-step RACH configurations | per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 9. NR_2step_RACH | [9-3] | [Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA] | [Parallel MsgA and SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell] | 9-1 (TBD) | Yes | N/A | UE cannot transmit an MsgA and other UL transmissions in parallel across CCs in inter-band CA | Per BC | N/A | N/A | N/A |  | Optional with capability signalling |
| 9. NR_2step_RACH | [9-4] | [MsgA operation in a band combination including SUL] | [MsgA operations in a band combination including SUL] | 9-1, 6-16 (TBD) | Yes | N/A | UE does not support msgA operations in a band combination including SUL | Per BC | N/A | N/A | N/A |  | Optional with capability signalling |
| 9. NR_2step_RACH | [9-5] | [intra-slot msgA PUSCH FH with non-zero GP] | [intra-slot msgA PUSCH FH with non-zero GP] | TBD | Yes | N/A |  | [Per band] | N/A | N/A | N/A |  | Optional with capability signalling |
| 9. NR_2step_RACH | [9-6] | [up to X of msgBs per slot/within the msgB window] | [up to X of msgBs per slot/within the msgB window] | TBD | Yes | N/A |  | [Per band] | N/A | N/A | N/A |  | Optional with capability signalling |

- NR-unlicensed

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-1 | UL channel access for dynamic channel access mode | 1. Type 1 channel access 2. Type 2A channel access 3. Type 2B channel access 4. Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol for PUSCH/PUCCH transmission | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-1a | UL channel access for semi-static channel access mode | 1. Type 2C channel access 2. Single sensing slot of 9us channel access 3. 20MHz LBT bandwidth | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2 | SSB-based RRM [for dynamic channel access mode] | 1. SSB-based RRM with Q [for dynamic channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2a | SSB-based RRM [for semi-static channel access mode] | 1. SSB-based RRM with Q [for semi-static channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2b | MIB reading on unlicensed cell | 1. MIB reading on unlicensed cell | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2c | SSB-based RLM [for dynamic channel access mode] | 1. SSB-based RLM with Q [for dynamic channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2d | SSB-based RLM [for semi-static channel access mode] | 1. SSB-based RLM with Q [for semi-static channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2e | SIB1 reception on unlicensed cell | 1. SIB1 reception on unlicensed cell | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2f | Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0 | 1. Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0 | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-7 | UL channel access for 10 MHz SCell | - 10 MHz LBT bandwidth | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-10 | RSSI and channel occupancy measurement and reporting | - RSSI measurement - Channel occupancy reporting | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-11 | SRS starting position at any OFDM symbol in a slot | - Support transmitting SRS starting in all symbols (0,…,13) of a slot | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-20 | Support search space set configuration with freqMonitorLocation-r16 | 1. Support search space set configuration with freqMonitorLocations-r16 | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-20a | Support coreset configuration with rb-Offset | 1. Support coreset configuration with rb-Offset | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-23 | CGI reading on unlicensed cell [based on off-sync raster SSB] for ANR functionality | 1. Support acquisition of relevant information from a neighbouring NR unlicensed cell in an unlicensed carrier by reading the RMSI of the neighbouring unlicensed cell and reporting the acquired information to the network | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Support reading RMSI from SCell from an off-sync raster SSB for ANR | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-25 | Enable configured UL transmissions when DCI 2_0 is configured but not detected | 1. Support configuration of enableConfiguredUL-r16 and enable transmission of higher-layer configured UL *SRS, PUCCH, CG-PUSCH etc) when DCI 2_0 is configured but not detected | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-27 | Wideband PRACH | - Enhanced PRACH design for NR-U by adopting a single long ZC sequence, with ZC sequence = 1151 for 15kHz and ZC sequence = 571 for 30kHz | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-29 | Support available RB set indicator field in DCI 2_0 | - Support monitoring DCI 2_0 to read availableRB-Sets-r16 | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-30 | Support channel occupancy duration indicator field in DCI 2_0 | - Support monitoring DCI 2_0 to read COT duration | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-8 | Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision | - Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A | Note length 9/10 with DMRS shift due to CRS collision are already covered by 14-3 | Optional with capability signalling |
| 10. NR-unlicensed | 10-9 | Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring | 1. Two groups of search space sets 2. Monitor DCI 2_0 with a search space set switching field 3. Support switching the search space set group with PDCCH decoding in group 1 4. Support a timer to switch back to original search space set group 5. Monitor DCI 2_0 for channel occupancy time and use the end of channel occupancy time to switch back to the original search space set group | TBD | Yes | N/A |  | FFS: Per UE or per band or per BC | N/A | N/A |  | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
| 10. NR-unlicensed | 10-9b | Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring | 1. Two groups of search space sets 2. Support switching the search space set group with PDCCH decoding in group 1 3. Support a timer to switch back to original search space set group | TBD | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A |  | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
| 10. NR-unlicensed | 10-9c | Joint search space group switching across multiple cells | - Configured with a group of cells and switch search space set group jointly over these cells | TBD | Yes | N/A |  | Per band | N/A | N/A |  | Without this capability, the UE will switch search space set groups for different cells independently | Optional with capability signalling |
| 10. NR-unlicensed | 10-9d | Support Search space set group switching capability 2 | - Search space set group switching Capability-2: P=10/12/22 symbols for µ = 0/1/2 SCS | TBD | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A |  |  | Optional with capability signalling |
| 10. NR-unlicensed | 10-14 | Non-numerical PDSCH to HARQ-ACK timing | - Support configuration of a value for dl-DataToUL-ACK indicating an imapplicable time to report HARQ ACK | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | If non-numerical K1 value is supported | Optional with capability signalling |
| 10. NR-unlicensed | 10-15 | Enhanced dynamic HARQ codebook | 1. Support of bit fields signalling PDSCH HARQ group index and NFI in DCI 1_1 (configuration of nfi-TotalDAI-Included) 2. Support of bit field in DCI 0_1 for other group total DAI if configured. (configuration of ul-TotalDAI-Included) 3. Support the retransmission of HARQ ACK (pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16) | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Enhanced dynamic HARQ codebook supporting grouping of HARQ ACK and triggering the retransmission of HARQ ACK in each groups | Optional with capability signalling |
| 10. NR-unlicensed | 10-16 | One-shot HARQ ACK feedback | - Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_1 scheduling a PDSCH - Support feedback of type 3 HARQ-ACK codebook , triggered by a DCI 1_1 without scheduling a PDSCH using a reserved FDRA value | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A | Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group. | Optional with capability signalling |
| 10. NR-unlicensed | 10-17 | Multi-PUSCH UL grant | 1. Support of scheduling up to 8 PUSCH with a single DCI 0_1 | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A |  | Optional with capability signalling |
| 10. NR-unlicensed | [10-19a] | [Support DL reception in a carrier with intra-cell guard-bands] | 1. [When DL BWP has multiple RB sets, support using the available RB set bitmap in DCI 2_0 to validate the periodic CSI-RS transmission if the CSI-RS is over multiple RB-sets] | TBD | Yes | N/A |  | Per band | N/A | N/A |  | Without this capability, UE will assume all RB sets in the DL BWP are all transmitted or none of them are transmitted | Optional with capability signalling |
| 10. NR-unlicensed | [10-19b] | [Support UL transmission with subset of RB sets passing LBT] | 1. [When UL BWP has multiple RB sets, support transmission of UL signal or channels when LBT passes for only the RB sets the UL signals or channels are located] | TBD | Yes | N/A |  | Per band | N/A | N/A |  | Without this capability, UE will transmit UL when all RB sets in the UL BWP pass LBT | Optional with capability signalling |
| 10. NR-unlicensed | 10-26 | CSI-RS based RLM for NR-U | [CSI-RS based RLM for NR-U] | TBD | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 10. NR-unlicensed | 10-26a | CSI-RS based RRM for NR-U | [CSI-RS based RRM for NR-U] | TBD | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 10. NR-unlicensed | [10-31] | [Support of CSI-RS measurements for CSI reporting and tracking without COT duration from DCI 2_0] | [· Perform CSI measurements for reporting and tracking using CSI-RS resources that are not within a COT duration indicated by DCI 2_0 · Note: This includes the cases when DCI 2_0 is not configured and when DCI 2_0 is configured but COT duration is not provided by either CO duration field or SFI.] | TBD | Yes | N/A |  | [Per band] | N/A | N/A |  |  | Optional with capability signaling |
| 10. NR-unlicensed | 10-3 | PRB interlace mapping for PUSCH | 1. PRB interlace frequency domain resource allocation for PUSCH | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Support of PRB interlace PUSCH | Optional with capability signalling |
| 10. NR-unlicensed | 10-3a | PRB interlace mapping for PUCCH | - PRB interlace frequency domain resource allocation for PUCCH format 0 and format 1 - PRB interlace frequency domain resource allocation for PUCCH format 2 - PRB interlace frequency domain resource allocation for PUCCH format 3 | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Support of PRB interlace PUCCH format 0/1 | Optional with capability signalling |
| 10. NR-unlicensed | 10-12 | OCC for PRB interlace mapping for PF2 and PF3 | 1. OCC2 2. OCC4 | TBD | Yes | N/A |  | Per band | N/A | N/A |  | UE OCC capability for EPF2/EFP3 | Optional with capability signalling |
| 10. NR-unlicensed | 10-13a | Extended CP range of more than one symbol for CG-PUSCH | - UE supports generating a CP extension of length longer than 1 symbol for Configured Grant PUSCH transmission | TBD 5-19 or 5-20 | Yes | N/A |  | Per band | N/A | N/A | N/A | How long a UE can generate the CP extension beyond 1 symbol for CG-PUSCH | Optional with capability signalling |
| 10. NR-unlicensed | 10-18 | Configured grant with retransmission in CG resources | 1. Support retransmission in CG resources 2. Support configured grant retransmission timer 3. Support DFI monitoring 4. Support CG-UCI in CG-PUSCH | TBD 5-19 or 5-20 | Yes | N/A |  | Per band | N/A | N/A |  | Support configured grant with retransmission in configured grant resource | Optional with capability signalling |
| 10. NR-unlicensed | 10-21a | Support using ED threshold given by gNB for UL to DL COT sharing | 1. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for scheduled UL to share COT with gNB for DL 2. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for CG-PUSCH to share COT with gNB for DL 3. Indicate in CG-UCI the COT sharing information | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 10. NR-unlicensed | 10-24 | CG-UCI multiplexing with HARQ ACK | 1. Support multiplexing CG-UCI with HARQ ACK | TBD 5-19 or 5-20 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 10. NR-unlicensed | 10-28 | Configured grant with Rel-16 enhanced resource configuration | 1. Support configuration of resources with cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, | TBD 5-19 or 5-20 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |

- NR_L1enh_URLLC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11. NR_L1enh_URLLC | 11-1 | Monitoring DCI format 1_2 and DCI format 0_2 | - Supports monitoring DCI format 1_2 for DL scheduling - Supports monitoring DCI format 0_2 for UL scheduling | TBD | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-1a | Monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space | - Supports monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space | 11-1 (TBD) | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-2 | Rel-16 PDCCH monitoring capability | - Supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span for combination (X, Y, ) - If UE reports the support of more than one combination of C(X, Y) for a given SCS, and if multiple combinations of C(X, Y) are valid for the span pattern, the maximum value of C of the valid combinations is applied - Supports the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span for combination (X, Y, ) - If UE reports the support of more than one combination of M(X, Y) for a given SCS, and if multiple combinations of M(X, Y) are valid for the span pattern, the maximum value of M of the valid combinations is applied - Capability on the number of CCs with Rel-16 PDCCH monitoring capability on all the serving cells. | 3-5b (TBD) | Yes | N/A |  | [FSPC] FFS: Compoent 5) reported per UE | [N/A] | [N/A] | [N/A] | This capability is necessary for SCS 15 kHz and 30 kHz. FFS: Adding a component for “supported combination(s) (X, Y, ), which may depend on how to report C, M and (X, Y, ) A list of separate UE capabilities C(X, Y, ), M(X, Y, ) for processing capability #1; A list of separate UE capabilities C(X, Y, ), M(X, Y, ) for processing capability #2; For component 5), if UE supports carrier aggregation with more than [x] DL carriers with Rel-16 PDCCH monitoring capability on all the carriers, UE should report this capability. Value of x (can be < 4) is TBD. FFS: Whether to merge component 1) and 3), and accordingly merge component 2) and 4) | Optional with capability signalling Candidate value set for (X, Y): {(7, 3), (4, 3), (2, 2)} The value of C for combination (7, 3) for 15 kHz and 30 kHz is 56 FFS the value of C for combination (4, 3) and (2, 2) FFS the value of M for combination (7, 3), (4, 3) and (2, 2) Candidate value for component 5): { x, x+1, …, 16} |
| 11. NR_L1enh_URLLC | 11-3 | More than one PUCCH for HARQ-ACK transmission within a slot | 1) Supports sub-slot based HARQ-ACK feedback procedure. • A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot.• At least one sub-slot configuration for PUCCH can be UE specifically configured to a UE. • Supports a single configuration for PUCCH resource for all sub-slots in a slot. The starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot. Any sub-slot PUCCH resource is not across sub-slot boundaries. 2) Supported sub-slot configuration [3) Supported combinations of (A, B), where A is the minimum gap between sub-slots containing actual PUCCH transmissions measured from beginning to beginning of the sub-slots, including across slots, and B is the sub-slot duration, with both A and B in units of symbols] | TBD | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] | Candidate value set for component 2): { 7-symbol*2, 2-symbol*7 and 7-symbol*2} [Candidate value set for component 3): (A, B) = {(7, 7), (4, 2) and (7, 7), (2, 2) and (7, 7)}] FFS: Whether to keep component 3) and accordingly the above note for component 3) FFS: Any relationship between FG 11-3 and CBG-based PUSCH with minimum processing time capability #2? | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-4 | Two HARQ-ACK codebooks simultaneously constructed for supporting PDSCH reception with different priorities at a UE (slot + slot, slot + sub-slot) | 1) Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed with the restriction up to one sub-slot based HARQ-ACK codebook. 2) Supports separate PUCCH configuration for different HARQ-ACK codebooks 3) Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. [4) Supports a DCI format (from the formats 1_1/1_2) scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured per BWP] 5) Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. [6) Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot] | TBD | Yes | N/A |  | FFS [Per UE or Per FS] | [No] | [No] | [N/A] |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-4x | Two HARQ-ACK codebooks simultaneously constructed for supporting PDSCH reception with different priorities at a UE (sub-slot + sub-slot) | 1) Supports two sub-slot based HARQ-ACK codebooks with different priorities to be simultaneously constructed. 2) Supports separate PUCCH configuration for different HARQ-ACK codebooks 3) Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4) Supports a DCI format (from the formats /1_1/1_2) scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured in USS per BWP 5) Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. | 11-3 (TBD) | Yes | N/A |  | FFS [Per UE or Per FS] | [No] | [No] | [N/A] |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | [11-4a] | [DL priority indication in DCI with mixed DCI formats] | [DL priority indication in DCI with mixed DCI formats] | 11-1a, 11-4 (TBD) | Yes | N/A |  | Per UE | [No] | [No] | [N/A] |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-9 | Multiple active configured grant configurations for a BWP of a serving cell | - Supports up to 12 configured/active configured grant configurations in a BWP of a serving cell. • Separate RRC parameters for different configured grant configurations • Separate activation for different configured grant Type 2 configurations • Separate release for different configured grant Type 2 configurations - [Supported maximum number of configured grant configurations in a BWP of a serving cell] - [Supported maximum number of configured grant configurations across all serving cells] | TBD | Yes | N/A |  | [Per UE] FFS: FSPC | [No] | [No] | [N/A] |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-9a | Joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell | 1. M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, where the association between each state indicated by the indication and the CG configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple CG configurations to be released - • In case of no higher layer configured state(s), separate release is used where the release corresponds to the CG configuration index indicated by the indication | 11-9 (TBD) | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] | FFS: A UE supporting this feature shall also support 11-10 (Type 2 configured grant release by DCI format 0_1). A UE supporting this feature and 11-1 (DCI format 0_2/1_2) shall also support 11-11 (Type 2 configured grant release by DCI format 0_2). | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-10 | Type 2 configured grant release by DCI format 0_1 | - Support of type 2 configured grant release by DCI format 0_1 | TBD | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] FFS: The capability interpretation is from the perspective of a carrier on which the release DCI is received | [A UE supporting this feature and 11-1 (DCI format 0_2/1_2) shall also support 11-11 (Type 2 configured grant release by DCI format 0_2).] | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-11 | Type 2 configured grant release by DCI format 0_2 | - Support of type 2 configured grant release by DCI format 0_2 | TBD | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] FFS: The capability interpretation is from the perspective of a carrier on which the release DCI is received | [A UE supporting this feature shall also support 11-10 (Type 2 configured grant release by DCI format 0_1).] | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-6 | PUSCH repetition Type A | - PUSCH transmission with Rel-15 behavior with or without slot aggregation. • With slot aggregation, the number of repetitions can be dynamically indicated (as agreed for Rel-16). • When dynamically indicated, the number of repetitions is jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. | 2-12, 2-13, 2-14, 2-15 (TBD) | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] | FFS: Whether to add a component for the supported maximum number of PUSCH repetitions | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-7 | UL cancelation scheme for self-carrier | - Supports group common DCI (i.e. DCI format 2_4) for cancelation indication [on the same CC as PUSCH or SRS] - UL cancelation for PUSCH - Cancellation is applied to each PUSCH repetition individually in case of PUSCH repetitions - UL cancelation for SRS symbols that overlap with the cancelled symbols [For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by higher layer.] | TBD | Yes | N/A |  | [Per UE] FFS: FS | [No] | [No] | [N/A] | [More than one monitoring occasion for DCI format 2_4 per slot is applied only if the UE reports to support FG 3-5 or FG 3-5a or FG 3-5b] | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-7b | UL cancelation scheme for cross-carrier | - Supports group common DCI (i.e. DCI format 2_4) for cancelation indication [on a different CC than PUSCH or SRS] - UL cancelation for PUSCH - Cancellation is applied to each PUSCH repetition individually in case of PUSCH repetitions - UL cancelation for SRS symbols that overlap with the cancelled symbols [For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by higher layer.] | TBD | Yes | N/A |  | [Per UE] FFS: FS | [No] | [No] | [N/A] | [More than one monitoring occasion for DCI format 2_4 per slot is applied only if the UE reports to support FG 3-5 or FG 3-5a or FG 3-5b] | Optional with capability signalling |
| 11. NR_L1enh_URLLC | [11-7a] | [Independent cancellation of the overlapping PUSCHs in an intra-band UL CA] | [For a UE indicating the capability of pa-PhaseDiscontinuityImpacts, and if the PUSCH on at least one serving cell is cancelled, the UE may cancel the (repetition of the) PUSCHs transmission on all other intra-band serving cell(s). The cancellation of the (repetition of the) PUSCH transmission on a the set of intra-band serving cell(s) includes all symbols from the earliest symbol that is overlapping with the first cancelled symbol of the PUSCH on the serving cell for which the DCI format 2_4 is applicable to.] | 6-23, 11-7 (TBD) | Yes | N/A |  | [Per band] | [N/A] | [N/A] | TBD | FFS: Whether to add this FG and the content for each column if added | Optional with capability signaling |

- NR_IIOT

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12. NR_IIOT | 12-1 | UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer | Support intra-UE multiplexing/prioritization of UL overlapping channels/signals with two priority levels in physical layer (PHY) - Configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH - Multiplexing/prioritization between UL channels/signals with the same PHY priority level - Prioritization between UL channels/signals with different PHY priority levels - Additional number of symbols (d1) needed beyond the PUSCH preparation time for cancelling a low priority UL transmission. - Additional number of symbols (d2) needed beyond the PUSCH preparation time for scheduling a high priority UL transmission that cancels a low priority UL transmission | TBD | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] | [A UE supporting this feature shall also support the LCP restriction based on DCI priority indication ([lch-ToGrantPriorityRestriction-r16]) and intra-UE prioritization in MAC ([lch-PriorityBasedPrioritization-r16]).] The relationship between this feature and the feature of up to two HARQ-ACK codebooks of 11-4x including merging these features should be further discussed. | Optional with capability signaling Candidate value set for component 4: {0, 1, 2} Candidate value set for component 5: {0, 1, 2} |
| 12. NR_IIOT | [12-1a] | [UL priority indication in DCI with mixed DCI formats] | [UL priority indication in DCI with mixed DCI formats] | TBD | Yes | N/A | FFS | Per UE | [No] | [No] | [N/A] |  | Optional with capability signalling |
| 12. NR_IIOT | 12-2 | Multiple SPS configurations | - Support of up to 8 configured SPS configurations in a BWP of a serving cell [and up to 32 configured SPS configurations in a cell group,] including separate RRC parameters and separate activation/release for different SPS configurations - The max number of active SPS configurations in a BWP of a serving cell - The max number of active SPS configurations across all serving cells - The related HARQ-ACK enhancements to support multiple active SPS configurations | downlinkSPS (TBD) | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling Component-2, candidate value set is {1, 2, …, 8} Component-3, candidate value set is [{2, …, [32]}] |
| 12. NR_IIOT | 12-2a | Joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell | - M<=4 bits indication in the Release DCI is used for indicating which SPS configuration(s) is/are released, where the association between each state indicated by the indication and the SPS configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple SPS configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the SPS configuration index indicated by the indication - The related HARQ-ACK enhancements to support joint release | 12-2 (TBD) | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 12. NR_IIOT | 12-3 | SPS release by DCI format 1_1 | - Support of SPS release by DCI format 1_1 | downlinkSPS (TBD) | Yes | N/A |  | [Per UE] | [No] | [No] | [TBD] | [A UE supporting this FG and 11-1 (DCI format 0_2/1_2) shall also support FG12-3a (SPS release by DCI format 1_2).] | Optional with capability signaling |
| 12. NR_IIOT | 12-3a | SPS release by DCI format 1_2 | Support of SPS release by DCI format 1_2 | downlinkSPS (TBD) | Yes | N/A |  | [Per UE] | [No] | [No] | [TBD] |  | Optional with capability signaling |
| 12. NR_IIOT | 12-5 | Configuration of aggregation factor per SPS configuration | 1. Support of configurable PDSCH aggregation factor ({1, 2, 4, 8}) per DL SPS configuration | downlinkSPS (TBD) | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 12. NR_IIOT | 12-6 | Support of SPS periodicity shorter than 10 ms | Support of SPS periodicity shorter than 10 ms | TBD | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signalling |

- NR positioning

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13. NR Positioning | 13-1 | Common DL PRS Processing Capability | - Duration of DL PRS symbol in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} - Duration of DL PRS symbol in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 8, 12, 16, 20, 25, 30, 35, 40, 45, 50} ms Notes: - UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value - UE DL PRS processing capability is defined for a single positioning frequency layer - UE DL PRS processing capability is agnostic to DL PRS comb factor configuration - FFS if UE DL PRS processing capability is agnostic to the configured SCS settings of DL PRS - FFS if reported values of T are the same across bands within a FR or across FRs - FFS cases w/ and w/o configuration of measurement gap - [Max number of positioning frequency layers supported by UE. Values = {1, 2, 3, 4}] - Max number of DL PRS resources that UE can process in a slot under it - FR1 bands: {1, 2, 4, 8, 16, 32, 64} for each SCS: 15kHz, 30kHz, 60kHz - FR2 bands: {1, 2, 4, 8, 16, 32, 64} for each SCS: 15kHz, 30kHz, 60kHz Note: The above are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than X% (FFS: X) | TBD | No | N/A |  | Per band [Per UE for component 3] | N/A | N/A [Yes for component 3] | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-2 | DL PRS Resources for DL AoD | - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE. Values = {1,2} - Max number of DL PRS Resources per DL PRS Resource Set Values = {[1], 4, 8, 16, 32, 64} - Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets. Values = {64, 128, 192, 256, 512, 1024, 2048} - Max number of TRPs across all positioning frequency layers per UE. Values = [{16, 32, 64, 128, 256} or {3, 12, 64, 256}] - Max number of DL PRS Resources per positioning frequency layer. Values = {32, 64, 128, 256, 512, 1024} - [Max number of DL PRS resources per TRP across all frequency layers. Value set: {4,8,16,32,64,128}] - [The number of positioning layer UE supports] Values = {1, 2, 3, 4} | 13-3 (TBD) | No | N/A |  | FFS: [Per band or Per UE] | N/A | [N/A or Yes] | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-3 | DL PRS Resources for DL-TDOA | - Max number of DL PRS Resource Sets per TRP per frequency layer. Values = {1, 2} - Max number of DL PRS Resources per DL PRS Resource Set. Values = {1, 4, 8, 16, 32, 64} - Max number of DL PRS Resources across all frequency layers, TRPs and DL PRS Resource Sets. Values = {64, 128, 192, 256, 512, 1024, 2048} - Max number of TRPs across all positioning frequency layers per UE. Values = [{16, 32, 64, 96, 128, 256} or {3, 12, 64, 256}] - Max number of DL PRS Resources per positioning frequency layer. Values = {32, 64, 128, 256, 512, 1024] - [Max number of DL PRS resources per TRP across all frequency layers. Value set: {4,8,16,32,64,128}] - [The number of positioning layer UE supports] Values = {1, 2, 3, 4} | 13-3 (TBD) | No | N/A |  | FFS: [Per band or Per UE] | N/A | [N/A or Yes] | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-4 | DL PRS Resources for Multi-RTT | - Max number of DL PRS Resource Sets per TRP per frequency layer. Values = {1, 2} - Max number of DL PRS Resources per DL PRS Resource Set. Values = {1, 4, 8, 16, 32, 64} - Max number of DL PRS Resources across all frequency layers, TRPs and DL PRS Resource Sets. Values = {64, 128, 192, 256, 512, 1024, 2048} - Max number of TRPs across all positioning frequency layers per UE. Values = [{16, 32, 64, 96, 128, 256} or {3, 12, 64, 256}] - Max number of DL PRS Resources per positioning frequency layer. Values = {32, 64, 128, 256, 512, 1024] - [Max number of DL PRS resources per TRP across all frequency layers. Value set: {4,8,16,32,64,128}] - [The number of positioning layer UE supports] Values = {1, 2, 3, 4} | 13-3 (TBD) | Yes | N/A |  | FFS: [Per band or Per UE] | N/A | [N/A or Yes] | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-5 | DL PRS Measurement Report for DL-AoD | - Max number of DL PRS RSRP measurements on different PRS resources from the same TRP supported by the UE Values = {1, 2, 3, 4, 5, 6, 7, 8} | 13-3, 13-5 (TBD) | No | N/A |  | FFS: [Per band or Per UE] | [N/A] | [N/A] | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-5a | Inter-frequency measurement for DL-AoD | - Support of inter-frequency measurement for DL-AoD | TBD | No | N/A |  | FFS: [Per UE or per band or per BC] | [No or N/A] | [No or Yes or N/A] | N/A |  | Optional with capability signalling {supported, notSupported} |
| 13. NR Positioning | [13-6] | [DL PRS RSTD/[RSRP] Measurement Report for DL-TDOA] | - [Max number of DL PRS measurements on different PRS resources from the same TRP supported by the UE Values = {[0], 1, 2, 3, 4, 5, 6, 7, 8}] | TBD | No | N/A |  | [Per band] | [N/A] | [N/A] | [N/A] | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-6a | Inter-frequency measurement for DL-TDOA | - Support of inter-frequency measurement for DL-TDOA | TBD | No | N/A |  | FFS: [Per UE or per band or per BC] | [No or N/A] | [No or Yes or N/A] | N/A |  | Optional with capability signalling {supported, notSupported} |
| 13. NR Positioning | [13-7] | [Support of SSB from neighbor cell as QCL source of a DL PRS] | - [Support of SSB from neighbor cell as QCL source of a DL PRS] | TBD | No | N/A |  | [Per band] | [N/A] | [N/A] | [N/A] | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | [13-7a] | [Support of DL PRS from serving/neighbor cell as QCL source of a DL PRS] | - [Support of DL PRS from serving/neighbor cell as QCL source of a DL PRS] | TBD | No | N/A |  | [Per band] | [N/A] | [N/A] | [N/A] | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-8 | SRS Resources for Positioning | - Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, [2], 4, [8], [12], 16}. - [Max number of periodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {1, 2, 4, 8, 16, 32, 64}] - [Max number of P/SP/AP SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64}] - [Max number of P/SP/AP SRS Resources including the SRS resources for positioning per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14}] - [Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64}] - [Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14}] | TBD | Yes | N/A |  | [Per FS] | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-8a | Support of Aperiodic SRS Resources for positioning | - Max number of aperiodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} - [Max number of aperiodic SRS Resources for positioning per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14}] | TBD | Yes | N/A |  | [Per FS] | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-8b | Support of Semi-persistent SRS Resources for positioning | - Max number of semi-persistent SRS Resources for positioning supported by UE per BWP. Values = {1,2,4,8,16,32,64} - [Max number of semi-persistent SRS Resources for positioning supported by UE per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14}] | TBD | Yes | N/A |  | [Per FS] | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-9 | OLPC for SRS for positioning based on PRS from the serving cell | - OLPC for SRS for positioning based on PRS from the serving cell | TBD | No | N/A |  | FFS: [Per band or Per UE] | N/A | [N/A or Yes] | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-9a | OLPC for SRS for positioning based on SSB from neighbouring cells | - OLPC for SRS for positioning based on SSB from neighbouring cells | TBD | No | N/A |  | FFS: [Per band or Per UE] | N/A | [N/A or Yes] | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-9b | OLPC for SRS for positioning based on PRS from the neighbouring cells | - OLPC for SRS for positioning based on PRS from the neighbouring cells | TBD | No | N/A |  | FFS: [Per band or Per UE] | N/A | [N/A or Yes] | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-9c | OLPC for SRS for positioning based on CSI-RS from serving cell | - OLPC for SRS for positioning based on CSI-RS from serving cell | TBD | No | N/A |  | [Per band] | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | [13-9d] | [OLPC for SRS for positioning based on SSB from serving cell] | - [OLPC for SRS for positioning based on SSB from serving cell] | TBD | No | N/A |  | [Per band] | [N/A] | [N/A] | [N/A] | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | [13-9e] | [PathLoss estimate maintenance] | - [Max Number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning across all serving cells in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions. Values = {0,1,4,8,16}] - [Max Number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning per serving cell in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions. Values = {0,1,4,8,16}] | TBD | No | N/A |  | FFS: [Per band or Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 13. NR Positioning | 13-10 | Spatial relation for SRS for positioning based on SSB from the serving cell | - Spatial relation for SRS for positioning based on SSB from the serving cell | TBD | No | N/A |  | [Per band] | N/A | N/A (FR2 only) | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-10a | Spatial relation for SRS for positioning based on CSI-RS from the serving cell | - Spatial relation for SRS for positioning based on CSI-RS from the serving cell | TBD | No | N/A |  | [Per band] | N/A | N/A (FR2 only) | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-10b | Spatial relation for SRS for positioning based on PRS from the serving cell | - Spatial relation for SRS for positioning based on PRS from the serving cell | TBD | No | N/A |  | [Per band] | N/A | N/A (FR2 only) | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-10c | Spatial relation for SRS for positioning based on SRS | - Spatial relation for SRS for positioning based on SRS | TBD | No | N/A |  | FFS: [Per band or Per UE] | [N/A or No] | [N/A or Yes] (FR2 only) | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-10d | Spatial relation for SRS for positioning based on SSB from the neighbouring cell | - Spatial relation for SRS for positioning based on SSB from the neighbouring cell | TBD | No | N/A |  | FFS: [Per band or Per UE] | [N/A or No] | [N/A or Yes] (FR2 only) | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-10e | Spatial relation for SRS for positioning based on PRS from the neighbouring cell | - Spatial relation for SRS for positioning based on PRS from the neighbouring cell | TBD | No | N/A |  | FFS: [Per band or Per UE] | [N/A or No] | [N/A or Yes] (FR2 only) | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | [13-10f] | [Spatial relation maintenance] | - [Component 1: Max Number of maintained spatial relations for all the SRS resource sets for positioning across all serving cells in addition to the spatial relations maintained spatial relations per serving cell for the PUSCH/PUCCH/SRS transmissions. Values = {0,1,2,4,8,16}] - [Component 2: Max Number of maintained spatial relations for all the SRS resource sets for positioning per serving cell in addition to the spatial relations maintained spatial relations per serving cell for the PUSCH/PUCCH/SRS transmissions. Values = {0,1,2,4,8,16}] | TBD | No | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 13. NR Positioning | 13-11a | Inter-frequency measurement for Multi-RTT | - Inter-frequency measurement for Multi-RTT | TBD | Yes | N/A |  | FFS: [Per Band or Per BC or Per UE] | [N/A or No] | [N/A or No or Yes] | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | [13-11] | [UE Rx-Tx Measurement Report for Multi-RTT] | - [UE Rx-Tx Measurement Report for Multi-RTT] | TBD | No | N/A |  | [Per band] | [N/A] | [N/A] | [N/A] | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | [13-12] | [NR E-CID DL SSB RRM measurements with LPP support for NR Positioning] | - [NR E-CID DL SSB RRM measurements with LPP support for NR Positioning] | TBD | No | N/A |  | [Per band] | [N/A] | [N/A] | [N/A] | Need for location server to know if the feature is supported. | TBD |
| 13. NR Positioning | [13-12a] | [NR E-CID DL CSI-RS RRM measurements with LPP support for NR Positioning] | - [NR E-CID DL CSI-RS RRM measurements with LPP support for NR Positioning] | TBD | No | N/A |  | [Per band] | [N/A] | [N/A] | [N/A] | Need for location server to know if the feature is supported. | TBD |

- NR TEI

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14. NR TEI | 14-1 | Multiple LTE-CRS rate matching patterns | - Maximum number of LTE-CRS rate matching patterns in total within a NR carrier using 15 kHz SCS - Maximum number of LTE-CRS non-overlapping rate matching patterns within a NR carrier using 15 kHz SCS | TBD [5-28 (Rate-matching around LTE CRS) ] | Yes | N/A |  | Per band | N/A | N/A (FR1 only) | N/A | For DSS The number of the additional CRS rate matching patterns reported in Rel-16 is accounted in the total number of rate matching pattern reported by the UE for Rel-15 by using pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot and pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot UE reporting component 1 for 14-1 also reports component 2. Reporting of values of Component 1 larger than three is only applicable when reporting values of Component 2 larger than one. | Optional with capability signalling Component 1:{2, 3, 4, 5, 6} Component 2: {1, 2, 3} |
| 14. NR TEI | 14-1a | Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier | - Support of two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz SCS overlapping with a LTE carrier | TBD [14-1 (indicating component 1 value larger than component 2 value), 16-2 (mTRP support)] | Yes | N/A |  | Per band | N/A | N/A (FR1 only) | N/A | For DSS The number of the additional CRS rate matching patterns reported in Rel-16 is accounted in the total number of rate matching pattern reported by the UE for Rel-15 by using pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot and pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot | Optional with capability signaling |
| 14. NR TEI | 14-2 | PDSCH Type B mapping of length 9 and 10 OFDM symbols | - Indicates whether the UE supports PDSCH Type B scheduling of length 9 and 10 OFDM symbols with DMRS shift due to CRS collision | TBD [5-6a (PDSCH mapping type B) ] | Yes | N/A |  | FFS: [Per band or Per UE] | [N/A or No] | [N/A or No] (FR1 only) | N/A | For DSS FG10-8 covers PDSCH type B mapping without DMRS shift due to CRS collision. | FFS: [Mandatory with capability signailng or Optional with capability signaling] |
| 14. NR TEI | 14-3 | One slot periodic TRS configuration for FR1 | - UE can be configured with one-slot periodic TRS configuration only when no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated | TBD [2-51 (CSI-RS for tracking)] | Yes | N/A | No TRS can be configured when no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated | Per band | N/A (TDD only) | N/A (FR1 only) | N/A | UE can be configured with one-slot periodic TRS configuration only when no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated. FFS: relationship with maxBurstLength for FG2-51 | Optional with capability signalling |
| 14. NR TEI | 14-4 | SRS Tx switch with allowing downgrading configuration | 1) Support SRS Tx port switch [2) Report whether the uplink Tx switching impact to downlink receiving in a band] [3) Report whether the UL Tx is switched together with UL Tx in another band] [Define affected DL and UL bands by using txSwitchImpactToRx and txSwitchWithAnotherBand for the new (downgraded) entries] | TBD [2-55] | Yes | N/A |  | FFS: [Per BC or Per FS or Per FSPC] | N/A | N/A | N/A | Agreement: •Rel-16 UE capability design for SRS antenna switching in conjunction with the existing Rel-15 UE capability should allow UE to indicate support of one of the following combinations o{t1r1, t1r2} o{t1r1, t1r2, t1r4} o{t1r1, t1r2, t2r2, t2r4} o{t1r1, t2r2} o{t1r1, t2r2, t4r4} o{t1r1, t1r2, t2r2, t1r4, t2r4} oNote: Detailed signaling design is up to RAN2 FFS: whether components 2 and 3 are necessary or not | Optional with capability signalling Component 1: Candidate value set: { o{t1r1, t1r2} o{t1r1, t1r2, t1r4} o{t1r1, t1r2, t2r2, t2r4} o{t1r1, t2r2} o{t1r1, t2r2, t4r4} o{t1r1, t1r2, t2r2, t1r4, t2r4} } Component2: Candidate value set: {yes, no} Component 3: Candidate value set: {yes, no} |
| 14. NR TEI | 14-5 | Half-duplex UE behaviour in TDD CA for same SCS | - Support for directional collision handling between reference and other cell(s) for half-duplex operation in CA with same SCS | TBD [6-5, 6-6, simultaneousRxTxInterBandCA not supported] | Yes | N/A |  | FFS: [Per BC or Per FS or Per UE] | [N/A or No] (TDD only) | [N/A or Yes or No] | [N/A] | Half duplex UEs that do not indicate this capability should still be able to operate half-duplex TDD CA (i.e. simultaneousRxTxInterBandCA not supported) per Rel15 specifications if network ensures same transmission direction across all the serving cells | FFS: [Mandatory with capability signaling for intra-band CA band and for inter-band CA in band combination without RAN4 FG 2-5 capability or Optional with capability signaling] |
| 14. NR TEI | [14-5a] | Half-duplex UE behaviour in TDD CA with different SCS | - Support for directional collision handling between reference and other cell(s) for half-duplex operation in CA with different SCS | TBD | Yes | N/A |  | FFS: [Per BC or Per FS or Per UE] | [N/A or No] (TDD only) | [N/A or Yes or No] | [N/A] | Half duplex UEs that do not indicate this capability should still be able to operate half-duplex TDD CA (i.e. simultaneousRxTxInterBandCA not supported) per Rel15 specifications if network ensures same transmission direction across all the serving cells | FFS: [Mandatory with capability signaling for intra-band CA band and for inter-band CA in band combination without RAN4 FG 2-5 capability or Optional with capability signaling] |
| 14. NR TEI | 14-6 | New RACH configuration for FR1 TDD | - new RACH configuration entries with subframe number 2 and/or 7 for RACH periodicity longer than 10 ms | TBD | No | N/A |  | N/A | N/A (TDD only) | N/A (FR1 only) | N/A | Agreement: •A new UE capability is not introduced for this TEI, i.e., it is a mandatory UE feature for Rel-16. | Mandatory without capability signalling |
| 14. NR TEI | [14-7] | New capability for beamSwitchTiming values of 224 and 336 | [48 is used as the beam switching threshold for UEs reporting 224 or 336 When using sym224 and sym336, beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI.] | TBD [2-28] | Yes | N/A |  | Per band | N/A | N/A (FR2 only) | N/A | FFS: relationship with beamSwitchTiming for FG2-28 Agreements: ・48 is used as the beam switching threshold for UEs reporting 224 or 336 When using the higher values of the feature (sym224 and sym336), beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI. | Optional with capability signaling |
| 14. NR TEI | [14-8] | Active BWP when receiving the CSI triggering DCI and when receiving the associated CSI-RS | - For a given CSI report, whether UE supports to receive the CSI triggering DCI in a different active DL BWP from receiving the associated CSI-RS, in the carrier of the serving cell expecting to receive the associated CSI-RS. | TBD | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

- 5G_V2X_NRSL

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-1 | Receiving NR sidelink | 1) UE can receive NR PSCCH/PSSCH. Up to [A] sidelink HARQ processes are supported. 2) UE can receive [X] PSCCH in a slot. 3) UE can decode [Y] RBs per slot (FFS: counting both PSCCH and PSSCH). 4) UE supports reception based on the normal 64QAM MCS table [and 256QAM MCS table in FR1]. 5) UE supports PT-RS reception in FR2. FFS: 6) The UE can receive [Z] total number of soft channel bits in a slot. [7) minimum two receive antennas] 8) UE can receive using the subcarrier spacing [and CP length] [defined for a given band in R15 in RAN4] [as configured for NR UL] FFS: 9) CP length 10) Supports 14-symbol SL slot with [all/some] DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH [11) UE can receive PSSCH with 256QAM in NR sidelink] 12) [for NR SL by preconfiguration] UE can receive using 30 kHz subcarrier spacing in FR1, FFS FR2 | None | FFS | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 8 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 FFS: details for component (10) Note: Component 12 is required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Component-2 candidate value set: {value1, value2, …} FFS: whether to report different value for each SCS indicated in component-8 Component-3 candidate value set: {value1, value2, …} FFS: whether to report different value for each SCS indicated in component-8 FFS: Component-6 candidate value set: {value1, value2, …} [Component-8 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-8 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}}] [Component-12 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-8 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}}] Candidate values for A are {value1, value2 …} | Optional with capability signaling. For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-2 | Transmitting NR sidelink mode 1 scheduled by NR Uu | 1) UE can transmit PSCCH/PSSCH using dynamic scheduling or configured grant type 1 and 2 in NR sidelink mode 1 scheduled by NR Uu. Up to [8] configured grants can be configured for a UE. Up to [C] sidelink processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can monitor DCI format 3_0 for NR sidelink dynamic scheduling and configured grant type 2. 6) UE can transmit using the subcarrier spacing [and CP length] it reports. FFS: 7) CP length 8) Supports 14-symbol SL slot with [all/some] DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH [9) Support downlink pathloss based open loop power control] [10) SL HARQ feedback on Uu] |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. FFS: This is the basic FG for sidelink in licensed spectrum where gNB is operating on or managing that spectrum and optional FG otherwise Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} FFS: whether to mandate an SCS. | Optional with capability signalling FFS: For UE supports NR sidelink in licensed spectrum where gNB is defined, UE must indicate this FG is supported. Candidate values for C are {value1, value2 …} |
| 15-3 | Transmitting NR sidelink mode 2 | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by NR Uu or preconfiguration. Up to [B] sidelink processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. [4) UE can perform sensing and resource allocation operations.] [5) UE supports rank 1 PSSCH transmissions.] 6) UE can transmit using the subcarrier spacing [and CP length] it reports for FG 15-1. FFS: 7) CP length 8) Supports 14-symbol SL slot with [all/some] DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH 9) default SCS with pre-configuration: 30 kHz with normal CP: [operator managed] same as Rel. 15 Uu 10) UE can transmit using 30 kHz [and normal CP] subcarrier spacing in FR1, FFS FR2 [11) DL pathloss based open loop power control when mode 2 is configured by NR Uu] | 15-1 | FFS | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 This is the basic FG for sidelink [in ITS spectrum where gNB is not defined and optional FG for licensed spectrum where gNB is defined] [Note: Component 5 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1] Note: Component 6 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 10 is required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 FFS: all details for component (11) | Optional with capability signalling For UE supports NR sidelink, [for UE supports NR sidelink in ITS spectrum where gNB is not defined, UE must indicate this FG is supported,] UE must indicate this FG is supported. Candidate values for B are {value1, value2 …} |
| 15-4 | Synchronization sources for NR sidelink | 1) UE can receive S-SSB in NR sidelink if it supports 15-1. 2) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3. 3) UE supports GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 4) UE can transmit or receive NR sidelink based on the synchronization to an gNB 5) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 6) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 4 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 5 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 6 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-5 | Sidelink congestion control | 1) UE can report CBR measurement to gNB [when operating in Mode 1] [and mode 2] (FFS: delete component 1) 2) UE can adjust its radio parameters based on CBR measurement and CRlimit. 3) UE can process CBR and CR within the time it indicates | 15-1 and at least one of 15-2 and 15-3 | Yes | FFS |  | Per band | N.A. | N.A. | N.A. | FFS: This is the basic FG for NR sidelink FFS: details of components (1) Component-3 candidate value set {Congestion process time 1, Congestion process time 2} where Congestion process time 1: 2, 2, 4, 8 slots for 15, 30, 60, 120 kHz subcarrier spacing. Congestion process time 2: 2, 4, 8, 16 slots for 15, 30, 60, 120 kHz subcarrier spacing | Optional with capability signalling |
| 15-6 | Short-term time-scale TDM for in-device coexistence | - Support prioritization between LTE sidelink transmission/reception and NR sidelink transmission/reception | At least one of 15-1, 15-2, 15-3 | No | No |  | Per band | N.A. | N.A. | N.A. | FFS whether a set of candidate values need to be defined for the time required for the inter-RAT conflict resolution | Optional with capability signalling |
| 15-7 | Transmitting LTE sidelink mode 3 scheduled by NR Uu | 1) UE can be scheduled over NR Uu for LTE sidelink mode 3 transmission.. 2) UE reports a value ‘X’ for the minimum value it supports for the additional time indicated in the NR DCI scheduling LTE sidelink mode 3. - 3) UE can monitor DCI format 3_1 for LTE sidelink SPS grant. |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Component-2 candidate value set: {0ms, 0.25ms, 0.5ms, 0.625ms, 0.75ms, 1ms, 1.25ms, 1.5ms,1.75ms, 2ms, 2.5ms, 3ms, 4ms, 5ms, 6ms, 8ms, 10ms, 20 ms } | Optional with capability signalling |
| 15-9 | Transmitting LTE sidelink mode 4 configured by NR Uu | 1) UE can be configured over NR Uu for LTE sidelink mode 4 operation |  | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-10 | 256QAM sidelink transmission | 1) UE can transmit PSSCH with 256QAM in NR sidelink | At least one of 15-2, 15-3 | Yes | Yes | UE supports QPSK, 16QAM, and 64 QAM for transmission only. | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-11 | PSFCH format 0 | 1) UE can transmit and receive NR PSFCH format 0 2) UE can receive [N] PSFCH(s) resources in a slot. 3) UE can transmit [M] PSFCH(s) resources in a slot. [4) UE can report sidelink HARQ-ACK to gNB via PUCCH and PUSCH when it is operating in NR sidelink mode 1.] [FFS: move to 15-2] | At least one of 15-1, 15-2, 15-3 | FFS | FFS |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 4 is not required to be supported in a band indicated with the PC5 interface in 38.101-1 Table 5.2E-1 | Optional with capability signalling FFS: For UE supports NR sidelink, UE must indicate this FG is supported. ALT 1) Candidate values for N are {5, [10,] 15, [20,] 25, [30,] 35, [40,] 45, 50 } ALT 2) Candidate values for N are {32, 64} Candidate values for M are {1, 4, [5,] 8, 16} |
| 15-12 | Low-spectral efficiency 64QAM MCS table | 1) UE can transmit or receive PSSCH with low-spectral efficiency 64QAM MCS table. | At least one of 15-1, 15-2, 15-3 | Yes | Yes | UE supports normal 64QAM MCS table and 256QAM MCS table only. | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-14 | Sidelink CSI report | 1) UE can transmit and receive sidelink CSI-RS with 1 or 2 antenna port(s). 2) UE supports RI and CQI feedback on sidelink. | 15-1 and at least one of 15-2 and 15-3 | No | FFS |  | Per band | N.A. | N.A. | N.A. | FFS: This is the basic FG for NR sidelink | Optional with capability signalling. |
| 15-15 | eNB type synchronization source for NR sidelink | 1) UE can transmit or receive NR sidelink based on the synchronization to an eNB. 2) If UE supports 15-4, UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports 15-4, UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 15-16 | Simultaneous transmission of uplink and sidelink | 1) UE supports simultaneous transmission of NR uplink and NR sidelink (on different carriers) in a band combination for which the UE indicated simultaneous sidelink and uplink support in a band combination. | At least one of 15-2 and 15-3 | Yes | No |  | Per band combination | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 15-18 | Support of rank 2 transmission | 1) UE additionally supports rank 2 PSSCH transmission | [At least one of 15-2 and 15-3] | FFS | FFS | UE supports rank 1 PSSCH transmission only. | Per band | N.A. | N.A. | N.A. | This FG is a WA | Optional with capability signalling |
| 15-19 | Support of rank 2 reception | 1) UE additionally supports rank 2 PSSCH reception | [15-1] | FFS | FFS | UE supports rank 1 PSSCH reception only. | Per band | N.A. | N.A. | N.A. | This FG is a WA. FFS: This is the basic FG for NR sidelink | [Optional with capability signalling] |
| 15-22 | Support of SL slot less than 14 consecutive symbols | - UE additionally supports transmission/reception of SL slot configured with 7, 8, 9, 10, 11, 12, 13 consecutive symbols [and the corresponding DMRS patterns it reports.] - [UE supports [some/all] applicable DMRS patterns for the number of consecutive Sl symbols it reports] | At least one of 15-1, 15-2, 15-3 | Yes | No | UE supports SL only in a SL slot configured with 14 consecutive symbols. | Per band | N.A. | N/A | N.A. | FFS: This is the basic FG for NR sidelink The component-1 candidate value set can be DRMS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {{12,2},{12,1}, {11,4},{11,3},{11,2}, {10,4},{10,3},{10,2}, {9,2},{8,3},{8,2},{7,2},{6,2}, {5,2}} | Optional with capability signalling |
| 15-23 | Support of open loop SL power control and RSRP report | - Support sidelink pathloss based open loop power control and RSRP report in case of unicast - [downlink pathloss based OLPC] | FFS | FFS | Yes |  | Per band | N.A. | N/A | N.A. | Working assumption: This FG is a basic UE FG [at least] for UEs supporting mode 1 FFS: all details for component (2) FFS: whether this is a basic FG also for UEs not supporting mode 1 | Optional with capability signalling |
| 15-24 | [Support of multiple synchronization references] | [1) UE can support sidelink reception using up to A synchronziaion references in a carrier/BWP] | At least one of 15-1, 15-2, 15-3 | Yes | No | UE supports only a single synchronization reference in a carrier/BWP. | Per band | N.A. | N.A. | N.A. | Component-1 candidate value set: {1, 2, 3, 4} Note: RAN1 is still discussing whether this FG is needed | Optional with capability signalling |

- NR_eMIMO

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | - The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-1 - The max number of NZP-IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) within a slot shall not exceed MB_1-3 - The max number of SSB/CSI-RS (1Tx)/CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR (including CMR and IMR) shall not exceed MC_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-1 - The max number of NZP IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MC_1-3 - The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_2 - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 | 2-21, 2-22 or 2-23, 2-23a |  |  |  | Per band |  |  |  |  | Optional with capability signalling Component-1 to 4, candidate value set for MB_1 and MB_1-x is {0, 8, 16, 32, 64} Component-5 to 8, candidate value set for MC_1 and MC_1-x is {0, 4, 8, 16, 32, 64} Component-9, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-10: candidate value set: {"not supported", "1 only", "3 only", "both 1 and 3"} Component-11, candidate value set for MD_2 is {0, 1, 4, 8, 16, 32, 64} |
|  | 16-1a-2 | Non-group based L1-SINR reporting | - Support of non-group based L1-SINR reporting with N_max L1-SINR values reported | 16-1a-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling Candidate value set is {1, 2, 4} |
|  | 16-1a-3 | Group based L1-SINR reporting | - Support of group based L1-SINR reporting | 16-1a-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
|  | 16-1b | TCI state activation and spatial relation update | - Support of Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH - FFS: details on whether/how to indicate band pairs which can share the same DL TCI state | Component 1: 2-1, 2-4 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
|  | 16-1b-2 | Spatial relation update across multiple CCs | - Support of Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - FFS: details on whether/how to indicate band pairs which can share the same UL spatial relation info | Component 1: 2-59, 2-60 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
|  | 16-1b-3 | Spatial relation update for PUCCH group | - Support of PUCCH resource groups per BWP for simultaneous spatial relation update | 2-53, 2-59, 4-24 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
|  | 16-1c | Default spatial relation | Support of default spatial relation and pathloss reference RS for dedicated-PUCCH/SRS and PUSCH scheduled by DCI format 0_0 | 2-53, 2-59 |  | N/A |  | TBD [Per band] | N | Y |  |  | TBD |
|  | 16-1d | MAC CE spatial relation update for AP-SRS | [Support of / The maximum number of] spatial relation update for AP-SRS via MAC CE | 2-53, 2-59 |  | N/A |  | TBD [Per UE] | N | N |  |  | TBD |
|  | 16-1e | Pathloss reference RS activation via MAC CE | - The maximum number of configured pathloss reference RSs for PUSCH/SRS by RRC for MAC-CE based pathloss reference RS update - FFS: The maximum number of activated pathloss reference RS update for PUSCH/SRS/PUCCH [across CCs / within a slot across all CCs / per CC] - FFS: Number of measurement samples N to apply newly activated pathloss reference RS | 8-2, 8-3 |  | N/A |  | TBD [Per UE] | N | N |  |  | TBD |
|  | 16-1f | SCell beam failure recovery | - The maximum number of SCells for SCell beam failure recovery - FFS: Support of PUCCH-BFR - FFS: The maximum number of CSI-RS and/or SSB resources for new beam identification of SCell BFR [across all CCs / within a slot across all CCs / per CC] - FFS: Density of CSI-RS for new beam identification for SCell BFR | 2-31 |  | N/A |  | TBD | N |  |  |  | TBD |
|  | 16-1g | Resources for beam management, [pathloss measurement, BFD, and BFR] | - The maximum number of SSB/CSI-RS/[CSI-IM] resources [within a slot] across all CCs for any of L1-RSRP measurement, L1-SINR measurement, [pathloss measurement, BFD, and new beam identification] - FFS: The maximum number of SSB/CSI-RS resources within a slot across all CCs for pathloss measurement - FFS: The maximum number of SSB/CSI-RS resources within a slot across all CCs for BFD - FFS: The maximum number of SSB/CSI-RS resources across all CCs for new beam identification |  |  | N/A |  | TBD | N |  |  |  | TBD |
|  | 16-2a | Multi-DCI based multi-TRP | - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” - The value of R=[1,2] for BD/CCE - [Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency] [for FR1] FFS whether default value of R is needed | TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
|  | 16-2a-0 | Overlapping PDSCHs in time and non-overlapping in frequency | - Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency [merge with 16-2a? Merge with 16-2a-8?] [FR2 only?] |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-1 | Simultaneous reception of Multiple PDSCHs overlapping in frequency | - Support fully overlapping PDSCHs at frequency and partially/fully overlapping at time - Support partially overlapping PDSCHs in frequency and partially/fully overlapping in time - The maximum number of MIMO layers of each scheduled PDSCH [if PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain] - The maximal number of PDSCH scrambling sequences per serving cell FFS whether default values of component 3/4 to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-2 | Out-of-order operation for DL | 1. Support out-of-order operation for PDCCH to PDSCH 2. Support out-of-order operation for PDSCH to HARQ-ACK |  |  |  |  |  |  |  |  | Note: this FG is a WA in RAN1 |  |
|  | 16-2a-3 | Out-of-order operation for UL | 1. Support out-of-order operation for PDCCH to PUSCH |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-4 | HARQ-ACK for multi-DCI based multi-TRP - separate | - Support of separate HARQ-ACK - [Support of two TDMed long PUCCHs in a slot] |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-4a | HARQ-ACK for multi-DCI based multi-TRP - joint | - Support of joint HARQ-ACK |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-5 | Separate CRS rate matching | Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-6 | Default QCL enhancement for multi-DCI based multi-TRP | Support of default QCL assumption per CORESETPoolIndex |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-7 | Maximum number of activated TCI states | - Support of maximum per CORESETPoolIndex - Support of total maximum FFS whether default values to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-8 | Simultaneous reception with different Type-D | whether UE supports receiving time-overlapping [PDSCHs]/PDCCHs with different Type-D FFS whether default values to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-9 | [Simultaneous reception across CCs with Multi-DCI] | [The maximum number of CCs supporting multi-DCI based multi-TRP simultaneously] |  |  |  |  |  |  |  |  | Note: If the type of 16-2a is agreed to be FSPC or Fs this FG will be removed |  |
|  | 16-2a-10 | Value of BD factor | R=[(1,2] for BD/CCE |  |  |  |  |  |  |  |  |  |  |
|  | 16-2b-0 | Two default beams for single-DCI based multi-TRP | Support of default QCL assumption with two TCI states | 16-2b |  | N/A |  | Per band | TDD only | FR2 only |  |  | TBD |
|  | 16-2b-1 | Single-DCI based SDM scheme | - FFS: Support of DCI indication of of 2 TCI states by a codepoint and DMRS ports within two CDM groups - Support of two PTRS ports - FFS Support of DMRS entry {0, 2, 3} | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  |  | TBD |
|  | 16-2b-2 | Single-DCI based FDMSchemeA | Support of FDMSchemeA | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  |  | TBD |
|  | 16-2b-3 | Single-DCI based FDMSchemeB | - Support of FDMSchemeB - For FDMSchemeB, whether the UE can support CW soft combining | 16-2b, TBD |  | N/A |  | [per FSPC] | N | TBD |  |  | TBD |
|  | 16-2b-4 | Single-DCI based TDMSchemeA | - Support of TDMSchemeA - Supported maximum TBS size for TDMSchemeA | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  | Component 2 candidate values {10 CBs, TBD} | TBD |
|  | 16-2b-5 | Single-DCI based inter-slot TDM | - Support of MAC CE to activate two TCI states for a TCI codepoint - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  | Component 1 candidate values: {8,16} Component 2 candidate values {10 CBs, TBD} | TBD |
|  | 16-3a | Regular eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=1 - Support of parameter combinations 1-6 - Support of rank 1,2 - FFS: CBSR with hard amplitude restriction | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3a-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=2 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3a-2 | Support of parameter combinations 7-8 | Support of parameter combinations 7-8 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3a-3 | Support of rank 3,4 | Support of rank 3,4 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3a-4 | CBSR | ALT 1) CBSR with soft amplitude restriction (capture consequence if not supported hard amplitude restriction is supported) ALT 2) CBSR |  |  |  |  |  |  |  |  |  |  |
|  | 16-3b | Port selection eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=1 - 6 parameter combinations (combos with L=6 don’t apply) - Support of rank 1,2 | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3b-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=2 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3b-2 | Support of rank 3,4 | Support of rank 3,4 |  |  |  |  |  |  |  |  |  |  |
|  | 16-4 | Low PAPR DMRS for DL | Low PAPR DMRS for PDSCH | TBD |  | N/A |  | Per UE | N | N |  |  | [Optional] |
|  | 16-5a | UL full power transmission [mode 0] | - Supported UL full power transmission [mode 0] | 2-13, 2-14 | Y | N/A | Y | FFS: Per FS or Per band or Per band per BC | N/A | N/A |  |  | TBD |
|  | 16-5b | UL full power transmission mode 1 | - Supported UL full power transmission mode 1 - Number of Tx to support mode 1: {2Tx, 4Tx, 2Tx_4Tx} - FFS: New UL codebook set(s) per supported Tx | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N | N |  |  | TBD |
|  | 16-5c | [UL full power transmission mode 2] | - [Supported UL full power transmission mode 2 - Number of Tx to support mode 2: {2Tx, 4Tx, 2Tx_4Tx} - The maximum number of SRS resources in set with different number of ports [for usage set to ‘codebook’]. FFS on details for supported number of Tx. - FFS: Number of ports per SRS resource - FFS: Maximum number of different spatial relation info for all SRS resources for usage set to ‘codebook’ in a resource set - TPMI group which delivers full power. FFS on details for supported number of Tx. Note: UE indicating mode 2 shall support full power transmission for 1 antenna port] | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N | N |  |  | TBD |
|  | 16-6a | Low PAPR DMRS for PUSCH without transform precoding | - For PUSCH without transform precoding | TBD | Y | N/A | Y | FFS: Per band | N/A | N/A |  |  | FFS: Optional with capability signalling |
|  | 16-6b | Low PAPR DMRS for PUCCH | For PUCCH format 3 and/or PUCCH format 4 with transform precoding and with pi/2 BPSK modulation | TBD |  | N/A |  | FFS: Per band | N | N |  |  | FFS: Optional with capability signalling |
|  | 16-6c | Low PAPR DMRS for PUSCH with transform precoding and with pi/2 BPSK | For PUSCH with transform precoding and with pi/2 BPSK modulation | TBD | Y | N/A | Y | FFS: Per band | N/A | N/A |  |  | FFS: Optional with capability signalling |
|  | 16-7 | Extension of the maximum number of configured aperiodic CSI report settings | Extension of the maximum number of configured aperiodic CSI report settings for all codebook types |  |  |  |  | [Per band] |  |  |  |  |  |
|  | 16-8 | Active CSI-RS resources and ports for mixed codebook types in any slot | FFS: the detailed component design |  |  |  |  |  |  |  |  |  |  |

- NR_CLI_RIM

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17. NR_CLI_RIM | 17-1 | CLI-RSSI measurement | 1. Support CLI-RSSI measurement. The max number of resources across all CCs configured to measure RSSI simultaneously shall not exceed 64. 2. Maximum number of measurement resources configured for CLI-RSSI measurement | TBD | Yes | N/A |  | Per UE | TDD only | Yes | N/A | Candidate values for component 2 are {8, 16, 32, 64}. FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
| 17. NR_CLI_RIM | 17-2 | SRS-RSRP measurement | 1. Support SRS-RSRP measurement. The max number of SRS resources across all CCs configured to measure SRS-RSRP simultaneously shall not exceed 32. 2. Maximum number of measurement resources configured for SRS-RSRP measurement 3. Maximum number of measurement resources configured for SRS-RSRP measurement within a slot | TBD | Yes | N/A |  | Per UE | TDD only | Yes |  | Candidate values for component 2 are {4, 8, 16, 32}. Candidate values for component 3 are {2, 4, 8}. FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |

- MR-DC/CA enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18. MR-DC/CA enhancement | 18-1 | Basic UL power sharing for DC | Semi-static power sharing mode1 between MCG and SCG cells of same FR for NR dual connectivity. | TBD | Yes | N/A |  | Per BC | N/A | N/A | N/A | Absence means intra-FR DC is not supported. | Optional with capability signalling |
|  | 18-1a | Semi-static UL power sharing mode 2 for DC | Semi-static power sharing mode 2 between MCG and SCG cells of same FR for NR dual connectivity. | 18-1 (TBD) | Yes | N/A |  | Per BC | N/A | N/A | N/A | Semi-static power sharing mode 2 between MCG and SCG cells of same FR is applicable only for synchronous NR dual connectivity | Optional with capability signalling |
|  | 18-1b | Dynamic UL power sharing for DC | Dynamic power sharing between MCG and SCG cells of same FR for NR dual connectivity. - T_offset | 18-1 (TBD) | Yes | N/A |  | Per BC | N/A | N/A | N/A | 1) {short, long} | Optional with capability signalling |
| 18. MR-DC/CA enhancement | 18-4 | SCell dormancy within active time | Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_1/1_1 | TBD | Yes | N/A |  | FFS [Per UE or Per BC] | No | [Yes or N/A] | N/A |  | Optional with capability signalling |
|  | 18-4a | SCell dormancy outside active time | Support for SCell dormancy indication sent outside the active time on PCell with DCI format 2_6 | [19-1] (TBD) | Yes | N/A |  | FFS [Per UE or Per BC] | No | [Yes or N/A] | N/A |  | Optional with capability signalling |
|  | [18-4b] | [Support of SCell dormancy indication without data scheduling within active time] | [Support of SCell dormancy indication without data scheduling within active time] | TBD | Yes | N/A |  | FFS [Per UE or Per BC] | No | [Yes or N/A] | N/A |  | Optional with capability signaling |
| 18. MR-DC/CA enhancement | 18-5 | DL cross-carrier scheduling with different SCS | 1. The UE supports DL cross carrier scheduling for the different numerologies with carrier indicator field (CIF) in DL carrier aggregation where numerologies for the scheduling cell and scheduled cell are different {Scheduling cell of lower SCS and scheduled cell of higher SCS, Scheduling cell of higher SCS and scheduled cell of lower SCS, both} [2. Processing up to X unicast DCI scheduling for DL per scheduled CC ] X is based on pair of (scheduling CC SCS, scheduled CC SCS): X=[4] for (15,120), (15,60), (30,120), X=[2] for (15,30), (30,60), (60,120 kHz), X applies per span in a slot of scheduling CC | TBD | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per FS] | N/A | [Yes or N/A] | N/A | crossCarrierScheduling-OtherSCS Note: This applies also to the case where there is a single span in the slot for the scheduling CC. In case UE supports 3-5b, the limits apply for each span for FDD scheduling cell and TDD scheduling cell. | Optional with capability signalling |
|  | 18-5a | Default QCL assumption for cross-carrier scheduling | Indicates whether the UE can be configured with enabledDefaultBeamForCCS for default QCL assumption for cross-carrier scheduling. | TBD | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per band] | N/A | [Yes or N/A] | N/A |  | Optional with capability signalling |
|  | 18-5b | UL cross-carrier scheduling with different SCS | 1. The UE supports UL cross carrier scheduling for the different numerologies with carrier indicator field (CIF) in UL carrier aggregation where numerologies for the scheduling cell and scheduled cell are different {Scheduling cell of lower SCS and scheduled cell of higher SCS, Scheduling cell of higher SCS and scheduled cell of lower SCS, both} [2. Processing up to X unicast DCI scheduling for UL per scheduled CC ] X is based on pair of (scheduling CC SCS, scheduled CC SCS): X=[4] for (15,120), (15,60), (30,120), X=[2] for (15,30), (30,60), (60,120 kHz), X applies per span in a slot of scheduling CC | TBD | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per FS] | N/A | [Yes or N/A] | N/A | crossCarrierScheduling-OtherSCS Note: This applies also to the case where there is a single span in the slot for the scheduling CC. In case UE supports 3-5b, the limits apply for each span for FDD scheduling cell and TDD scheduling cell. | Optional with capability signalling |
|  | [18-5c] | [DL cross-carrier scheduling with different SCS and PDSCH processing capability 2] | [DL cross-carrier scheduling with different SCS and PDSCH processing capability 2] | TBD | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per FS] | N/A | [Yes or N/A] | N/A |  | [Optional with capability signaling] |
|  | [18-5d] | [UL cross-carrier scheduling with different SCS and PDSCH processing capability 2] | [UL cross-carrier scheduling with different SCS and PDSCH processing capability 2] | TBD | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per FS] | N/A | [Yes or N/A] | N/A |  | [Optional with capability signaling] |
| 18. MR-DC/CA enhancement | 18-6 | Cross-carrier A-CSI RS triggering with different SCS | Cross-carrier A-CSI RS triggering with different SCS | 2-33 (TBD) | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per FS or Per BC] | N/A | [Yes or N/A] | N/A | 1) {PDCCH cell of lower SCS and A-CSI RS cell of higher SCS, PDCCH cell of higher SCS and A-CSI-RS of lower SCS, both} . | Optional with capability signalling |
|  | 18-6a | Default QCL assumption for cross-carrier A-CSI-RS triggering | Indicates whether the UE can be configured with enabledDefaultBeamForCCS for default QCL assumption for cross-carrier A-CSI-RS triggering. | TBD | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per band or Per BC] | N/A | [Yes or N/A] | N/A |  | Optional with capability signalling |
| 18. MR-DC/CA enhancement | 18-7 | CA with non-aligned frame boundaries | CA with non-aligned frame boundaries for inter-band CA | TBD | Yes | N/A |  | Per BC | N/A | N/A | N/A | Defines whether the UE supports carrier aggregation operation where the frame boundaries of the Pcell and the Scell are not aligned, while the slot boundaries are. | Optional with capability signalling |
| 18. MR-DC/CA enhancement | 18-8 | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | 6-7 (TBD) | Yes | N/A |  | Per UE | No | No | N/A | Support HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group. Rel-15 had this per cell group | [TBD] |
| 18. MR-DC/CA enhancement | 18-2 | Single UL TX operation for TDD PCell in EN-DC | TDM restriction to LTE TDD PCell in EN-DC for single UL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) TDD UL/DL configuration#2, #4, #5 configured as DL-reference UL/DL configuration 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern (for type 1 UE) [4) if UE indicates that it does not support simultaneous UL transmissions as defined in TS 38.101-3 [4] using singleUL-Transmission, NR (SCG) UL transmission is dropped when an overlapping LTE (MCG) UL transmission is present (for type 1 UE).”] | EN-DC (TBD) | Yes | N/A |  | Per band combination | Applicable to TDD-TDD EN-DC only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to TDD PCell | Optional with capability signalling |
|  | 18-2a | Enhanced single UL TX operation for FDD Pcell EN-DC | TDM restriction to LTE FDD Pcell in EN-DC for single UL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern (for type 1 UE) [4) if UE indicates that it does not support simultaneous UL transmissions as defined in TS 38.101-3 [4] using singleUL-Transmission, NR (SCG) UL transmission is dropped when an overlapping LTE (MCG) UL transmission is present (for type 1 UE).”] | 6-13 (TBD) | Yes | N/A |  | Per band combination | Applicable to in FDD-LTE -NR EN-DC | Applicable to FR1 only |  | Enhancement to the R15 capability tdm-Pattern | Optional with capability signalling |
|  | 18-2b | Support of HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell for type 1 UE | Support of HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell for type 1 UE | TBD | Yes | N/A |  | Per band combination | N/A | N/A | N/A |  | Optional with capability signaling |
|  | 18-3 | Dual Tx transmission for EN-DC with FDD PCell(TDM pattern for dual Tx UE) | TDM restriction to LTE FDD PCell in EN-DC for dual UL Tx operation when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern | EN-DC (TBD) | Yes | N/A |  | Per band combination | Applicable to EN-DC with LTE FDD PCell only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to a 2Tx UE | Optional with capability signalling |
|  | 18-3a | Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern | UE configured with tdm-patternConfig-r16 can be semi-statically configured with LTE UL transmissions in all UL subframes not limited to the reference tdm-pattern (only for type 1 UE) | 18-2, 18-2a (TBD) | Yes | N/A |  | [Per band combination] | Applicable to EN-DC only | Applicable to FR1 only |  |  | Optional with capability signaling |

- UE Power Saving

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19.UE Power Saving | 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON | N/A |  | Yes |  | N/A |  | Per UE | No | Yes | N/A | The minimum time gap between the end of the slot of last DCI format 3_0 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
|  | 19-2 | Cross Slot Scheduling | (1) Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset (2) minimumSchedulingOffset K2 configuration for PUSCH |  | FFS |  | Yes | N/A | Dynamic adaptation of the minimum value of K0min/K2min for cross-slot scheduling is not supported | Per UE | No | No | N/A | FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
|  | 19-3 | Maximum MIMO Layer Adaptation | - 1. Support of maximum number of MIMO layer configuration per DL BWP |  | See Note |  | Yes | N/A |  | [Per UE ] | No | [Yes] | N/A | This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
|  | 19-4a | UE assistance information | Support of reporting preferred minimum K0/K2 via UE assistance information - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots |  | 19-2 |  | Yes | N/A |  | Per UE | No | No | N/A | The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |

- NR_IAB

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20. NR_IAB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 20-2 | Inter-IAB-node discovery and measurements: SSB reception configuration | Support up to 4 SMTCs configured for an IAB node MT per frequency location, including IAB-specific SMTC window periodicities |  |  |  | Yes | N/A | Separate configuration of SMTC windows for Inter-IAB node discovery and measurement is not possible | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. [Devices supporting IAB backhaul must report this FG as supported] |
|  | 20-3 | Extension of RACH occasions and periodicities for backhaul RACH resources | Support RACH configuration for IAB-MT separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors |  |  |  | Yes | N/A | Separate configuration of RACH transmissions for access UEs and IAB nodes is not possible | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. [Devices supporting IAB backhaul must report this FG as supported] |
|  | 20-5a | UL-Flexible-DL slot formats | Support semi-static configuration/indication of UL-Flexible-DL slot formats for IAB-MT resources |  | 5-1a |  | Yes | N/A | Only Rel-15 slot formats can be configured for backhaul links | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
|  | 20-5b | UL-Flexible-DL slot formats | Support dynamic indication of UL-Flexible-DL slot formats for IAB-MT resources |  | 3-6 |  | Yes | N/A | Dynamic indication of UL-Flexible-DL slot formats for IAB-MT resources is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
|  | 20-6 | Dynamic indication of soft resource availability | Support monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of soft resource availability to an IAB node |  |  |  | Yes | N/A | Explicit indication of soft resource availability is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. [Devices supporting IAB backhaul must report this FG as supported] |
|  | 20-7 | Case 1 OTA timing alignment | Support T_delta reception. |  |  |  | Yes | N/A | Case-1 OTA timing alignment is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. [Devices supporting IAB backhaul must report this FG as supported] |
|  | 20-8 | Guard symbols | 1) Support DesiredGuardSymbols reporting 2) Support ProvidedGuardSymbols reception |  |  |  | Yes | N/A | Guard symbols reporting and reception is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |

- Mobility Enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21. Mobility Enhancement | 21-1a | Intra-frequency DAPS HO | Support of intra-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The network cannot configure UE with DAPS HO | Per Band | No | N/A | N/A |  | [Optional with capability signalling] |
|  | 21-1b | Inter-frequency DAPS HO | Support of inter-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The network cannot configure UE with DAPS HO | Per BC | No | N/A | N/A |  | [Optional with capability signalling] |
|  | 21-2 | Basic UE power sharing for DAPS HO | Support of semi-static power sharing mode1 |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | FFS: The UE is only able to drop the transmission to the source. | Per BC | No | N/A | N/A |  | Optional with capability signalling |
|  | 21-2a | Semi-static UL power sharing mode 2 for DAPS HO | Support of semi-static power sharing mode 2 |  | 21-2 |  | Yes | N/A |  | Per BC | No | N/A | N/A |  | Optional with capability signalling |
|  | 21-2b | Dynamic UL power sharing for DAPS HO | Support of dynamic power sharing |  | 21-2 |  | Yes | N/A |  | Per BC | No | N/A | N/A |  | Optional with capability signalling |
|  | [21-2d] | [UL transmission cancellation] | [Indicates support of cancelling UL transmission to the source cell] |  | DAPS (Note: RAN2 feature) |  |  |  |  |  |  |  |  |  |  |

- Potential change/update on existing UE features for Rel-16 UE

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8. UL TPC | 8-1 | Dynamic power sharing for LTE-NR DC | When total transmission power exceeds Pcmax, UE scales NR transmission power. |  | EN-DC |  | No | N/A |  | Per UE | No | No |  |  | Mandatory with capability signalling set to 1 |