3GPP TSG RAN WG1 Meeting #100bis-e	R1-2003149

# e-Meeting, April 20th- 30th, 2020

Title:	LS on IAB-DU features

Response to:

Release:	Rel-16

Work Item:	NR_IAB-Core

Source:	RAN1

To:	RAN3

Cc:

# Contact Person:

# Name:	                 Luca Blessent

# E-mail Address:	  lblessen [at] qti [dot] qualcomm [dot] com

# 1. Overall Description:

During RAN WG1 #100bis-e it was agreed the following RAN WG1 features apply to the IAB-DU. RAN WG1 would like to inform RAN WG3 of such features.

- Inter-IAB-node discovery and measurements: SSB transmission configuration: support up to 4 STCs configured for an IAB-DU per cell per frequency location, including IAB-specific SSB transmission periodicities.

- Extension of RACH occasions and periodicities for backhaul RACH resources: support RACH configuration separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors.

- IAB-node non-TDM multiplexing capability: TDM not required between IAB-MT and IAB-DU functions.

- UL-Flexible-DL slot formats: support semi-static and dynamic configuration/indication of UL-Flexible-DL slot formats for IAB-DU resources.

- Dynamic indication of soft resource availability: support DCI Format 2_5 based indication of soft resource availability to an IAB-node.

- IAB-DU resource configuration: per-cell D/U/F resource type configuration + H/S/NA attributes per-resource type.

- Insertion of guard symbols by the IAB-DU.

# 2. Actions:

# To RAN3

# ACTION:

# RAN WG1 informs RAN WG3 of the aforementioned IAB-DU features and respectfully asks RAN WG3 to take them into consideration in any related decision on how to handle IAB-DU features.

# 4. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #101-e	25 May – 5 June 2020, 		e-Meeting

TSG-RAN WG1 Meeting #102	24 – 28 August 2020, 		Toulouse, France