3GPP TSG RAN WG1 Meeting #101-e	R1-2003579

e-Meeting, May 25th – June 5th, 2020

Agenda item:		7.2.5.3

Source:		Nokia, Nokia Shanghai Bell

Title:	Maintenance of PUSCH enhancements for Rel-16 NR URLLC

Document for:		Discussion and Decision

# 1	Introduction

The URLLC L1 enhancements work item was declared as complete in December 2019. Many open issues for PUSCH enhancements have been resolved in RAN1#100-e and especially in RAN1#100bis-e, but there are still some open issues, e.g. CSI multiplexing on PUSCH, UCI scaling procedures and for some agreement’s the corresponding TPs are still missing.

In this contribution, we discuss these remaining issues and the related text proposals based on the latest specifications.

# 2		Identified remaining issues

## 2.1 	A-CSI/SP-CSI on PUSCH repetition Type B

There had been good progress on HARQ-Ack multiplexing (in terms of the actual PUSCH repetition) during RAN1#100bis-e:

Agreements:

In case of PUCCH overlapping with PUSCH with repetition Type B,

- Option A: Multiplexing timeline conditions in Clause 9.2.5 of TS 38.213 shall be satisfied for all the overlapping actual repetitions. Otherwise it is considered as an error case.

Agreements:

In case PUCCH overlaps with multiple repetitions of PUSCH repetition Type B that satisfy the multiplexing timeline conditions, UCI is multiplexed on only one actual repetition (including the case where a PUCCH overlaps with a PUSCH with repetition Type B in multiple slots).

- Option 1: the first overlapping actual repetition that satisfies the multiplexing timeline

There had also been good discussion on the related mapping of A-CSI and SP-CSI with and without UL-SCH but no agreement had been reached there. We would like to note here, that in the decision process on HARQ, the arguments when the final decision has been made in the online session was: (a) keeping it simple, and (b) following the legacy principles. We think the same should be equally applied when deciding the A-CSI and SP-CSI mapping.

A-CSI/SP-CSI on PUSCH repetition Type B without UL-SCH

The summary of the related discussion during RAN1#100bis-e can be found in Sec. 2.1 of the FL summary in R1-2003006. Specifically, we would like to use the latest related proposals from the FL that had been developing during the meeting weeks as a starting point for discussions here:

Proposal 1a:

For A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH, A-CSI/SP-CSI is transmitted on the first actual repetition, and other actual repetitions are discarded. Down-select among the following:

- Option A: there is no scheduling restriction.

- Option B: UE is not expected to be indicated with numberofrepetitions >1.

- Option C: The first nominal repetition is not expected to be segmented into multiple actual repetitions.

Proposal 8:

For A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH, A-CSI/SP-CSI is transmitted on the first actual repetition and other actual repetitions are discarded. UE is not expected to be indicated with numberofrepetitions >1.

We are fine with the intention of the latest proposal to have the mapping on the first actual repetition (similar as decided for HARQ-Ack as noted above), but we do not see the need to restrict the number of repetitions (as added in the second sentence of Proposal 8) as it is anyhow clear that any other repetitions are to be discarded. Actually, this is the same as in Rel-15 for PUSCH with aggregation factor, that there can be ‘repetition’ scheduled but actually only the first PUSCH containing the A-CSI or SP-CSI is transmitted. Therefore, we suggest to adopt only the first sentence of the proposal 8 (according to Option A of Proposal 1a above) as the additional restriction is not reducing UE complexity but without the additional restriction will give the gNB more flexibility to select the appropriate S & L combination from the overall set of TDRA allocations of any K value here. Moreover, this directly follows the Rel-15 behavior for PUSCH repetition Type A.

Proposal 1: For A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH, A-CSI/SP-CSI is transmitted on the first actual repetition and other actual repetitions are discarded.

A-CSI on PUSCH repetition Type B with UL-SCH

The summary of the related discussion during RAN1#100bis-e can be found in Sec. 2.1 of the FL summary in R1-2003006. Specifically, we would like to use the latest related proposals from the FL that had been developing during the meeting weeks as a starting point for discussions here:

- Proposal 2: For A-CSI on PUSCH with repetition Type B with UL-SCH, A-CSI is transmitted on the first actual repetition.

- Proposal 7: For A-CSI on PUSCH with repetition Type B with UL-SCH, A-CSI is transmitted on the last actual repetition.

- Proposal 7a: For A-CSI on PUSCH with repetition Type B with UL-SCH, A-CSI is transmitted on the first actual repetition that fulfills the CSI multiplexing timeline.

Proposal 2 would basically go somehow hand in hand with the agreement on the HARQ-Ack mapping, i.e. the multiplexing timeline would need to be fulfilled for all the (actual & nominal) repetitions as well as the Rel-15 PUSCH repetition Type A operation, where the CSI is only to be mapped on the first PUSCH transmission. In this respect, in the spirit of the agreements on the HARQ-Ack mapping – we think Proposal 2 should be supported.

There had been companies raising the issue, that there may be a delay in the PUSCH – as the multiplexing timeline needs to be fulfilled and therefore Proposal 7 was brought up. As commented during the RAN1#100bis-e email discussions of the discussion thread “[100b-e-NR-L1enh-URLLC-PUSCH-04]”, this is not really true as it is fully under gNB control and the gNB can schedule urgent data without A-CSI in the first place and schedule (even at the same time) for later on, where the multiplexing timeline is fulfilled already for the start of the overall PUSCH transmission burst. Thus, we do not see any need for any latency optimization here – as this can be handled by gNB implementation. Moreover, as brought up by us in the email discussion, if some latency optimizations should be done, then actually Proposal 7a should be adopted as Proposal 7 would artificially delay the A-CSI overall.  For illustration, let’s look at two examples shown in Figure 1 below.

Figure 1: Additional A-CSI latency of Proposal 7 compared to Proposal 2/7a of Case A (in case the multiplexing timeline is fulfilled for the first repetition - based on Rel-15 requirements) 
or compared to Proposal 7a of Case B (in case the mux. Timeline is not fulfilled 
for the first repetition, contradicting the related Rel-17 requirement)

The figure nicely demonstrates the unnecessary additional A-CSI delay introduced by adopting Proposal 7 here, without any actual need for it. In case we assume the Rel-15 multiplexing timeline is to be kept, Proposal 2 should be agreed (which is clearly our first preference) as noted above we do not see any issue with latency here (as under full gNB control). But if this assumption is not kept (contradicting Rel-16), then clearly Proposal 7a should be applied as it provides the same ‘claimed’ advantages of the Proposal 7 camp in terms of PUSCH latency but does not have the detrimental delay of A-CSI on PUSCH of Proposal 7 here.

Observation 1: Proposal 7 of mapping the A-CSI on the last actual PUSCH repetition with UL-SCH introduces unnecessary, artificial delay of A-CSI delivery by the UE – which is contradicting the latency / reliability (CQI) requirements of URLLC operation.

Therefore, the following proposal is made:

Proposal 2: For A-CSI on PUSCH with repetition Type B with UL-SCH, A-CSI is transmitted on the first actual repetition (first preference). If PUSCH latency would be of concern, A-CSI should be transmitted on the first actual repetition that fulfills the CSI multiplexing timeline.

## 2.2 	UCI resource determination for PUSCH repetition Type B

In Rel-15, the UCI resources depend on the available REs in PUSCH and the TBS. As an example, the number of coded modulation symbols per layer for HARQ-ACK transmission is calculated as (Section 6.3.2.4.1.1 in TS 38.212)

,

where denotes the available RE number of the current PUSCH piggybacking the UCI, and  denotes the information bits (e.g. TBS + CRC when CBG based retransmission is not used) of the PUSCH. Note that there are similar equations for CSI.

For PUSCH repetition Type B, TBS is determined based on the nominal repetition duration, which in turn control the information bits. The question is then how the number of REs for UCI should be calculated, based on the duration of the nominal repetition or actual repetition?

The following options have been discussed in RAN1#100-e:

- Option 1: The calculation is based on the nominal repetition.

  - This was later updated to add “the additional limit of no more than the resources available in the actual repetition”. Otherwise the number of REs could exceed the available REs in an actual repetition.

  - This corresponds to FL proposal 2-1a during RAN1#100bis-e as noted in Sec. 3.2 of R1-2003004.

- Option 2: The calculation is based on the actual repetition.

- Option 3: The first part of the equation is based on the nominal repetition, and the second part of the equation is based on the actual repetition.

When nominal repetition is used in the first and second parts of the equation (option 1), that may provide a larger number of coded modulation symbols per layer for UCI compared to using actual repetition in the first and second parts of the equation (option 2). However, option 1 is not a complete solution as it further requires a conditional calculation to limit the number of coded modulation symbols per layer for UCI not more than the resources available in the actual repetition (as was also visible in the FL proposal 2-1a from RAN1#100bis-e). Moreover, with the addition to not exceed the number of REs available in an actual repetition as third condition in the min{…} operation, for a short length of the actual repetition there may be actually not any PUSCH REs transmitted. So, for option 1, basically the configured value of the ‘scaling’, i.e. α, in the second part will not really have an effect for such cases.

With option 2, the changes in the specifications are minimal, but as pointed out during the email discussions, the number of REs spent for HARQ-ACK may be rather low in case there is a large difference in number of symbols /REs of a nominal repetition and an actual repetition. Therefore, with option 2 the reliability of the HARQ-ACK multiplexed on PUSCH will be very much dependent on the ratio of number of REs of the actual repetition and the nominal repetition. This could only be controlled by gNB with option 2 by scheduling in a way that the repetition carrying the HARQ-ACK would be as long as possible (with preferably having the length of a nominal repetition). This would clearly have an impact on the potential start of a PUSCH repetition Type B transmission and therefore may introduce scheduling delays, as e.g. 2 symbol actual repetitions with HARQ-Ack should be avoided potentially deferring the start to a new slot or after the UL/DL switching period. From this perspective we don’t think that option 2 should be supported.

In option 3, nominal repetition is assumed in the first part and actual repetition is assumed in the second part of the equation. From principle point of view, as with option 1, more REs will be spent on HARQ-ACK compared to option 2 in the first part. But in contrast to option 1, the configured ‘scaling’ will still be applicable also for shorter actual repetitions. In this respect, we see option 3 as having a slight advantage compared to option 1 (or FL Proposal 2-1a from RAN1#100bis-e) in this respect.

Proposal 3: For UCI resource determination for PUSCH repetition Type B adopt Option 3, i.e. the first part of the equation is based on the nominal repetition, and the second part of the equation is based on the actual repetition. Adopt the following text proposal on the number of coded modulation symbols per layer for UCI to Sec. 6.3.2.4.1.1 for HARQ-ACK in TS 38.212 with changes marked in red. If adopted, similar changes along the lines of the TP below for HARQ-ACK will also be required for CSI part 1 / 2 as well as for CG in Sec. 6.3.2.4.1.2, 6.3.2.4.1.3, Sec. 6.3.2.4.1.4 and Sec. 6.3.2.4.1.5.

TP to TS 38.212, Sec. 6.3.2.4.1.1 on HARQ-ACK resource determination for PUSCH repetition Type B: ###### 6.3.2.4.1.1 HARQ-ACK For HARQ-ACK transmission on PUSCH with UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK transmission, denoted as , is determined as follows: - for PUSCH repetition Type A: - for PUSCH repetition Type B: where - is the number of HARQ-ACK bits; - if , ; otherwise is the number of CRC bits for HARQ-ACK determined according to Clause 6.3.1.2.1; - ; - is the number of code blocks for UL-SCH of the PUSCH transmission; - if the DCI format scheduling the PUSCH transmission includes a CBGTI field indicating that the UE shall not transmit the -th code block, =0; otherwise, is the -th code block size for UL-SCH of the PUSCH transmission; - is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers; - is the number of subcarriers in OFDM symbol that carries PTRS, in the PUSCH transmission; - is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and is the total number of OFDM symbols of the PUSCH or the total number of symbols of the actual repetition for PUSCH repetition Type B, for , is the total number of symbols of a pseudo nominal repetition for PUSCH repetition Type B, including all OFDM symbols used for DMRS; - for any OFDM symbol that carries DMRS of the PUSCH, ; - for any OFDM symbol that does not carry DMRS of the PUSCH, ; - is configured by higher layer parameter scaling; - is the symbol index of the first OFDM symbol that does not carry DMRS of the PUSCH, after the first DMRS symbol(s), in the PUSCH transmission. For HARQ-ACK transmission on PUSCH without UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK transmission, denoted as , is determined as follows: where - is the number of HARQ-ACK bits; - if , ; otherwise is the number of CRC bits for HARQ-ACK defined according to Clause 6.3.1.2.1;; - ; - is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers; - is the number of subcarriers in OFDM symbol that carries PTRS, in the PUSCH transmission; - is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and is the total number of OFDM symbols of the PUSCH or the total number of symbols of the actual repetition for PUSCH repetition Type B, including all OFDM symbols used for DMRS; - for any OFDM symbol that carries DMRS of the PUSCH, ; - for any OFDM symbol that does not carry DMRS of the PUSCH, ; - is the symbol index of the first OFDM symbol that does not carry DMRS of the PUSCH, after the first DMRS symbol(s), in the PUSCH transmission; - is the code rate of the PUSCH, determined according to Clause 6.1.4.1 of [6, TS38.214]; - is the modulation order of the PUSCH; - is configured by higher layer parameter scaling. < text omitted>

## 2.3	TP on invalid symbols based on SSB, CORESET#0 in MIB and gap handling (38.214, Sec. 6.1.2.1)

At RAN1#100bis-e, the following agreements have been made without having the respective TP(s) discussed and agreed (as noted in the FL summary in R1-2003005):

Agreements:
For operation in unpaired spectrum, symbols that are indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks are considered invalid symbols for PUSCH repetition Type B, and segmentation occurs around these invalid symbols.

Agreements:

For operation in unpaired spectrum, symbols indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS are considered as invalid symbols for PUSCH repetition Type B, and segmentation occurs around these symbols.

Agreements:

For operation in unpaired spectrum, introduce a new RRC parameter numberInvallidSymbolsForDL-UL-Switching to indicate the number of symbols after the last semi-static DL symbol that are invalid symbols for PUSCH repetition Type B.

- The candidate values include {1, 2, 3, 4}.

- If not configured, it means no symbols are explicitly defined for DL-to-UL switching.

We therefore propose the following TP to cover these three agreements in 38.214, Sec. 6.1.2.1.

Proposal 4: Adopt the following text proposal based on the related agreements from RAN1#100bis-e on the symbols for SSB and CORESET#0 in MIB as well as gap handling to Sec. 6.1.2.1 TS 38.214 with changes marked in red.

TP to TS 38.214, Sec. 6.1.2.1: < text omitted> For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows: - A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. is considered as an invalid symbol for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, - symbols indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, and - symbols indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, and - a number of symbols after the last semi-static DL symbol, where the number of symbols is provided by numberInvalidSymbolsForDL-UL-Switching if configured and zero otherwise, are considered as invalid symbols for PUSCH repetition Type B transmission. < text omitted>

# 3	Conclusions

In this contribution, we discussed the remaining issues for PUSCH enhancements in eURLLC. The discussion can be summarized in the following proposals:

- Proposal 1: For A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH, A-CSI/SP-CSI is transmitted on the first actual repetition and other actual repetitions are discarded.

- Observation 1: Proposal 7 of mapping the A-CSI on the last actual PUSCH repetition with UL-SCH introduces unnecessary, artificial delay of A-CSI delivery by the UE – which is contradicting the latency / reliability (CQI) requirements of URLLC operation.

- Proposal 2: For A-CSI on PUSCH with repetition Type B with UL-SCH, A-CSI is transmitted on the first actual repetition (first preference). If PUSCH latency would be of concern, A-CSI should be transmitted on the first actual repetition that fulfills the CSI multiplexing timeline.

- Proposal 3: For UCI resource determination for PUSCH repetition Type B adopt Option 3, i.e. the first part of the equation is based on the nominal repetition, and the second part of the equation is based on the actual repetition.

Adopt the related text proposal on the number of coded modulation symbols per layer for UCI to Sec. 6.3.2.4.1.1 for HARQ-ACK in TS 38.212 as described in Sec. 2.2. If adopted, similar changes along the lines of the TP below for HARQ-ACK will also be required for CSI part 1 / 2 as well as for CG in Sec. 6.3.2.4.1.2, 6.3.2.4.1.3, Sec. 6.3.2.4.1.4 and Sec. 6.3.2.4.1.5.

- Proposal 4: Adopt the text proposal of Sec. 2.3 to Sec. 6.1.2.1 TS 38.214 based on the related agreements from RAN1#100bis-e on the symbols for SSB and CORESET#0 in MIB as well as gap handling.