3GPP TSG RAN WG1 #101		                                                    R1-2003965

e-Meeting, May 25th – June 5th, 2020

Agenda item:	8.2.3

Source: 	CMCC

Title: 	Discussions on potential positioning enhancements

Document for:	Discussion and Decision

Introduction

In RAN#87 meeting, a new study item for NR positioning enhancements was approved [1], in which the motivation to provide higher accuracy location requirements resulting from new applications and industry verticals was justified. Due to a more stringent positioning accuracy and latency requirements for the IIoT use cases than that of the commercial ones, potential enhancements should be identified and studied.

In this contribution, we provide our views on some candidate enhancements that can be further studied in Rel-17 to improve the positioning performance.

Potential enhancements

2.1 Positioning for UEs in idle/inactive state

In Rel-16 positioning study item phase, positioning for UEs in idle/inactive mode was raised and discussed by some companies [2]; however, this enhancement was not included in the work item phase.

Note that without supporting the positioning for UEs in idle/inactive state, for a UE with positioning request, it has to frequently switch from idle/inactive mode tso RRC_CONNECTED mode to detect RS and report measurements. In such a case, even if the UE has no data transmissions, it is mandatory to switch into connected mode and monitor PDCCH, which costs high power consumption and signalling overhead. Considering that the positioning enhancements in Rel-17 are for IIoT scenarios, where the reduction of the power consumption is a critical factor to extend the battery life of the industrial devices, positioning for UEs in idle/inactive mode should be supported.

To support positioning in idle/inactive state, both measuring and reporting in idle/inactive mode should be investigated. For the measurement procedure, in LTE Rel-14, the idle state positioning measurement has already been specified for NB-IoT, which can be used as a starting point in the study of positioning for UEs in idle/inactive mode in NR. On the other hand, for the measurement reporting, note that the Rel-15 IoT introduced an early data transmission (EDT) to reduced power consumption and latency, in our view, it is also a good starting point for the idle/inactive state positioning enhancements.

Proposal 1: Positioning for UEs in idle/inactive state should be supported.

2.2 On-demand PRS

In Rel-16 positioning, the concept of on demand resource allocations were agreed for the DL PRS design. However, due to the limited timing budget, no details were discussed and agreed.

In general, the on demand PRS can be alternatively configured by LMF and triggered by the UE. To be specific, when a network client requests a high accuracy and low latency positioning requirements, which cannot be satisfied by the periodically configured PRS resources, the LMF can additionally allocate on demand PRS resources to meet the requirements. On the other hand, the UE itself should be able to request the network/serving cell to trigger on demand PRS resources to improve the measurement quality and reduce signaling overhead. One possible solution to support the UE-triggered on demand PRS is shown in Figure 1.

Figure 1: Illustration with UE-triggered on demand PRS

The UE can report the resource indices along with the measurements through the physical-layer procedures, based on which the gNB decides to trigger the configuration of the on demand DL PRS resources. For the transmission of the on demand DL PRS, the gNB can use selected beams toward the UE to provide high positioning accuracy.

Proposal 2: On demand PRS configurations should be supported.

Proposal 3: NR positioning should support the physical-layer procedures to trigger the on demand DL PRS configurations.

Conclusions

In this contribution, we discuss the potential NR positioning enhancements, and the following proposals are provided:

Proposal 1: Positioning for UEs in idle/inactive state should be supported.

Proposal 2: On demand PRS configurations should be supported.

Proposal 3: NR positioning should support the physical-layer procedures to trigger the on demand DL PRS configurations.

References

- RP-193237, New SID NR positioning enhancements, Qualcomm Incorporated, Sitges, Spain.

- TR 38.855, “Study on NR positioning support”, v16.0.0.