3GPP TSG RAN WG1 #101			R1-2004466

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.6.5

Source: 	Qualcomm Incorporated

Title: 	Remaining issues on low PAPR RS

Document for:	Discussion/Decision

# Introduction

In this contribution, one remaining issue is discussed on low PAPR RS, which is introduced for NR Rel-16.

In this contribution, Section 2 discussed an identified issue on ambiguity of PTRS sequence when Pi/2 BPSK is enabled for PUSCH. A TP is provided to clarify the issue. Conclusions are drawn in Section 3.

# PTRS sequence for PUSCH with Pi/2 BPSK modulation

In PTRS sequence generation for PUSCH with Pi/2 BPSK modulation, in TS38.211 clause 6.4.1.2.1.1, the following behaviour is specified

###### 6.4.1.2.1.1	Sequence generation if transform precoding is not enabled

If transform precoding is not enabled, the precoded phase-tracking reference signal for subcarrier  on layer  is given by

r(pj)m=rmif j=j' or j=j"0otherwise

where

-	antenna ports  or  associated with PT-RS transmission are given by clause 6.2.3 of [6, TS 38.214]

-	 is given by clause 6.4.1.1.1.1

Accordingly,  is given by clause 6.4.1.1.1.1 in TS38.211 as below.

- 6.4.1.1.1.1	Sequence generation when transform precoding is disabled

If transform precoding for PUSCH is not enabled, the sequence  shall be generated according to

.

where the pseudo-random sequence  is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with

cinit=217Nsymbslotns,fμ+l+12NIDnSCIDλ+1+217λ2+2NIDnSCIDλ+nSCIDλmod 231

where  is the OFDM symbol number within the slot, ns,fμ is the slot number within a frame, and

-	NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_1 or 0_2, or by a PUSCH transmission with a configured grant;

-	NID0∈0,1,…,65535 is given by the higher-layer parameter scramblingID0 in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI;

-	NID0,NID1∈0,1,…,65535 are, for each msgA PUSCH configuration, given by the higher-layer parameters msgA-scramblingID0 and msgA-scramblingID1, respectively, in the msgA-DMRS-Configuration IE if provided and the PUSCH transmission is triggered by a Type-2 random access procedure as described in clause 8.1A of [5, TS 38.213];

-	 otherwise;

-	nSCIDλ and λ are given by

-	if the higher-layer parameter DMRSuplink-r16 in the DMRS-UplinkConfig IE is provided

nSCIDλ=nSCIDλ=0 or λ=21-nSCIDλ=1
λ=λ

where λ is the CDM group defined in clause 6.4.1.1.3.

-	otherwise

nSCIDλ=nSCID
λ=0

The quantity nSCID∈0,1 is indicated by the DM-RS initialization field, if present, either in the DCI associated with the PUSCH transmission if DCI format 0_1 or 0_2, in [4, TS 38.212] is used or by the higher layer parameter dmrs-SeqInitialization, if present, for a Type 1 PUSCH transmission with a configured grant, otherwise nSCID=0.

When Pi/2 BPSK is used for the PUSCH transmission, i.e., DMRSuplink-r16 in the DMRS-UplinkConfig IE is provided, different DMRS sequence  is generated on different DMRS ports associated with different CDM group λ. It might be straightforward. But the specification in clause 6.4.1.2.1.1 does not clearly specify  for a PTRS port  or  is based on which CDM group. To clarifying this issue, a simple TP can adopt as proposed below

Proposal 1: Adopt the following TP for Section 6.4.1.2.1.1 of TS38.211

-----------------------------------------------------------------------------------------------------------------------------------------------------

< Unchanged parts are omitted >

###### 6.4.1.2.1.1	Sequence generation if transform precoding is not enabled

If transform precoding is not enabled, the precoded phase-tracking reference signal for subcarrier  on layer  is given by

r(pj)m=rmif j=j' or j=j"0otherwise

where

-	antenna ports  or  associated with PT-RS transmission are given by clause 6.2.3 of [6, TS 38.214]

-	 is given by clause 6.4.1.1.1.1 based on the CDM group index λ associated with pj' or pj',pj'' respectively.

< Unchanged parts are omitted >

-----------------------------------------------------------------------------------------------------------------------------------------------------

# Conclusions

For the remaining issues of low PAPR RS, the following two proposals are proposed.

Proposal 1: Adopt the following TP for Section 6.4.1.2.1.1 of TS38.211

-----------------------------------------------------------------------------------------------------------------------------------------------------

< Unchanged parts are omitted >

###### 6.4.1.2.1.1	Sequence generation if transform precoding is not enabled

If transform precoding is not enabled, the precoded phase-tracking reference signal for subcarrier  on layer  is given by

r(pj)m=rmif j=j' or j=j"0otherwise

where

-	antenna ports  or  associated with PT-RS transmission are given by clause 6.2.3 of [6, TS 38.214]

-	 is given by clause 6.4.1.1.1.1 based on the CDM group index λ associated with pj' or pj',pj'' respectively.

< Unchanged parts are omitted >

-----------------------------------------------------------------------------------------------------------------------------------------------------

# References

- 3GPP TSG RAN WG1 Meeting #99, “RAN1 Chairman’s notes,” Reno, USA, November 18th – 22nd, 2019.