3GPP TSG RAN WG1 Meeting #101-e														R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of switched uplink operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| mmWork item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2020-06-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of switched uplink operation between an 1-port uplink and 2-port ulink |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of a feature |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The switched uplink operation is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.0 (new), 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | 38.101-1, 38.101-3, 38.133, 38.331 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.1.0 	Uplink switching

The UE may omit uplink transmission during the uplink switching gap NTx1-Tx2 if the conditions defined in this sub-clause are met and the UE is configured with uplinkTxSwitchingPeriod-r16. The switching gap NTx1-Tx2 is indicated by UE capability [UplinkTxSwitchingPeriod-r16]:

-	If a UE indicated a capability for uplink switching with [uplinkTxSwitchRequested-r16] for a band combination, and if it is for that band combination

- 	Configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), or

-	Configured with uplink carrier aggregation, or

-	Configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink.

The conditions under which the switching gap may be present and the location of the switchin gap are defined for each of the cases in sections 6.1.0.1, 6.1.0.2, and 6.1.0.3 respectively.

6.1.0.1	Uplink switching for EN-DC

For a UE indicating a capability for uplink switching with [uplinkTxSwitchRequested-r16] for a band combination, and if it is for that band combination configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC),  if the UE is configured with uplink switching with parameter [uplinkTxSwitchingPeriod-r16],

- for the UE configured with [uplinkTxSwitchingOption1],

- when the UE is to transmit an NR uplink that takes place after an LTE uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers [immediately after] the LTE transmission is scheduled or configured to end.

- when the UE is to transmit an LTE uplink that takes place after an NR uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers [immediately before] the LTE transmission is scheduled or configured to start.

- the UE is not expected to transmit simultaneously on the NR uplink and the LTE uplink. If the UE is scheduled or configured to transmit any NR uplink transmission overlapping with an LTE uplink transmission, the NR uplink transmission is dropped,

- for the UE configured with [uplinkTxSwitchingOption2],

- when the UE is to transmit an NR two-port uplink that takes place after an LTE uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers [immediately after] the LTE transmission is scheduled or configured to end.

- when the UE is to transmit an LTE uplink that takes place after an NR two-port uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers [immediately before] the LTE transmission is scheduled or configured to start.

- the UE is not expected to transmit simultaneously a two- port transmission on the NR uplink and the LTE uplink.

-	in all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

-	when the UE is configured with tdm-PatternConfig-r15 or by tdm-PatternConfig-r16

-	for the LTE subframes designated as uplink by the higher layer parameter subframeAssignment-r15, the UE assumes one-port LTE uplink and one-port NR uplink.

-	for the LTE subframes other than the ones designated as uplink by the higher layer parameter subframeAssignment-r15, the UE assumes two port NR uplink only.

NTx1-Tx2

NTx1-Tx2

6.1.0.2	Uplink switching for Carrier Aggregation

For a UE indicating a capability for uplink switching with [uplinkTxSwitchRequested-r16] for a band combination, and if it is for that band combination configured with uplink carrier aggregation:

-	If the UE is configured with uplink switching with parameter [uplinkTxSwitchingPeriod-r16],

-	Based on DCI(s) received before T0-Toffset or based on a higher layer configurations(s), when the UE is to transmit a 2-port NR uplink channel on one uplink carrier starting at T0 and if the last uplink transmission before the NR uplink channel is  a 1-port transmission on another uplink carrier, then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2 and the UE is not expected to [be scheduled or configured to] transmit for the duration of NTx1-Tx2 on any of the two carriers [immediately before] the 2-port NR uplink channel is scheduled or configured to start at T0.

-	Based on DCI(s) received before T0-Toffset or based on a higher layer configurations(s), when the UE is to transmit a 1-port NR uplink channel on one uplink carrier starting at T0 and if the last uplink transmission before the NR uplink channel is a 2-port transmission on another uplink carrier, then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2 and the UE is not expected to [be scheduled or configured to] transmit for the duration of NTx1-Tx2 on any of the two carriers [immediately before] the 1-port NR uplink channel is scheduled or configured to start at T0.

-    For the UE configured with [uplinkTxSwitchingOption1], based on DCI(s) received before T0-Toffset or based on a higher layer configurations(s), when the UE is to transmit a 1-port NR uplink channel on one uplink carrier starting at T0 and if the last uplink transmission before the NR uplink channel is a 1-port transmission on another uplink carrier, then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2 and the UE is not expected to [be scheduled or configured to] transmit for the duration of NTx1-Tx2 on any of the two carriers [immediately before] the 1-port NR uplink channel is scheduled or configured to start at T0.

-	For the UE configured with [uplinkTxSwitchingOption2],  based on DCI(s) received before T0-Toffset or based on a higher layer configurations(s), when the UE is to transmit a 2-port NR uplink channel on one uplink carrier starting at T0  and if the last uplink transmission before the NR uplink channel is a 1-port transmission on the same uplink carrier and the UE is under the operation state in which 2-port transmission cannot be supported in the same uplink carrier, then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2 and the UE is not expected to [be scheduled or configured to] transmit for the duration of NTx1-Tx2 on any of the two carriers [immediately before] the 2-port NR uplink channel is scheduled or configured to start at T0.

-	For the UE configured with [uplinkTxSwitchingOption2], based on DCI(s) received before T0-Toffset or based on a higher layer configurations(s), when the UE is to transmit a 1-port NR uplink channel on one uplink carrier starting at T0 and if the last uplink transmission before the NR uplink channel is a 1-port transmission on another uplink carrier and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier, then the UE is not expected to [be scheduled or configured to] transmit for the duration of NTx1-Tx2 on any of the two carriers [immediately before] the 1-port NR uplink channel is scheduled or configured to start at T0.

-    where T0 is the start time of the first symbol of the NR uplink channel and Toffset is the preparation procedure time of the NR uplink channel given in subclause 5.3, subclause 5.4, subclause 6.2.1, subclause 6.4 and in subclause 9 of [6, TS 38.213], respectively.

-	The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous 1-port transmission and 2-port transmission on the two uplink carriers.

-	In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

6.1.0.3	Uplink switching for Supplementary Uplink

For a UE not indicating a capability of simultaneousTxSUL-NonSUL for any carrier of a band combination and indicating a capability for uplink switching with [uplinkTxSwitchRequested-r16] for a band combination, and if it is for that band combination configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink:

-	If the UE is configured with uplink switching with parameter [uplinkTxSwitchingPeriod-r16],

-	If the UE is to transmit any NR uplink channel on a different uplink from the last transmission occasion based on DCI(s) received before T0-Toffset or based on a higher layer configurations(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2, where T0 is the start time of the first symbol of the transmission occasion of the NR uplink channel and Toffset is the preparation procedure time of the transmission occasion of the NR uplink channel given in subclause 5.3, subclause 5.4, subclause 6.2.1, subclause 6.4 and in subclause 9 of [6, TS 38.213], respectively. During the switching gap NTx1-Tx2, the UE is not expected to [be scheduled or configured to] transmit on any of the two uplinks.

- 	[When the UE is to transmit a 2-port transmission occasion on a non-SUL carrier, and if the previous uplink transmission occasion was a 1-port transmission, then the UE is not expected to [be scheduled or configured to] transmit for the duration of NTx1-Tx2 on any of the two carriers [immediately before] the 2-port transmission is scheduled or configured to start.]

- 	[When the UE is to transmit a 2-port transmission occasion on a non-SUL carrier followed by a an 1-port uplink transmission occasion on the SUL carrier, then the UE is not expected to [be scheduled or configured to] transmit for the duration of NTx1-Tx2 on any of the two carriers [immediately before] the 1-port transmission is scheduled or configured to start.]

-	In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

## 6.4	UE PUSCH preparation procedure time

If the first uplink symbol in the PUSCH allocation for a transport block, including the DM-RS, as defined by the slot offset K2 and the start and length indicator SLIV of the scheduling DCI and including the effect of the timing advance, is no earlier than at symbol L2, where L2 is defined as the next uplink symbol with its CP starting after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, then the UE shall transmit the transport block.

-	N2 is based on µ of Table 6.4-1 and Table 6.4-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µDL, µUL) resulting with the largest Tproc,2, where the µDL corresponds to the subcarrier spacing of the downlink with which the PDCCH carrying the DCI scheduling the PUSCH was transmitted and µUL corresponds to the subcarrier spacing of the uplink channel with which the PUSCH is to be transmitted, and κ is defined in clause 4.1 of [4, TS 38.211].

-	If the first symbol of the PUSCH allocation consists of DM-RS only, then d2,1 = 0, otherwise d2,1 = 1.

-	If the UE is configured with multiple active component carriers, the first uplink symbol in the PUSCH allocation further includes the effect of timing difference between component carriers as given in [11, TS 38.133].

-	If the scheduling DCI triggered a switch of BWP, d2,2 equals to the switching time as defined in [11, TS 38.133], otherwise d2,2=0.

-	For a UE that supports capability 2 on a given cell, the processing time according to UE processing capability 2 is applied if the high layer parameter processingType2Enabled in PUSCH-ServingCellConfig is configured for the cell and set to enable,

-	If the PUSCH indicated by the DCI is overlapping with one or more PUCCH channels, then the transport block is multiplexed following the procedure in clause 9.2.5 of [6, TS 38.213], otherwise the transport block is transmitted on the PUSCH indicated by the DCI.

-	If uplink switching gap is triggered as defined in subclause 6.1.0, d2,3 equals to the switching gap duration, otherwise d2,3=0.

Otherwise the UE may ignore the scheduling DCI.

The value of  is used both in the case of normal and extended cyclic prefix.

Table 6.4-1: PUSCH preparation time for PUSCH timing capability 1

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |

Table 6.4-2: PUSCH preparation time for PUSCH timing capability 2

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 5 |
| 1 | 5.5 |
| 2 | 11 for frequency range 1 |