3GPP TSG RAN WG1 #101			                                   					       												        R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.6

Source:	Moderator (AT&T)

Title:	Summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-04]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-04] during RAN1 #101-e. According to the Chairman’s Notes:

[101-e-NR-eMIMO-UEFeature-04] Email discussion/approval till 5/29 – Ralf (AT&T) - Finalize all FGs in the 16-4, 16-5, and 16-6 families of features as in x4285

The following was discussed and agreed during RAN1 #101-e within the scope of [101-e-NR-eMIMO-UEFeature-04].

# Summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-04]

This email discussion/approval aims to finalize all FGs in the 16-4, 16-5, and 16-6 families of features. Open issues are highlighted in yellow [1] and summarized in Error: Reference source not found based on the contributions submitted to RAN1 #101-e. Additional aspects can be raised as part of this email discussion.

Companies are asked to prioritize the following:

  - Any change to the number of rows, i.e., deletion of rows, merging of rows, splitting of rows …

  - Any change to a component that impacts signalling design, e.g., because the component requires candidate values to be signalled incl. {enabled, disabled}

  - Any change to the type

  - Any change to xDD/FRx differentiation

  - Any change to whether the gNB needs to know if the feature is supported

  - Any change to whether capability exchange between UEs (V2X only) is applicable

  - Any change to a note that impacts signalling design, e.g., because a component requires candidate values to be signalled incl. {enabled, disabled}

The following should only be discussed if relevant or necessary in order to finalize aspects with ASN.1 impact:

  - Any change to a component that does not impact signalling design

  - Any change to a note that does not impact signalling design

  - Any change to whether a feature group is mandatory or optional

  - Any change to consequences if a feature is not supported by a UE

  - Any change to prerequisite feature groups for a feature

The following tables represent the latest version of the FGs in the 16-4, 16-5, and 16-6 families of features [1]. Companies are invited to provide their views on how to finalize the respective FG in the 16-4, 16-5, and 16-6 families of features in the following tables.

| 16-4 | Low PAPR DMRS for DL | Low PAPR DMRS for PDSCH | FFS |  | N/A |  | [Per UE] | No | No |  |  | [Optional] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| QC | This FG should be “per band”, as a UE may be able to support it in one band, but not in another band. For example, UE can support this feature in licensed band, but not in unlicensed band. This FG should be “optional with capability signaling” The “FFS” on pre-requisite can be removed, as we don’t see the need of any pre-requisite for this FG. |
| ZTE | This feature should be per UE and optional. That’s because low PAPR design for DL DMRS does not impact hardware implementation. So per UE is enough. |
| Spreadtrum | This FG should be optional. |
| Samsung | Suggest to remove FFS on pre-requisite. Support “per band” Support “optional with capability signaling”. |
| vivo | Should be per band, optional with capability signaling |
| Ericsson | The feature is per UE as we don’t see why it has to be per band? Can the proponent explain why it can be supported in one band and not in another. Unclear why unlicensed band have less issue with high PAPR?? The feature must be mandatory with capability signaling. The reason for mandatory is that this is a critical correction of a flaw in Rel.15 specifications, it is not an add-on enhancement. The FFS can be removed. |
| Intel | We do not see the see the relevance of licensed/unlicensed band for this feature. The feature should be “per UE” and “optional with capability signaling” |
| Huawei, HiSilicon | The FG should be per band to leave flexibility to UE implementation, the same understanding with QC. Since the feature is with increasing the complexity on sequence generation and also applicable for different scenarios for different band. |
| Apple | The FG can be “per band” and “optional with capability signaling” |
| OPPO | This FG should be per band and optional with capability signaling |
| LG | For this FG, we prefer the “optional with capability signaling” and “per band” |

| 16-5a | UL full power transmission [mode 0] | - Supported UL full power transmission [mode 0] | 2-13, 2-14 | Yes | N/A | Yes | FFS: Per FS or Per band or Per band per BC | N/A | N/A |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| QC | The square bracket around mode 0 can be removed. This FG need to be “per FS”, i.e. per band per band combination. A simple example is that, due to UE’s PA, a UE can only support full power on one band. Suppose this UE can support UL CA on two bands. Then depends the band combination is band A + band B, or band A + band C, UE may choose to do full power on different band. For example, for band A + band B, UE may choose to support UL full power on band A. While for band A + band C, UE may choose to support full power on band C. This FG should be “optional with capability signaling” |
| ZTE | We do not have any definitions for ‘[mode0]’. So, we prefer to reuse the titles of RRC parameters for all Mode0, 1, 2. ul-FullPowerTransmission-r16 ENUMERATED {fullpower, fullpowerMode1, fullpoweMode2} OPTIONAL -- Need R |
| Spreadtrum | - ‘[mode 0]’ should be removed - This FG is optional |
| Samsung | - Remove bracket around mode 0 |
| vivo | - Removing [mode0] is fine, should be per band per BC |
| Intel | Regarding [mode 0], we suggest the following change to align with RRC parameter UL full power transmission [mode 0] mode of fullpower - Supported UL full power transmission [mode 0] mode of fullpower |
| Huawei, HiSilicon | Aligned with the RRC parameter, the same understanding with ZTE and Intel. |
| Apple | The square bracket should be removed This FG can be “per FS” and “optional with capability signaling” |
| OPPO | Mode 2 can be changed to mode of fullpower (Same name as RRC parameter) This FG should be optional |
| LG | Remove [Mode 0], and prefer to use title of RRC parameter. This FG should be “optional with capability signaling”. |
| NTT DOCOMO | It is better to align with RRC parameter for clarity. Hence Mode0 should be replaced by fullpower (no brackets) |

| 16-5b | UL full power transmission mode 1 | - Supported UL full power transmission mode 1 - [Number of Tx to support mode 1: {2Tx, 4Tx, 2Tx_4Tx}] - [New UL codebook set(s) per supported Tx] | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | No | No |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - Component 2 is not needed given Rel-15 capability indication already - Component 3 should be removed as there is no consensus to include it in RAN1 eMIMO. |
| QC | Same comment as for 16-5a, this FG need to be “per FS”, i.e. per band per band combination. A simple example is that, due to UE’s PA, a UE can only support full power on one band. Suppose this UE can support UL CA on two bands. Then depends the band combination is band A + band B, or band A + band C, UE may choose to do full power on different band. For example, for band A + band B, UE may choose to support UL full power on band A. While for band A + band C, UE may choose to support full power on band C. Support to keep component 2. This FG should be “optional with capability signaling” |
| ZTE | We share the same views with Nokia/NSB that component 2 and 3 should be removed. - The motivation for “16-5b: Number of Tx to support mode 0/1/2: {2Tx, 4Tx, 2Tx_4Tx}” is unclear with the following analysis, and those three components also restrict the flexibility of gNB configuration significantly. - For mode 1, the scaling factor is the same as Rel-15, and only extension of the UL codebook is supported herein. It does not make sense that the extension of codebook is only applied to a specific number of Tx ports. |
| Spreadtrum | This FG should be optional. |
| Samsung | Remove 2Tx_4Tx from component 2 |
| vivo | Component 3 should be removed. 2Tx_4Tx can be kept in component 2, the reason is 4Tx UE may or may not support full power transmission with 2 ports SRS configured |
| Intel | Component 2 is not necessary. Similar view with Nokia Component 3 should be removed. It’s not necessary for Mode 1. |
| Huawei, HiSilicon | Component-2 should be clarified, what’s the meaning. Why there is 2Tx_4Tx? |
| Apple | This FG can be “per FS” and “optional with capability signaling” - Component 2: we need to cover the possibilities of downgrade configuration including {4 port non-coherent, 4 port partial-coherent, 2 port non-coherent}, therefore, component 2 is needed |
| OPPO | This FG should be optional |
| LG | Component 2 and 3 should be removed: Same view with Nokia |
| NTT DOCOMO | It is not clear for us the need of 2Tx_4Tx under component 2. At the moment, we do not see any clear use of it |

| 16-5c | [UL full power transmission mode 2] | - [Supported UL full power transmission mode 2 - Number of Tx to support mode 2: {2Tx, 4Tx, 2Tx_4Tx} - The maximum number of SRS resources in set with different number of ports [for usage set to ‘codebook’]. FFS on details for supported number of Tx. - FFS: Number of ports per SRS resource - FFS: Maximum number of different spatial relation info for all SRS resources for usage set to ‘codebook’ in a resource set - TPMI group which delivers full power. FFS on details for supported number of Tx. Note: UE indicating mode 2 shall support full power transmission for 1 antenna port] | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | No | No |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | One single FG is enough from our point of view, but it might be better to wait for RAN2 feedback before making further decisions. In any case we would like to observe that the note and component 5 are not necessary in our view. |
| QC | Support to split 16-5c to two separate FGs, one for SRS resource related and one for TPMI reporting related, because SRS virtualization and TPMI reporting are two different ways to support mode 2. Also, same comment as for 16-5a and 16-5b, the two split FGs for mode 2 need to be “per FS” The two split FGs for mode 2 should be “optional with capability signaling” |
| ZTE | One single FG is sufficient and can also well handle the feature of UL-Tx-Switching. Some more detailed analysis between Tx mode 2 and UL-Tx-Switching can be found in our contribution R1-2003476. - Component 2 should be removed, since this component is also redundant and similar to Component 2 in FG16-5b, considering that the component “16-5-6: TPMI group which delivers full power” can fully provide the UE capability of which TPMIs can support full power. As a compromise, we can consider how to draft the requirement in component 3 and component 4. - Regarding component 5, We think this component is needed. We can consider the candidate values of “1”, “2” or “4” for this component. - Regarding the case that UE does not support additional TPMI with full power, one entry of “NULL” (i.e., no further TPMI group is supported for full power) can be considered as a candidate value for TPMI group reporting. Consequently, in order to save the FG signaling, the single FG 16-5c seems to be sufficient. |
| Spreadtrum | - Regarding that both virtualization and TPMI group based reporting could achieve full power transmission, from the perspective of UE flexibility, we prefer to split FG 16-5c into two FGs: one for SRS resource related, one for TPMI grouping which delivers full power. - This FG should be optional. |
| Samsung | - Split this FG into two rows with component 6 being a separate row or FG - Remove 2Tx_4Tx from component 2 - Remove FFS from component 3 - Remove component 4 and 5 |
| Vivo | - Although RAN2 feedback is yet to arrive, we propose to split into 2 rows. - Remove FFS part from component 3 - Remove component 4 and 5 - Per band |
| Intel | For component 2, the meaning of 2Tx_4Tx is not clear and should be clarified. Also, the relationship between component 2 and Rel-15 UE capability parameter maxNumberSRS-Ports-PerResource should be clarified. For component 3, the text of ‘with different number of ports’ should be removed to align with the RAN1 agreement. Also, the FFS part in component 3 should be removed. Component 4 is not necessary and can be removed. Component 5 can be removed. Regarding the number of rows for 16-5c, we are fine to have two rows. But it’s better to wait for RAN2 feedback. |
| Huawei, HiSilicon | Support single row on Mode-2. It is clear that Mode-2 is a complete solution for full power transmission, similar as Mode-1. In Mode-2, two components included, one is SRS resources with different number of SRS ports, another is full power TPMIs. For example, UE is partial coherent with 20+20+20+17dBm Pas, then UE may report supporting 2 different SRS resources with different ports, and report full power TPMIs [1 0], and [1 0 0 0; 0 1 0 0]; [1 0 0 0; 0 1 0 0; 0 0 1 0], then gNB schedule rank-1 for 2-port case for full power and higher rank case for 4-port. It is the intention to design the feature. Although, one of the component can be optional reported from UE side, but there are not separate features |
| Apple | This FG can be “per FS” and “optional with capability signaling” - Component 2: we need to cover the possibilities of downgrade configuration including {4 port non-coherent, 4 port partial-coherent, 4 port full-coherent, 2 port non-coherent, 2 port coherent} - For each configuration, UE should be able to flexibly indicates the following 3 components independently - The TPMI bitmap or list that UE supports full power transmission, including the possibility that UE does not support any - The maximum number of SRS resources per SRS resource set, prefer to have value {1, 2, 4} - Whether different number of ports can be configured per SRS resource set - At least the case when 4 ports SRS resource is configured in an SRS resource set, whether 2 ports SRS resource can be configured in the same SRS resource set. |
| OPPO | Support to split the FG into 2 rows Component 4 and 5 are not needed This two sub-FG should be optional with capability signaling |
| LG | Support a single row on Mode-2 with the same view with Huawei. Remove component 4. |
| NTT DOCOMO | It seems a single FG is sufficient. This allows a UE to simultaneously support SRS resources with different no. of ports and to report full power TPMIs. |

| 16-6a | Low PAPR DMRS for PUSCH without transform precoding | - For PUSCH without transform precoding | FFS | Yes | N/A | Yes | FFS: Per band | N/A | N/A |  |  | FFS: Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| QC | This FG should be “per band”, as a UE may be able to support it in one band, but not in another band. For example, UE can support this feature in licensed band, but not in unlicensed band. This FG should be “optional with capability signaling” The “FFS” on pre-requisite can be removed, as we don’t see the need of any pre-requisite for this FG. |
| ZTE | This feature should be per UE and optional. |
| Spreadtrum | This FG should be per band and optional. |
| Samsung | Suggest to remove FFS on pre-requisite. Support “per band” Support “optional with capability signaling”. |
| vivo | Per band, optional with capability signaling |
| Ericsson | The feature is per UE as we don’t see why it has to be per band? Can the proponent explain why it can be supported in one band and not in another. Unclear why unlicensed band have less issue with high PAPR?? The feature must be mandatory with capability signaling. The reason for mandatory is that this is a critical correction of a flaw in Rel.15 specifications, it is not an add-on enhancement. The FFS can be removed. |
| Intel | The feature should be “per band” and “optional with capability signaling”. |
| Huawei, HiSilicon | The feature should be per Band. |
| Apple | The FG can be “per band” and “optional with capability signaling” |
| OPPO | This FG should be per band and optional with capability signaling |
| LG | For this FG, we prefer the “optional with capability signaling” and “per band” |

| 16-6b | Low PAPR DMRS for PUCCH | For PUCCH format 3 and[/or] PUCCH format 4, [if capable,] with transform precoding and with pi/2 BPSK modulation | [FG 1-7, 4-4, 4-5] |  | N/A |  | FFS: Per band | No | No |  |  | FFS: Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| QC | This FG should be “per band”, as a UE may be able to support it in one band, but not in another band. For example, UE can support this feature in licensed band, but not in unlicensed band. This FG should be “optional with capability signaling” Regarding the pre-requisite “[FG 1-7, 4-4, 4-5]”, we don’t see they are necessary and they should be deleted. |
| ZTE | This feature should be per band and optional. That’s because low PAPR DMRS for PUCCH may impact hardware implementation, so per band is better. Suggested wording is For PUCCH format 3 and[/or] PUCCH format 4, [if capable,] with transform precoding and with pi/2 BPSK modulation |
| Spreadtrum | This FG should be per band and optional. |
| Samsung | [/or] and [if capable] can be removed Suggest to remove the pre-requisite. Support “per band” Support “optional with capability signaling”. |
| vivo | Per band, optional with capability signaling |
| Ericsson | The feature is per UE as we don’t see why it has to be per band? Can the proponent explain why it can be supported in one band and not in another. Unclear why PUSCH transmission in unlicensed band have less issue with high PAPR?? The feature can be optional with capability signaling. The reason for optional is that it anyway depends on several other FGs. |
| Intel | We are ok with suggestion from ZTE i.e., [/or] and [if capable] are removed. The feature should be “per band” and “optional with capability signaling”. |
| Huawei, HiSilicon | The FG should be “Per band”. Then, the pre-requisite for 1-7, 4-4, 4-5 should be there, since the use case is restricted in PUCCH format-3/4 and pi/2 BPSK is used. |
| Apple | The FG can be “per band” and “optional with capability signaling” |
| OPPO | This FG should be per band and optional with capability signaling |
| LG | Support “per-band” and “optional with capability signaling” |

| 16-6c | Low PAPR DMRS for PUSCH with transform precoding and with pi/2 BPSK | For PUSCH with transform precoding and with pi/2 BPSK modulation | FFS | Yes | N/A | Yes | FFS: Per band | N/A | N/A |  |  | FFS: Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| QC | This FG should be “per band”, as a UE may be able to support it in one band, but not in another band. For example, UE can support this feature in licensed band, but not in unlicensed band. This FG should be “optional with capability signaling” The “FFS” on pre-requisite can be removed, as we don’t see the need of any pre-requisite for this FG. |
| ZTE | This feature should be per band and optional. |
| Spreadtrum | This FG should be per band and optional. |
| Samsung | Suggest to remove FFS on pre-requisite. Support “per band” Support “optional with capability signaling”. |
| vivo | Per band, optional with capability signaling |
| Ericsson | The feature is per UE as we don’t see why it has to be per band? Can the proponent explain why it can be supported in one band and not in another. Unclear why unlicensed band have less issue with high PAPR?? The feature must be mandatory with capability signaling. The reason for mandatory is that this is a critical correction of a flaw in Rel.15 specifications, it is not an add-on enhancement. The FFS can be removed. |
| Intel | The feature should be “per band” and “optional with capability signaling”. |
| Huawei, HiSilicon | The FG should be per band. The pre-requisite for 1-6 and 2-12 should be added, since the use case for the DMRS is for PUSCH with pi/2 BPSK only. |
| Apple | The FG can be “per band” and “optional with capability signaling” |
| OPPO | This FG should be per band and optional with capability signaling |
| LG | Support “per-band” and “optional with capability signaling” |

# Conclusion

…

# References

- R1-2003194, Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — MIMO Enhancement Aspects, Moderator (AT&T)

- R1-2004285, Summary of UE features for eMIMO, Moderator (AT&T)