3GPP TSG RAN WG1 #101-e	R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.4.2.1

Source:	Ericsson (Moderator)

Title:	TPs for 38.212 for the agreements in [101-e- NR-5G_V2X_NRSL-Mode-1-02]

Document for:	Discussion, Decision

# 1	Introduction

This contribution includes text proposals for 38.213 to implement the following agreements from [101-e- NR-5G_V2X_NRSL-Mode-1-02]. Track changes for TS 38.212 are with respect to the endorsed CRs in [1].

Agreements:

- (working assumption) The resource pool index (if present) is the first information field in DCI format 3_0.

- The size of DCI format 3_0 before alignment with other DCI format sizes is determined based on the configured resource pool resulting the largest DCI size. If necessary, zero padding, added after all the fields, is used to align to this size when other resource pool indices are signaled.

# 2	TP for 38.212

The motivation of the TP is as follows:

- Reasons for change:

  - Implement agreements on size alignment for DCI format 3_0 when multiple transmit pools are configured.

- Summary of changes:

  - Size of DCI format 3_0 is given by the pool configuration resulting in a largest field.

- Specs/Sections impacted: TS 38.212 Clause 7.3.1.4.1

- Consequences if not approved: The size of DCI format 3_0 is not fixed. The UE needs to blind decode multiple options.

< Start of TP for TS 38.212 >

### 7.3.1	DCI formats

< Unchanged parts are omitted >

##### 7.3.1.4.1	Format 3_0

DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell.

The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI:

-	Time gap – [x] bits determined by higher layer parameter sl-DCI-ToSL-Trans, as defined in clause 8.1.2.1 of [6, TS 38.214]

-	HARQ process ID – [x] bits as defined in clause 16.4 of [5, TS 38.213]

-	New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213]

-	Lowest index of the subchannel allocation to the initial transmission –log2(N subChannel SL) bits as defined in clause 8.1.2.2 of [6, TS 38.214]

-	SCI format 1-A fields according to clause 8.3.1.1:

-	Frequency resource assignment.

-	Time resource assignment.

-	PSFCH-to-HARQ feedback timing indicator – 3 bits as defined in clause 16.5 of [5, TS 38.213].-	PUCCH resource indicator – 3 bits as defined in clause 16.5 of [5, TS 38.213].

-	Configuration index – 0 bit if the UE is not configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI; otherwise [x] bits as defined in clause x.x.x of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, this field is reserved for DCI format 3_0 with CRC scrambled by SL-RNTI.

-	Counter sidelink assignment index – [1 or 2] bits

-	2 bits as defined in clause 15.x of [5, TS 38.213] if the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic

-	[1 or 2] bits as defined in clause 15.x of [5, TS 38.213] if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static

-	Padding bits, if required

If multiple transmit resource pools are provided in sl-TxPoolScheduling, the size of DCI format 3_0 is given by a configuration of the transmit resource pool resulting in the largest number of information bits in DCI format 3_0. Zeros shall be appended to the DCI format 3_0 with smaller number of information bits until the payload size is equal to the size of DCI format 3_0.

If the UE is configured to monitor DCI format 3_1 and the number of information bits in DCI format 3_0 is less than the payload of DCI format 3_1, zeros shall be appended to DCI format 3_0 until the payload size equals that of DCI format 3_1.

< End of TP for TS 38.212 >

# References

- R1-2003169, “Corrections on 5G V2X sidelink features after RAN1#100bis-e”, Huawei, RAN1#100bis-e, April 2020.