3GPP TSG-RAN WG4 Meeting # 94-e-Bis 	R4-2005193

Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:	7

Source: 	NTT DOCOMO, INC.

Title: 	RAN4 UE features list for Rel-16

Document for:	Information

- Introduction

This is a document for Rel-16 LTE and NR UE features. Companies are encouraged to check the lists and to provide feedbacks during 94-e-bis meeting if any.

- NR UE feature

  - NR-based access to unlicensed spectrum

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 4-1 | Transmission in intra-carrier guardband | Capability of transmission in the intra-band carrier guardband if the UE is scheduled a contiguous allocation that is wider than a 20MHz subband. | None | yes | no | UE cannot transmit in the guardband, it could only transmit in the subbands | per UE | No | No |  |  | Optional |
|  | [4-x] | [Number of candidate SSB positions to be monitored for 1 SSB index for RLM/BFD/CBD] | For RLM/BFD/CBD, UE capability of monitoring at least N1 candidate SSB positions from the set of SSBs that are QCLed with each other within the set of configured resources. | FFS | FFS |  | FFS | Per UE or per band | No | No |  | Candidate N1 values [1, 2, …] is up to RAN1 (LS R4-2005418) | [Optional] |
|  | [4-y] | [Number of candidate SSB positions to be monitored for 1 SSB index for intra and inter-frequency measurements] | For intra and inter-frequency measurements, UE capability of monitoring at least N2 candidate SSB positions from the set of SSBs that are QCLed with each other within discovery burst transmission window | FFS | FFS |  | FFS | Per UE or per band | No | No |  | Candidate N2 values [1, 2, …] is up to RAN1 (LS R4-2005418) | [Optional] |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | - Propose to add a separate capability to support 25 PRB maximum transmission BW for 60kHz SCS and 20 MHz CBW. The capability can be made mandatory for UEs supporting 60kHz and applicable for NR-U bands only. 2. 4-1: Recommend renaming to “Transmission/Reception in intra-carrier guardband” since the feature is applicable to both TX and RX operation. |
| Ericsson | 4-1: we agree on optionality. Prerequisite groups may be support of (optional) features related to wideband operation. Is this capability linked to other features like simultaneous measurements on multiple LBT bandwidths? |
| Nokia | 4-1: We agree “Capability of transmission in the intra-band carrier guardband if the UE is scheduled a contiguous allocation that is wider than a 20MHz subband” optional. However, for DL we think that reception at the UE also in the intra-band carrier guardband should be mandatory. |
| MTK | 4-1. The Prerequisite feature groups should be wideband operation. Suggest to consider also DL reception. |
| Huawei: | The capability is only applied for NR-U, hence the type of the UE feature should not be per-UE but per-band. - Furthermore, we think the capability should be mandatory for the UE supporting wideband operation. |
| NTT DOCOMO (Moderator) | In our understanding, the detail is still under discussed in WI. To rapporteur, - Please update above list based on the outcome in 1st round and above company’s comments, if needed. Please provide your comments as a rapporteur to each company’s comment or below. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm (Rapporteur) | To Intel: 1.We think that a SU related capability can be discussed after the corresponding discussion in the system parameter thread is concluded. 2. We tentatively added a separate capability for reception. We can discuss further how to handle this. To Ericsson: we changed the pre-requisite to TBD to further discuss this. We would like to check the exact status of that in RAN1 To Nokia: see comment 2. to Intel. We can further discuss mandatory vs. optional To MTK: please see comments to Intel To Huawei: This capability should apply to all bands in which NR-U is supported so we believe per UE is the right applicability. It is obvious that this only applies to NR-U operation. |
| MTK | From GTW RRM session in Tuesday, there is an agreement to add the number of SSB candidate positions to be monitored for RLM/BFD/CBD and intra and inter-frequency measurements. We added 4-x and 4-y as the place holder for now Agreement: - Define the following UE capabilities - For RLM/BFD/CBD UE is required to monitor at least N1 candidate SSB positions from the set of SSBs that are QCLed with each other within the set of configured resources - For intra and inter-frequency measurements UE is required to monitor at least N2 candidate SSB positions from the set of SSBs that are QCLed with each other within SMTC - FFS for the case Q is not provided to the UE - FFS how to handle IDLE mode capabilities - Candidate N1 and N2 values are [1, 2, …] - FFS whether N1 = N2 - FFS whether to have different capabilities for FBE and LBE modes - Send LS to RAN1 to ask for feedback on candidate values N1 and N2 taking into account impact on the overall system performance - Further discuss other cases Nokia will prepare LS |
| Intel | We propose to add a separate capability to support 25 PRB maximum transmission BW for 60kHz SCS and 20 MHz CBW as one of the candidate options for further discussion. The capability can be added in the brackets and removed further if we don’t reach consensus. Also SSB monitoring capabilities N1 and N2 were agreed in the RRM session and shall be included into the list. RAN4 agreements - Define the following UE capabilities - For RLM/BFD/CBD UE is required to monitor at least N1 candidate SSB positions from the set of SSBs that are QCLed with each other within the set of configured resources For intra and inter-frequency measurements UE is required to monitor at least N2 candidate SSB positions from the set of SSBs that are QCLed with each other within SMTC |

  - NR mobility enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5. Mobility Enhancement | 5-1 | Support of intra-frequency DAPS HO | - Support of synchronous DAPS handover for intra-frequency case - Support of asynchronous DAPS handover for intra-frequency case |  | Yes | N/A | The network cannot configure UE with DAPS HO | [Per Band] | No | No | N/A |  | [Optional with capability signalling] |
|  | 5-2 | Support of inter-frequency DAPS HO | - Support of synchronous DAPS handover for inter-frequency case - Support of asynchronous DAPS handover for inter-frequency case |  | Yes | N/A | The network cannot configure UE with DAPS HO | [Per BC] | No | No | N/A | For inter-band combination, only component for inter-frequency case is valid. | Optional with capability signalling |
|  | 5-3 | Support of simultaneous UL transmission | - Support of simultaneous UL transmission for DAPS handover for intra-frequency case - Support of simultaneous UL transmission for DAPS handover for inter-frequency case |  | Yes | N/A | The network cannot configure the UE simultaneous uplink transmission for DAPS handover | [Per BC] | No | No | N/A | For inter-band combination, only component for inter-frequency case is valid. UE can indicate differently for intra-band intra-frequency and intra-band contiguous inter-frequency. | Optional with capability signalling |
|  | 5-4 | Support of multi TAG | - Support of different TAGs in source and target cells for intra-frequency case - Support of different TAGs in source and target cells for inter-frequency case |  | Yes | N/A | The network cannot configure the different TAGs for uplink transmission for DAPS HO | [Per BC] | No | No | N/A | For inter-band combination, only component for inter-frequency case is valid | Optional with capability signalling |
|  | 5-5 | Support of different SCS-s in source and target cells for combination | - Support of different SCS-s in source and target cells for intra-frequency case - Support of different SCS-s in source and target cells for inter-frequency case |  | Yes | N/A | The network cannot configure the different SCS-s in source and target cells | [Per BC] | No | No | N/A | This is valid for intra-band band combination | Optional with capability signalling |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel (rapporteur) | RAN4 already sent LS to RAN2 with the list of relevant RAN4-related capabilities (R4-1915781). A number of modifications and clarifications are suggested on top of the original LS: - Synchronous DAPS HO should be supported as baseline (if the feature is supported). Support of asynchronous DAPS HO can be optionally indicated. - We would like to highlight that for support of asynchronous DAPS HO, UE can indicate differently for intra-band intra-frequency and intra-band contiguous inter-frequency. The reason is under current signaling structure (according to RAN2 discussion) only 1 bit to indicate the support of asynchronous DAPS for intra-band intra-frequency and intra-band contiguous inter-frequency. In fact, RAN4 discussion was general from intra-frequency and inter-frequency perspective. When RAN4 confirmed different capabilities for intra-frequency and inter-frequency, the agreement should also apply to the mentioned scenario. - Different capabilities are needed to indicate support of different SCS in source and target cells for different BC, not only for intra-band. Recommend to modify the capabilities as follows 5-1; DAPS handover; Capability for DAPS HO per combination of bands where simultaneous downlink connectivity with source and target cells are supported: - Support of sSynchronous DAPS HO - Support of vs. asynchronous support for DAPS HO - Support of simultaneous UL transmission to source and target cell per band combination - Support of indication of intra-band (separate signalling for intra-frequency and inter-frequency) HO - Support of multi TAG (i.e., different TAG in source and target cells) - Support of different SCS in source and target cells for DAPS HO in intra-band (separate signalling for intra-frequency and inter-frequency) HO; Yes; N/A; The network cannot configure UE with DAPS HO in corresponding scenarios; [Per BC]; No; N/A; N/A; RAN4 assumes RAN2 will capture these capabilities in their UE feature list discussion, since RAN4 already sent LS (R4-1915781) to RAN2 on these UE capabilities in previous meetings. For support of asynchronous DAPS HO, UE can indicate differently for intra-band intra-frequency and intra-band contiguous inter-frequency. RAN4 assumes synchronous DAPS is supported in case UE supports DAPS HO; [Optional with capability signalling]. |
| Apple | The capability is limited to single CC case. How to extend it to CA/DC is FFS. We also need to check RAN2 feature list to avoid conflicts. The following UE capability should be specified separately. - Sync intra-frequency DAPS (per band) - Async intra-frequency DAPS (ongoing discussion) - Sync inter-frequency DAPS(per band or per band combination) - Async inter-frequency DAPS (per band or per band combination) - Simultaneous UL for sync intra-frequency, inter-frequency (per band or per band combination) - Simultaneous UL for async intra-frequency, inter-frequency (per band or per band combination) - Support of multi TAG - Different numerologies for intra-frequency - Different numerologies for intra-band inter-frequency - Different numerologies for inter-band (i.e. FR1+FR1) |
| MTK | RAN4 can have some suggestion to RAN2, but the final features should be determined in RAN2 Regarding asynchronous intra-freq DAPS, our preference is not to support it in Rel-16, rather than make it an optional feature. |
| Huawei | It seems all the features are put under the same feature group. We would like to consider having separate feature groups. As mentioned in LS [R4-1915781], any UE baseband or RF capability that is per CC, per band, per band combination, or per band of band combination, which is currently used for the purpose of dual connectivity capabilities, need to be signalled and shared between source gNB/eNB part and target gNB/eNB part of the UE. How to handle those capabilities for feature list needs more discussion. Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (V2X WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Mandatory/Optional. 5. Mobility Enhancement; 5-1; Support of sync DAPS HO; - Support of synchronous DAPS handover for intra-frequency case - Support of synchronous DAPS handover for inter-frequency case; Yes; N/A; The network cannot configure UE with DAPS HO; [Per BC]; No; Yes; N/A; For inter-band combination, only component for inter-frequency case is valid; [Optional with capability signalling]. 5-2; Support of sync and async DAPS HO; - Support of synchronous and asynchronous DAPS handover for intra-frequency case - Support of synchronous and asynchronous DAPS handover for inter-frequency case; Yes; N/A; The network cannot configure UE with DAPS HO; [Per BC]; No; Yes; N/A; For inter-band combination, only component for inter-frequency case is valid; [Optional with capability signalling]. 5-3; Support of simultaneous UL transmission; - Support of simultaneous UL transmission for DAPS handover for intra-frequency case - Support of simultaneous UL transmission for DAPS handover for inter-frequency case; Yes; N/A; The network cannot configure the UE simultaneous uplink transmission for DAPS handover; Per BC; No; Yes; N/A; For inter-band combination, only component for inter-frequency case is valid. UE can indicate differently for intra-band intra-frequency and intra-band contiguous inter-frequency.; Optional with capability signalling. 5-4; Support of multi TAG; - Support of different TAGs in source and target cells for intra-frequency case - Support of different TAGs in source and target cells for inter-frequency case; Yes; N/A; The network cannot configure the different TAGs for uplink transmission for DAPS HO; Per BC; No; Yes; N/A; For inter-band combination, only component for inter-frequency case is valid; Optional with capability signalling. 5-5; Support of different SCS-s in source and target cells for intra-band combination; - Support of different SCS-s in source and target cells for intra-frequency case - Support of different SCS-s in source and target cells for inter-frequency case; Yes; N/A; The network cannot configure the different SCS-s in source and target cells; Per BC; No; Yes; N/A; This is valid for intra-band band combination; Optional with capability signalling. 5-X; General UE baseband and RF capability ( NOTE1); - Maximum supported channel bandwidth per CC for source and target cell - MIMO layers per CC for source and target cell - SCS per CC for source and target cell - …; Yes; N/A; The network cannot configure the DAPS handover; Per BC; No; Yes; N/A; NOTE1: As mentioned in LS [R4-1915781], any UE baseband and RF capability that is per CC, per band, per band combination, or per band of band combination, which is currently used for the purpose of dual connectivity capabilities, need to be signalled and shared between source gNB/eNB part and target gNB/eNB part of the UE.; Optional with capability signalling. |
| NTT DOCOMO (Moderator) | To rapporteur, - Please update above list based on the outcome in 1st round and above company’s comments, if needed. - Please provide your comments as a rapporteur to each company’s comment or below |
| Intel (Rapporteur) | To Apple: actually, in current RAN2 discussion not all the capabilities are limited to single CC case, some of the capabilities are put under CA-parameter. Overall, final features should be determined in RAN2. We agree with Apple proposals regarding capability list. To MTK: we share similar view that RAN4 can have some suggestion to RAN2, but the final features should be determined in RAN2, since RAN2 is the leading group and they has already started their work based on previous LS from RAN4. Regarding async intra-frequency DAPS HO, whether to define RRM requirements is still under discussion in RAN4. It seems at least some companies disagree not to have requirement for async case. As rapporteur, we suggest not to revisit previous RAN4 agreement (async is feasible depending UE capability). To Huawei: regarding the 1st comment, we are fine with way Huawei groups the feature. However, in our view RAN4 can at most have some suggestion to RAN2, the final features should be determined in RAN2 regarding how to group the features. As for the 2nd comment on handle capabilities that are currently used for dual connectivity is pure RAN2 issue. We don’t think we should discuss this in RAN4. Since RAN2 already got this information in previous RAN4 LS, we don’t think it is necessary to mention 5-X again in the UE feature list. Regarding the suggested capability list, comments are made directly on the table. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Suggest put all feature types in bracket since it is hard to decide they are per band or per band combination |
| Huawei | We are OK to put all the feature types in []. We would like to respond to Intel for the comment in the 1st round: - We are not sure that RAN4 should group all the features in one feature group and let RAN2 to make decision. We check with RAN1 that in RAN1 they define a separate feature group such that this feature group can be simply disabled or enabled by 1 IoT bit. If we group all the features in one group then there would be misunderstanding in RAN2 all the features are linked to each other and can be disabled or enabled together. - Regarding the 2nd comment, we need check with RAN2 colleagues and would like to come back in the next meeting. According to our understanding of current running CR in RAN2, it seemed they have no idea on how to capture the agreement in the LS since no specific features are given in the LS. We would like to further discuss it in the next meeting In general, we agree that RAN2 should design the signaling. But in our view RAN4 should provide the detailed information about what will be designed. |
| Intel (Rapporteur) | No progress related to UE capability discussion in the 2nd round RRM discussion. To Apple: we are fine to put signalling type into brackets We updated the feature list below accordingly 1) added signalling type into [] and 2) removed note for 5-1 Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (V2X WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 5-1; Support of intra-frequency DAPS HO; - Support of synchronous DAPS handover for intra-frequency case - Support of asynchronous DAPS handover for intra-frequency case; Yes; N/A; The network cannot configure UE with DAPS HO; [Per Band]; No; No; N/A; [Optional with capability signalling]. 5-2; Support of inter-frequency DAPS HO; Support of synchronous DAPS handover for inter-frequency case Support of asynchronous DAPS handover for inter-frequency case; Yes; N/A; The network cannot configure UE with DAPS HO; [Per BC]; No; No; N/A; For inter-band combination, only component for inter-frequency case is valid.. 5-3; Support of simultaneous UL transmission; - Support of simultaneous UL transmission for DAPS handover for intra-frequency case - Support of simultaneous UL transmission for DAPS handover for inter-frequency case; Yes; N/A; The network cannot configure the UE simultaneous uplink transmission for DAPS handover; [Per BC]; No; No; N/A; For inter-band combination, only component for inter-frequency case is valid. UE can indicate differently for intra-band intra-frequency and intra-band contiguous inter-frequency.; Optional with capability signalling. 5-4; Support of multi TAG; - Support of different TAGs in source and target cells for intra-frequency case - Support of different TAGs in source and target cells for inter-frequency case; Yes; N/A; The network cannot configure the different TAGs for uplink transmission for DAPS HO; [Per BC]; No; No; N/A; For inter-band combination, only component for inter-frequency case is valid; Optional with capability signalling. 5-5; Support of different SCS-s in source and target cells for combination; - Support of different SCS-s in source and target cells for intra-frequency case - Support of different SCS-s in source and target cells for inter-frequency case; Yes; N/A; The network cannot configure the different SCS-s in source and target cells; [Per BC]; No; No; N/A; This is valid for intra-band band combination; Optional with capability signalling. |
| NTT DOCOMO (Moderator) | We tried to reflect above list. Please check it. |

  - Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6. LTE_NR_DC_CA_enh | [6-1] | [Interruption in EN-DC and NE-DC due to LTE SCell activation/hibernation] | - Interruptions at direct SCell activation and hibernation in EN-DC - Interruptions at SCell hibernation in EN-DC - Interruptions at direct SCell activation and hibernation in NE-DC - Interruptions at SCell hibernation in NE-DC | Rel-15 euCA | No | N/A | UE implementation complexity. | [Per UE] | No | No | N/A |  | [Optional without capability signaling] |
|  | [6-2] | [Configured output power for NR-DC] | NR-DC within FR1 (FR1-FR1) - Synchronous/Non-synchronous - Semi-static power sharing/Dynamic power sharing - Intra band NR-DC - Inter band NR-DC NR-DC between FR1 and FR2 - Non-synchronous - Independent power control | N/A | Yes | N/A | Impact on gNB Scheduling, UE is not compliant to necessary UE requirements | [Per BC] | No | Yes | Yes |  | [Mandatory for a UE that supports NR-DC within FR1 (FR1-FR1) and/or NR-DC between FR1 and FR2 (non-synchronous)] |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson (Rapporteur) | For early measurements and dormancy, the UE capabilities endorsed in RAN2 are sufficient (see the TP for 38.306 in R2-2001192 and 36.306 in R2-2002335). |
| Intel | - 6-1: RRM requirements for Direct SCell activation are not limited to Interruption requirements. The motivation to focus on interruptions is unclear. All RRM requirements for Direct SCell activation shall be applicable for UEs supporting Direct SCell Activation feature (directSCellActivation-r16) defined by RAN2. Propose to remove 6-1. - 6-2: RF discussions have just started and the decision on features shall be postponed till May. The following is suggested: - Support of semi-static power sharing or dynamic power sharing is optional with capability signaling. UE needs to indicate support of either dynamic power sharing or semi-static power sharing, one or the other, since dynamic power sharing is a more advanced feature. That means if UE supports dynamic power sharing, then semi-static power sharing is not needed. - Separate capabilities for sync/async NR-DC may be considered - Proposal for mandatory feature is unclear. Is it suggested to mandate support of sync/async and different power schemes for all UEs capable of NR-DC within FR1? - RF requirements may not be limited by Configured output power. Suggest to rename the feature group to “NR-DC within FR1” and “NR-DC between FR1 and FR2 for non-sync scenarios” instead |
| Qualcomm | 6-2: semi-static and dynamic power sharing should be different capabilities and should be further split in synchronous and asynchronous. The complexity of supporting these features is totally different. These should be optional, not mandatory. |
| Nokia | 6-1: How does the network know the UE performance if these interruption cases are optional without capability signaling? 6-2: OK |
| MTK | 6-1 is not needed. Although RRM session is now discussing the requirement. In our understanding for each scenario there will be one single requirement that should be followed by UE. In this case, no additional feature or capability is needed. |
| Huawei | 6-1 needs to be clarified. We understand if UE supports the capability it shall meet the requirements defined for the interruptions, otherwise it may not meet the requirements. Therefore, we suggest to describe this capability in the same way as X-5 in section 2.15. |
| NTT DOCOMO (Moderator) | To rapporteur, - Please update above list based on the outcome in 1st round and above company’s comments, if needed. - Please provide your comments as a rapporteur to each company’s comment or below. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | On feature 6-1, interruption across FR can be different from the case within FR. Feature type is FFS |
| MTK | To Nokia: [6-1] There are already several interruption requirements in TS38.133, e.g., due to DRX, due to SCell addion/activation, due to BWP switch. For those requirements, there were no capability to network. In our understanding, network assumes UE is always allowed to cause interruption. |
| Intel | No updates were made after the first round by rapporteur and comments not addressed. Recommend to continue discussion before providing information to RAN2. |
| NTT DOCOMO (Moderator) | Can we add square brackets in 6-1 and 6-2 as above? |

  - NR Positioning Support

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | x-1 |  |  |  |  |  |  |  |  |  |  | RAN4 assumes all UE capabilities are being discussed in RAN1/2. |  |
|  | x-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | … |  |  |  |  |  |  |  |  |  |  |  |  |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel (rapporteur) | Further discussion in RAN4 #94-ebis needed on whether any UE capabilities related to RSTD/UE Rx-Tx timing difference measurement granularity shall be introduced (e.g. supported K values) |
| Apple | Still ongoing discussion and the related feature list discussion should be hold until the conclusion of K. |
| MTK | RRM session is now discussing some options related to measurement gap. One feature could be whether UE can do PRS measurement without gap assistance. The other could be whether UE can support a measurement gap with MGL longer than 6ms. We should wait for the outcome of RRM session. |
| Huawei | RRM discussion for this WI is ongoing, and some UE capabilities may be defined based on the outcome of the discussions. For example, we think it is necessary to define a capability indicating whether UE can measure FR1 PRS layer within serving cell active BWP, which has different SCS than the serving cell BWP, without measurement gap. |
| NTT DOCOMO (Moderator) | In our understanding, the detail is still under discussed in WI. To rapporteur, - Please update above list based on the outcome in 1st round and above company’s comments, if needed. - Please provide your comments as a rapporteur to each company’s comment or below. |
| Intel (Rapporteur) | All these capabilities are candidate capabilities and are still under discussion. We don’t think the situation is stable enough to put these capabilities in the list, including: - K in mapping table, and - New measurement gap, and - UE capabilities regarding support of measuring FR1 PRS layer within serving cell active BWP, which has different SCS than the serving cell BWP, without measurement gap We can wait for conclusions in April meeting and update it accordingly if needed. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| MTK | Agree with Intel that the current discussion is not converged yet to figure out a stable feature list. |
| Intel | No new UE capabilities agreed in the 2nd round and we can continue discussion in the May meeting. |
| Qualcomm | RAN4 has been discussing the need for new MG patterns with longer than 6ms duration for the purpose of positioning for a few meetings. Majority of companies view these new gap patterns necessary for the successful deployment of NR positioning. Introduction of UE capabilities for each new gap pattern or group of gap patterns (e.g., 6 ms < MGL < 10ms associated with one capability and MGL > 10ms associated with another capability) should be discussed. |
| Huawei | Agree that further discussion is needed. |
| NTT DOCOMO (Moderator) | - Keep this table. |

  - Physical layer enhancements for NR URLLC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | y-1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | y-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | … |  |  |  |  |  |  |  |  |  |  |  |  |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | The UE demodulations requirements are currently being defined for the existing Rel-15 UE features. Further discussion on requirements applicability is needed. In case the requirements are defined for Rel-15 UEs then it is important to ensure a mechanism that only UEs compliant with conformance requirements can indicate the respective capabilities. In case the requirements are defined from Rel-16, additional features shall be introduced. Further discussion on requirements applicability for Ultra-low BLER requirements is needed and RAN4 needs to decide whether requirements apply for generic NR devices, Rel-15 NR devices supporting one of the URLLC related features or a new Rel-16 capability is needed. |
| Apple | Agree with Intel’s comments |
| Qualcomm | Further discussion is needed once the demod discussion will get closer to conclusion |
| Huawei | In our view for each URLLC feature there will be capability specified in RAN1/RAN2. It is not clear what feature list RAN4 should provide. RAN4 can discuss the applicability rule based on the specified capability for URLLC for either Rel-15 or Rel-16, and if needed the release independency. But in our view there is no RAN4 new feature list needed. |
| NTT DOCOMO (Moderator) | In our understanding, the detail is still under discussed in WI. To rapporteur, - Please update above list based on the outcome in 1st round and above company’s comments, if needed. - Please provide your comments as a rapporteur to each company’s comment or below. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Remove “No RAN4 specific feature is needed.” on top of the page since it is pending on the ongoing discussion. |
| Intel | Suggest to clarify requirements applicability first for URLLC requirements. To Huawei: Can you share your view on the applicability of Ultra-low BLER requirements (i.e. to which Rel-5 feature it shall correspond to)? |
| Huawei | As rapporteur, we are open to discussion. So far there is no need to update the feature list. We are OK not to remove this feature list. But honestly we have no idea what feature group should be specified. Maybe the applicability rule or the release independent approach can address the issue. |
| NTT DOCOMO (Moderator) | Keep this table. |

  - Enhancements on MIMO for NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | z-1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | z-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | … |  |  |  |  |  |  |  |  |  |  |  |  |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | No RAN4 specific features needed |
| Apple | Although the outcome of a potential study of whether additional signaling is needed to distinguish UEs which are capable of high power classes only in UL MIMO or Tx diversity vs. UEs which are capable of high power classes per antenna port is not currently known, the Rel-16 UE feature list should capture the outcome, if the outcome is to define a new UE capability |
| NTT DOCOMO (Moderator) | In our understanding, the topic as mentioned by Apple is still under discussion. To rapporteur, - Please update above list based on the outcome in 1st round and above company’s comments, if needed. - Please provide your comments as a rapporteur to each company’s comment or below. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| MTK | Agree with Apple that the capability is needed to resolve the ambiguity of HPUE. But we are not sure whether to capture the feature here or somewhere else. |
| Huawei | During the webinar session discussion for NR eMIMO, companies agreed that “SA UE declaring PC2 HPUE shall have 26dBm MOP for both 1TX port transmission and 2TX UL-MIMO (if supported)”, which means that the power class does not distinguish the transmission mode. In that sense, no need to introduce a new power-class capability. |
| NTT DOCOMO (Moderator) | Keep this table. |

  - RF requirements for NR frequency range 1 (FR1)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE-RF | 7-1 | Tx switching period between two uplink carriers | Switching time capability for Tx switching between two uplink carriers in inter-band EN-DC, inter-band UL CA or SUL band combinations |  | Yes | N/A | UE does not support Tx switching between two uplink carriers for inter-band EN-DC, inter-band UL CA and SUL band combinations. | UE signals supported switching period per pair of UL bands per UL band combination | No need | Applicable only to FR1 | Support mixture of FDD/TDD | Candidate value set for UL CA and SUL combinations: {35us, 140 us, 210us} Candidate value set for EN-DC: {35us, 140 us} NOTE: Signalling structure is up to RAN2 | Optional with capability signalling |
|  | 7-2 | [Application of DL interruptions due to UL Tx switching between two uplink carriers] | [Capability to indicate that for the band where DL interruption is needed, the RRM interruption requirements defined in RAN4 shall be applied for duplex mode combinations except the combinations - SUL+TDD - TDD+TDD CA with the same UL-DL pattern - TDD+TDD EN-DC with the same UL-DL pattern] | Tx switching period between two uplink carriers | Yes | N/A | [UE not reporting this capability means DL interruption is not required] | [UE capability is defined as per band per band combination for each band pair supporting UL Tx switching] | FFS | Applicable only to FR1 | Support mixture of FDD/TDD | The capability is introduced according to the agreement in R4-2005665. NOTE: Signalling structure is up to RAN2 | [Optional with capability signalling] |
|  | 7-3 | [UL Separation class List for intra-band non-contiguous CA Note: the maximum UL CC number for intra-band UL CA is 2 in Rel-16, which is agreed in R4-1910273] | [Report maximum UL frequency separation UE can support for each UL transmission chain for intra-band non-contiguous CA Note1: maximum UL frequency separation means maximum frequency span between lower edge of lowest component carrier and upper edge of highest component carrier that UE can support in uplink Note2: separation class set for FR1 depends on decision in RAN4 {UL separation class for UL chain 1, UL separation class for UL chain 2} ] | Intra-band UL non-contiguous CA band combination | Yes | N/A | [Network cannot schedule NC UL CA transmission with single PA architecture] | 3 | No need | FR1 | N/A | It is consensused in WF R4-2005660 that both 1PA and 2PA architecture for intra-band non-contiguous UL CA will be considered for UE capability, and MIMO supporting with 4TX for 2PA UL NC CA should not be excluded. | [Optional with capability signalling, mandatory for UE only support single PA architecture] |
|  | 7-4 | [In-gap ACLR and SEM relax] | [Report whether the in-gap ACLR and/or SEM need to be relaxed for intra-band non-contiguous UL CA {Yes, No} Note: In-gap ACLR requirement depends on RAN4 decision.] | Intra-band UL non-contiguous CA band combination | Yes | N/A |  | 3 | No need | FR1 |  |  | [Optional with capability signalling, Mandatory to support either Yes or No] |
|  | 7-5 | [Transient period] Note: Whether to introduce this feature group depends on RAN4 agreement | FFS |  | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | - Tx switching period between two uplink carriers: The signalling method should be clarified as “UE signals supported switching period per pair of UL bands per UL band combination” - DL interruptions in Tx switching between two uplink carriers: The feature description contradicts to RAN2 recommendations in R2-2002378 (i.e. “Absence of a capability bit shall not imply support for something that is not yet defined”). Having interruptions is not UE feature. Current way of signalling may cause ambiguity and should be streamlined. Propose to define feature as “Support of interruption-less Tx switching” instead. UEs which report such capability shall not cause interruptions. The signalling overhead can be quite big for BCs involving many CCs and further discussion on the simplification of signalling is needed. - UL Separation class List for intra-band non-contiguous CA: We are ok in general. However, more clarification or discussion would be required. Since a single PA architecture has limited (narrower) separation compared to two PAs, it is not clear what does mean by “mandatory for UE only support single PA architecture”. - In-gap ACLR relax: Do not think that there is enough time to define requirements and capability in Rel-16. - Transient period: If introduced, the feature shall be defined as optional |
| Ericsson | [In-gap ACLR relax]: what is the gNB supposed to do with this? |
| China Telecom | Regarding Intel’s comment #2, the consequence if UE report this capability is being discussed in FR1 RF WI, and the UE feature can be updated later based on the agreement in this RAN4 meeting. |
| Skyworks | UL CA non-contiguous: needs 1PA/2PA signaling and instantaneous TX BW class (rather than gap): for example below/above 200MHz. IN gap ACLR relaxation if agreed (at least for 1PA) may not need signaling (1PA/2PA can do this) |
| Nokia | Tx switching period between two uplink carriers: LS to RAN2 already sent. Follow the LS agreement. [DL interruptions in Tx switching between two uplink carriers]: DL interruption has not been agreed. Thus, we cannot agree on this capability. [Transient period]: RAN#86 already agreed that RAN4 will only discuss testability aspects not about this capability. This capability should be introduced. |
| MTK | [Tx switching period between two uplink carriers] We assume that if UE does not report this capability, it means that UE does not support this feature, rather than UE support a default value for switching period [Intel on DL interruptions in Tx switching between two uplink carriers]: “per DL band within supported band combination per UL band pair per band combination” could be a very large burden in RAN2 to introduce the signaling. It is better to work on some simplification. [UL Separation class List for intra-band non-contiguous CA] The requirement is not yet clear at this moment. OK to have a placeholder for now. Also not clear why it is mandatory for single PA. [In-gap ACLR relax] Requirement is not clear at this moment. |
| Huawei | To Intel second comment, our solution would be - Specify the RRM downlink interruption requirements caused by uplink switching - Specify the signaling to indicate whether the requirement above applies to UE. - For transient period, more discussion is needed before agreeing on the feature group. Adding in 2nd round: Record the consensus making in R4-2005660 as one note for reference. |
| NTT DOCOMO (Moderator) | We think some features are still under discussion in WI. To rapporteur, - Please update above list based on the outcome in 1st round and above company’s comments, if needed. - Please provide your comments as a rapporteur to each company’s comment or below. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | No updates were made after the first round by rapporteur. Recommend to put feature groups into brackets before providing response to RAN2. DL interruptions in Tx switching between two uplink carriers: The capability is needed for duplexer mode combination except the SUL+TDD, TDD+TDD CA with the same UL-DL pattern, and TDD+TDD EN-DC with the same UL-DL pattern which are not requiring DL interruption. The UE capability is defined as per band per band combination for each band pair supporting UL Tx switching. |
| Huawei | - Add the modification for feature group 7-3 UL Separation class List for intra-band non-contiguous CA in 2nd round: Record the consensus making in R4-2005660 as one note for reference. - Add the modification for new capability according to the agreement in R4-2005665 for UE capability on DL interruption for UL Tx switching. All the contents are in [ ] for further checking. |
| NTT DOCOMO (Moderator) | Please check Huawei’s update. |

  - NR RF requirement enhancements for frequency range 2 (FR2)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR RF Requirement Enhancements for FR2 | [8-1] | [MPE] | 1 P-MPR reporting 2 FFS if other reporting is defined |  | yes |  |  |  | TDD only | FR2 only | N/A |  |  |
|  | [8-2] | [Beam correspondence] | [1 SSB based BC 2 CSI-RS based BC] |  | yes |  |  |  | TDD only | FR2 only | N/A |  |  |
|  | [8-3] | [Non-contiguous intra-DL CA] | 1 FFS RAN4 is also considering not having this feature, and enhancing existing R15 capability instead |  | yes |  |  |  | TDD only | FR2 only | N/A |  |  |
|  | [8-4] | [Non-contiguous intra-band UL CA] | 1 FFS, RAN4 is also considering not having this feature, and enhancing existing R15 capability instead |  | yes |  |  |  | TDD only | FR2 only | N/A |  |  |
|  | [8-5] | [Inter-band DL CA] | 1 FFS |  | yes |  |  |  | TDD only | FR2 only | N/A |  |  |
|  | [8-6] | [MPR Enhancement] | 1 zero dB MPR region extension |  | yes |  |  |  | TDD only | FR2 only | N/A |  |  |
|  |  |  | 2 FFS if Tx power is boosted when IBE is suspended |  | yes |  |  |  | TDD only | FR2 only | N/A |  |  |
|  | [8-7] | [Spherical coverage] | 1 FFS |  | FFS |  |  |  | TDD only | FR2 only | N/A |  |  |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | - MPE: Agree with feature. Components besides P-MPR reporting need further discussion in RAN4 94e-bis. Per-UE reporting granularity is suggested. - Beam correspondence: Prefer to define separate capabilities for SSB only based BC and CSI-RS based BC. Per-UE reporting granularity is suggested. - Non-contiguous intra-band DL CA: The feature name should be corrected from “intra-DL” to “intra-band DL”. 1) CA DL-only extension is optional with capability signaling since UE has freedom to not have DL-only spectrum. Further discussion on signalling of DL only extension is needed. - Non-contiguous intra-band UL CA: Support of non-contiguous UL CA requires RAN1/2 impact and has RRM impact. Based on current progress we do not think it is feasible to fit in Rel-16. Propose to remove the feature from the list. - Inter-band DL CA: Further discussion is needed on whether separate capabilities for common/independent beam are needed (e.g. for 28+28) or a single capability with independent beam shall always be applied for FR2 inter-band DL CA. In case multiple capabilities shall be defined, then UE needs to report whether a common / independent beams are used per a pair of bands in a CA BC. The decision on the support of common beams requirements shall be made in RAN4 94-ebis to ensure that corresponding RRM requirements can be introduced. Otherwise, the requirements for independent beams shall be defined only. - MPR enhancement: - 0 dB MPR improvement: Agree - Tx power boosting when IBE is suspended: Further discussion is needed whether the feature shall be introduced - Spherical coverage: So far, there is no decision to modify the requirement. Proposed to remove the feature. Discussion can continue in the next release. |
| Qualcomm | More discussion on all these capabilities is needed since WI has not advanced enough to have clear agreements MPR Enhancement with IBE relaxation, our understanding is that it was agree to introduce this, some details are still under discussion |
| Ericsson | MPE: P-MPR reporting only (optional) |
| Apple | The discussion on whether to discuss capability reporting for enhanced BC is ongoing this meeting: if RAN4 agress to treat the capability reporting independently, then two separate components (for SSB based and CSI-RS based) are needed. Furthermore, RAN4 is still discussing the feasibility of the SSB based BC, and the introduction of the related signaling is pending on that outcome. Suggested wording to capture these components is provided with tracked changes in the table above. The existing capability reporting for frequency separation class needs to be updated to refer to the latest version of the specification, since Fs was enhanced. A new component is proposed to be introduced to capture the agreement on the DL-only frequency separation class, Fsd. Suggested wording to capture these components is provided with tracked changes in the table above. Discussion is ongoing whether to confirm the feasibility of the non-simultaneous uplink feature for non-contiguous UL CA in FR2. Depending on its outcome, the related capability signaling is needed. Suggested wording to capture these components is provided with tracked changes in the table above. MBR is a set of manufacturer declarations which are not signaling. These should be captured in the feature list for clarity. Suggested wording to capture these components is provided with tracked changes in the table above. Regarding the components proposed by the moderator, we have the following comments: Non-contiguous intra-DL CA: this feature group is already defined in Rel-15, and in Rel-16 we have approved an extension of the value range; we should update the value range in the same component Non-contiguous intra-band UL CA: this feature group is already defined in Rel-15, and in Rel-16 we have approved an extension of the value range; we should update the value range in the same component Inter-band DL CA: How is this parameter different from the already existing signaling on supported band combination lists? Is this capability necessary? Spherical coverage: this capability is not needed, since RAN4 has not agreed to introduce any modification of spherical coverage requirement to PC3 (or any other PC) DOCOMO (Moderator) We think following table is provided by Apple, right? At this point, we would like to remove this table from above, and keep it in this comment box. I hope rapporteur takes this into account Beam correspondence; [1) UE is capable of a beam refinement procedure based only on the SSB reference signal (FFS whether this component will be introduced)] 2) UE is capable of a beam refinement procedure based only on the CSI reference signal; 2-20; Yes; N/A; UE is not capable of enhanced beam refinement procedures; Type 1 (per UE); N/A; Applicable to FR2 only; N/A; Optional with capability signaling. Non-contiguous intra-band CA frequency separation class for FR2; 1) Support of frequency separation classes to handle the total frequency span for DL for intra-band non-contiguous CA as specified in TS38.101-2 2) Support of frequency separation classes to handle the total frequency span for UL for intra-band non-contiguous CA as specified in TS38.101-2 3) Support of Frequency separation class for DL-only frequency separation span of DL CCs that UE can support per band. The spectrum covered by the DL-only frequency separation extends on one-side relative to the bidirectional spectrum. The sum value of the frequency separation (Fs) and frequency separation class for DL-only spectrum (Fsd) cannot exceed 2400 MHz; Yes; N/A; UE is not capable of supporting non-contiguous carrier aggregation configurations; Type 3 (per band per band combination); N/A; Applicable to FR2 only; N/A; Mandatory to support a frequency separation class within {I, II, III} specified in TS38.101-2 with capability if UE supports non-contiguous CA in FR2. Non-simultaneous UL; 1) UE is capable of non-simultaneous uplink in non-contiguous sub-blocks of an UL intra-band CA configuration; Yes; N/A; UE can support simultaneous uplink on non-contiguous UL sub-blocks; Type 3 (per band per band combination); N/A; Applicable to FR2 only; N/A; Optional with capability signaling. Multi-band relaxation framework; 1) Total multi-band relaxation for peak EIRP and peak EIS (Table 6.2.1.3-4 in TS38.101-2) 2) Total multi-band relaxation for spherical coverage EIRP and spherical coverage EIS(Table 6.2.1.3-4 in TS38.101-2); No; N/A; N/A; Type 2 (per band); N/A; Applicable to FR2 only; N/A; This is a manufacturer declaration during certification and is not a signaled capability. For the UEs that support multiple FR2 bands, minimum requirement for peak EIRP and EIRP spherical coverage in Tables 6.2.1.3-1 and 6.2.1.3-3 shall be decreased per band, respectively, by the peak EIRP relaxation parameter ∆MBP,n and EIRP spherical coverage relaxation parameter ∆MBS,n. For each combination of supported bands ΔMBP,n and ΔMBS,n apply to each supported band n, such that the total relaxations, ∑MBP and ∑MBS, across all supported bands shall not exceed the total value indicated in Table 6.2.1.3-4.. Tx power boost with IBE relaxation; UE is capable of boosting Tx power by relaxing IBE under network control; Yes; N/A; Power boost is not supported; Type 2(per band); N/A; FR2 only; N/A; Optional with capability signaling. |
| Samsung: | On beam correspondence, given the discussion on requirements is still ongoing, suggest to remove this feature or leave the component as FFS until clear requirements are defined. |
| Skyworks | 0 dB MPR improvement: Agree for mandatory |
| Nokia: | Agree on the proposed UE capability. Decide FFS cases once the corresponding decisions are made under the WID |
| MTK | MPE: Type 1 Beam correspondence: In our understanding, this is the enhancement on testing method rather than new UE features. The basic functionalities were already developed in R15. Suggest to remove this. FFS DL only extension for Non-contiguous intra-band DL CA Inter-band DL CA: Suggest to add a feature for common beam and independent beam implementation. Spherical coverage: the requirement is not clear at this moment. |
| Huawei | - MPE: - PMPR specific value was agreed to be indicated with PHR via MAC CE, in our understanding it does not need report as UE capability. - The reference PCMAX which is under discussion to indicate by UE capability, where reference PCMAX means the PCMAX without adding MPR, AMPR and PMPR. gNB can utilize it to get the PMPR status without event triggering report on PMPR. - Beam correspondence: In RAN#86 meeting, WF was agreed that no new signaling will be considered, the enhanced BC will be based on Rel-15 feature. - DL only is an optional capability, the UE type have no limitation on DL-only should be also reported. Furthermore, since two-sided has been dropped in Rel-16, the DLseparation can be reported per RF chain which explicitly reflect what the UE can support. - UL separation class should be reported separately, UL and DL capability can be different. In rel-16, we already agree that UL separation class is confined within 1400MHz. We propose to define UL separation class level different from DL, the list can be {400, 600,800,1000, 1200,1400} with 200MHz granularity. - In our understanding, 2 UE capabilities are to be defined for FR2 inter-band CA: - whether UE support non-collocated deployment, it can be reflected as: - Type 1: support MRTD=8us and independent beam management, PSD difference is [30]dB - Type 2: MRTD=[TBD], common beam management and max PSD difference≤ [6]dB,where MRTD depends on RAN4 discussion - Which RF requirement UE can support - Bit 0: Spherical coverage for each band determinedly separately without common range definition - Bit 1: common EIS spherical coverage range between the two bands shall be 50% for power class 3 - MPR enhancement: Feasibility of IBE suspended power boost is still under discussion under RF session, we propose to remove it since power boost is not sensitive to IBE. |
| NTT DOCOMO (Moderator) | We think some features are still under discussion in WI. To rapporteur, - Please update above list based on the outcome in 1st round and above company’s comments, if needed. - Please provide your comments as a rapporteur to each company’s comment or below. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | We added Tx power boost with IBE relaxation based on agreements in R4-1912975. |
| Intel | No updates were made after the first round by rapporteur. The current table includes duplicated features. Recommend to continue RAN4 discussion and put feature groups into brackets before providing a response to RAN2. Some comments on specific features: - Beam correspondence: SSB based beam correspondence is optional. - Non-contiguous intra-band CA frequency separation class for FR2: DL and UL in bidirectional spectrum are mandated and one an applicable frequency separation class value should be selected from 38.101-2 for DL and UL separately. DL-only spectrum is optional. If UE supports, then UE needs to select an applicable frequency separation class from 38.101-2 - Non-simultaneous UL: the feature status is unclear and cannot be added unless all requirements are finalize - Multi-band relaxation: For Rel-16, framework discussions led to the removal of the total relaxations ∑MBP and ∑MBS in Table 6.2.1.3-4 (table is now based on ΔMBP,n and ΔMBS,n). The wording in the Note column should be updated to reflect this. |
| NTT DOCOMO (Moderator) | It is very difficult to merge above list. Can we add square brackets to all features? We move The list provided by Apple to 1st comment box. |

  - NR RRM requirement enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9. Rel-16 NR RRM Enhancement | 9-1 | BWP switching on multiple CCs | 1) Support of BWP switching on multiple CCs. |  | Yes | N/A | Network should not trigger BWP switching on multiple CC simultaneously. Otherwise there may be additional unclear BWP switching delay. | [Per UE] | No | [No] | N/A |  | [Optional with capability signalling] |
|  | 9-2 | Mandatory gap pattern] | 1) Support of additional mandatory gap patterns for NR-only measurements in NR SA and NR DC, 2) Support of mandatory additional gap patterns defined for NR SA and NR-DC for NR measurement only in LTE SA, EN-DC, NE-DC |  | Yes | N/A | Network cannot configure corresponding gap patterns for the UE. | Per UE | No | No | N/A | Note: Agreements are provided in [R4-2005346]. According to RAN4 agreement, a bitmap should be introduced for 1). A single bit should be introduced for 2) | [Mandatory with capability for component 1] [Optional with capability for component 2] |
|  | 9-3 | Inter-frequency measurement without measurement gap | 1) Support of inter-frequency measurement without MG when the inter-frequency SSB is completely contained in the active DL BWP of the UE [2) support of SSB based measurement on inter-frequency without MG and data reception of PDCCH/PDSCH in serving with different SCS] |  | Yes | N/A | 1] gNB has to configure measurement gap for inter-frequency measurement [2) UE cannot support of SSB based measurement on inter-frequency without MG and data reception of PDCCH/PDSCH in serving with different SCS] | Per UE | No | [No/Yes] | N/A | Details of 2) can be found in RAN4 LS R4-2005350 to RAN2, wherein two options are listed, i.e.1) update existing IE (simultaneousRxDataSSB-DiffNumerology); 2) introduce a new UE capability | FFS |
|  | 9-4 | [CGI reading] | 1) Support of autonomous gap-based CGI reading of an NR neighbour cell for EN-DC, NR SA, LTE SA, NR-DC, NE-DC 2) Support of autonomous gap-based CGI reading of an E-UTRA neighbour cell for EN-DC, NR SA, LTE SA, NR-DC, NE-DC |  | Yes | N/A | [gNB cannot configure CGI reading of neighbor cell] | Per UE | No | Yes | N/A | RAN2 already specified UE capability for LTE SA, EN-DC and NR SA scenarios. However CGI reading of NR and E-UTRA neighbor cell in NR-DC and NE-DC scenarios are also supported from RAN4 RRM requirements perspective. Need to discuss if UE capability for NE-DC and NR-DC are to be introduced. | Optional with capability signalling |
|  | 9-5 | [SRS carrier switching] | 1) Support of SRS carrier switching RRM requirements |  | Yes | N/A | [Network cannot know the SRS carrier switching delay and corresponding interruption length for this UE. Therefore, either network may not trigger SRS carrier switch or there will be performance degradation] | Per UE | No | [No/Yes] | N/A | Functionality of SRS carrier switching has already been supported since R15. RRM requirement is expected to be introduced in R16. Thus, R16 UE shall meet corresponding RRM requirement. The requirements apply when SRS carrier switching is on NR carrier or on LTE carrier i.e. it should cover also EN-DC and NE-DC scenarios. | [Mandatory with capability signalling for Rel-16 UEs supporting SRS carrier switching] |
|  | 9-6 | [Multiple SCell activation] | 1) Support of multiple SCell activation RRM requirement |  | Yes | N/A | [Network cannot know the multiple SCell activation delay and corresponding interruption length for this UE. Therefore, either network may not trigger multiple SCell activation or there will be performance degradation] | Per UE | No | [No/Yes] | N/A | Functionality of multiple SCell activation has already been supported since R15. RRM requirement is expected to be introduced in R16. Thus, R16 UE shall meet corresponding RRM requirement. | [Optional with capability signalling] |
|  | 9-7 | [UE specific CBW change] | 1) Support of UE-specific CBW change RRM requirement |  | Yes | N/A | [Network cannot know the UE specific CBW change delay and corresponding interruption length for this UE. There will be performance degradation when UE specific CBW changes] | Per UE | No | No | N/A | Functionality of UE specific CBW change has already been supported since R15. RRM requirement is expected to be introduced in R16. Thus, R16 UE shall meet corresponding RRM requirement. | [Optional with capability signalling] |
|  | 9-8 | [Spatial relation switch for uplink] | 1) Support of UL spatial relation switch RRM requirement |  | Yes | N/A | [Network cannot know the uplink spatial relation switch delay for this UE. There will be performance degradation when uplink spatial relation changes] | Per UE | No | No | N/A | Functionality of uplink spatial relation change has already been supported since R15. RRM requirement is expected to be introduced in R16. Thus, R16 UE shall meet corresponding RRM requirement. | [Optional with capability signalling ] |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | X-0: To allow early Rel-16 UE implementation, it is better to keep corresponding RRM requirement optional since UE cannot disable the feature if it cannot meet the requirements, since the feature itself was mandatorily supported in R15 while there was no requirement in R15. X-1: according to RAN4 discussion, a potential new UE capability regarding “NR only measurement gap pattern” may be needed. Although the additional mandatory GP should be mandatorily supported, the new capability for “NR only measurement” can be optional with capability signaling. New mandatory GP can be implemented by updating corresponding description in 38.306. if UE doesn’t indicate new capability for “NR only measurement”, network would consider these GP can be used for both NR and LTE measurement by default. X-4: RAN2 already specified UE capability for LTE SA, EN-DC and NR SA scenarios. In general, separate capabilities can be defined for NR-DC and NE-DC. Alternative approach is to avoid differentiating the capabilities and define a single capability for NR and LTE neighbor cell CGI reading. X-5: It is proposed to define the requirements as mandatory for Rel-16 UE which support SRS carrier switching feature. X-0; BWP switching on multiple CCs; 1) Support of RRM requirements for BWP switching on multiple CCs (FFS) 2) Number of CCs K1 that can be processed simultaneously when DCI based BWP switching is triggered on multiple CCs simultaneously. (FFS) 3) Number of CCs K2 that can be processed simultaneously when timer-based BWP switching is triggered on multiple CCs simultaneously. (FFS) 4) Number of CCs K3 that can be processed simultaneously when RRC based BWP switching is triggered on multiple CCs simultaneously. (FFS); Yes; N/A; Network should not trigger BWP switching on multiple CC simultaneously. Otherwise there may be additional unclear BWP switching delay.; [Per UE]; No; [No]; N/A; K1, K2 and K3 are still under discussion.; Optional with capability signalling. X-1; Mandatory gap pattern]; 1) Support of NR only measurement gap patterns; Yes; N/A; Network cannot configure corresponding gap patterns for the UE.; Per UE; No; No; N/A; Additional MG patterns will be required to be supported as mandatory. UE can also indicate if some of MG are supported for NR only measurements.; [Optional with capability signalling]. X-3; Inter-frequency measurement without measurement gap; 1) Support of inter-frequency measurement without MG when the inter-frequency SSB is completely contained in the DL active BWP of the UE; Yes; N/A; [gNB has to configure measurement gap for inter-frequency measurement]; Per UE; No; Yes; N/A; It’s still under discussing in RAN4, and will be decided in this April meeting.; FFS. X-4; [CGI reading]; 1) Support of autonomous gap based CGI reading of an NR neighbour cell for EN-DC, NR SA, LTE SA, NR-DC, NE-DC 2) Support of autonomous gap based CGI reading of an E-UTRA neighbour cell for EN-DC, NR SA, LTE SA, NR-DC, NE-DC; Yes; N/A; [gNB cannot configure CGI reading of neighbor cell]; Per UE; No; Yes; N/A; Optional with capability signalling. X-5; [SRS carrier switching]; 1) Support of SRS carrier switching RRM requirements; Yes; N/A; [Network cannot know the SRS carrier switching delay and corresponding interruption length for this UE. Therefore, either network may not trigger SRS carrier switch or there will be performance degradation]; Per UE; No; No; N/A; Functionality of SRS carrier switching has already been supported since R15. RRM requirement is expected to be introduced in R16. Thus, R16 UE shall meet corresponding RRM requirement.; Mandatory for Rel-16 UEs supporting SRS carrier switching. X-6; [Multiple SCell activation]; 1) Support of multiple SCell activation RRM requirement; Yes; N/A; [Network cannot know the multiple SCell activation delay and corresponding interruption length for this UE. Therefore, either network may not trigger multiple SCell activation or there will be performance degradation]; Per UE; No; No; N/A; Functionality of multiple SCell activation has already been supported since R15. RRM requirement is expected to be introduced in R16. Thus, R16 UE shall meet corresponding RRM requirement.; Optional with capability signalling. X-7; [UE specific CBW change]; 1) Support of UE-specific CBW change RRM requirement; Yes; N/A; [Network cannot know the UE specific CBW change delay and corresponding interruption length for this UE. There will be performance degradation when UE specific CBW changes]; Per UE; No; No; N/A; Functionality of UE specific CBW change has already been supported since R15. RRM requirement is expected to be introduced in R16. Thus, R16 UE shall meet corresponding RRM requirement.; Optional with capability signalling. X-8; [Spatial relation switch for uplink]; 1) Support of UL spatial relation switch RRM requirement; Yes; N/A; [Network cannot know the uplink spatial relation switch delay for this UE. There will be performance degradation when uplink spatial relation changes]; Per UE; No; No; N/A; Functionality of uplink spatial relation change has already been supported since R15. RRM requirement is expected to be introduced in R16. Thus, R16 UE shall meet corresponding RRM requirement.; Optional with capability signalling. |
| Apple | X-0: we need to investigate if UE should indicate the same value of k for different band combinations, e.g. intra-band CA vs. inter-band CA. Also, the need of “Support of RRM requirements for BWP switching on multiple CCs” should be FFS since BWP switching feature has been specified in Rel-15. UE’s capability to handle multiple BWP switching can be indicated by the value of “k” Intel: according to the 1st round discussion in #121, majority view is K=1 for DCI based BWP switch. Some companies also propose the same for RRC based BWP switch. If K=1 then corresponding capability is not needed. It is suggested to come back later in this meeting. X-1: we need to clarify the capability bit is used to indicate the new mandatory gap is supported for either NR MO only or NR+LTE MO. Intel: same understanding. Details are still under discussion in #122. We can update later if there is any agreement. X-3: this feature should be indicated per FR. In FR2, scheduling restriction is always assumed for intra-frequency measurement. That means UE cannot receive data while measuring SSB. In this case, UE may choose to reduce the bandwidth only to cover SSB. Consequently, UE cannot support inter-frequency measurement without gap. Situation for FR1 is different since it is possible for simultaneous data and SSB reception, UE has motivation to keep the bandwidth to cover whole BWP all the time. Intel: OK with the feature to be indicated per FR. X-8:Propose to remove it since it has been speified from R15 already. In R16 WI, only the requirements are specified and it is not used to differentiate capability of R16 and R15 UE on this feature. Intel: the situation is similar with BWP switching on multiple CC and multiple SCell activation. We prefer to handle them in the same way. Based on our understanding we have several options to handle this: for functionalities which have already been supported in R15 w/o RRM requirement, RAN4 is to 1) introduce R16 requirement and R16 optional capability 2) introduce R16 requirement and R16 mandatory capability 3) introduce R16 requirement w/o UE capability 4) introduce R16 requirement and release independent capability. We are open to further discuss and would like to collect further comments in the 2nd round. |
| Qualcomm | X-5, X-6, X-7: What does mandatory with capability signaling mean? These features were already defined in Rel.15, some are optional or supported without any capability. For example, X-7 is supported impicilily otherwise UE will not work. Intel: X-5, SRS carrier switch is an optional feature in R15 without RRM requirement. RRM requirement is being introduced in R16. A R16 UE which support this optional feature shall meet corresponding RRM requirement. X-6 and X-7 have been changed to optional with capability signaling in our 1st comment. |
| Ericsson | X-1: Component clarified: 1) Support of additional mandatory gap patterns for NR-only measurements. Intel: same understanding. Details are still under discussion in #122. We can update later if there is any agreement X-5: Elaborated the note: The requirements apply when SRS carrier switching is on NR carrier or on LTE carrier i.e. it should cover also EN-DC and NE-DC scenarios. Intel: requirement applicability can be specified in TS38.133. From UE capability perspective, UE only needs to indicate if it can meet corresponding RRM requirement (including both EN-DC and NE-DC). |
| Nokia | Mandatory gap pattern measurement should be mandatory. Intel: agree that the new mandatory gap pattern should be mandatory. It can be achieved by updating related statement in TS38.306. However, X-1 is a new UE capability regarding support of “NR only MO”. This new IE can be optional. If a UE doesn’t report this, network would consider the gap can be used for both NR and LTE measurement. |
| MTK | X-0: Suggest to delete this feature. This functionality is already developed in R15. RRM session is only working on its corresponding requirement. Unless RRM session can not conclude on a single requirement for each scenario. Otherwise, we do not see the need of this feature. Intel: the feature was developed in R15 without RRM requirement. RRM requirement are being developed in R16. Different network behaviours can be foreseen for UE can or cannot meet corresponding RRM requirement. thus we think it is helpful to introduce this capability to let network know if UE can meet RRM requirement. however, we are open to discuss how to handle this issue. X-1: RRM session discussion is on-going. We expect some clarification such as for NR-only measurement and whether this mandatory gap also applicable to E-UTRAN. Intel: agree. Details are still under discussion in #122. We can come back later if there is any agreement. X-3: In the Components column, it should mention active DL BWP. Whether it is mandatory or optional is still FFS at this moment. Intel: agree. X-4: This should be updated as autonomous gap based CGI reading. In our understanding, DRX-based CGI reading was already introduced by RAN2 in R15. Intel: agree X-5: The functionality was already defined in R15. RRM session is only working its corresponding requirement in R16 time frame. Suggest to delete this feature. Intel: same response under X-0. X-6: The functionality was already defined in R15. RRM session is only working its corresponding requirement in R16 time frame. Suggest to delete this feature. Intel: same response under X-0. X-7: The functionality was already defined in R15. RRM session is only working its corresponding requirement in R16 time frame. Suggest to delete this feature. Intel: same response under X-0. X-8: The functionality was already defined in R15. RRM session is only working its corresponding requirement in R16 time frame. Suggest to delete this feature. Intel: same response under X-0. |
| Huawei | X-0 (2-4) seems not needed. For BWP switching on multiple CCs, we suggest not to define and report the number of CCs that can be processed simultaneously (K1, K2 and K3). The requirement is still under discussion in RAN4, and it is more reasonable to define the requirements based on K = 1 with incremental time normalized to one CC. In this way, we will have one unified requirements; otherwise, there will be multiple requirements for different combinations of K and D values. Intel: issues are being discussed in #121. If K=1, then we agree that related capability is not needed. X-4 seems not needed, as RAN2 has already defined the related capabilities, as indicated in R2-2002219. The introduction of capability for CGI reading under NE-DC and NR-DC should also be up to RAN2. Intel: it is fine if RAN2 has already cover all the scenarios. However, we see no harm to provide RAN4 agreement to RAN2, just to avoid any missing of scenarios. |
| NTT DOCOMO (Moderator) | We think some features are still under discussion in WI. To rapporteur, - Please update above list based on the outcome in 1st round and above company’s comments, if needed. - Please provide your comments as a rapporteur to each company’s comment or below. |
| Intel (Rapporteur) | Recommend the scope of the 2nd round discussion: According to the 1st round discussion, most comments are on the functionalities which have already been supported in R15 while there is no RRM requirement in R15. We see several options to move forward and would like collect views from companies. Companies are encouraged to provide views on the following options (other options are also welcome): 1) introduce R16 requirement and R16 optional capability 2) introduce R16 requirement and R16 mandatory capability 3) introduce R16 requirement w/o UE capability 4) introduce R16 requirement and release independent capability. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | 9-1: since we have agreed k=1, we can remove 2), 3) and 4) 9-2: it seems LTE/EN-DC and NR SA/DC have different signaling format. Further discussion is needed to define a single or two features to reflect this 9-3: as commented in 1st round, we propose “yes” to the column “Need of FR1/FR2 differentiation” 9-5: we propose “yes” to the column “Need of FR1/FR2 differentiation” 9-6: we propose “yes” to the column “Need of FR1/FR2 differentiation” 9-8: since this is a rel-15 feature, do we need to include it? |
| MTK | 9-2: The agreement in Wed RRM GTW session is Agreement Support of “mandatory MG patterns” for NR measurement only for NR SA and NR DC is mandatory feature with capability signalling. UE capability shall be indicated for each gap pattern. Agreement For LTE SA, EN-DC, NE-DC Support of a set of “mandatory additional gap patterns defined for NR SA and NR-DC” for NR measurement only is an optional UE capability Introduce a new UE capability signalling where UE may indicate the support for a full set of “mandatory additional gap patterns defined for NR SA and NR-DC” for NR measurement only Therefore, for NR SA and NR DC, a bit map should be introduced which is mandatory feature with capability signaling. For LTE SA, EN-DC, NE-DC, a single bit should be introduced, which is optional |
| Intel (Rapporteur) | Regarding how to handle functionalities which have already been supported in R15 while there is no RRM requirement in R15. We provided several options in the 1st round discussion: 1) introduce R16 requirement and R16 optional capability 2) introduce R16 requirement and R16 mandatory capability 3) introduce R16 requirement w/o UE capability 4) introduce R16 requirement and release independent capability. We would like to elaborate more on our view. - Option 1) means the requirement is optionally supported. If UE doesn’t indicate support of RRM requirement, network may need to be careful when triggering corresponding procedure. Undefined UE behavior may happen. If UE indicates support of RRM requirement, UE behavior is clearly defined and no ambiguity. Intel prefers this option for most of the R16 requirement, e.g. BWP switching on multiple CC, multiple SCell activation and so on. - Option 2) means all R16 UE are mandated to support this feature. Introducing capability signaling is to allow early UE implementation to support this feature. - Option 3) also means all R16 UE are mandated to support this feature. The difference from option 2) is that early UE implementation may not be possible. R16 UE may need to wait until all the features are fully developed such that it won’t cause any ambiguity at network side. It should be used very carefully. - Option 4) can allow early release UE to indicate support of this feature in future design. To Apple: address the comment in the table Propose the following updates after 2nd round Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (V2X WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 9-1; BWP switching on multiple CCs; 1) Support of BWP switching on multiple CCs.; Yes; N/A; Network should not trigger BWP switching on multiple CC simultaneously. Otherwise there may be additional unclear BWP switching delay.; [Per UE]; No; [No]; N/A; [Optional with capability signalling]. 9-2; Mandatory gap pattern]; 1) Support of additional mandatory gap patterns for NR-only measurements in NR SA and NR DC, 2) Support of mandatory additional gap patterns defined for NR SA and NR-DC for NR measurement only in LTE SA, EN-DC, NE-DC; Yes; N/A; Network cannot configure corresponding gap patterns for the UE.; Per UE; No; No; N/A; Note: Agreements are provided in [R4-2005346]. According to RAN4 agreement, a bitmap should be introduced for 1). A single bit should be introduced for 2); [Mandatory with capability signalling for NR SA and NR DC Optional with UE capability signalling for LTE SA, EN-DC and NE-DC]. 9-3; Inter-frequency measurement without measurement gap; 1) Support of inter-frequency measurement without MG when the inter-frequency SSB is completely contained in the active DL BWP of the UE [2) support of SSB based measurement on inter-frequency without MG and data reception of PDCCH/PDSCH in serving with different SCS]; Yes; N/A; 1] gNB has to configure measurement gap for inter-frequency measurement [2) UE cannot support of SSB based measurement on inter-frequency without MG and data reception of PDCCH/PDSCH in serving with different SCS]; Per UE; No; [No/Yes]; N/A; . Details of 2) can be found in RAN4 LS R4-2005350 to RAN2, wherein two options are listed, i.e.1) update existing IE (simultaneousRxDataSSB-DiffNumerology); 2) introduce a new UE capability; FFS. 9-4; [CGI reading]; 1) Support of autonomous gap-based CGI reading of an NR neighbour cell for EN-DC, NR SA, LTE SA, NR-DC, NE-DC 2) Support of autonomous gap-based CGI reading of an E-UTRA neighbour cell for EN-DC, NR SA, LTE SA, NR-DC, NE-DC; Yes; N/A; [gNB cannot configure CGI reading of neighbor cell]; Per UE; No; Yes; N/A; Optional with capability signalling. 9-5; [SRS carrier switching]; 1) Support of SRS carrier switching RRM requirements; Yes; N/A; [Network cannot know the SRS carrier switching delay and corresponding interruption length for this UE. Therefore, either network may not trigger SRS carrier switch or there will be performance degradation]; Per UE; No; [No/Yes]; N/A; Functionality of SRS carrier switching has already been supported since R15. RRM requirement is expected to be introduced in R16. Thus, R16 UE shall meet corresponding RRM requirement. The requirements apply when SRS carrier switching is on NR carrier or on LTE carrier i.e. it should cover also EN-DC and NE-DC scenarios.; [Mandatory with capability signalling for Rel-16 UEs supporting SRS carrier switching]. 9-6; [Multiple SCell activation]; 1) Support of multiple SCell activation RRM requirement; Yes; N/A; [Network cannot know the multiple SCell activation delay and corresponding interruption length for this UE. Therefore, either network may not trigger multiple SCell activation or there will be performance degradation]; Per UE; No; [No/Yes]; N/A; Functionality of multiple SCell activation has already been supported since R15. RRM requirement is expected to be introduced in R16. Thus, R16 UE shall meet corresponding RRM requirement.; [Optional with capability signalling]. 9-7; [UE specific CBW change]; 1) Support of UE-specific CBW change RRM requirement; Yes; N/A; [Network cannot know the UE specific CBW change delay and corresponding interruption length for this UE. There will be performance degradation when UE specific CBW changes]; Per UE; No; No; N/A; Functionality of UE specific CBW change has already been supported since R15. RRM requirement is expected to be introduced in R16. Thus, R16 UE shall meet corresponding RRM requirement.; [Optional with capability signalling]. 9-8; [Spatial relation switch for uplink]; 1) Support of UL spatial relation switch RRM requirement; Yes; N/A; [Network cannot know the uplink spatial relation switch delay for this UE. There will be performance degradation when uplink spatial relation changes]; Per UE; No; No; N/A; Functionality of uplink spatial relation change has already been supported since R15. RRM requirement is expected to be introduced in R16. Thus, R16 UE shall meet corresponding RRM requirement.; Optional with capability signalling. |
| ZTE | Thanks Intel for the detailed analysis. Regarding how to handle functionalities which have already been supported in R15 while there is no RRM requirement in R15. We provided several options in the 1st round discussion: 1) introduce R16 requirement and R16 optional capability 2) introduce R16 requirement and R16 mandatory capability 3) introduce R16 requirement w/o UE capability 4) introduce R16 requirement and release independent capability. Since there is no requirements specified for a feature in R15 it is not clear if the feature is actually supported by the UE. The R16 UE capability is to indicate if the feature is supported for this Rel-16 UE. So in our view the Rel-16 UE capability should follow the feature itself in Rel-15 in terms of mandatory or optional. For example if BWP switching on multiple CCs are optional in Rel-15 it should be optional in Rel-16. If it is mandatory in Rel-15, it should be mandatory in Rel-16 either. For the feature mandatory gap pattern, we made some revisions based on Intel’s input. 9-2; Mandatory gap pattern]; 1) Support of additional mandatory gap patterns for NR-only measurement in NR SA and NR DC 2) Support of additional mandatory gap patterns defined for NR SA and NR-DC for NR only measurement in LTE SA, EN-DC, NE-DC; Yes; N/A; Network cannot configure corresponding gap patterns for the UE.; Per UE; No; No; N/A; Note: Agreements are provided in [R4-200xxxx].; [Mandatory with capability for component 1] [Optional with capability for component 2]. |
| NTT DOCOMO (Moderator) | We would like to know rapporteur’s suggestion is fine or not. We reflected Intel’s version to above list. To Intel, please check ZTE’s view. |
| Intel (Rapporteur) | Thanks ZTE for the feedback. We agree with ZTE for functionalities introduced in R15 with capability (mandatory or optional). For instance, SRS carrier switching is an optional feature in R15. It is quite straightforward that R16 requirement should only apply for UE which support SRS carrier switching. However, there are also some functionalities introduced in R15 without capabilities, e.g. multiple SCell activation. How to handle this is may need to be discussed. If we don’t introduce UE capability for this feature, then early UE implementation may not be possible. R16 UE may need to wait until all the features are fully developed such that it won’t cause any ambiguity at network side. |

  - NR RRM requirements for CSI-RS based L3 measurement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | The requirements for L3 CSI-RS measurements are introduced for an existing Rel-15 feature. There are no Rel-15 requirements defined and UE behavior is not known to the UE. Propose to introduce a new Rel-16 feature “Support of RRM requirements for CSI-RS based L3 measurement” and capability signalling to ensure the gNB is aware that UE can fulfil the requirements. |
| Apple | Depending on the conclusion on “associatedSSB”, an additional UE capability may need to be introduced. |
| Qualcomm | Not clear if anything is needed here. |
| MTK | If RRM session cannot conclude on a single measurement bandwidth to define the requirement, we may need a feature here. |
| Huawei | RRM discussion for this WI is ongoing, and some UE capabilities may be defined based on the outcome of the discussions. For example, we have proposed to introduce UE capability in number of CSI-RS resources per slot per MO UE can measure, as well as the minimum separation between two slots with CSI-RS resources. |
| NTT DOCOMO (Moderator) | We think some features are still under discussion in WI. To rapporteur, - Please update above list based on the outcome in 1st round and above company’s comments, if needed. - Please provide your comments as a rapporteur to each company’s comment or below. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | Discussion should be postponed |
| MTK | To Apple, whether to support CSI-RS for L3 measurement with or without associated SSB is already a feature in R15. Detail can be found in TS38.306. csi-RSRP-AndRSRQ-MeasWithSSB Indicates whether the UE can perform CSI-RSRP and CSI-RSRQ measurement as specified in TS 38.215 [13], where CSI-RS resource is configured with an associated SS/PBCH. This parameter needs FR1 and FR2 differentiation. If the UE supports this feature, the UE needs to report maxNumberCSI-RS-RRM-RS-SINR.. csi-RSRP-AndRSRQ-MeasWithoutSSB Indicates whether the UE can perform CSI-RSRP and CSI-RSRQ measurement as specified in TS 38.215 [13], where CSI-RS resource is configured for a cell that transmits SS/PBCH block and without an associated SS/PBCH block. This parameter needs FR1 and FR2 differentiation. If the UE supports this feature, the UE needs to report maxNumberCSI-RS-RRM-RS-SINR.. Agree with Huawei that some potential UE capability discussion in now ongoing in RRM session. It is not 100% clear at this moment. |
| Intel | Further RAN4 discussion is needed. No response provided by rapporteurs. |
| NTT DOCOMO (Moderator) | Keep this table. |

  - NR support for high speed train scenario

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 NR HST | 10-1 | RRM enhanced requirements specified within NR and NR-E-UTRAN inter-RAT measurement for NR HST | The enhanced RRM requirements specified within NR and NR-E-UTRAN inter-RAT measurement to support high speed up to 500 km/h, as specified in TS 38.133 |  | No |  | The performance of RRM in NR HST scenario cannot be guaranteed | Per UE | NO | FR1 only |  |  | TBD |
|  | 10-2 | Demodulation enhancement for HST-SFN joint transmission scheme | The enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h, as specified in TS 38.101-4 |  | No |  | The demodulation performance of HST-SFN joint transmission cannot be guaranteed | Per UE | NO | FR1 only |  |  | TBD |
|  | 10-3 | RRM enhancement for E-UTRAN -NR inter-RAT measurement for NR HST | The enhanced RRM requirements specified for E-UTRAN-NR inter-RAT measurement to support high speed up to 500 km/h, as specified in TS 36.133 |  | No |  | The performance of RRM in NR HST scenario cannot be guaranteed | Per UE | NO | FR1 only |  |  | TBD |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | - Agree with RRM capabilities - Suggest to define 3 separate capabilities for Demodulation enhancements including HST-SFN, HST Single tap and HST multi-path fading. Otherwise the requirements applicability for Single tap and fading conditions becomes ambiguous. The latter two capabilities can be defined as optional without capability signalling 1-2; Demodulation enhancement for HST-SFN joint transmission scheme; Enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h; Yes; The demodulation performance of HST-SFN joint transmission cannot be guaranteed; Per UE; NO; FR1 only; Optional with capability signalling. 1-X; Demodulation enhancement for HST single tap conditions; Enhanced demodulation processing for HST single tap conditions with velocity up to 500km/h; No; The demodulation performance of HST single tap conditions cannot be guaranteed; Per UE; NO; FR1 only; Optional without capability signalling. 1-Y; Demodulation enhancement for HST multi-path fading; Enhanced demodulation processing for HST multi-path fading conditions with velocity up to 500km/h; No; The demodulation performance of HST multi-path fading conditions cannot be guaranteed; Per UE; NO; FR1 only; Optional without capability signalling. |
| Apple | Agree with Intel’s comments |
| Samsung | For enhanced RRM requirements to support high speed, we suggest to keep this feature as optional feature. Capability signaling can be designed for optional feature to inform gNB on UE capability For demod requirements, we agreed the capability signaling is needed for SFN scenarios but justification of mandating this feature is needed. At this stage, similar with LTE SFN, we suggest it as optional with capability signalling Regarding the requirement for single tap and multi-path fading condition, requirements is still discussing in NR HST WI. It is not clear whether these proposed two requirements should belong to the UE features are needed? |
| Qualcomm | Features should be optional |
| Ericsson | Added a note: The requirements are being defined within NR, from NR to E-UTRA, and from E-UTRA to NR. |
| MTK | RRM session is now discussing whether additional capability is needed for inter-RAT measurement. We should wait for the conclusion. |
| Huawei | The feature list provided is similar to LTE. The proposal from rapporteur is OK for us. We are not convinced for adding the feature list for single tap and multi-path fading condition. In our view there is no special implementations for single tap and multi-path, which is different from HST-SFN where the advanced receiver is specified. The single tap and multi-path conditions need mandatorily be supported by UE. |
| NTT DOCOMO (Moderator) | To rapporteur, - Please update above list based on the outcome in 1st round and above company’s comments, if needed. - Please provide your comments as a rapporteur to each company’s comment or below. |
| CMCC (Rapporteur) | Thanks all the companies for the discussion. As rapporteur, we would like to provide our consideration and feedback to companies’ comments as following. To Intel and Apple, according to current agreements, only UE capability on HST-SFN joint transmission scheme was agreed. As for HST single tap, multi-path fading, we share similar view as HW, there is no special implementations for single tap and multi-path, even in LTE HST, both of them are not added to the UE feature list. Anyway, for single tap and multi-path, whether UE capability is needed or not is under discussion in NR HST WI. To avoid overlapped discussion, after coordination with RAN4 leadership, it is suggested to focus on the related discussion in the email discussion for [222] NR_HST_Demod_UE. If needed, the feature list for NR HST can be updated pending on the outcome of demod session. To Samsung and Qualcomm, our consideration to mandate this feature is that high speed scenario is an important deployed scenario. And the improvement of UE experience is very necessary. It is preferred to have this feature mandatory. We would like to have further discussion. At current stage, we keep mandatory with capability signaling with square bracket. To Ericsson, we are OK with the update. To MTK, we are OK that the suggestion. Whether additional capability is needed for inter-RAT measurement is under discussion in RRM session. To avoid overlapped discussion, it is suggested to focus on the related discussion in the email discussion for [126] NR_HST_RRM. The feature list for NR HST can be updated based on the outcome of RRM session. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| MTK | As agreed in RRM GTW session, a new per-UE capability to indicate that UE is capable of supporting the enhanced EUTRAN-NR inter-RAT measurement requirements is needed. We believe CMCC’s change reflects this agreement. The only FFS part is Mandatory/Optional |
| Intel | RRM Inter-RAT measurement feature shall be added based on the 2nd round conclusion in the HST RRM thread For demodulation features – RAN4 shall continue discussion on “Demodulation enhancement for HST single tap conditions”, “Demodulation enhancement for HST multi-path fading” in RAN4 #95e. . |
| Samsung | Update To CMCC: My understanding a LS about UE feature list will be sent to RAN2 in this meeting. Considering the parallel discussion for NR HST WI, a LS about supporting Rel-16 NR HST from Rel-15 UEs to RAN2 for checking whether the early implementation is applicable for NR, and the open issue for HST single tap about whether to configure existing RRM signalling is still under discussion, at this stage, I would like to suggest using TBD, which may be more proper compared with [mandatory with capability signaling] for both RRM and demodulation enhancement. We can further discuss in the next meeting. |
| NTT DOCOMO (Moderator) | We think rapporteur has already reflected the latest agreement. |

  - NR performance requirement enhancement

No RAN4 specific feature is needed.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | Further discussion on requirements applicability is needed. In general, Rel-15 UEs are already expected to support the respective features but the performance may not be guaranteed. In case the requirements are defined from Rel-15 it is important to ensure a mechanism that only UEs compliant with the newly defined conformance requirements can indicate the respective capabilities. In case the requirements are defined from Rel-16, additional features/capabilities shall be introduced to inform gNB that UE can fulfill the respective requirements. |
| Huawei | We can discuss the applicability rule. But we are not sure what feature list RAN4 should provide. |
| NTT DOCOMO (Moderator) | To rapporteur, - Please update above list based on the outcome in 1st round and above company’s comments, if needed. - Please provide your comments as a rapporteur to each company’s comment or below. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| China Telecom (rapporteur) | No new UE features identified as far. |
| Intel | Suggest to continue discussion in RAN4 on requirements applicability first. |
| NTT DOCOMO (Moderator) | Keep this table. |

  - Others

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Maximum uplink duty cycle for TDD+TDD EN-DC power class 2 (maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16) | Indicates the maximum percentage of symbols during a certain evaluation period that can be scheduled for NR uplink transmission under different EUTRA TDD uplink-downlink configurations so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. This field is only applicable for inter-band TDD+TDD EN-DC power class 2 UE as specified in TS 38.101-3 [4]. If the field is absent, 30% shall be applied to all EUTRA TDD uplink-downlink configurations. If eutra-TDD-Configx is absent, 30% shall be applied to the corresponding EUTRA TDD uplink-downlink configuration. Value n20 corresponds to 20%, value n40 corresponds to 40% and so on. |  | Yes | N/A | If UE does not report this capability, 30% shall be applied to all EUTRA TDD uplink-downlink configurations. | Per band combination | TDD only | FR1 only |  |  | Optional |
|  |  | 29 dBm UE Power Class for band 41/n41 | Modified MPR behavior for 29 dBm UE Power Class (modifiedMPRbehavior IE). 1. (bit0) EN-DC contiguous intraband MPR as defined in clausue 6.2B.2.1 of 38.101-3 v15.5.0 2. (bit1) EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 3. (bit2) EN-DC contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 vTBD 4. (bit3) EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 vTBD | 2-8 | Yes | N/A | Table 6.5.2.3.2-3 with unmodified requirements applies | Type 3 (per band and band combination) | N/A | FR1 only | N/A | Applicable to band n41 in an EN-DC configuration with band 41 | Optional |
|  |  | support co-located scenario only for inter-band EN-DC | Indicates the inter-band EN-DC combination supported by the UE can only work at co-located scenario, and in this scenario the PSD difference between DL carriers and MRTD can be guaranteed. candidate values set: {type1, type2} type 1 UE: performance guaranteed with PSD difference between DL carriers < 6dB, and MRTD=3us (current only DC_20_n28 has this limitation) type 2 UE: performance guaranteed without limitation on PSD difference between DL carriers and MRTD=33us |  | Yes | N/A | If UE does not reports this capability, the performance cannot be guaranteed under inter-band non-collocated scenario. | Per band combination | N/A | FR1 only | N/A |  | Optional |
|  | [2-15] | [Maximum uplink duty cycle for FR2] | [1) Maximum percentage of uplink transmission time that can be scheduled within 1s time window at the UE maximum total radiated power in order to ensure compliance with applicable electromagnetic power density exposure requirements provided by regulatory bodies. The value range is {15%, 20%, 25%, 30%, 40%, 50%, 60%, 70%, 80%, 90%, 100%}.] |  | [Yes] | [N/A] | [UE is not able to provide the maximum UL duty cycle information to the network, thereby leading to less efficient scheduling of UL resources] | [Type 2 (per band)] | [No] | [Applicable only to FR2] | [N/A] | [Indicates the maximum percentage of symbols during 1s that can be scheduled for uplink transmission at the UE maximum total radiated power so as to ensure compliance with applicable electromagnetic power density exposure requirements provided by regulatory bodies. This field is applicable for all power classes UE in FR2 as specified in TS38.101-2 [3]. Value n15 corresponds to 15%, value n20 corresponds to 20% and so on. If the field is absent or the percentage of uplink symbols transmitted within any 1s evaluation period is larger than maxUplinkDutyCycle-FR2, the UE behaviour is specified in TS38.101-2 [3].] | [Optional with capability signaling] |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| CMCC | To add the UE feature of Maximum uplink duty cycle for TDD+TDD EN-DC power class 2. The capability was already introduced in RAN2. Just to capture it in RAN4 feature list for information. |
| Apple | Introduce modified MPR behavior bits for the 29 dBm UE power class in band 41/n41. For FG 2-15 in the Rel-15 feature list, introduce the correction of the UE capability for maxUplinkDutyCycle-FR2 |
| NTT DOCOMO (Moderator) | Thank you for providing above list. To companies, please review them and provide comments in 2nd round. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | 29 dBm UE Power Class for band 41/n41: Optional Maximum uplink duty cycle for FR2: this related to Rel-15 and shall not be a part of Rel-16 feature list discussion? |
| OPPO | For FG 2-15 in the Rel-15 feature list, the reference power of maxUplinkDutyCycle-FR2 has not been agreed. And this is independent of Rel-16 feature list. If finally change is needed then normal LS approach can be used, rather than mix Rel-15 feature with Rel-16 features. |
| NTT DOCOMO (Moderator) | Regarding Maximum uplink duty cycle for FR2, we are not sure whether it should be removed or not. We attached square brackets at this point. |

- LTE UE feature

  - Additional MTC enhancements for LTE

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. LTE_eMTC5 | 1-1 | Relaxed RRM measurements | 1. Relaxation of RRM measurements for serving cell for UEs using MWUS for at least low mobility UEs | Rel-15 MWUS | No | N/A | RRM measurement requirements will not be relaxed. | Per UE | No | N/A | No |  | Optional without capability signaling |
|  | 1-2 | RRS-based measurement improvement in RRC_IDLE | 1. Improved DL RSRP measurement accuracy through use of RSS in RRC_IDLE | Rel-15 RSS | No | N/A | Measurements will be based on CRS only (not RSS). | Per UE | Yes | N/A | No |  | Optional without capability signaling |
|  | 1-3 | RRS-based measurement improvement in RRC_CONNECTED | 1. Improved DL RSRP measurement accuracy through use of RSS in RRC_CONNECTED | Rel-15 RSS | Yes | N/A | Measurements will be based on CRS only (not RSS). | Per UE | Yes | N/A | No |  | Optional with capability signaling |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Rapporteur (Ericsson) | Rel-16 LTE-MTC features that are under the responsibility of other WGs are not listed in this RAN4 feature list. |
| Intel | Agree with rapporteur proposal. |
| NTT DOCOMO (Moderator) | Above lists looks feasible. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| None |  |

  - Additional enhancements for NB-IoT

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB-IoT | 2-1 | NRSRP measurement on non-anchor carrier | NRSRP measurement on non-anchor carrier using NRS | NRS on a non-anchor carrier for paging | No | N/A | UE cannot do measurement and evaluation of serving cell on non-anchor carrier | Per UE | FDD only | Applicable only to FR1 | N/A |  | Optional without capability signalling |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | Agree with rapporteur proposal. |
| NTT DOCOMO (Moderator) | Above lists looks feasible. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei | No more comment from rapporteur. |

  - Further performance enhancement for LTE in high speed scenario

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.LTE HST | 3-1 | Further RRM enhancement for LTE HST | The enhanced RRM requirements to support high speed with 500 km/h |  | No | N/A | The performance of RRM in LTE HST scenario with 500km/h cannot be guaranteed | FFS | No | N/A | No |  | Optional with capability signalling |
|  | 3-2 | Further RRM enhancement for LTE HST with CA | The enhanced RRM requirements to support high speed with 350 km/h for SCell measurements |  | No | N/A | The performance of RRM in LTE HST scenario with 350km/h with CA cannot be guaranteed | FFS | No | N/A | No |  | Optional with capability signalling |
|  | 3-3 | Further demodulation enhancement for HST-SFN | The enhanced demodulation processing for HST-SFN with 500km/h |  | No | N/A | The demodulation performance of HST-SFN with 500km/h cannot be guaranteed | FFS | No | N/A | No |  | Optional with capability signalling |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | Agree with the proposed feature set |
| Ericsson | In our view CA support for HST is a separate sub-feature (3-2 is added) since not every UE may support CA and HST |
| NTT DOCOMO (Moderator and Rapporteur) | Thank you very much for Ericsson’s update. Your suggestion is fine for us. Please give us feedback in 2nd round if above list is not acceptable. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei | Basically we are OK with the feature list. Regarding to 3-2, since it is related how many CCs will be supported, it may be related to band combination. Regarding 3-1 and 3-3, we are thinking that they may be related to band, since the requirement is specified relatively to Doppler rather than velocity. More discussion would be needed for the feature type, or we can still keep the feature as per-UE but change the description of feature to support speed with 500km/h for the frequency bands below xxx Hz. So we change feature type to FFS. |
| NTT DOCOMO (Moderator and Rapporteur) | Huawei’s suggestion is OK. |

  - Even further Mobility enhancement in E-UTRAN

No RAN4 specific feature is needed.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4. LTE feMob | 4-1 | Support of intra-frequency DAPS HO | 1) Support of synchronous DAPS handover for intra-frequency case 2) Support of asynchronous DAPS handover for intra-frequency case |  | Yes | N/A | The network cannot configure UE with DAPS HO | Per Band/Per BC | No | No | N/A | For inter-band combination, only component for inter-frequency case is valid | Optional with capability signalling |
|  | 4-2 | Support of inter-frequency DAPS HO | 1) Support of synchronous DAPS handover for inter-frequency case 2) Support of asynchronous DAPS handover for inter-frequency case |  | Yes | N/A | The network cannot configure UE with DAPS HO | Per BC | No | No | N/A | For inter-band combination, only component for inter-frequency case is valid. | Optional with capability signalling |
|  | 4-3 | Support of simultaneous UL transmission | 1) Support of simultaneous UL transmission for DAPS handover for intra-frequency case 2) Support of simultaneous UL transmission for DAPS handover for inter-frequency case |  | Yes | N/A | The network cannot configure the UE simultaneous uplink transmission for DAPS HO | Per BC | No | No | N/A | For inter-band combination, only component for inter-frequency case is valid. UE can indicate differently for intra-band intra-frequency and intra-band contiguous inter-frequency. | Optional with capability signalling |
|  | 4-4 | Support of multi TAG | 1) Support of different TAGs in source and target cells for intra-frequency case 2) Support of different TAGs in source and target cells for inter-frequency case |  | Yes | N/A | The network cannot configure the different TAGs for uplink transmission for DAPS HO | Per BC | No | No | N/A | For inter-band combination, only component for inter-frequency case is valid | Optional with capability signalling |

1st round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Intel | The capabilities shall be aligned with NR Mobility enhancements. |
| China Telecom | RAN4 already sent LS to RAN2 with the list of RAN4-related capabilities in R4-1915781, and RAN4 assumes that RAN2 have captured these capabilities in RAN2’s UE feature list discussion. Agree with Intel to align the UE feature with NR mobility enhancements, if further modification and clarification are agreed in this RAN4 meeting. |
| NTT DOCOMO (Moderator) | To rapporteur, please update it based on the outcome in NR mobility enhancements WI, if needed. |

2nd round

| Company | Comments/Questions/Suggestions |
| --- | --- |
| China Telecom (rapporteur) | Aligned with NR mobility enhancements. |
| Huawei | We are basically OK to align them with NR, and further discussion is still needed for NR. More discussion is needed especially for how to capture the agreements in the previous LS. |

Appendix: RF and RRM features in TR38.822 v15.0.1

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. System parameter | 1-1 | 60kHz of subcarrier spacing for FR1 | 60kHz subcarrier spacing for data channel in FR1 |  | scs-60kHz | Phy-ParametersFR1 | No | Applicable only to FR1 |  | Optional with capability signalling |
|  | 1-2 | 64QAM modulation for FR2 PDSCH | 64QAM modulation for FR2 PDSCH |  | n/a | n/a | No | Applicable only to FR2 | Capability can be discussed in future, e.g. when low cost device (e.g. IoT) and/or higher frequency band in FR2 are introduced | Mandatory without capability signalling |
|  | 1-3 | 64QAM for PUSCH | 64QAM for PUSCH |  | n/a | n/a | No | No | Capability can be discussed in future, e.g. when low cost device (e.g. IoT) and/or higher frequency band in FR2 are introduced | Mandatory without capability signalling |
|  | 1-4 | 256QAM for PDSCH | 256QAM for PDSCH |  | pdsch-256QAM-FR1 | Phy-ParametersFR1 | No | Yes | For FR1, it can be revisited in the future whether the 256QAM is mandated in all UE types or categories | Mandatory with capability signalling for FR1 |
|  |  |  |  |  | pdsch-256QAM-FR2 | BandNR |  |  | For FR2, RAN4 agreed that no BS and UE requirements will be introduced in Rel.15. | Optional with capability signalling for FR2 |
|  | 1-5 | 256QAM for PUSCH | 256QAM for PUSCH |  | pusch-256QAM | BandNR | No | Yes | For FR1, RAN4 can further discuss to mandate 256QAM for PUSCH for FR1 in future release. For FR2, RAN4 agreed that no BS and UE requirements will be introduced in Rel.15. | Optional with capability signalling (for both FR1 and FR2) |
|  | 1-6 | pi/2-BPSK for PUSCH | pi/2-BPSK for PUSCH |  | pusch-HalfPi-BPSK | Phy-ParametersFRX-Diff | No | Yes | RAN4 will define the same minimum requirements for pulse-shaped pi/2 BPSK and non-pulse shaped pi/2 BPSK for FR2. | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 |
|  | 1-7 | pi/2-BPSK for PUCCH format 3/4 | pi/2-BPSK for PUCCH format 3/4 |  | pucch-F3-4-HalfPi-BPSK | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 |
|  | 1-8 | Active BWP switching delay | Support of active BWP switching delay specified in TS38.133, candidate values set: {type1, type2} |  | bwp-SwitchingDelay | Phy-ParametersCommon | No | No | For this feature, RAN4 also sent another LS (R4-1803283). Network cannot configure the shorter delay for certain UE type. | Mandatory to support either type 1 or type 2 with capability signalling |
|  | 1-9 | Support of EN-DC with LTE-NR coexistence in UL sharing from UE perspective | 1) LTE and NR UL Transmission in the shared carrier via TDM only 2) LTE and NR UL Transmission in the shared carrier via FDM only 3) LTE and NR UL transmission in the shared carrier via FDM or TDM |  | ul-SharingEUTRA-NR | MRDC-Parameters | No | Applicable only to FR1 |  | Optional with capability signalling |
|  | 1-10 | Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Support of switching type between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective. Type 1: <0.5us Type 2: <20us | 1-9 | ul-SwitchingTimeEUTRA-NR | MRDC-Parameters | No | Applicable only to FR1 | This feature is the switching time between LTE UL and NR UL in the same carrier Per band combination signalling UE Capability signalling elements. 1: <0.5us switching type. 2: <20us switching type. | Mandatory to support either type 1 or type 2 with capability signalling if UE reports its capability in 1-10 as 1) LTE and NR UL Transmission in the shared carrier via TDM only, or 3) LTE and NR UL transmission in the shared carrier via FDM or TDM |
|  | 1-11 | 7.5kHz UL raster shift | 7.5kHz UL raster shift |  | n/a | n/a | No | No |  | Mandatory in the SUL bands with uplink sharing either from UE perspective or from network perspective 7.5KHz raster shift as mandatory without capability signalling. 7.5kHz UL raster shift is mandatory for the bands described in clause 5.4.2.1 of Release 15 TS 38.101-1. RAN4 can revisit the above bands in the future release. 7.5KHz raster shift is not mandatory for other LTE refarming band except the bands which were agreed to support 7.5kHz UL raster shift as mandatory |
| 2. UE RF | 2-1 | Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC | 1) FR1 channel bandwidths in TS38.101-1 Table 5.3.5-1 2) FR2 channel bandwidths in TS38.101-2 Table 5.3.5-1 |  | channelBWs-DL channelBWs-UL | BandNR | No | No | UE capability signalling shall follow RP-172832 (Per-band capability signalling, separately for DL and UL and for each SCS) Whether a bandwidth newly introduced in future is mandatory for UE shall be discussed case by case. | For FR1, all the bandwidths listed in TS38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC. The bandwidths listed in the slide #3 of R4-1805985 are mandatory with a single CC. 90MHz is optional for n41, n77, n78. For FR2, the set of mandatory CBW is 50, 100, 200 MHz. |
|  |  |  |  |  | supportedBandwidthDL channelBW-90mhz | FeatureSetDownlinkPerCC |  |  |  |  |
|  |  |  |  |  | supportedBandwidthUL channelBW-90mhz | FeatureSetUplinkPerCC |  |  |  |  |
|  | 2-2 | Simultaneous reception or transmission with same or different numerologies in CA | Support of simultaneous reception or transmission with same or different numerologies in CA |  | supportedSubcarrierSpacingDL | FeatureSetDownlinkPerCC | No | No | From RAN4 perspective UE shall be able to signal the supported SCS per CC for each band combination Same numerology for intra-band NR CA including both continuous and non-continuous is mandatory support for Rel15 The capability of supporting SCS within the single carrier in the CA configuration will be signalled separately, i.e., there is no need to mandatory UE to support mixed numerologies in CA case If a UE supports inter-band NR CA including both FR1 band(s) and FR2 band(s), the UE shall support two mixed numerologies between FR1 band(s) and FR2 band(s) in DL and UL with capability signalling. | Same numerology for intra-band NR CA including both continuous and non-continuous is mandatory with capability in both FR1 and FR2. Two mixed numerologies between FR1 band(s) and FR2 band(s) in DL and UL are mandatory with capability if UE supports inter-band NR CA including both FR1 band(s) and FR2 band(s). Optional for other cases. |
|  |  |  |  |  | supportedSubcarrierSpacingUL | FeatureSetUplinkPerCC |  |  |  |  |
|  | 2-3 | Non-contiguous intra-band CA frequency separation class for FR2 | 1) Support of frequency separation classes to handle the total frequency span for DL for intra-band non-contiguous CA 2) Support of frequency separation classes to handle the total frequency span for UL for intra-band non-contiguous CA |  | intraBandFreqSeparationDL | FeatureSetDownlink | No | Applicable only to FR2 | UE signals the supported Frequency separation classes with per band granularity (Type 1) based on R4-1803363 Separate Frequency separation classes can be signalled for DL and UL | Mandatory to support a frequency separation class within {I, II, III} specified in TS38.101-2 with capability if UE supports non-contiguous CA in FR2 |
|  |  |  |  |  | intraBandFreqSeparationUL | FeatureSetUplink |  |  |  |  |
|  | 2-4 | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) |  | simultaneousRxTxInterBandENDC | MRDC-Parameters | No | No | For TDD-FDD and TDD-TDD band combinations for which simultaneous RxTx capability is agreed to be supported, corresponding capability indication must be set to "supported". Band combinations for which simultaneous RxTx capability is mandatory EN-DC combinations (Both FR1 LTE – FR1 NR and FR1 LTE- FR2 NR) are captured in TS 38.101-3. | Mandatory/Optional support depends on band combination and captured in TS 38.101-3 |
|  | 2-5 | Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD) | Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD) |  | simultaneousRxTxInterBandCA | CA-ParametersNR | No | No | For TDD-FDD and TDD-TDD band combinations for which simultaneous RxTx capability is agreed to be supported, corresponding capability indication must be set to "supported". Band combinations for which simultaneous RxTx capability is mandatory are captured in TS 38.101-1, TS 38.101-2 and TS 38.101-3. | Mandatory/Optional support depends on band combination and captured in TS 38.101-1, TS 38.101-2 and TS 38.101-3 |
|  | 2-6 | Asynchronous FDD-FDD intra-band EN-DC DC | Asynchronous FDD-FDD intra-band EN-DC |  | asyncIntraBandENDC | MRDC-Parameters | Applicable only to FDD | Applicable only to FR1 |  | Optional with capability signalling |
|  | 2-7 | Almost contiguous UL CP-OFDM | Support of almost contiguous UL CP-OFDM transmissions |  | almostContiguousCP-OFDM-UL | Phy-ParametersFRX-Diff | No | Yes | RAN4 had defined the requirements for "Almost contiguous UL CP-OFDM" in Rel-15. | Optional with capability signalling |
|  | 2-8 | UE power class | 1) Support of FR1 UE power class 2) Support of FR2 UE power class 3) Support of FR1 UE power class for EN-DC 4) Support of FR1 UE power class for NR-CA |  | ue-PowerClass | BandNR | No | No | Capability signalling - FR1 UE power class (per band) - FR2 UE power class (per band) - FR1 UE power class for EN-DC (per band combination) - FR1 UE power class for NR CA (per band combination) Default power class for each component is indicated in TS38.101-1/2/3. If the default power class is not indicated, UE shall report supported power class. The component 2) is also used as power class for intra-band NR-CA in FR2 | Mandatory to support at least one power class with capability. The capability signalling is absent if UE supports only default power class |
|  |  |  |  |  | powerClass | BandCombination |  |  |  |  |
|  | 2-9 | Simultaneous reception and transmission for SA SUL band combinations | Simultaneous reception and transmission for SA SUL band combinations |  | simultaneousRxTxSUL | CA-ParametersNR | No | No |  | Mandatory/Optional support depends on band combination and captured in TS 38.101-1 |
|  | 2-10 | Multiple frequency band indication | Multiple frequency band indication |  | n/a | n/a | No | No | Per UE capability | Mandatory without capability signalling |
|  | 2-11 | Modified MPR behaviour | Modified MPR behaviour |  | modifiedMPR-Behaviour | BandNR | No | No | Per band capability | Optional with capability signalling |
|  | 2-12 | Multiple NS/P-Max | Multiple NS/P-Max |  | n/a | n/a | No | No | Per UE capability | Mandatory without capability signalling |
|  | 2-13 | Maximum uplink duty cycle for FR1 power class 2 UE | Maximum percentage of uplink symbols can be scheduled within a certain evaluation period provided by regulatory bodies. The value range is {60%, 70%, 80%, 90%, 100%}. If the field is absent, 50% shall be applied. |  | maxUplinkDutyCycle-PC2-FR1 | BandNR | No | Applicable only to FR1 | Per band capability. If this capability is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%, or this capability is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than this capability, apply all requirements for the default power class. The evaluation period is up to UE implementation, no less than one radio frame. UE do not need to do UL duty cycle calculation when it's transmit power is below 23dBm and all the UL/DL configurations can be scheduled. | Optional with capability signalling. The capability signalling is absent if UE supports 50% |
|  | 2-14 | Power boosting for Pi/2 BPSK for power class 3 UE | Power boosting for Pi/2 BPSK for power class 3 UE in TDD bands n40, n77, n78 and n79 with duty cycle less than 40% | 1-6, 1-7 | powerBoosting-pi2BPSK | BandNR | Applicable only to TDD | Applicable only to FR1 | Per band capability | Optional with capability signalling |
|  | 2-15 | Maximum uplink duty cycle for FR2 | 1) Maximum percentage of uplink transmission time that can be scheduled within 1s time window in order to ensure compliance with applicable electromagnetic power density exposure requirements provided by regulatory bodies. The value range is {15%, 20%, 25%, 30%, 40%, 50%, 60%, 70%, 80%, 90%, 100%}. |  | maxUplinkDutyCycle-FR2 | BandNR | No | Applicable only to FR2 | Per band capability. If the field of UE capability is present and the percentage of uplink symbols transmitted within any 1 s evaluation period is larger than this capability, the UE follows the uplink scheduling and can apply P-MPR as in TS38.101-2. If the field of UE capability is absent, the compliance to electromagnetic power density exposure requirements are ensured by means of scaling down the power density or by other means. This capability is applicable for all power classes in FR2 | Optional with capability signalling |
|  | 2-16 | PA architectures for intra-band EN-DC | Support of dual PA |  | dualPA-Architecture | MRDC-Parameters | No | No | Per band per band combination capability Single PA is default architecture The following requirements are involved by this capability - A-MPR/MPR and MSD values for dual uplink. Whether two sets of requirements will be introduced in RAN4 can be further discussed for each specific band combination - Switching time between LTE UL and NR UL in single switched UL operation mode for intra-band EN-DC | Mandatory to support either single or dual PA architectures with capability if UE supports intra-band EN-DC configuration in uplink. The capability signalling is absent if UE supports single PA architecture. |
|  | 2-17 | PA architectures for intra-band UL CA | Support of dual PA |  | dualPA-Architecture | CA-ParametersNR-v1540 | No | No | Per band per band combination capability Single PA is default architecture The following requirements are involved by this capability - A-MPR/MPR and MSD values for dual uplink. Whether two sets of requirements will be introduced in RAN4 can be further discussed for each specific band combination | Mandatory to support either single or dual PA architectures with capability if UE supports intra-band CA configuration in uplink. The capability signalling is absent if UE supports single PA architecture |
| 3. Baseband | 3-1 | Independent measurement gap configurations for FR1 and FR2 | Measurement gaps for FR1 and FR2 are configured independently. |  | independentGapConfig | MeasAndMobParametersMRDC-Common | No | No |  | Optional with capability signalling |
|  | 3-2 | Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement | Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement |  | simultaneousRxDataSSB-DiffNumerology | MeasAndMobParametersFRX-Diff MeasAndMobParametersMRDC-FRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 3-3 | Short measurement gap | Measurement gap patterns with short MGL (gap pattern#2, 3, 6, 7, 8, 10) are supported for E-UTRAN measurement. Gap patterns #6, 7, 8, 10 only apply to E-UTRAN measurement when MO includes both E-UTRAN and NR. |  | supportedGapPattern | MeasAndMobParametersCommon | No | No | Per UE capability This capability is signalled as a part of supportedGapPattern in TS38.306. | Optional with capability signalling |
|  | 3-4 | SU-MIMO Interference Mitigation advanced receiver | 1) R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2 RX antennas. 2) R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4 RX antennas. |  | n/a | n/a | No | No | UE supporting the feature is required to meet the Enhanced Receiver Type requirements in TS 38.101-4 | Optional without capability signalling |