3GPP TSG RAN WG1 Meeting #102-e	R1-2005249

E-meeting, August 17-28, 2020

Agenda Item:	8.12.1

Source:	Huawei, HiSilicon

Title:	Resource configuration and group scheduling for RRC_CONNECTED UEs

Document for:	Discussion and Decision

Introduction

The new Rel-17 work item for broadcast/multicast [1] was approved with objectives as follows:

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]: - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2] - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception. - Specify support for dynamic change of Broadcast/Multicast service delivery between multicast (PTM) and unicast (PTP) with service continuity for a given UE [RAN2, RAN3] - Specify support for basic mobility with service continuity [RAN2, RAN3] - Assuming that the necessary coordination function (like functions hosted by MCE, if any) resides in the gNB-CU, specify required changes on the RAN architecture and interfaces, considering the results of the SA2 SI on Broadcast/Multicast (SP-190625) [RAN3] - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided.[RAN1, RAN2] - Study the support for dynamic control of the Broadcast/Multicast transmission area within one gNB-DU and specify what is needed to enable it, if anything [RAN2, RAN3] - Specify RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states [RAN2, RAN1]: - Specify required changes to enable the reception of Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, with the aim of keeping maximum commonality between RRC_CONNECTED state and RRC_IDLE/RRC_INACTIVE state for the configuration of PTM reception. [RAN2, RAN1]. - Note: the possibility of receiving Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, without the need for those UEs to get the configuration of the PTM bearer carrying the Broadcast/Multicast service while in RRC CONNECTED state

This contribution mainly focuses on scheduling MBS for RRC_CONNECTED UEs, and the discussion for IDLE/INACTIVE UEs can refer to contribution [2]. HARQ feedback for NR multicast will be discussed in another contribution [3].

PDCCH scheduling scheme for MBS

For enabling group scheduling for MBS, two straightforward mechanisms are group-common PDCCH based and UE-specific PDCCH based. The former uses a group common PDCCH and the latter uses UE-specific PDCCH to schedule a group common PDSCH.

Comparing to UE-specific PDCCH based scheduling, group-common PDCCH scheduling is beneficial in terms of efficiency or low PDCCH overhead because high PDCCH overhead would lead to high PDCCH blocking that degrades system performance usually.

In addition, for supporting scheduling MBS for UEs in RRC_IDLE/RRC_INACTIVE states and keeping commonality between RRC_CONNECTED UEs, group-common PDCCH based scheduling seems more appropriate.

Proposal 1：Group-common and UE-specific PDCCH scheduling are two options to enable group scheduling for MBS.

- Group-common based PDCCH scheduling is more appropriate in terms of low PDCCH overhead and keeping scheduling commonality between RRC_CONNECTED UEs and RRC_IDLE/RRC_INACTIVE UEs.

Scheduling MBS for RRC_CONNECTED UEs

To enable the reception of MBS by UEs in RRC_CONNECTED states, necessary configurations shall be transmitted to UE, e.g., the service(s) that UE is interested in. As we discussed in [2], for UEs in RRC_IDLE/ RRC_INACTIVE states, two applicable approaches for the network to transmit configurations to the UEs are by: 1) ‘broadcasting’ message, i.e. SI or/and MCCH message; 2) dedicated RRC message.

If option 1 is used for RRC_IDLE/ RRC_INACTIVE UEs, these configurations could be used also for RRC_CONNECTED UEs, but some additional parameters are needed for RRC_CONNECTED UE, such as the HARQ-ACK feedback related configurations. In addition, when transitioning from RRC_IDLE/ RRC_INACTIVE state to RRC_CONNECTED state, UE may switch from initial BWP to dedicated BWP for unicast. Accordingly, the frequency resources configured for receiving MBS may also be different for UEs in RRC_IDLE/ RRC_INACTIVE and RRC_CONNECTED states.

How configurations are configured to RRC_IDLE/ RRC_INACTIVE UEs needs RAN2 discussion or determination. This paper focuses on resource configuration for RRC_CONNECTED UEs.

  - Frequency resource configuration for NR multicast

UEs in RRC_CONNECTED can be configured up to four dedicated BWPs for unicast and the BWPs can be different for different UEs. Then how the frequency resource is configured for MBS among a group of UEs is a question. There are some options that can be considered:

- Additional BWP separately configured for MBS

From network perspective, a common BWP can be separately configured which is also the active BWP if only one common BWP is configured to a group of UEs for transmitting MBS independent of the dedicated BWPs configured for unicast.

For this option, the BWP for MBS may be or may not be overlapped with the dedicated BWP for unicast. Therefore, for UEs with dedicated BWP non-overlapped with MBS BWP, UE will be required to support two active BWPs for simultaneous operation with unicast reception. If UE does not support two active BWPs, UE has to switch BWPs back and forth for receiving MBS or unicast, which is usually not desirable due to unnecessary latency caused by BWP switching.

- Frequency resource for MBS confined within dedicated unicast BWP

In order to avoid BWP switching, the frequency resource for MBS to be confined within dedicated BWP can be implemented in two manners. The most restrictive way is up to network to configure the identical dedicated unicast BWPs for all UEs in the group which is also the BWP for scheduling MBS as illustrated in Fig. 1.

Fig. 1: Identical dedicated unicast BWP for scheduling MBS

Another less restrictive approach is configuring a common sub-band for scheduling MBS within dedicated unicast BWP as shown in Fig. 2.

Fig. 2: A common sub-band for MBS within dedicated unicast BWP

Comparing the approaches discussed above, a common sub-band within dedicated unicast BWP is a good option with less restriction and without BWP switching for receiving both unicast and MBS.

Proposal 2: Frequency resource for MBS can be configured independently from dedicated unicast BWP, or confined within dedicated unicast BWP.

Proposal 3: A common sub-band for MBS configured within dedicated unicast BWPs of UEs of the group is more proper in terms of less restriction and without BWP switching for receiving both unicast and MBS.

  - CORESET and search space configuration

When a common sub-band for MBS is configured within dedicated unicast BWPs of UEs of the group and a group common PDCCH is used for scheduling a common PDSCH, the CORESET and search space for UEs’ monitoring will be accordingly configured within the sub-band as well. CORESET and search space configuration framework in NR Rel-15 and Rel-16 could be re-used for multicast scheduling. Whether the number of CORESETs that can be configured per UE per BWP is extended or kept the same as Rel-16 for UEs supporting MBS could be discussed.

Regarding the search space for scheduling MBS, it is more like a CSS because all UEs will determine the same DCI scheduling the PDSCH, and the configuration of the search space is the same for all UEs within the group.

Proposal 4: For a common sub-band for MBS configured within dedicated unicast BWP and a group-common PDCCH based scheduling:

- The CORESET and search space is configured within the common sub-band;

- The configurations for the CORESET and search space are kept the same for all UEs of the group.

  - Scheduling DCI format(s) for MBS

In NR Rel-15 and Rel-16, three DCI formats including DCI formats 1_0, 1_1 and 1_2 have been defined for scheduling unicast PDSCH. Scheduling MBS PDSCH is similar to scheduling unicast MBS in a large extent, hence defining a new DCI format does not seem necessary.

One difference for scheduling a MBS PDSCH from unicast PDSCH is the FDRA field in the DCI. The FDRA is dimensioned per initial BWP or dedicated unicast BWP when scheduling unicast but should be per sub-band when scheduling MBS.

Proposal 5: DCI formats 1_0, 1_1 and 1_2 can be used for scheduling MBS with necessary modifications:

- For a common sub-band for MBS configured within dedicated unicast BWP and a group-common PDCCH based scheduling, the FDRA field in DCI is dimensioned per the common sub-band.

  - DCI size budget

In order to keep blind decoding to a reasonable level for a UE, the DCI formats that a UE monitors per slot/span should be subject to size alignment within a budget. The current DCI size budget is that the total number of different DCI sizes configured to monitor is no more than 4 for the cell and the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell.

Whether such budget is kept or extended should be discussed, as well as the DCI size alignment for DCI formats scheduling MBS and unicast transmissions.

Proposal 6: DCI size alignment for monitoring DCI for MBS scheduling needs to be determined including whether the DCI size budget is kept or can be extended.

Conclusions

This paper focuses on discussion on MBS scheduling for RRC_CONNECTED UEs, which leads to the proposals as follows:

Proposal 1：Group-common and UE-specific PDCCH scheduling are two options to enable group scheduling for MBS.

- Group-common based PDCCH scheduling is more appropriate in terms of low PDCCH overhead and keeping scheduling commonality between RRC_CONNECTED UEs and RRC_IDLE/RRC_INACTIVE UEs.

Proposal 2: Frequency resource for MBS can be configured independently from dedicated unicast BWP, or confined within dedicated unicast BWP.

Proposal 3: A common sub-band for MBS configured within dedicated unicast BWPs of UEs of the group is more proper in terms of less restriction and without BWP switching for receiving both unicast and MBS.

Proposal 4: For a common sub-band for MBS configured within dedicated unicast BWP and a group-common PDCCH based scheduling:

- The CORESET and search space is configured within the common sub-band;

- The configurations for the CORESET and search space are kept the same for all UEs of the group.

Proposal 5: DCI formats 1_0, 1_1 and 1_2 can be used for scheduling MBS with necessary modifications:

- For a common sub-band for MBS configured within dedicated unicast BWP and a group-common PDCCH based scheduling, the FDRA field in DCI is dimensioned per the common sub-band.

Proposal 6: DCI size alignment for monitoring DCI for MBS scheduling needs to be determined including whether the DCI size budget is kept or can be extended.

- References

- RP-201039, WID revision: NR Multicast and Broadcast Services. Huawei, HiSilicon, June 29-July 2, 2020.

- R1-2005272, “Discussion on multicast support for IDLE/INACTIVE UEs,” Huawei, HiSilicon, RAN1#102-e, August 17th – 28th, 2020.

- R1-2005250, “Mechanisms to improve reliability for RRC_CONNECTED UEs,” Huawei, HiSilicon, RAN1#102-e, August 17th – 28th, 2020.