3GPP TSG RAN WG1 #102-e	                                              									   R1-2005328

e-Meeting, August 17th – 28th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on PUSCH skipping with overlapping UCI on PUCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-8-5 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Following two cases on dynamic UL skipping were discussed in RAN1 and related conclusions were made below. - Case 1: dynamic PUSCH skipping without overlapping CSI/HARQ-ACK on PUCCH - Case 2: dynamic PUSCH skipping with overlapping CSI/HARQ-ACK on PUCCH For Case 1, it was agreed in RAN1#100-e meeting that when a UL grant without UL-SCH field or UL-SCH =1 (if present) is detected by a UE configured with skipUplinkTxDynamic, the corresponding PUSCH transmission is skipped by the UE if no transport block for the PUSCH transmission is generated by MAC and there is no CSI/HARQ-ACK on PUCCH overlapping with the PUSCH. For Case 2, RAN1 agreed that in Rel-15, the UE behavior is undefined and Case 2 can be addressed for Rel-16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To define the UE behavior for Case 2 in 38.213 section 9 that layer 1 shall notify higher layers that there is UCI to be multiplexed on a PUSCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent UE beahvior in RAN1 and RAN2 specification and ambiguous UE behavior for case 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: The CR has isolated impact on dynamic PUSCH skipping with overlapping CSI/HARQ-ACK on PUCCH. 1. If the gNB is implemented according to this CR while the UE is not, when there is UCI on PUCCH overlapping with a PUSCH with skipUplinkTxDynamic set to true, there may be misunderstanding for gNB which PUSCH the UCI is multiplexed on. 2. If the UE is implemented according to this CR while the gNB is not, there is no impact on gNB. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9	UE procedure for reporting control information

<unchanged part omitted>

If a UE

-	would multiplex UCI in a PUCCH transmission that overlaps with a PUSCH transmission, and

-	the PUSCH and PUCCH transmissions fulfill the conditions in Clause 9.2.5 for UCI multiplexing,

the UE

-	notifies higher layers there is UCI to be multiplexed on the PUSCH by Layer 1 when skipUplinkTxDynamic provided by higher layers is set to true.

-	multiplexes only HARQ-ACK information, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE multiplexes aperiodic or semi-persistent CSI reports in the PUSCH;

-	multiplexes only HARQ-ACK information and CSI reports, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE does not multiplex aperiodic or semi-persistent CSI reports in the PUSCH.

<unchanged part omitted>