3GPP TSG RAN WG1 #102-e	                                              									   R1-2005328

e-Meeting, August 17th – 28th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR |  | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PUSCH skipping with overlapping UCI on PUCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-8-5 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the LS from RAN2 in R1-2000163 (R2-1916572) RAN1 made conclusions on dynamic UL skipping for the following two cases. - Case 1: dynamic PUSCH skipping without overlapping CSI/HARQ-ACK on PUCCH - Case 2: dynamic PUSCH skipping with overlapping CSI/HARQ-ACK on PUCCH For Case 1, RAN1 agreed in RAN1#100-e meeting that when a UL grant without UL-SCH field or UL-SCH =1 (if present) is detected by a UE configured with skipUplinkTxDynamic, the corresponding PUSCH transmission is skipped by the UE if no transport block for the PUSCH transmission is generated by MAC and there is no CSI/HARQ-ACK on PUCCH overlapping with the PUSCH. For Case 2, RAN1 agreed that in Rel-15, the UE behavior is undefined and Case 2 can be addressed for Rel-16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To define the UE behavior for Case 2 in 38.321 section 5.4.3.1.3 that MAC entity shall not generate a MAC PDU for the HARQ entity if there is no UCI to be multiplexed on this PUSCH transmission as indicated by Layer 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent UE beahvior in RAN1 and RAN2 specification about UL skipping and ambiguous UE behavior for case 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: The CR has isolated impact on dynamic PUSCH skipping with overlapping CSI/HARQ-ACK on PUCCH. 1. If the gNB is implemented according to this CR while the UE is not, when there is UCI on PUCCH overlapping with a PUSCH, UE may or may not generate a MAC PDU. Then it may cause misunderstanding for gNB which PUSCH the UCI is multiplexed on. 2. If the UE is implemented according to this CR while the gNB is not, there is no impact on gNB. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.4.3.1.3			Allocation of resources

<unchanged part omitted>

The MAC entity shall not generate a MAC PDU for the HARQ entity if the following conditions are satisfied:

-	the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant; and

-	there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

-	there is no notification of UCI to be multiplexed on this PUSCH transmission received from lower layers; and

-	the MAC PDU includes zero MAC SDUs; and

-	the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR.

<unchanged part omitted>