3GPP TSG RAN WG1 #102e		R1-2005372

e-Meeting, August 17th – 28th, 2020

Source:	vivo

Title:	Discussion on channel access mechanism

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

In RAN#86 meeting, the study item of beyond 52.6GHz was approved [1]. The objectives of the study item are shown as below:

- Study of required changes to NR using existing DL/UL NR waveform to support operation between 52.6 GHz and 71 GHz

  - Study of applicable numerology including subcarrier spacing, channel BW (including maximum BW), and their impact to FR2 physical layer design to support system functionality considering practical RF impairments [RAN1, RAN4].

  - Identify potential critical problems to physical signal/channels, if any [RAN1].

- Study of channel access mechanism, considering potential interference to/from other nodes, assuming beam-based operation, in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6 GHz and 71 GHz [RAN1].

  - Note: It is clarified that potential interference impact, if identified, may require interference mitigation solutions as part of channel access mechanism.

In this contribution, we present system level evaluation results for co-channel coexistence in the 60 GHz band according to the evaluation assumptions agreed in RAN1 101-e [2]. Besides, the channel access scheme related issues are discussed as well.

- Discussion

System level evaluation result for no LBT case

In this section, we provide some coexistence evaluation results for no LBT when access the channel. The detailed simulation assumptions are provided in Table 5 of Appendix. Table 1 shows evaluation results for the indoor scenario with 1 or 2 operators, where DL only FTP 3 traffic model is used.

Table 1 The performance of no LBT with FTP 3 traffic model

| Number of Operators |  | 1 |  |  | 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load | Medium load | High load | Low load | Medium load | High load |
| DL UPT (Mbps) | 5%ile | 3511.53 | 2398.51 | 2114.52 | 2828.87 | 967.261 | 658.576 |
|  | 50%ile | 11980.30 | 8596.12 | 6182.75 | 10032.40 | 7210.65 | 3950.26 |
|  | 95%ile | 16748.90 | 15898.00 | 14704.20 | 15945.70 | 15310.50 | 11805.40 |
|  | mean | 11254.48 | 9182.38 | 7183.21 | 10319.99 | 7987.82 | 4613.32 |
| DL delay (s) | 5%ile | 0.012893 | 0.012894 | 0.012896 | 0.012892 | 0.012896 | 0.012906 |
|  | 50%ile | 0.018842 | 0.027168 | 0.038860 | 0.021868 | 0.032531 | 0.063088 |
|  | 95%ile | 0.057902 | 0.104559 | 0.139622 | 0.085918 | 0.260433 | 0.427172 |
|  | mean | 0.024549 | 0.037402 | 0.052655 | 0.030984 | 0.075809 | 0.119493 |
| Arrival rate (files/s) |  | 1 | 2 | 4 | 1 | 2 | 4 |
| 𝜌DL |  | 0.996612 | 0.998534 | 0.984599 | 0.997896 | 0.973332 | 0.921757 |
| BO |  | 0.108923 | 0.251225 | 0.520603 | 0.13337 | 0.388629 | 0.745707 |
| RU |  | 0.108748 | 0.250911 | 0.520125 | 0.133179 | 0.388323 | 0.745366 |

With the same served traffic load per UE and the same number of UEs per operator, if assuming that there is no interference between the two operators, the UPT should be the same for both scenarios.  However, as we see in Table 1, the mean delay of two operators’ scenario is more than doubled compared with that of one operator’s scenario. The mean UPT of two operators’ scenario decreases about 55.7% compared with that of one operator’s scenario when traffic load is high. Moreover, the 5%ile UPT result shows that the cell-edge UE performance degrades more when the arrival rate increases.

Observation 1: Interference is observed and degrades UE performance for DL-only traffic in two operators’ scenario for no LBT case.

Observation 2: The cell edge UEs observe strong interference and degrade the UPT, especially when traffic load increases for no LBT case.

ED (Energy Detection)-based LBT mechanism

Omni-directional LBT

A straight forward channel access scheme for NR beyond 52.6 GHz is to inherit a similar LBT mechanism as that in unlicensed 5GHz, i.e. a transmission node listens to the channel using an omni-directional antenna and then performs transmission to the reception node(s). In this case, as long as LBT succeeds, the transmission node could transmit data to its reception node(s) in any direction. The LBT procedure is almost the same as that in 5GHz [3], i.e. generating a random back off counter that is decreasing upon CCA succeeds and channel is considered as available when the counter becomes 0. The difference lies in that only one category is defined for 60GHz band here instead of 4 categories in 5GHz. However, the LBT procedure and parameters are not discussed in last meeting. To facilitate the calibration, the LBT procedure and parameters are better to be aligned between companies. The proposed detailed LBT parameters from our side are given in the table below based on [4].

Table 2  LBT related parameters

| Parameter | Value |
| --- | --- |
| ED threshold | -47dBm |
| CCA slot length (Tsl in [3]) | 5us |
| Maximum Channel Occupancy Time | 5ms |
| Contention Window Size | [3,7] |
| mp in [3] | 1 |
| Td in [3] | 8us |

Proposal 1: The LBT procedure and parameters in coexistence evaluation should be aligned between companies to facilitate the calibration with Table 2 as a starting point.

With simulation settings in Table 5 of Appendix and LBT settings in Table 2, the evaluation result for omni-directional LBT case in the indoor scenario with 2 operators is provided in Table 3:

Table 3  The performance of omni-directional LBT with FTP 3 traffic model

| Traffic load Metrics |  | DL-only |  |  |
| --- | --- | --- | --- | --- |
|  |  | Low load | Medium load | High load |
| DL UPT (Mbps) | 5%ile | 2696.53 | 1675.99 | 439.678 |
|  | 50%ile | 8026.89 | 6276.24 | 3978.61 |
|  | 95%ile | 16074 | 14496 | 11754.1 |
|  | mean | 9074.521 | 7230.419 | 5009.495 |
| DL delay (s) | 5%ile | 0.013395 | 0.013399 | 0.013429 |
|  | 50%ile | 0.026188 | 0.038917 | 0.052611 |
|  | 95%ile | 0.194774 | 0.22536 | 0.384937 |
|  | mean | 0.054551 | 0.066566 | 0.103715 |
| Arrival rate (files/s) |  | 1 | 2 | 4 |
| 𝜌DL |  | 0.99691 | 0.978412 | 0.89118 |
| BO |  | 0.614761 | 0.662126 | 0.82921 |
| RU |  | 0.14884 | 0.353069 | 0.638369 |

From Table 3, we can observe that compared with no LBT case results in Table 1, the omni-directional LBT scheme makes the BO larger because of the channel access procedure. Furthermore, the omni-directional LBT actually causes some performance loss for both UPT and delay when the traffic load is low. In high load, there’s some gain in terms of average UPT and delay. However, the omni-directional LBT is not effective to mitigate the strong interface observed by the edge UEs especially with high traffic load.

Observation 3: Compared to no-LBT scheme, the omni-directional LBT scheme causes some UPT performance loss when the traffic load is low. In high load, there’s some gain in terms of average UPT.

Directional LBT

Figure 1-1 					                                                     Figure 1-2

Figure 1 Example for directional LBT

Besides omni-directional LBT, directional LBT is another channel access scheme which needs to be investigated for 60GHz band. This is because most of the nodes operated in 60 GHz band likely will transmit using beam forming with massive MIMO antennas, including NR nodes, WiFi with 802.11ad/ay nodes, etc. Beamforming can strengthen the signal to the desired beam direction with minimum interference to other directions in spatial domain thus enable the spatial resource reuse. For directional LBT, the key difference with omni-directional LBT lies in that a transmission node listens to the channel using a directional antenna and then it is only allowed to perform transmission to the reception node(s) in the succeeding direction. In this way, directional LBT could offer benefits for spatial reuse in such high gain beamforming environment. Particularly, if traditional omni-directional channel sensing is performed, the transmission in any direction from other nodes will block the channel. For example, as shown in Figure 1-1, both the gNB and the WiFi AP transmit with beamforming. If omni-directional LBT is performed, only one node is allowed to transmit in the unlicensed spectrum, e.g. WiFi as shown in the left part of Figure 1-2. If beam specific LBT is performed at the gNB, then it is possible for both of the nodes to transmit at the same time as shown in the right part of Figure 1-2. The transmission of the gNB with beam 2 and 3 will not interfere with the transmission of the WiFi AP. Therefore, the gNB can transmit simultaneously with the WiFi AP. The spectrum efficiency is greatly increased compared with that of the omini-directional LBT scheme. However, the ED-based LBT is performed at the transmitter side, which cannot reflect the interference at the receiver side, especially when directional LBT is applied, the interference at the receiver side may be more severe. Therefore, it is not clear how much gain would be obtained and some evaluation is needed to facilitate the determination of channel access scheme in 60GHz.

Regarding LBT parameters, it should be mostly the same as that for omni-directional LBT. Though the way of calculating CCA energy need further discussion. There are two alternatives to do this:

- Alt. 1: CCA energy=Energy from one specific beamforming direction;

- Alt. 2: CCA energy=Energy from one specific beamforming direction – beamforming gain in the direction.

In our view, Alt. 1 could provide reasonable protection range for the coexistence since the transmission is also with beamforming gain. Then the CCA energy will be compared to ED threshold to determine whether the channel is available or not.

Proposal 2: Directional LBT should be studied and evaluated in 60 GHz band, where the way of calculating CCA energy should be clarified.

With simulation settings in Table 5 of Appendix and LBT settings in Table 2, the evaluation result for directional LBT case is provided in Table 4:

Table 4 Directional LBT performance with FTP 3 traffic model

| Traffic load Metrics |  | DL-only |  |  |
| --- | --- | --- | --- | --- |
|  |  | Low load | Medium load | High load |
| DL UPT (Mbps) | 5%ile | 1818.51 | 1442.62 | 266.586 |
|  | 50%ile | 6845.65 | 6138.78 | 3095.1 |
|  | 95%ile | 16088.4 | 15027.9 | 10452.9 |
|  | mean | 8068.932 | 6997.088 | 4064.593 |
| DL delay (s) | 5%ile | 0.013394 | 0.013397 | 0.013415 |
|  | 50%ile | 0.030708 | 0.037793 | 0.075792 |
|  | 95%ile | 0.911984 | 0.449702 | 0.660212 |
|  | mean | 0.145075 | 0.107469 | 0.164558 |
| Arrival rate (files/s) |  | 1 | 2 | 4 |
| 𝜌DL |  | 0.983044 | 0.977919 | 0.7508 |
| BO |  | 0.640702 | 0.722158 | 0.867895 |
| RU |  | 0.141522 | 0.332171 | 0.599902 |

Besides, the UPT bar graph to compare no LBT, omni-directional LBT and directional LBT is provided below:

From the above results, we can observe that, for the cell-centre UE, the no LBT scheme and the two ED-based LBT schemes have similar performances with different traffic loads. For median UE, the UPT performance of no LBT is better than that of omni-directional LBT and the directional-LBT, and the directional-LBT shows the worst performance.

Moreover, for the cell-edge UE, the three schemes have different performance gains with different traffic loads. Furthermore, the mean UPT of no LBT scheme is better than the other two with low and medium traffic load, and the omni-directional LBT scheme has better UPT performance with high traffic load. In a word, no LBT, omni LBT and directional LBT have different performance gain for different UEs in different scenarios. Though both the omni-directional LBT and the directional-LBT are not effective to mitigate the interface observed by the edge UEs especially with high traffic load.

Observation 4: No LBT, the omni-directional LBT and the directional LBT schemes have different performance gain for different UEs in different scenarios.

Observation 5: The omni-directional LBT and the directional-LBT are not effective to mitigate the interface observed by the edge UEs with high traffic load.

Receiver-assisted listen before talk (LBT) mechanism

In current regulation [4], LBT is mandatory for some regions in unlicensed 60 GHz band. Before a transmission or a burst of transmissions on an Operating Channel, the equipment shall perform a Clear Channel Assessment (CCA) check using "energy detect". The equipment shall observe the Operating Channel(s) for the duration of the CCA observation time measured by multiple CCA slot times of 5 μs. The Operating Channel shall be considered occupied for a slot time if the energy level in the channel exceeds the threshold corresponding to the power level. An extended CCA check is initiated at end of operating channel occupied. The transmission shall not start earlier than 8 μs while observing CCA empty. The transmission deferring continues for a random number of empty slots period.

The ED based LBT is performed at the transmitter side, which cannot reflect the interference at the receiver side, especially when directional LBT is applied, the interference at the receiver side may be more severe. As our evaluation results shown in the above section, the omni-directional LBT and the directional-LBT at the transmitter side are not effective to reflect the actual interface observed by the UEs with high traffic load. Therefore, to avoid the hidden node problem, a receiver assisted channel access scheme can be considered in the 60 GHz band.

Figure 2  Receiver assisted channel access scheme

An RTS/CTS-like scheme can be introduced in the NR unlicensed band as shown in Figure 2. There are 2 transmitters and two receivers, among them, transmitter 1 and receiver 2 will have interference with each other. When data arrives at the transmitter 1, it performs LBT and sends out RTS. The receiver 1 responds with CTS if the interference is not severe and the RTS can be decoded. Then the transmitter will start to transmit the data. During this data transmission period, data arrives at the transmitter 2. Transmitter 2 also performs LBT and then sends out RTS. However, due to the interference from transmitter 1, receiver 2 cannot correctly decode the RTS and is not able to respond with CTS at this time. After transmitter 1 finishes data transmission, transmitter 2 and receiver 2 can shake hands and start the data transmission and reception.

The RTS/CTS can be sent either in a dedicated RTS/CTS control channel or in PDCCH and PUCCH respectively. To differentiate with other channels, dedicated RNTI can be used for RTS/CTS. In the RTS, at least the receiver information should be included, e.g. UE ID or group ID if multiple UEs are the target receivers. Furthermore, the time duration that the channel will be occupied should also be indicated, like the NAV in WiFi. The time duration can also be included in the CTS, and the transmitter will detect the CTS in the scheduled resources.

Proposal 3: The receiver assisted channel access scheme should be considered in 60 GHz band and how to implement this handshaking mechanism in NR systems should be studied.

- Conclusion

In this contribution, we presented system level evaluation results and discussed some channel access schemes in 60GHz band. The proposals and observations are summarized below.

Observation 1: Interference is observed and degrades UE performance for DL-only traffic in two operators’ scenario for no LBT case.

Observation 2: The cell edge UEs observe strong interference and degrade the UPT, especially when traffic load increases for no LBT case.

Observation 3: Compared to no-LBT scheme, the omni-directional LBT scheme causes some UPT performance loss when the traffic load is low. In high load, there’s some gain in terms of average UPT.

Observation 4: No LBT, the omni-directional LBT and the directional LBT schemes have different performance gain for different UEs in different scenarios.

Observation 5: The omni-directional LBT and the directional-LBT are not effective to mitigate the interface observed by the edge UEs with high traffic load.

Proposal 1: The LBT procedure and parameters in coexistence evaluation should be aligned between companies to facilitate the calibration with Table 2 as a starting point.

Proposal 2: Directional LBT should be studied and evaluated in 60 GHz band, where the way of calculating CCA energy should be clarified.

Proposal 3: The receiver assisted channel access scheme should be considered in 60 GHz band and how to implement this handshaking mechanism in NR systems should be studied.

References

- RP-193259, “New SID: Study on supporting NR from 52.6GHz to 71 GHz”, RAN#86, Sitges, Spain, Dec. 2019.

- Chairman Notes, 3GPP TSG RAN WG1 #101 meeting.

- 3GPP TS 37.213 V16.2.0 (2020-06)

- ETSI EN 302 567 V2.1.1, 2017.

- 3GPP TR 38.901 V16.1.0 (2019-12)

- 3GPP TR 38.802 V14.2.0 (2017-09)

Appendix

Table 5  Summary of simulation assumptions for indoor scenario

| Parameters | Assumptions |
| --- | --- |
| Layout | 24 BSs are deployed in 120m * 50m box. 2 operators deploy 2 BSs randomly in each 10m * 10m box. |  |
| UE distribution | 100% Indoor, 3km/h. Average 5 users per BS. |  |
| Carrier frequency | 60 GHz |  |
| Carrier bandwidth | 2000 MHz CC |  |
| SCS | 960 kHz |  |
| Channel model | NR InH Open Office model in 38.901 [5] |  |
| Max. allowed BS Tx power | 40 dBm EIRP 20 dBm for 128 transmit antennas |  |
| Max. allowed UE Tx Power | 25 dBm EIRP 14 dBm for 16 transmit antennas |  |
| BS Antenna gain | 5 dBi |  |
| UE Antenna gain | 5 dBi |  |
| BS antenna height | 3 m |  |  |
| BS receiver noise figure | 7 dB |  |  |
| UE receiver noise figure | 10 dB |  |  |
| BS antenna pattern | InH ceiling mount in TR38.802 [6] |  |  |
| UE antenna pattern | UE Mode 1 in TR38.802 [6] |  |  |
| BS antenna array configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 2), dH = dV = 0.5 λ |  |
| UE antenna array configuration | (M, N, P, Mg, Ng) = (2, 2, 2, 2, 1), dH = dV = 0.5 λ Mechanic tilt: 0 degree |  |
| Traffic model | FTP3 with packet size of 27Mbyte. For FTP3 traffic model, low, median, high traffic load are needed. |  |
| UE receiver | MMSE-IRC as the baseline receiver |  |
| Data Processing Latency | K1=192 symbol |  |
| Channel estimation | ideal |  |
| Traffic type | DL only |  |
| Metric | UPT, Latency, Average BO, RU, Ratio of mean served throughput and offered cell throughput |  |
| Overhead | PDCCH, PUCCH: 2 symbol per slot CSI_RS, SRS: 0 |  |