3GPP TSG RAN WG1 #102-e			R1-2005378

e-Meeting, August 17th – 28th, 2020

Source: 	vivo

Title:  	Other issues for Rel-17 URLLC

Agenda Item:	8.3.4

Document for:	Discussion and Decision

Introduction

According to the IIOT/URLLC WID [1] as quoted below, one major objective is to support the time synchronization when a UE which is attached with a clock master transmits the gPTP message to another UE.

- Enhancements for support of time synchronization: - RAN impacts of SA2 work on uplink time synchronization for TSN, if any. [RAN2] - Propagation delay compensation enhancements (including mobility issues, if any). [RAN2, RAN1, RAN3, RAN4]

In this contribution, we provide our understanding on the potential enhancements of the TSN network.

Discussions

Figure 1: UE-to-UE gPTP message transmission

An example of UE-to-UE gPTP message transmission is illustrated in Figure 1. As two UE(s) need to be synchronized with the 5G GM via the reference time provided by the gNB, then the errors added by the two UEs would be doubled compared to the synchronization errors from gNB to UE.

Figure 2: Procedure of reference time provisioning and propagation delay compensation for CF RA

The following inaccuracy analysis is calculated after removing the propagation delay by using NTA/2. As shown in Figure 2, the whole procedure of reference time provisioning to the UE includes two separate procedures:

- Procedure 1: The gNB provides the referenceTime (which is pointed to a specific SFN) to the UE via SIB/DLInfomationTransfer.

- Procedure 2: The UE triggers RACH to obtain an NTA closest to the provided referenceTime, after receiving the propagation compensation indication required by the network.

The structure and error components of the following table are based on [2][3][4].

Table 1: Timing error in clock synchronisation

| Factors | Timing error type | Requirement for different SCS (kHz) (unit: ns) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15 | 30 | 60 | 120 |
| 1 | Inaccuracy caused by downlink/uplink frame timing alignment |  |  |  |  |
| 1.1 | Time alignment error of gNB transmitter (TETAE) | 65 | 65 | 65 | 65 |
| 1.2 | Inaccuracy caused by UE detection (TEUE-DL-RX) | 260 | 260 | 163 | 98 |
| 1.3 | Inaccuracy caused by propagation delay when propagation delay is decided from TA. (1.3.1+1.3.2+1.3.3)/2 | 375 | 245 | 192.5 | 119 |
| 1.3.1 | Inaccuracy of UE transmitting (TEUE-DL-to-TX) | 390 | 260 | 228 | 114 |
| 1.3.2 | Inaccuracy of gNB detection (TEUL-RX) | 100 | 100 | 92 | 92 |
| 1.3.3 | Inaccuracy caused by TA indication (TETA-G) | 260 | 130 | 65 | 32 |
| 1.4 | Inaccuracy caused by propagation delay when considering 20km2 service area (TEPD) | 8410 | 8410 | 8410 | 8410 |
| 2 | Inaccuracy caused by time reference information delivery from gNB to UE (TERT) | 5 | 5 | 5 | 5 |
| 3 | Inaccuracy caused by the synchronisation between the gNB and external clock (TEEC) | 100 | 100 | 100 | 100 |
| Total error | Total inaccuracy for 20 km2 service area (with propagation compensation) = (2*(1.1+1.2+1.3+2+3)) | 1610 | 1350 | 1051 | 774 |

Firstly, we can confirm that the propagation delay compensation is based on NTA/2. Then we do not have to introduce other measurements and procedures, given that the TA which is always needed for the uplink synchronization can be obtained via the RACH procedure.

Proposal 1: NTA/2 is used for the propagation delay compensation of the reference time.

According to the analysis given in the above table, the inaccuracy of 15/30/60 KHz SCS cannot fulfil the TSN clock synchronization requirements of 1us.

Observation 1: After the propagation delay compensation, the inaccuracy of 15/30/60 KHz SCS cannot fulfil the TSN clock synchronization requirements.

Therefore, even with the propagation delay compensation, additional enhancements are necessary and following aspects can be considered:

- More accurate DL signal detection (1.2)

- More accurate UL signal transmission (1.3.1)

- More accurate UL signal detection (1.3.2)

- Finer granularity of TA indication (1.3.3)

With above analysis, RAN1 should discuss the potential enhancements for the propagation delay compensation.

Proposal 2: RAN1 should discuss the potential enhancements for the propagation delay compensation:

- More accurate DL signal detection (1.2)

- More accurate UL signal transmission (1.3.1)

- More accurate UL signal detection (1.3.2)

- Finer granularity of TA indication (1.3.3)

Conclusions

Based on the analysis given above, we have the following Observations and Proposals:

Observation 1: After the propagation delay compensation, the inaccuracy of 15/30/60 KHz SCS cannot fulfil the TSN clock synchronization requirements.

Proposal 1: NTA/2 is used for the propagation delay compensation of the reference time.

Proposal 2: RAN1 should discuss the potential enhancements for the propagation delay compensation:

- More accurate DL signal detection (1.2)

- More accurate UL signal transmission (1.3.1)

- More accurate UL signal detection (1.3.2)

- Finer granularity of TA indication (1.3.3)

Reference

[1] RP-201310, Nokia, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”.

[2] R2-1900838, vivo, “Potential enhancements on the reference time accuracy”.

[3] R2-1818254, NTT DOCOMO, INC., “Time Synchronisation for IIOT”.

[4] R1-1900935, Nokia, “Discussion on the RAN2 LS on TSN requirements evaluation”.

[5] 3GPP TR 22.104, “Service requirements for cyber-physical control applications in vertical domains”.

Annex A

The following texts are extracted from [4]:

- 3.2.1 gNB transmit time alignment error (TETAE) The maximum gNB transmit time alignment error is defined in 38.104 Sec. 6.5.3. At a single carrier, the maximum error is to be within ±65ns. - 3.2.2 Propagation delay (TEPD) This error source is only applicable for the case of small service areas without propagation delay / timing advance compensation. - 3.2.4 UE transmit timing error (TEUE-DL-to-TX) The maximum transmit timing error which is allowed according to 38.133 Sec. 7.1.2 is given by the following table: Table 7.1.2-1: Te Timing Error Limit Frequency Range; SCS of SSB signals (KHz); SCS of uplink signals s(KHz); Te. 1; 15; 15; [12]*64*Tc. 30; [10]*64*Tc. 60; [10]*64*Tc. 30; 15; [8]*64*Tc. 30; [8]*64*Tc. 60; [7]*64*Tc. 2; 120; 60; [3.5]*64*Tc. 120; [3.5]*64*Tc. 240; 60; [3]*64*Tc. 120; [3]*64*Tc. Note 1: Tc is the basic timing unit defined in TS 38.211 [6] Editor’s note: The final values of Te for 120KHz SSB SCS are subject to further discussions in further meeting, and may not be outside 3*64*Tc to 3.5*64*Tc.. - 3.2.5 UE reception timing error (TEUE-DL-RX) The achievable synchronization accuracy will be very much dependent on the (bandwidth of the) used reference signal and the possible related time domain resolution in the estimation process. For FR1 and 30kHz SCS, the t-sync accuracy based on PSS/SSS will be in principle limited to about ±8Ts (~260ns) whereas for PBCH DM-RS the limitation is reduced to ~130ns. - 3.2.6 gNB UL receive timing estimation error (TEUL-RX) The UL receive timing of UE can be estimated at gNB based on any UL reference signal, such as PUSCH DM-RS, PUCCH DM-RS, or SRS. In applications such as closed-loop motion control, we may assume that PUSCH will be available in each motion control cycle. Here we analyse the error of short-term, single-shot UL receive timing estimation on PUSCH. The short-term UL receive timing estimation error on PUSCH consists of two components: CIR peak detection error and error due to time sample granularity. The peak detection error is determined based on simulations and shown below, expressed as the deviation from the true TO in terms of number of FFT samples. The simulations results shown in Figure 3.2-1 assume 5.4 GHz carrier frequency, AWGN channel, and 2Tx2Rx. The results show close to perfect detection for a larger PUSCH allocation of #PRB≥64. For a very small PUSCH allocation of 4 PRBs and low SNR, e.g. -5 dB ≤ SNR ≤ 0 dB, the detection error is in the order of ~±5 time samples, e.g. ~±5·64∙Tc/2μ corresponding to ~±160ns for 15kHz SCS. Specifically, for highly-reliable communication in the low SNR regime a larger PRB allocation is needed to support low MCS and therefore, we think that the higher PRB allocations are more typical in this respect. - 3.2.7 Timing advance adjustment granularity (TETA-G) The timing adjustment granularity (as specified in 38.213 Sec. 4.2) is dependent on the applied SCS and is defined as 16∙64∙Tc/2μ. The quantization error is half the indication granularity, meaning that the maximum error is given by: TETA-G=8∙64∙Tc/2μ Specifically, for lower SCS values such as 15kHz the TETA-G,15kHz=260ns which will take a large part of the UE timing accuracy budget of 1μs (13%) whereas for higher SCS such as 120kHz this contribution will play only have a minor effect (TETA-G,120kHz=32ns). - 3.2.8 Relative Timing Advance adjustment accuracy (TETA-err) The maximum allowed values are specified in TS 38.133 Sec. 7.3.2.2 in the following table: Table 7.3.2.2-1: UE Timing Advance adjustment accuracy Sub Carrier Spacing, SCS kHz; 15; 30; 60; 120. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc.

Annex B

The following requirements are extracted from the TR 22.104:

- 5.6.2 Clock synchronisation service performance requirements Table 5.6.2-1: Clock synchronization service performance requirements User-specific clock synchronicity accuracy level; Number of devices in one Communication group for clock synchronisation; Clock synchronicity requirement; Service area; Scenario. 1; Up to 300 UEs; < 1 µs; ≤ 100 m x 100 m; -Motion control -Control-to-control communication for industrial controller. 2; Up to 10 UEs; < 10 µs; ≤ 2500 m2; - -High data rate video streaming. 3; Up to 100 UEs; < 1 µs; < 20 km2; - -Smart Grid: synchronicity between PMUs.