3GPP TSG RAN WG1 Meeting #102-e	R1-2005420

e-Meeting, August 17th – 28th, 2020

Source: 	ZTE

Title: 	Remaining Issues of Power Control for NR-DC

Agenda item:	7.2.10

Document for:	Discussion/Decision

# Introduction

After extensive discussion during Q1 and Q2, most of the issues on Rel-16 MRDC/CA WI have been resolved. In this contribution, discussion and analysis on the remaining maintenance issues of power control for NR-DC are presented.

# Discussion

## Power Control for NR-DC

During RAN1#101e meeting, RAN2 sent a reply LS to inform RAN1 that RAN2 has introduced new inter-node signaling for T_offset exchange between nodes (as shown below) to support dynamic power sharing with look-ahead.

- MN signals the maxToffset restriction (i.e. maxToffset) in CG-ConfigInfo to SN, and SN shall respect the restriction when deciding the SCG configuration, such that Tproc,SCGmax <= maxToffset. - RAN2 understanding is that if SN cannot accept the maxToffset restriction set by MN, SN can at least reject the procedure. RAN2 companies assume that current procedures will be reused. - RAN2 understanding is that upon receiving and accepting maxToffset restriction from MN, SN can provide the actual maxToffsetSCG (e.g.Tproc,SCGmax) in IE requestedToffset according to the SCG configuration. - SN may request, in CG-Config, a change in the maxToffset restriction imposed by MN. The SN may request MN to increase/decrease maxToffset and It is up to the MN to decide whether to and how to respond to the SN request.

With the above inter-node signalling and RAN1’s previous working assumption for determination of T_offset, the whole procedure of determining T_offset can be summarized as below.

1. MN computes the Tproc,MCGmax according to the MN’s configuration;

2. MN determines its preferred maxToffset (Toffset1) and send it to SN via CG-ConfigInfo, where Toffset1 ≥Tproc,MCGmax;

3. If SN accepts Toffset1, then SN shall respect the restriction when deciding the SCG configuration, such that Tproc,MCGmax≤Toffset1; or

If SN can NOT accept Tofset1, the SN can reject the procedure; or

If SN can NOT accept Toffset1, the SN may request a change in Toffset1 imposed by MN via CG-Config.

4. It is up to the MN to decide whether to and how to respond to the SN request.

Proposal 1: Confirm previous RAN1 working assumption on determination of T_offset made in RAN1#100e together with the updates in RAN1#101bis-e.

Based on the avoid analysis, the newly introduced inter-node signaling for T_offset exchange is feasible from RAN1 perspective to make the whole dynamic power sharing with look-ahead work smoothly. In this case, the newly introduced inter-node signaling for T_offset exchange can be confirmed from RAN1 perspective.

Another question from RAN2 is the value range of T_offset. Based on the current specification, the range of Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and Tproc,CSImux for different numerologies is from 0.34ms to 3ms. In this case, the range of T_offset can be 0.375ms to 3ms and granularity can be 0.125ms, which is corresponding to the slot length of 120 KHz SCS.

Proposal 2: The range of maxToffset is 0.375ms to 3ms and the granularity of maxToffset is 0.125ms.

## TP

As Rel-16 NR-DC covers both the FR1 and FR2 cases, it seems the following update is needed for FR2.

Proposal 3: Introduce the following TP1 for 38.213.

TP1: {38.213: 7.6.2 NR-DC}

7.6.2 NR-DC If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5. If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2-r16 and a maximum power for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2-r16 and with an inter-CG power sharing mode by nrdc-PCmode-FR1-r16 for FR1 and/or by nrdc-PCmode-FR2-r16 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range. If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR2-r16, or semi-static-mode2 for nrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR2-r16, the UE does not expect and to be configured such that , where is the linear value of , is the linear value of , and is the linear value of a configured maximum transmission power for NR-DC operation in FR1 or FR2 as defined in [8-3, TS 38.101-3].

# Conclusion

In this contribution, remaining issues on power control for NR-DC are discussed with the following proposals.

Proposal 1: Confirm previous RAN1 working assumption on determination of T_offset made in RAN1#100e together with the updates in RAN1#101bis-e.

Proposal 2: The range of maxToffset is 0.375ms to 3ms and the granularity of maxToffset is 0.125ms.

Proposal 3: Introduce the following TP1 for 38.213.

TP1: {38.213: 7.6.2 NR-DC}

7.6.2 NR-DC If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5. If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2-r16 and a maximum power for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2-r16 and with an inter-CG power sharing mode by nrdc-PCmode-FR1-r16 for FR1 and/or by nrdc-PCmode-FR2-r16 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range. If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR2-r16, or semi-static-mode2 for nrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR2-r16, the UE does not expect and to be configured such that , where is the linear value of , is the linear value of , and is the linear value of a configured maximum transmission power for NR-DC operation in FR1 or FR2 as defined in [8-3, TS 38.101-3].

# Reference

R1-2004922, LS Reply to RAN1 on UL PC for NR-DC, RAN1#101e.