3GPP TSG RAN WG1 #102-e                                                 R1-2005428

e-Meeting, August 17th – 28th, 2020

Source: 	ZTE Corporation

Title: 	Discussion on potential techniques for PUCCH coverage enhancement

Agenda item:	8.8.2.2

Document for:	Discussion/Decision

Introduction

In the RAN plenary #86 meeting, a new SID on NR coverage enhancement was approved [1]. One objective of this study item is to identify the performance target for coverage enhancement for specific scenarios for both FR1 and FR2 and study the potential solutions for both DL and UL.

The baseline coverage performance for both UL and DL based on link-level simulation is provided in our companion contribution [2]~[3]. In this contribution, we discuss the potential techniques for coverage enhancements for PUSCH and PUCCH.

Potential techniques for PUCCH

Based on the evaluation results provided in our companion contribution [2][3], there still exists some scenarios that need coverage enhancement for PUCCH. Therefore, in this section, we give some potential techniques for PUCCH coverage enhancement

Dynamic PUCCH repetition

In Rel-16, PUSCH repetition number can be dynamically indicated. However, the repetition number for PUCCH is RRC configured based on large scale characteristic of propagation condition. Such a semi-static configuration cannot be adaptive to the instantaneous radio channel condition. To ensure the reliability of PUCCH, a conservative repetition factor will be set up. This wastes radio resources which otherwise can be used for accommodating more coverage limited UE. In this regard, dynamic indication of PUCCH repetition can be considered.

To investigate the necessity of dynamic PUCCH repetition, we first evaluate the required SNR for different repetitions. Figure 3 shows the performance of different UCI payload with different repetition for urban scenario. And Table 2 lists the required SNR for different cases. The simulation assumptions are listed in Table A1 in the Annex.

Figure 3. Performance of PUCCH format 3 with different repetition factors.

Table 2. Required SNR (dB) to satisfy BLER <=0.01

| Repetition number | Urban_TDL-C_3km/h_4Rx |  |  |
| --- | --- | --- | --- |
|  | 3bits | 11bits | 22bits |
| RP=2 | -13.48 | -10.82 | -8.76 |
| RP=4 | -15.46 | -12.8 | -10.71 |
| RP=8 | -17.6 | -14.52 | -12.65 |

Based on Table 2, a simulation is conducted at an SNR of -12.8dB, which is the required SNR for the case with 11bits UCI and RP=4. The simulation is to get the distribution of instantaneous received SNR at certain RBs (which are assumed to be configured for PUCCH transmission) and to see the percentage of instantaneous received SNR higher than the required SNR for RP=4. Without loss of generality, the RBs configured for PUCCH transmission is randomly selected within the whole bandwidth. Figure 4 gives the distribution of the instantaneous SNR over interested RBs. From Figure-4, we find that at least 90% of the instantaneous SNR on the RBs assumed to be configured for PUCCH transmission exceed the required SNR and more than 70% cases can be indicated to 2 repetitions instead. Therefore, dynamic repetition should be considered if the instantaneous SNR can be obtained by gNB.

Figure 4. Distribution of instantaneous SNR on each RB

Proposal 1: Dynamic PUCCH repetition can be considered for NR coverage enhancement to increase the resource utilization efficiency.

Sequence based design for PUCCH with more than 2 bits

In current NR specification, DMRS less design is adopted for PUCCH format 0 to support up to 2bits. That is, different information bits are carried by different sequences transmitted on the PUCCH. This is also referred as “Sequence based PUCCH”. For more than 2 bits, channel coding based design is adopted for PUCCH format 2/3/4. That is, RM coding is used for 3~11 bits, and polar coding is applied for more than 11 bits. To evaluate the possible extension of sequence based design to more than 2 bits, we simulate the case with PUCCH format 3 carrying 11 bits UCI in Figure 5. Compared to legacy NR PUCCH format 3, about 2~3 dB gain at BLER@10^-3 for sequenced based PUCCH with 11 bits payload size is achieved. The simulation assumptions are listed in Table A1 in the Annex.

Figure 5. Performance comparison of sequence based PUCCH and PUCCH format 3

The performance gain of sequence based PUCCH may come from several factors, such as all energy/resources are spent for useful information transmission, and non-coherent detection is applied to avoid poor channel estimation at low SNR. Considering the clear performance gain, we suggest that sequences based design should be considered for PUCCH coverage enhancement, especially for small to medium UCI payload, i.e., 3~11bits.

Observation 1: For UCI of 11 bits, sequence based PUCCH can provide about 2~3 dB gain over legacy PUCCH format 3.

Proposal 2: Sequence based PUCCH should be considered for coverage enhancement for small to medium UCI payload.

In the following, we give some preliminary considerations on the design aspects for sequence based PUCCH. First of all, for sequence based design with larger UCI payload, we should determine how many resources that sequences can be mapped to. In current NR, the time-frequency resources that can be used for PUCCH format 3 vary from 4 symbols to 14 symbols in time domain, and 1 RB to 16 RBs in frequency domain. If the sequence based PUCCH also has such configurable resources (up to 11*16 combinations) for sequences to map to, it would be a great challenge since we need to design sequences with different length for each time-frequency combination. Therefore, we should determine the total resources for sequence mapping first, then study the potential methods for sequence design. Considering it is for coverage enhancement, the time domain symbols within a slot should be used as many as possible. And due to the limit UE transmit power, the transmit power per RE reduces as the number of transmitted RB increases. Therefore, we take 1RB with 14 symbols as a baseline for the sequence based PUCCH design. That is, there are 168 REs available for sequence mapping.

Secondly, we should consider what type of sequence should be used. There are at least two types of sequence can be considered.

- Option 1: long sequence based

- Option 2: short sequence based

For Option 1, a long sequence with length 168 shall be mapped to all PUCCH resource elements in both time and frequency domain, which is somewhat similar to the NB-SSS. In such design, low PAPR/CM property cannot be guaranteed when DFT-S-OFDM is used even if the long sequence has low PAPR/CM. Therefore, it may not be suitable for the coverage enhancement scenario. So we think the sequence design based on long sequence should not be considered.

For Option 2, a short sequence with length-12 is mapped to each symbol. This method enable low PAPR when the short sequence with length-12 is based on low PAPR/CM design. The low PAPR sequence adopted in current NR can be a starting point.

Proposal 3: Short sequence with length equal to the number of RE in one OFDM symbol is the starting point for sequence based PUCCH design.

Conclusion

According to the analysis given above, we have the following observations and proposals:

Proposal 1: Dynamic PUCCH repetition can be considered for NR coverage enhancement to increase the resource utilization efficiency.

Observation 1: For UCI of 11 bits, sequence based PUCCH can provide about 2~3 dB gain over legacy PUCCH format 3.

Proposal 2: Sequence based PUCCH should be considered for coverage enhancement for small to medium UCI payload.

Proposal 3: Short sequence with length equal to the number of RE in one OFDM symbol is the starting point for sequence based PUCCH design.

Reference

3GPP RP-193240, “New SID on NR coverage enhancement”, China Telecom, RAN#86, Sitges, Spain, December 9th – 12th, 2019.

3GPP RAN1#102-e, R1-2005425, Discussion on baseline coverage performance for FR1, ZTE

3GPP RAN1#102-e, R1-2005426, Discussion on baseline coverage performance for FR2, ZTE

- Appendix

- Table A-1 Simulation assumption for PUCCH

| - Parameters | - Value |
| --- | --- |
| - Carrier frequency | - 4 GHz |
| - Channel | - TDL-C , DS = 300ns |
| - UE speed | - 3 km/h |
| - Antenna configuration | - 1Tx, 4Rx |
| - Number of symbols | - 14 |
| - Number of RBs | - 1 |
| - Number of repetition | - 1/2/4/8 |
| - DMRS location | - The 3rd ,10th symbol |
| - Payload | - 3/11/22 bits |
| - Coding | - RM for legacy PUCCH format 3, or - Sequence based PUCCH with 2^11 sequences. |
| - BLER | - 1% |