3GPP TSG RAN WG1 Meeting #102-e	R1-2005439

e-Meeting, August 17th – 28th, 2020

Source: 	ZTE

Title: 	Preliminary Simulation Results of Rel-17 MBS

Agenda item:	8.12.4

Document for:	Discussion/Decision

Introduction

During RAN#88e meeting, the revised WI on NR MBS has been approved with the following objectives [1]. There are mainly three objectives related to RAN1.

1. Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service for RRC_CONNECTED UEs;

2. Specify required changes to improve reliability of Broadcast/Multicast service for RRC_CONNECTED UEs;

3. Specify RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states.

In this contribution, regarding the objective of improving reliability of Broadcast/Multicast service for RRC_CONNECTED UEs, some preliminary simulation results of CSI feedback are given.

Discussion

Based on the analysis in [2], in NR system, gNB can perform better link adaptation by flexible CSI-RS resource configuration and CSI Report configuration. So, in order to improve the reliability of Broadcast/Multicast service for RRC_CONNECTED UEs, CSI feedback can be considered for MBS PDSCH.

In the following sections, preliminary simulation results with or without CSI feedback are provided.

Simulation assumption

For MBS PDSCH, three simulation schemes are evaluated and the detailed simulation assumptions in Table A-1 in the appendix.

- Scheme 1: MBS transmission without CQI and PMI feedback. gNB determines CQI and PMI based on the RSRP feedback and PMI cycling, respectively.

- Scheme 2: MBS transmission with CQI feedback but without PMI feedback. Network determines CQI and PMI based on the CQI feedback and PMI cycling, respectively.

- Scheme 3: MBS transmission with CQI and PMI feedback. Network determines CQI and PMI based on the CQI and PMI feedback, respectively.

- Note: in our simulation, 2 Tx rank1 precoding codebook is used.

UEs are separated into different MBS groups, and the UEs within the same MBS group apply the same beam direction. Scheme 1 is for broadcast especially for RRC idle/inactive UEs. Scheme 2 is for multicast for RRC connected UE where PMI feedback is disabled. Scheme 3 is for multicast for RRC connected UE where PMI feedback is enabled.

Simulation results

The MBS PDSCH simulation results of comparison between 2-port MBS transmission without CQI/PMI feedback and 2-port MBS transmission with CQI/PMI feedback is summarized in Table 1.

Table 1: Comparison between 2-port MBS transmission without CQI/PMI feedback and 2-port MBS transmission with CQI/PMI feedback

| Simulation schemes |  | Cell spectrum efficiency |
| --- | --- | --- |
| Scheme 1 (Baseline) | Spectrum efficiency (bps/Hz) (Baseline: 2-port MBS transmission without CQI/PMI feedback) | 1.0646 |
| Scheme 2 | Spectrum efficiency (bps/Hz) | 1.4724 |
|  | Gain compared with scheme 1 | 38.31% |
| Scheme 3 | Spectrum efficiency (bps/Hz) | 2.1759 |
|  | Gain compared with scheme 1 | 104.4% |

Based on the results, it can be observed that,

- The performance of multicast transmission in RRC connected state with CSI feedback outperforms that of broadcast in RRC idle/inactive state.

- The performance of the MBS transmission with both CQI and PMI feedback is much better than that of pure CQI feedback.

From performance perspective, NR MBS should be served in RRC connected state as much as possible. CQI and PMI feedback should be supported for MBS transmission.

Observation 1:

- The performance of multicast transmission in RRC connected state with CSI feedback outperform that of broadcast in RRC idle/inactive state.

- The performance of the MBS transmission with both CQI and PMI feedback is much better than that of pure CQI feedback.

Proposal 1: From performance perspective, NR MBS should be served in RRC connected state as much as possible. CQI and PMI feedback should be supported for MBS transmission.

Conclusion

In this contribution, some preliminary simulation results of CSI feedback for NR MBS are presented with the following observation and proposal.

Observation 1:

- The performance of multicast transmission in RRC connected state with CSI feedback outperforms that of broadcast in RRC idle/inactive state.

- The performance of the MBS transmission with both CQI and PMI feedback is much better than that of pure CQI feedback.

Proposal 1: From performance perspective, NR MBS should be served in RRC connected state as much as possible. CQI and PMI feedback should be supported for MBS transmission.

Reference

RP-201038, WID revision: NR Multicast and Broadcast Services, RAN#88e.

R1-2005437, Mechanisms to Improve Reliability for RRC_CONNECTED UEs, RAN1#102e.

Appendix

Table A-1 Simulation assumptions of CSI feedback

| Parameters | Value |
| --- | --- |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| UE Tx power | 23dBm |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| BS antenna configurations | 2 Tx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 1) |
| BS antenna height | 25 m |
| UE antenna configuration | 2 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) |
| UE antenna height | 1.5m |
| UE antenna gain | 0dBi |
| BS Tx power | 49 dBm per 10 MHz |
| BS receiver | MMSE-IRC |
| UE receiver noise figure | 9 dB |
| SCS | 15 kHz |
| Simulation bandwidth | 10 MHz |
| Layout | Single layer - Macro layer: Hex. Grid |
| Channel model | UMa in TR 38.901 |
| Number of UEs per MBS group | 4 |
| UE distribution | 80% of users are indoor: 3 km/h |
| HARQ | Max number of transmissions = 1 with target BLER = 1% |
| Channel estimation | Ideal |
| CSI feedback Period and scheduling delay | 5 , 2 |
| CSI feedback mode | Sub-band feedback |
| Traffic model | Full buffer |