3GPP TSG RAN WG1 #102-e																   R1-2005489

e-Meeting, August 17th – 28th, 2020

Agenda Item:		8.1.5

Source:				InterDigital, Inc.

Title:						Evaluation of Multi-TRP MIMO Deployment with Multi-panel UEs

Document for:		Discussion and Decision

# Introduction

Per the agreed WID for Rel-17 eMIMO at the RANP #86 [1], four main objectives were identified to be studied by RAN1. This sub-agenda item is focused on parts of the second objective related to beam management enhancements for multi-TRP:

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception.

In this contribution, we provide some system level evaluation results in NR MIMO deployments. Specifically, we study the potential gains achievable by using multi-panel UEs in a multi-TRP transmission scenario.

# Background

The use cases of MIMO evolve as new scenarios emerge. For example, FD-MIMO was introduced in LTE by employing two-dimensional antenna arrays. By giving a TRP the ability to steer its beams, FD-MIMO provides a better spatial coverage of UEs located in a 3D space. The work in NR MIMO is built upon the significant progress achieved with FD-MIMO that is being extended to operate in the FR2 range. This means that new UE architectures with multi-panel capability should be further studied to better understand their potential use cases and benefits. In this contribution, we provide some system level simulation results in different NR MIMO FR2 deployments where UEs are equipped with multi-panels. We also study the impact of using multi-TRPs with different assumptions on UE panel usage.

# Simulation assumptions

We use the simulation assumptions from the NR MIMO Phase I system level calibration [2]. The deployments are evaluated at the carrier frequency of 30 GHz. The UEs are uniformly distributed within the deployment area. UEs are assumed to be equipped with two antenna panels that are placed on opposite sides of the device. In other words, each panel is oriented 180 degrees away from the other one such that their boresights face opposite directions. Employed antenna arrays are based on uniformly distributed structures.

Both UEs and TRPs are assumed to have analog beamforming capability covering multiple spatial directions. UE and TRP sweep through all possible TX/RX beam-pairs to find the one which yields the highest RSRP. We consider several cases for pairing a UE with a TRP:

Single TRP

Case 1:

- UE activates only one panel. The selection of the panel is random to simulate random holding/orientation by/of the user.

- UE finds the TRP that results in the best beam-pair to the activated panel.

Case 2:

- UE activates both panels.

- UE finds the TRP that results in best beam-pairs per panel, but only selects the best of the two beam-pairs.

Case 2a:

- UE activates both panels.

- UE finds the TRP that results in best beam-pairs per panel, UE combines the signal received by the panels.

Multiple TRP

Case 3:

- UE activates only one panel. The selection of the panel is random to simulate random holding/orientation by/of the user.

- UE finds the two TRPs that result in the best beam-pair for the activated panel. UE receives and processes the over-the-air combined signals.

Case 3a:

- The UE activates both panels.

- UE finds the two TRPs and the panel that results in the best beam-pair for the activated panel. UE receives and processes the over-the-air combined signals.

Case 4:

- The UE activates both panels.

- UE finds the two TRPs that results in best beam-pairs per panel, UE receives and processes the signal received by the panels.

A detailed list of simulation assumptions is available in the Appendix.

# Evaluation results

In Figure 1, we show the results for cases with a single TRP in an open office scenario, and compare the performance with different assumptions on activated panels and combining mechanism. We notice that case 2 provides a substantial 4-5 dB gain over case 1 because both panels are active and allow the UE to search for the best possible beam-pair. In an open office scenario, the TRPs are densely packed within a room which gives several potential candidate TRPs. Nevertheless, being able to find the best panel orientation towards a serving TRP makes a significant difference. In Case 2a, where both panels are activated and combining is used, a slight additional gain is obtained. Given the panel placement at the UE, it is always likely that one of the panels is facing more towards a serving TRP than the other one. The panel with less favorable orientation might be still helpful in reception, but obviously its benefit is limited due to its direction. Figure 3 shows the same cases as Figure 1assuming UMa scenario. We notice a similar 4-5 dB gain from using the best panel pair over using a random panel pair.

Observation 1: Multiple activated panels yield significant gain when single TRP is used.

In Figure 2, we plot the cases with multi-TRP in an open office scenario. We notice that similar to the single TRP case, case 3a with best panel to different TRPs provides a significant gain of 2-4 dB compared to random panel activation. Given the density of cells, it is easy for a UE to find two TRPs with the best possible panel orientation and proper beam directions. Case 4 shows that using another panel may help in some cases to find a different TRP than the serving TRP for the other panel. However, in some cases, there are UEs for which both best beam-pairs are on the same panel. Comparing to Figure 1, we see that case 2a with single TRP and multi-panel yields almost the same performance as case 3a and 4. Substantial gains can be achieved when both panels are activated to search for best beam-pairs without needing a second TRP. A similar observation is made in the UMa scenario between Figures 3 and 4.

Observation 2: Similar gains can be achieved with single TRP multi-panel compared to multiple TRPs.

In Figure 4, we plot the same cases as Figure 2 but assuming the UMa scenario. The best panel activated yields ~0.7-1 dB gain over random activation. The TRPs are spaced wide apart, resulting a small number of TRPs compared to the open office. This results having few possible beam-pair combinations. Then, compared to the open office case, it is hard for a UE to find a second TRP that can significantly help the serving cell. Therefore, there may not be a beam-pair with a significantly better performance than the others. As shown in Figure 4, case 4 performs slightly better than case 3a, because of using the second active panel could provide some gain for most UEs. With a sparse TRP deployment and less beam options, it’s less likely to find two good beam-pairs from one panel other than the best one to the closest TRP. Then a second panel with a different orientation helps to find alternative TRPs that are in other directions.

Observation 3: Multi-panel is beneficial for multi-TRP when TRPs are sparsely deployed.

Figure 1																Figure 2

Figure 3 																Figure 4

# Conclusions

This contribution evaluated system level performance of NR MIMO in an FR2 deployment with multi-panel UEs and multi-TRP. Based on the presented discussion, we make the following observations:

Observation 1: Multiple activated panels yield significant gains when single TRP is used.

Observation 2: Similar gains can be achieved with single TRP multi-panel compared to multiple TRPs.

Observation 3: Multi-panel is beneficial for multi-TRP when TRPs are sparsely deployed.

# References

- RP-193133, “New WI: Further enhancements on MIMO for NR”, Samsung, 3GPP RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- R1-1703534, “Evaluation assumptions for Phase 1 NR MIMO system level calibration”, ZTE, 3GPP RAN1 Meeting #88, Athens, Greece, February 13th-17th 2017

# Appendix

Table 1 InH Open Office deployment

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Channel Model | Indoor in TR 38.901 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection for serving TRP | Select the best beam-pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. |
| Constraints for the range of selective beams per TRP sector | With analog TX/RX beamforming, using a single digital TX/RX port. The first TXU/RXU in a single fixed panel is used with single-pol. - Beam directions for TRP: - Azimuth angle [pi/16 3*pi/16 5*pi/16 7*pi/16 9*pi/16 11*pi/16 13*pi/16 15*pi/16] - Zenith angle [pi/8 3*pi/8 5*pi/8 7*pi/8] - Beam directions for UE: - Azimuth angle [-3*pi/8 -pi/8 pi/8 3*pi/8] - Zenith angle [pi/4 3*pi/4] |
| ISD | 20m |
| BS Tx power | 23dBm |
| Number of sites | 12 |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,1,1). (dV,dH) = (0.5, 0.5)λ Boresight direction is perpendicular to the ceiling |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; The polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See wall-mount in Table A.2.1-7 in TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |
| BS antenna height | 3m |
| UE antenna height | 1.5 m |
| UE antenna gain | 5dBi |
| Noise figure for BS | 7dB |
| UE receiver noise figure | 10dB |
| UE distribution | 100% Indoor, 3km/h, 10 users per BS |
| Metric | CDF of SNR with beamforming |

Table 2 UMa deployment

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Channel Model | UMa in TR 38.901 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection for serving TRP | Select the best beam-pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. |
| Constraints for the range of selective beams per TRP sector | With analog TX/RX beamforming, using a single digital TX/RX port. The first TXU/RXU in a single fixed panel is used with single-pol. - Beam directions for TRP: - Azimuth angle [-5*pi/16 -3*pi/16 -pi/16 pi/16 3*pi/16 5*pi/16] - Zenith angle [5*pi/8 7*pi/8] - Beam directions for UE: - Azimuth angle [-3*pi/8 -pi/8 pi/8 3*pi/8] - Zenith angle [pi/4 3*pi/4] |
| ISD | 500m |
| BS Tx power | 43dBm |
| Number of sites | 19 |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,2,2) (dH,dV) = (0.5, 0.5)λ (dg,H,dg,V) = (4.0, 2.0)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; The polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See Table A.2.1-6 in TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |
| BS antenna height | 25m |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| UE antenna gain | 5dBi |
| Noise figure for BS | 7dB |
| UE receiver noise figure | 10dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |
| Metric | CDF of SNR with beamforming |