3GPP TSG RAN WG1#102-e											           R1-2005501

e-Meeting, August 17th – 28th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion of sidelink resource allocation mode 2 enhancements

Agenda item:	8.11.2.2

Document for:	Discussion and Decision

# Introduction

At RAN#86, a new work item “NR Sidelink enhancement” (NR_SL_enh) was approved ‎[1]. One of the objectives of this work item is relevant for the present agenda item:

2. Resource allocation enhancement: - Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following until RAN#90. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The study scope after RAN#90 is to be decided in RAN#90. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. - Note: RAN2 work will start after [RAN#89].

In this contribution we provide our views on enhancements in sidelink resource allocation mode 2.

# Discussion

## Analysis of the work item objective

The objective is about mode 2 enhancements to improve reliability and latency. For reliability metrics, both PRR and PIR are mentioned; this implies that persistent collisions, which are measured by PIR, are to be addressed by the enhancements. The following known issues can be addressed by mode 2 enhancements:

- Persistent collisions

- Hidden node issue

- Half-duplex collisions

Regarding specific enhancements, currently only “Inter-UE coordination” as defined in the objective is within scope of the work item.

The definition of Inter-UE coordination is very wide:

A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission.

The definition is fully open on how UE-A determines the set of resources: This could involve sensing by UE-A, UE-A’s knowledge of its own intended transmissions, UE-A’s knowledge of other UEs’ intended transmissions,…

The definition is fully open on what UE-B does with the resource set determined by UE-A: This could range from the extreme case that UE-A schedules UE-B (i.e. UE-B uses the exact resource set determined by UE-A) to UE-B treating the set determined by UE-B as just a recommendation of resources to use preferentially or to avoid.

Proposal 1: Inter-UE coordination shall address at least the following issues:

- Persistent collisions

- Hidden node issue

- Half-duplex collisions

Proposal 2: Inter-UE coordination shall address at least the following scenarios:

- Unicast

- Groupcast (e.g. platooning)

## Progress during Study Item FS_NR_V2X

During the FS_NR_V2X study item, two sub-modes of resource allocation mode 2 were defined which fit within the enhancement of “Inter-UE coordination” as defined in the WID objective, namely

- 2b)	UE assists sidelink resource selection for other UE(s)

- 2d)	UE schedules sidelink transmissions of other UEs

Below we summarize the FS_NR_V2X agreements relevant for these two submodes:

Agreements: … - Mode-2 definition covers potential sidelink radio-layer functionality or resource allocation sub-modes (subject to further refinement including merging of some or all of them) where - UE autonomously selects sidelink resource for transmission - UE assists sidelink resource selection for other UE(s) - UE is configured with NR configured grant (type-1 like) for sidelink transmission - UE schedules sidelink transmissions of other UEs

Agreements: - The following aspects about assistance information are studied for Mode 2(b) - Which assistance information is used and how it is acquired - Which UE sends assistance information - How to deliver assistance information, including physical channel and UE behavior - How assistance information is taken into account in determination of sidelink resource for transmission - RAN1 to further study whether some or all of Mode-2(b) functionality is a part of Mode-2(a)(c)(d)

Agreements: - The following aspects are studied for Mode 2(d) - In which use cases/scenarios this mode is applicable - What is the overall architecture for Mode-2(d) operation - How to decide which UE schedules which other UE(s) and how to maintain this relationship - What is the procedure of UE(s) when the scheduling UE disappears - What is the scheduling UE behavior and signaling mechanism to schedule sidelink resources for transmission/reception for other UEs - Which resources can be used to schedule other UEs - Inter- and intra-UE collision handling and sidelink resource allocation mechanisms across groups - RAN1 to further study whether or not some or all of the above aspects are applicable to 2(b)

- Agreements: For Mode-2(d) operation, further study the following potential radio-layer procedures including at least the following - Procedures to become/serve as a scheduling UE for in-coverage and out-of-coverage scenarios - The following options are identified for further study: - Scheduling UE is configured by gNB - Application layer or pre-configuration selects scheduling UE - Receiver UE schedules transmissions of the transmitter UE during the session - Scheduling UE is decided by multiple UEs including the one that is finally selected - UE may autonomously decide to serve as a scheduling UE (self-nomination) / offer scheduling UE functions

- Agreements: Initialization of Mode-2(d) operation is FFS For Mode-2(d) operation, further study the following potential radio-layer procedures including at least the following - Procedure to determine a set of sidelink resources a scheduling UE can use for scheduling of other UEs - The following options are identified: - Based on sensing procedure by scheduling UE - Configured by gNB if scheduling UE is in-coverage - Pre-configured if scheduling UE is out of coverage - Transmitting UE provides information about sidelink resources to scheduling UE - FFS behavior/algorithm of scheduling UE - Behavior of scheduling UE to signal scheduling decisions for transmission/reception of other UEs - The following options are identified: - Physical layer signaling - Higher layer signaling - UE behavior to (re)-select scheduling UE(s) - UE behavior to associate to scheduling UE(s) - UE behavior when scheduling UE stop scheduling - Resource management to address collision/interference and half-duplex issues b/w UEs scheduled by different scheduling UEs - Relationship between scheduling UE and UE groups from upper layer perspective - Whether UEs from the same upper layer group are served by the same scheduling UE - Resources used for communication before UE is associated with a scheduling UE - Procedures to switch between Mode-2(d) from/to other submodes

- Agreements: - In the context of Mode-2(d), NR V2X supports the following functionality: - A UE informs gNB about group members and gNB provides individual resource pool configuration and/or individual resource configuration through the same UE to each group member UE within the same group. It does not require connection between member UE and gNB - The UE cannot modify the configuration provided by gNB - Higher layer signaling is to be used to provide the configuration. No physical layer signaling is used - FFS if one or both options are supported (i.e. resource pool configuration(s) or resource configuration) - FFS which functionality defined as a part of Mode-2 is applicable for this feature - This functionality is up to UE capability(ies)

# Conclusions

In this contribution we discussed enhancements in sidelink resource allocation mode 2 and make the following observations and proposals:

Proposal 1: Inter-UE coordination shall address at least the following issues:

- Persistent collisions

- Hidden node issue

- Half-duplex collisions

Proposal 2: Inter-UE coordination shall address at least the following scenarios:

- Unicast

- Groupcast (e.g. platooning)

# References

- RP-201385, “WID revision: NR sidelink enhancement”

- TR 37.885, Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR

- TR 36.843, Study on LTE device to device proximity services; Radio aspects

- TR 38.840, Study on User Equipment (UE) power saving in NR