3GPP TSG-RAN WG1 Meeting #102-e	R1-2005508

eMeeting, August 17th –28th, 2020

Agenda Item:	7.2.5.3

Source:	Ericsson

Title:	Remaining Issue of PUSCH Enhancements for NR URLLC

Document for:	Discussion, Decision

# 1	Introduction

In RAN1 #101 e-Meeting, further maintenance work was done for Rel-16 PUSCH enhancement for NR URLLC. The issues were addressed in four email threads [1]. In this contribution, we discuss a couple of remaining issues. Text proposals are also provided for the corresponding specifications.

# 2	Discussion

## 2.1	Absence of a Dynamic PUSCH

In RAN1#101 e-meeting, the following agreement was made for Rel-15 maintenance. This agreement recognizes that MAC layer may not generate a transport block for a dynamically scheduled uplink grant. In this case, the PUSCH transmission is skipped unless other conditions still require a PUSCH transmission.

Agreement

The following text proposal is endorsed in R1-2005044 (TS38.214, Rel-15, CR#0105, Cat. F).

A UE shall upon detection of a DCI format scheduling a PUSCH PDCCH with a configured DCI format 0_0 or 0_1 transmit the corresponding PUSCH as indicated by that DCI. unless the UE does not generate a transport block as described in [10, TS38.321] and there is no PUCCH with CSI/HARQ-ACK that overlaps in time with the PUSCH. In this release of the specification, the UE behavior is undefined if there would be a PUCCH with CSI/HARQ-ACK overlapping in time with a PUSCH scheduled by a DCI format and if the UE does not generate a transport block as described in [10, TS38.321] when skipUplinkTxDynamic provided by higher layers is set to true.

For URLLC/IIoT scenarios, numerous scenarios can cause an uplink grant to be ignored, and no TB is provided for a dynamically scheduled PUSCH, see the Appendix. For example, due to two overlapping PUSCH resources, due to empty buffer, and due to prioritized vs deprioritized uplink grant. In other words, URLLC/IIoT scenarios can lead to an ignored uplink grant even when skipUplinkTxDynamic is not involved.

Thus a solution similar to the Rel-15 agreement above is needed to address the case of a dynamic scheduled PUSCH which does not obtain a TB from MAC. In the following, a text proposal to TS 38.214 Section 6.1 is provided.

Adopt the TP to address the case of a dynamic scheduled PUSCH for which MAC does not generate a transport block.

--------------------------- Text Proposal for 38.214 V16.2.0 Section 6.1 ------------------------------------------- ## *** Unchanged text is omitted *** A UE shall upon detection of a PDCCH with a configured DCI format 0_0, 0_1 or 0_2 transmit the corresponding PUSCH as indicated by that DCI unless the UE does not generate a transport block as described in [10, TS38.321]. Upon detection of a DCI format 0_1 or 0_2 with "UL-SCH indicator" set to "0" and with a non-zero "CSI request" where the associated "reportQuantity" in CSI-ReportConfig set to "none" for all CSI report(s) triggered by "CSI request" in this DCI format 0_1 or 0_2, the UE ignores all fields in this DCI except the "CSI request" and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1 or 0_2. When the UE is scheduled with multiple PUSCHs by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH, as described in clause 6.1.2.1, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo 16 operation applied. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1 or 0_2 scrambled by C-RNTI or MCS-C-RNTI for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process. ## *** Unchanged text is omitted *** ----------------------------------------------End of proposed TP ----------------------------------------------------

## 2.2	A-CSI on PUSCH without a TB

It is noted that the following TP was agreed with regard to A-CSI on PUSCH. With the understanding that MAC may not generated a transport block for a uplink grant, it should be discussed if/how to modify the “no transport block” phrase (yellow-highlighted) in the TP, such that it does not include the case that MAC ignored a uplink grant.

Discuss if/how to modify the TP in RAN1#101-e to exclude the case that no TB was provided by MAC for a dynamically scheduled uplink grant.

Agreement  (RAN1#101-e)

Adopt the following TP for TS 38.214 Clause 6.1.2.1:

### 6.1.2 Resource allocation #### 6.1.2.1 Resource allocation in time domain < Unchanged parts are omitted > For PUSCH repetition Type B, except for PUSCH transmitting CSI report(s) with no transport block, the number of nominal repetitions is given by numberofrepetitions. For the n-th nominal repetition, n = 0, …, numberofrepetitions - 1, - The slot where the nominal repetition starts is given by , and the starting symbol relative to the start of the slot is given by . - The slot where the nominal repetition ends is given by , and the ending symbol relative to the start of the slot is given by . < Unchanged parts are omitted > For PUSCH repetition Type B, after determining the invalid symbol(s) for PUSCH repetition type B transmission for each of the K nominal repetitions, the remaining symbols are considered as potentially valid symbols for PUSCH repetition Type B transmission. If the number of potentially valid symbols for PUSCH repetition type B transmission is greater than zero for a nominal repetition, the nominal repetition consists of one or more actual repetitions, where each actual repetition consists of a consecutive set of potentially valid symbols that can be used for PUSCH repetition Type B transmission within a slot. An actual repetition with a single symbol is omitted except for the case of L=1. An actual repetition is omitted according to the conditions in Clause 11.1 of [6, TS38.213]. The redundancy version to be applied on the nth actual repetition (with the counting including the actual repetitions that are omitted) is determined according to table 6.1.2.1-2. For PUSCH repetition Type B, when a UE receives a DCI that schedules aperiodic CSI report(s) or activates semi-persistent CSI report(s) on PUSCH with no transport block by a CSI request field on a DCI, the number of nominal repetitions is always assumed to be 1, regardless of the value of numberofrepetitions. When the UE is scheduled to transmit a PUSCH repetition Type B with no transport block and with aperiodic or semi-persistent CSI report(s) by a CSI request field on a DCI, the first nominal repetition is expected to be the same as the first actual repetition. For PUSCH repetition Type B carrying semi-persistent CSI report(s) without a corresponding PDCCH after being activated on PUSCH by a CSI request field on a DCI, if the first nominal repetition is not the same as the first actual repetition, the first nominal repetition is omitted; otherwise, the first nominal repetition is omitted according to the conditions in Clause 11.1 of [6, TS38.213]. For PUSCH repetition Type B, when a UE is scheduled to transmit a transport block and aperiodic CSI report(s) on PUSCH by a CSI request field on a DCI, the CSI report(s) is multiplexed only on the first actual repetition. The UE does not expect that the first actual repetition has a single symbol duration. < Unchanged parts are omitted >

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt the TP to address the case of a dynamic scheduled PUSCH for which MAC does not generate a transport block.

Proposal 2	Discuss if/how to modify the TP in RAN1#101-e to exclude the case that no TB was provided by MAC for a dynamically scheduled uplink grant.

# References

RAN1 Chairman’s Notes, RAN1 #101-e.

R1-1717990, RV Order for LDPC HARQ, Ericsson.

# Appendix

The following highlighted text from TS 38.321 V16.1.0 (2020-07) illustrate cases where an uplink grant is ignored, and no TB is provided for a dynamically scheduled PUSCH.

## 5.4 UL-SCH data transfer ### 5.4.1 UL Grant reception ... When the MAC entity is configured with lch-basedPrioritization, for each uplink grant whose associated PUSCH can be transmitted by lower layers, the MAC entity shall: 1> if this uplink grant is addressed to CS-RNTI with NDI = 1 or C-RNTI: 2> if there is no overlapping PUSCH duration of a configured uplink grant which was not already de-prioritized, in the same BWP whose priority is higher than the priority of the uplink grant; and 2> if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant: 3> consider this uplink grant as a prioritized uplink grant; 3> consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s); 3> consider the other overlapping SR transmission(s), if any, as a de-prioritized SR transmission(s). …

### 5.4.2 HARQ operation #### 5.4.2.1 HARQ Entity - … For each uplink grant, the HARQ entity shall: 1> identify the HARQ process associated with this grant, and for each identified HARQ process: 2> if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or 2> if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or 2> if the uplink grant was received in a Random Access Response (i.e. in a MAC RAR or a fallback RAR); or 2> if the uplink grant was determined as specified in clause 5.1.2a for the transmission of the MSGA payload; or 2> if the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery; or 2> if the uplink grant is part of a bundle of the configured uplink grant, and may be used for initial transmission according to clause 6.1.2.3 of TS 38.214 [7], and if no MAC PDU has been obtained for this bundle: 3> if there is a MAC PDU in the MSGA buffer and the uplink grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload was selected; or 3> if there is a MAC PDU in the MSGA buffer and the uplink grant was received in a fallbackRAR and this fallbackRAR successfully completed the Random Access procedure: 4> obtain the MAC PDU to transmit from the MSGA buffer. 3> else if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a fallbackRAR: 4> obtain the MAC PDU to transmit from the Msg3 buffer. 3> else if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a MAC RAR; or: 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery: 4> obtain the MAC PDU to transmit from the Msg3 buffer. 4> if the uplink grant size does not match with size of the obtained MAC PDU; and 4> if the Random Access procedure was successfully completed upon receiving the uplink grant: 5> indicate to the Multiplexing and assembly entity to include MAC subPDU(s) carrying MAC SDU from the obtained MAC PDU in the subsequent uplink transmission; 5> obtain the MAC PDU to transmit from the Multiplexing and assembly entity. 3> else if this uplink grant is a configured grant configured with autonomousTx; and 3> if the previous configured uplink grant, in the BWP, for this HARQ process was de-prioritized; and 3> if a MAC PDU had already been obtained for this HARQ process; and 3> if the uplink grant size matches with size of the obtained MAC PDU; and 3> if a transmission of the obtained MAC PDU has not been performed: 4> consider the MAC PDU has been obtained. 3> else if the MAC entity is not configured with lch-basedPrioritization; or 3> if this uplink grant is a prioritized uplink grant: 4> obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any; 3> if a MAC PDU to transmit has been obtained: 4> if the uplink grant is not a configured grant configured with autonomousTx; or 4> if the uplink grant is a prioritized uplink grant: 5> deliver the MAC PDU and the uplink grant and the HARQ information of the TB to the identified HARQ process; 5> instruct the identified HARQ process to trigger a new transmission; 5> if the uplink grant is a configured uplink grant: 6> start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed if LBT failure indication is not received from lower layers; 6> start or restart the cg-RetransmissionTimer, if configured, for the corresponding HARQ process when the transmission is performed if LBT failure indication is not received from lower layers. 5> if the uplink grant is addressed to C-RNTI, and the identified HARQ process is configured for a configured uplink grant: 6> start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed if LBT failure indication is not received from lower layers. 5> if cg-RetransmissionTimer is configured for the identified HARQ process: 6> if the transmission is performed and LBT failure indication is not received from lower layers: 7> consider the identified HARQ process as not pending. 6> else: 7> consider the identified HARQ process as pending. 3> else: 4> flush the HARQ buffer of the identified HARQ process. 2> else (i.e. retransmission): 3> if the uplink grant received on PDCCH was addressed to CS-RNTI and if the HARQ buffer of the identified process is empty; or 3> if the uplink grant is part of a bundle and if no MAC PDU has been obtained for this bundle; or 3> if the uplink grant is part of a bundle of the configured uplink grant, and the PUSCH duration of the uplink grant overlaps with a PUSCH duration of another uplink grant received on the PDCCH or an uplink grant received in a Random Access Response (i.e. MAC RAR or fallbackRAR) or an uplink grant determined as specified in clause 5.1.2a for MSGA payload for this Serving Cell; or: 3> if the MAC entity is configured with lch-basedPrioritization and this uplink grant is not a prioritized uplink grant: 4> ignore the uplink grant. 3> else: 4> deliver the uplink grant and the HARQ information (redundancy version) of the TB to the identified HARQ process; 4> instruct the identified HARQ process to trigger a retransmission; 4> if the uplink grant is addressed to CS-RNTI; or 4> if the uplink grant is addressed to C-RNTI, and the identified HARQ process is configured for a configured uplink grant: 5> start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed if LBT failure indication is not received from lower layers. 4> if the uplink grant is a configured uplink grant: 5> if the identified HARQ process is pending: 6> start or restart the configuredGrantTimer for the corresponding HARQ process when the transmission is performed if LBT failure indication is not received from lower layers; 5> start or restart the cg-RetransmissionTimer, if configured, for the corresponding HARQ process when the transmission is performed if LBT failure indication is not received from lower layers. 4> if the identified HARQ process is pending and the transmission is performed and LBT failure indication is not received from lower layers: 5> consider the identified HARQ process as not pending. - …