3GPP TSG RAN WG1 Meeting #102-e                                                                    R1-2005553

e-Meeting, August 17th – 28th,  2020

Source:              ZTE

Title:                   Discussion for NR SRS carrier switching on SUL

Agenda Item:     5.1

Document for:   Discussion and Decision

Introduction

In RAN2#110-e meeting, an LS R1-2005209(R2-2006362)[1] is sent to ask RAN1 whether the following two cases are allowed for NR Rel-15 SRS carrier switching as follows.

During the CR discussion, for typeA RAN2 didn’t achieve consensus on whether the following 2 cases on NR SRS carrier switching are allowed: - Case 1: SRS carrier switching to a SUL without PUCCH/PUSCH while PUCCH/PUSCH is configured in NUL; - Case 2: SRS carrier switching to both NUL and SUL, neither of which are configured with PUCCH/PUSCH.

In this contribution, we provide our analysis on these two cases.

Discussion

In LTE Rel-14, SRS carrier switching is introduced to acquire DL CSI via DL/UL channel reciprocity in some TDD carriers configured only for DL transmission, i.e. CCs without PUCCH/PUSCH configured in TDD band(s).

In NR Rel-15, because of limited time, it was agreed to reuse Rel-14 LTE SRS carrier-based switching design in RAN1#90bis meeting as shown in the following agreement. The motivation is the same as LTE, where SRS configured with antenna switching transmits in serving cells only for DL transmission in TDD band(s). Based on SRS carrier switching, gNB can acquire DL CSI in each serving cell, and further derive precoders for DL data transmission.

Agreement in RAN1#90bis [2]:

- Specify NR SRS switching among CCs similar to Rel-14 LTE SRS carrier-based switching design including

  - Periodic/aperiodic/semi-persistent SRS on a CC without PUCCH/PUSCH configured

  - TA (through PRACH) on TAG without PUSCH/PUCCH configured

  - Power control separated from that of PUSCH

  - Group common DCI for aperiodic SRS triggering and TPC

  - DL/UL interruptions and collision handling due to SRS switching

As shown in RAN1 specification 38.214 [4], the motivation of SRS antenna switching is for DL CSI acquisition as highlighted in the following box. For type A SRS carrier switching, SRS configured with usage ‘antenna switching’ is transmitted on each of the CCs within a CC set, the DL CSI acquisition for those CCs without PUCCH/PUSCH can be achieved.

- 6.2.1.2 UE sounding procedure for DL CSI acquisition When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', ... - 6.2.1.3 UE sounding procedure between component carriers For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, …. ... For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA', and given by SRS-CarrierSwitching, without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells in the indicated set of serving cells configured by higher layers, where the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'.

Observation1: For type A, NR SRS carrier switching is for DL CSI acquisition for a set of TDD serving cells without configured PUSCH/PUCCH.

In addition, the wording “with slot formats comprised of DL and UL symbols” means that this feature should be only applicable to TDD carriers.  However, a SUL carrier, which can be configured in a serving cell, is neither a TDD carrier nor a FDD carrier.  Instead, it is a new duplex mode introduced in NR Rel-15 as shown in Table 5.2-1 in TS38.101-1[3] as below.  In Rel-15, all the defined SUL bands overlap with FDD UL bands.  In Rel-16, it starts to have a new SUL band overlapping with a TDD band.  Regardless whether it overlaps with a TDD or FDD band, it does not have the corresponding downlink operating band i.e. N/A in the table.    At least in UE perspective, it does not have the corresponding DL symbols in any SUL carrier configured to the UE.     Consequently, it is an invalid case to support SRS carrier switching in SUL carrier for DL CSI acquisition.

Table 5.2-1 in [3]: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n50 | 1432 MHz – 1517 MHz | 1432 MHz – 1517 MHz | TDD1 |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n70 | 1695 MHz – 1710 MHz | 1995 MHz – 2020 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL |
| n76 | N/A | 1427 MHz – 1432 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL |
| n81 | 880 MHz – 915 MHz | N/A | SUL |
| n82 | 832 MHz – 862 MHz | N/A | SUL |
| n83 | 703 MHz – 748 MHz | N/A | SUL |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL |
| n86 | 1710 MHz – 1780 MHz | N/A | SUL |
| NOTE 1: UE that complies with the NR Band n50 minimum requirements in this specification shall also comply with the NR Band n51 minimum requirements. NOTE 2: UE that complies with the NR Band n75 minimum requirements in this specification shall also comply with the NR Band n76 minimum requirements. |  |  |  |

Observation2: NR SUL carriers do not have the corresponding downlink operating band.  DL CSI acquisition is invalid for SUL carriers.

The purpose of SUL configuration is for adding one more carrier for uplink transmission.  Although the current RRC signaling design does not preclude the configuration of SUL without configured PUCCH/PUSCH, this kind of configuration is meaningless.  It may be meaningful not to configure PUCCH/PUSCH in NUL and rely on SUL for uplink transmission.  However, Case 1 (i.e. SUL without PUCCH/PUSCH) and Case 2 (i.e. neither SUL nor NUL is configured with PUCCH/PUSCH) described in the LS are both meaningless configurations.

Observation3: SUL without PUCCH/PUSCH is a meaningless configuration.

Based on Observations 1,2 and 3, it is meaningless to achieve DL CSI acquisition in a SUL carrier which cannot support DL data transmission. Therefore, neither case 1 nor case 2 listed in RAN2 LS is allowed.

Further, since LTE has no SUL carrier, and it has never been any detailed discussion for NR on whether/how SRS carrier switching is supported on SUL, it is natural not to support NR carrier switching in SUL carrier.  Based on the analysis, we have the following proposal on replying to the RAN2 LS.

Proposal:  Reply to the LS R1-2005209(R2-2006362) that neither case 1 nor case 2 listed in the LS is allowed.

Conclusion

In this contribution, we discuss the questions listed in the LS R1-2005209(R2-2006362). The following observations and proposals are provided:

Observation1: For type A, NR SRS carrier switching is for DL CSI acquisition for a set of TDD serving cells without configured PUSCH/PUCCH.

Observation2: NR SUL carriers do not have the corresponding downlink operating band.  DL CSI acquisition is invalid for SUL carriers.

Observation3: SUL without PUCCH/PUSCH is a meaningless configuration.

Proposal:  Reply to the LS R1-2005209(R2-2006362) that neither case 1 nor case 2 listed in the LS is allowed.

References

- R1-2005209(R2-2006362), LS on NR SRS carrier switching, Release 15, TSG-RAN2 110-e, 1st - 12th June 2020

- Chairman notes in RAN1#90bis meeting, 9th – 13th October 2017

- 38.101-1-fa0, User Equipment (UE) radio transmission and reception, Part 1: Range 1 Standalone, (Release 15)

- TS 38.214 fa0, Physical layer procedures for data, V15.10.0.