3GPP TSG RAN WG1 #102-e	R1-2005607

e-Meeting, August 17th - 28th, 2020

Title:	Discussion on the required changes to NR for above 52.6GHz

Source: 	ZTE, Sanechips

Agenda Item:	8.2.1

Document for:	Discussion and Decision

# Introduction

In 3GPP RAN #86 plenary meeting in December 2019, a new SID RP-193259 [1] was approved to study required changes to NR using existing DL/UL NR waveform and channel access mechanism for supporting operation between 52.6 GHz and 71 GHz. The SID can be considered as a preliminary pre-research for new WID RP-193229 [2]. The detailed objectives of the SID are as follows.

SID RP-193259:

This study item will include the following objectives:

- Study of required changes to NR using existing DL/UL NR waveform to support operation between 52.6 GHz and 71 GHz

  - Study of applicable numerology including subcarrier spacing, channel BW (including maximum BW), and their impact to FR2 physical layer design to support system functionality considering practical RF impairments [RAN1, RAN4].

  - Identify potential critical problems to physical signal/channels, if any [RAN1].

- Study of channel access mechanism, considering potential interference to/from other nodes, assuming beam based operation, in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6 GHz and 71 GHz [RAN1].

  - Note: It is clarified that potential interference impact, if identified, may require interference mitigation solutions as part of channel access mechanism.

In last RAN1 #101 emeeting, a lot of physical layer aspects were identified for further study [3]. In this contribution, we give our views on required changes to NR using existing DL/UL NR waveform, such as numerology (SCS, CP and channel BW), as well as potential problems to physical signal/channels. The analysis on channel access mechanism and evaluation can refer to our companion contributions [4-5].

# Discussion on required changes to NR in 60 GHz band

To minimize the standardization complexity, reduce the expense of network/device re-design and accelerate the commercial process, on the premise of meeting the communication requirements on the bands from 52.6 GHz to 71 GHz, Rel-17 NR should reuse FR2 design as much as possible. In addition, Rel-17 NR also needs to consider the forward compatibility for the further evolution of NR HF, such as extending the supported bands to 71 GHz ~ 114.25 GHz in the future.

SID RP-193259 has a research prerequisite, that is, reusing existing NR waveform to support operation between 52.6 GHz and 71 GHz: CP-OFDM for DL and CP-OFDM / DFT-s-OFDM for UL. It will greatly reduce research difficulties and thus save research time. However, there are still many potential changes to be studied for above 52.6 GHz including channelization, numerology and impacts on existing physical channels/signals.

## 2.1 	Channel bandwidth

In 5 GHz band, ETSI EN 301 893 V1.8.1 [6] provides a clear channelization scheme:

- Service frequency bands: 5150 MHz to 5350 MHz; 5470 MHz to 5725 MHz.

- The Nominal Centre Frequencies (fc) for a Nominal Channel Bandwidth (NCB) of 20 MHz are defined by equation: fc = 5 160 + (g × 20) MHz, where 0 ≤ g ≤ 9 or 16 ≤ g ≤ 27 and where g shall be an integer.

Wi-Fi, LTE LAA and Rel-17 NR-U are all based on above channelization scheme, consider the impacts on incumbent RATs, and further carry out similar channelization. They have a same basic channel bandwidth with 20 MHz.

Unlike 5 GHz band, ETSI EN 302 567 V2.1.1 for 60 GHz band [7] does not offer any channelization scheme, as well as in other regional/international spectrum rule specification. However, channelization design of NR above 52.6 GHz may still need to take co-existence with other RATs into account, such as IEEE 802.11ad/ay systems, and various uses cases, such as ITS, IAB, and V2X.

The channelization used by IEEE 802.11ay [8] is shown in Figure 2.1-1.

Figure 2.1-1: Channelization used by IEEE 802.11ay

From Figure 1, we can see the channelization of IEEE 802.11ay is based on a basic channel bandwidth of 2.16 GHz. Other supported channel bandwidths can be K×2.16 GHz with K=1, 2, 3, 4, which means two, three or four continuous basic channels can form 4.32 GHz, 6.48 GHz or 8.64 GHz channel bandwidth. According to Figure 1, we can calculate detailed frequency ranges of each channel with 2.16 GHz bandwidth for frequency band 57 GHz - 71 GHz in IEEE 802.11ay, as shown in the Table 1 below.

Table 1: Channelization of 2.16 GHz channel in IEEE 802.11ay

|  | Channelization[GHz] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel #1 | Channel #2 | Channel #3 | Channel #4 | Channel #5 | Channel #6 |
| Center frequency | 58.32 | 60.48 | 62.64 | 64.80 | 66.96 | 69.12 |
| Frequency range | 57.24 - 59.40 | 59.40 - 61.56 | 61.56 - 63.72 | 63.72 - 65.88 | 65.88 - 68.04 | 68.04 - 70.20 |

For whether to consider co-existence with IEEE 802.11ad/ay channelization, one solution is that NR above 52.6 GHz only needs to meet requirements from regional/international spectrum rule specifications. But since such kind of specifications e.g. ETSI standard does not give any channelization scheme on this band range, so NR above 52.6 GHz can ignore Wi-Fi demand in principle. If based on this consideration, NR above 52.6 GHz design is relatively simple as there is no restriction from Wi-Fi. However, this will not only lead to uncertainty on the existing Wi-Fi systems that have been already deployed in the same region, but also cause poor NR operation especially in 57 GHz - 71 GHz unlicensed band.

Therefore, we suggest to align the Wi-Fi design at least in unlicensed band (e.g. 57 GHz - 71 GHz) and support 2.16 GHz bandwidth. In other licensed frequency band (e.g. 52.6 GHz - 57 GHz) or in a controlled environment without Wi-Fi devices, it can be designed uniformly with unlicensed band or independently. In above cases, Rel-17 NR can reuse the maximum 400 MHz bandwidth, or its integral multiple.

Proposal 1: When determining supported bandwidths for NR above 52.6 GHz, RAN1 should take co-existence of IEEE 802.11ad/ay into account at least in unlicensed band. In licensed frequency band or in a controlled environment, it can be designed in a unified way with unlicensed band or independently.

Proposal 2: 400 MHz (and/or its integral multiple e.g. 800/1600 MHz) and 2.16 GHz can be served as candidates of supported bandwidths for Rel-17 NR above 52.6 GHz.

## 2.2 	Numerology

The band from 52.6 GHz to 71 GHz has a wide and continuous available spectrum especially for unlicensed operation. For instance, 57 GHz - 71 GHz (14 GHz) supported by the United States, 57 GHz - 66 GHz (9 GHz) supported by Europe/Japan/South Korea and etc. Among the countries/regions that have planned unlicensed frequency band, China has the narrowest range, but it also supports up to 5 GHz unlicensed spectrum in the range of 59 GHz - 64 GHz. Furthermore, considering to align with channelization of IEEE 802.11 ad/ay as discussed in section 2.1, it is feasible and necessary to support channel bandwidths larger than 400 MHz, which is the largest bandwidth supported by NR at present.

### 2.2.1 Maximum FFT size

In order to support larger bandwidths (such as 2.16 GHz bandwidth), it can be generally realized by adjusting subcarrier spacing (SCS) and the maximum number of available RBs/REs. Increasing the number of RBs/REs will lead to a larger FFT size that will bring more complexity to the implementation. In current NR, the maximum IFFT size is 4096 corresponding to maximum 275 RBs / 3300 REs. If increasing the maximum number of RBs from 275 RBs to 550 RBs, it implies that the FFT size will be increased to 8192, which will cause a higher implementation burden for redesigning FFT engine. Therefore, we think Rel-17 NR should keep the same maximum FFT size 4096 as in Rel-15/16  NR to avoid higher PAPR and reduce implementation complexity.

Proposal 3: Keep the same maximum FFT size 4096 as in Rel-15/16 NR to avoid higher PAPR and reduce implementation complexity.

### 2.2.2 Subcarrier spacing

If we maintain the same limit on maximum FFT size, we need to consider how to scale SCS to support larger channel bandwidths. Table 2 and Table 3 give some options for numerology design of NR above 52.6 GHz.

Table 2: Candidate numerology for NR above 52.6 GHz based on FR2 numerology

| Candidate numerology for NR above 52.6 GHz | Δf [kHz] | N (IFFT size) | fs [MHz] (=Δf×N) | # of guard tones on edge | Bandwidth [MHz] | # of RBs |
| --- | --- | --- | --- | --- | --- | --- |
| Rel-15 FR2 | 120 | 4096 | 491.52 | 464 | 380.16 | 264 |
| Opt 1a | 240 | 2048 | 491.52 | 232 | 380.16 | 132 |
| Opt 1b | 240 | 4096 | 983.04 | 464 | 760.32 | 264 |
| Opt 1c | 240 | 8192 | 1966.08 | 928 | 1520.64 | 528 |
| Opt 2a | 480 | 2048 | 983.04 | 232 | 760.32 | 132 |
| Opt 2b | 480 | 4096 | 1966.08 | 464 | 1520.64 | 264 |
| Opt 2c | 480 | 8192 | 3932.16 | 928 | 3041.28 | 528 |
| Opt 3a | 960 | 1024 | 983.04 | 116 | 760.32 | 66 |
| Opt 3b | 960 | 2048 | 1966.08 | 232 | 1520.64 | 132 |
| Opt 3c | 960 | 4096 | 3932.16 | 464 | 3041.28 | 264 |
| Opt 4a | 1920 | 1024 | 1966.08 | 116 | 1520.64 | 66 |
| Opt 4b | 1920 | 2048 | 3932.16 | 232 | 3041.28 | 132 |

Table 3: Candidate numerology for NR above 52.6 GHz in evaluation parameter [3]

| Candidate numerology for NR above 52.6 GHz |  | Δf [kHz] | N (IFFT size) | fs [MHz] (=Δf×N) | # of guard tones on edge | Bandwidth [MHz] | # of RBs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 400 MHz BW | Opt 1a | 120 | 4096 | 491.52 | 512 | 368.64 | 256 |
|  | Opt 1b | 240 | 2048 | 491.52 | 256 | 368.64 | 128 |
|  | Opt 1c | 480 | 1024 | 491.52 | 128 | 368.64 | 64 |
|  | Opt 1d | 960 | 512 | 491.52 | 64 | 368.64 | 32 |
|  | N/A | 1920 | N/A | N/A | N/A | N/A | N/A |
| 2 GHz BW | N/A | 120 | N/A | N/A | N/A | N/A | N/A |
|  | N/A | 240 | N/A | N/A | N/A | N/A | N/A |
|  | FFS Opt 2a | 480 | 4096 | 1966.08 | 128 | 1843.2 | 320 |
|  | Opt 2b | 960 | 2048 | 1966.08 | 64 | 1843.2 | 160 |
|  | Opt 2b | 1920 | 1024 | 1966.08 | 32 | 1843.2 | 80 |

From the yellow highlighted part in above two tables, we can see that if the maximum FFT size 4096 is kept unchanged, the maximum transmission bandwidth with SCS 480 kHz and 264 RBs can only reach 1.52 GHz, which is close to 70% of the nominal bandwidth 2.16 GHz. Even if using interleaving scheme, it is still difficult for uplink transmission to meet the requirements of OCB. If the maximum number of available RBs is 320, the maximum transmission bandwidth with SCS 480 kHz is 1.84 GHz. It can achieve 85% of the nominal bandwidth 2.16 GHz However, 320 RBs exceeds the upper limit of 275 RBs in Rel-15/16 NR.

Observation 1: Considering the requirement of OCB and the limitation of the maximum number of available RBs, it is difficult to use 480 kHz as the candidate SCS for 2.16 GHz bandwidth if the maximum FFT size 4096 is kept unchanged.

For a high frequency band, the minimum value of SCS is mainly determined by phase noise and Doppler shift. If the CP length is fixed, the advantage of using a smaller SCS is reflected on lower CP overhead as the symbol length is relatively longer. But the smaller SCS will lead to higher phase noise, which will affect EVM. It also demands higher requirement for local oscillator which may limit the performance when the Doppler shift is relatively higher. Using a large SCS will shorten the symbol length and reduce the delay. But excessive larger SCS will lead to increased CP overhead if CP length is fixed,, or weaken the ability of resisting multi-path delay if CP and SCS scales at the same scale. An appropriate upper limit of SCS should be set according to the required CP overhead and multi-path delay spread.

SSB and/or Type0-PDCCH with QPSK modulation have good robustness to phase noise, irrespective of whether their SCSs are 120/240 kHz or larger. Thus Rel-17 NR can maintain the maximum SCS with 240 kHz for SSB and/or 120 kHz for Type0-PDCCH, and reuse the initial access procedure in Rel-15/16.

Proposal 4: Rel-17 NR can maintain the maximum SCS with 240 kHz for SSB and/or 120 kHz for Type0-PDCCH, and reuse the initial access procedure in Rel-15/16 NR.

Except for SSB, the maximum SCS for physical channels/signals supported by current Rel-16 NR is 120 kHz. At least for PDSCH/PUSCH, in order to support larger channel bandwidth (e.g. 2.16 GHz) and use higher-order modulation to support high data rate, it is necessary to use larger SCS. In order to reduce the complexity of the design, SCS above 52.6 GHz can be scaled by an integral multiple of current supported SCS, i.e. Δf = 2μ × 15 kHz (μ can be set as 4, 5, 6 or others). However, as we mentioned above, the selection of higher SCS needs to consider the impacts of frequency band, bandwidth, phase noise, CP overhead and multi-path delay.

Observation 2: The selection of SCSs for Rel-17 NR above 52.6 GHz needs to consider the impacts of frequency band, bandwidth, phase noise, CP overhead and multi-path delay.

Proposal 5: Numerology (SCS as well as CP) of NR above 52.6 GHz can be scaled by an integral multiple of current numerology supported by Rel-15/16 NR, i.e. Δf = 2μ × 15 kHz (FFS: μ can be set as 4, 5, 6 or others).

### 2.2.3 Cyclic prefix

If both CP and SCS are scaled according to the power of 2, the CP lengths corresponding to each SCS are shown in Table 2 below.

Table 4: CP lengths with different SCS

| u | SCS (kHz) | Slot length (us) | Symbol length | NCP length ( and ) | NCP length ( or ) | ECP |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 15 | 1000 | 66.667 us | 4.688 us | 5.208 us | 16.667 us |
| 1 | 30 | 500 | 33.333 us | 2.344 us | 2.865 us | 8.333 us |
| 2 | 60 | 250 | 16.667 us | 1.172 us | 1.693 us | 4.167 us |
| 3 | 120 | 125 | 8.333 us | 0.586 us | 1.107 us | 2.083 us |
| 4 | 240 | 62.5 | 4166.67 ns | 292.97 ns | 813.80 ns | 1041.66ns |
| 5 | 480 | 31.25 | 2083.33 ns | 146.48 ns | 667.32 ns | 520.83 ns |
| 6 | 960 | 15.625 | 1041.67 ns | 73.24 ns | 594.07 ns | 260.42 ns |
| 7 | 1920 | 7.8125 | 521.83 ns | 36.62 ns | 557.45 ns | 130.21 ns |

In OFDM mode of IEEE 802.11ay, it includes three kinds of guard interval (GI) durations: short GI with 18.18 ns, normal GI with 36.36 ns and long GI with 72.72 ns. According to Table 2, it can be seen that CP length with the largest SCS 1920 kHz is 36.62 ns, which is very close to the normal GI IEEE 802.11ay. The CP length of the SCS 960 kHz is 73.24 ns, which is similar to the long GI of 72.72 ns in IEEE 802.11ay. Compared with IEEE 802.11ay, in principle, NR above 52.6 GHz uses SCS(s) equal to or smaller than 1920 kHz, which may be feasible in the similar scenarios deployed with IEEE 802.11ay. Although smaller CP can maintain a constant CP overhead, CP length needs to be long enough to cover multi-path delay. Otherwise, it will lead to performance degradation. If SCSs larger than 240 kHz are supported, the short CP may be not enough to cover delay spread, beam switching time and timing errors. A gap for beam switching may be needed.

Observation 3: If SCSs larger than 240 kHz are supported, the short CP may be not enough to cover delay spread, beam switching time and possible timing errors.

### 2.2.4 Evaluation of candidate numerology

#### In this section, we provide some simulation results for CP-OFDM based PUSCH and PDSCH with different candidate SCSs, modulation orders and PN impacts (w/o PN, w/ PN but w/o compensation, or w/PN and w/ PTRS-based CPE compensation). Simulation assumptions can refer to Appendix.

#### 2.2.4.1 PUSCH

(1) TDL-A 5ns

- 16QAM

Figure 2.2.4.1-1 TDL-A 16QAM

It is shown from Figure 2.2.4.1-1, for 16QAM TDL-A channel:

- Without phase noise, different SCS shows similar performance.

- With phase noise and without PN compensation, phase noise will cause about 1 dB performance degradation on each SCS. Larger SCS shows better performance against PN, 120kHz is 0.5dB worse than 960kHz.

- With phase noise and with PTRS PN compensation, the performance is almost the same as the performance without phase noise.

- 64QAM

Figure 2.2.4.1-2 TDL-A 64QAM

It is shown from Figure 2.2.4.1-2, for 64QAM TDL-A channel:

- Without phase noise, different SCS shows similar performance.

- With phase noise and without PN compensation, phase noise will cause significant performance degradation on each SCS, larger SCS shows better resistance against the impairment caused by PN.

- With phase noise and with PTRS PN compensation, the performance is much better than without PN compensation. Although there is still some gap from the reference performance without PN, larger SCS shows better performance with PTRS PN compensation.

#### 2.2.4.2 PDSCH

(1) CDL-B 20ns

Considering 16QAM TDL-A performance of different SCS is similar, for DL evaluation, we only consider 64QAM in order to show the difference between different SCS.

Figure 2.2.4.2-1 CDL-B 64QAM

Figure 2.2.4.2-1 shows the evaluation results for 64QAM and CDL-B channel model with 20ns delay spread. Generally, similar observation as UL can be found as follows:

- Without phase noise, different SCS shows similar performance, 960kHz performance is slightly worse than others, probably due to the shortened CP length.

- With phase noise and if we do not do the PN compensation, significant performance loss can be observed on each curve with different SCS.

- Larger SCS shows better performance with PTRS PN compensation.

Observation 4: Phase noise has limited impact on 16QAM modulation, and with PTRS PN compensation, different SCS (120kHz, 240kHz, 480kHz, 960kHz) shows similar performance.

Observation 5: Phase noise has significant impact on 64QAM modulation, and with PTRS PN compensation, larger SCS shows better performance.

## 2.3 	Impacts on NR physical signal/channels

Rel-15/16 NR supports five cases for SS/PBCH block (SSB) patterns: Case A with 15 kHz and Case B/C with 30 kHz for FR1, Case D with 120 kHz and Case E with 240 kHz for FR2. Figure 2.3-1 shows the SSB patterns in Case D and Case E. As discussed in section 2.2, the maximum SCS with 240 kHz for SSB can be maintained as Rel-15/16 NR as QPSK demodulation has good robustness to phase noise. However, the existing patterns of case D/E can not multiplexed in time domain with PDCCH with higher SCS (e.g. equal to or larger than 240 kHz). Thus the patterns of SSB with 120/240 kHz still need to be re-considered.  If higher SSB SCSs needs to be supported in Rel-17 NR, SSB patterns with higher SCS than 240 kHz need to be re-designed. The existing patterns of case D/E can be used as a starting point. In addition, transmission opportunities, sync raster in a wider bandwidth, timing and QCI with the new SCS also may also need to be considered.

Figure 2.3-1: SSB patterns in Case D and Case E

Proposal 6: SSB pattern could be re-designed whether higher SCS is supported or not.

Proposal 7: Transmission opportunities, timing and QCI of Rel-17 SSB should be considered.

The association of SSB, Type0-PDCCH and RMSI should be studied for better initial access. Rel-17 NR can maintain the maximum SCS with 240 kHz for SSB and/or 120 kHz for Type0-PDCCH. The existing PDCCH/PDSCH supports up to 120 kHz SCS, which needs to be extended as well. For PDSCH, the necessity and duration(s) of Type-B mapping with higher SCS could be considered. DL burst detection and dynamic PDCCH monitoring can refer to Rel-16 NR-U conclusions. For a wider carrier bandwidth, how to configure BWP, RB-set, SSS and CORESET should also be studied.

In sub-7GHz NR-U, new long PRACH sequences are introduced to satisfy the OCB requirements. In above 52.6GHz, if the channel bandwidth for LBT is extended to for example 2.16GHz, the existing PRACH sequences are not appropriate as well. The higher SCS and longer sequence of PRACH may need to be considered.

For NR UL, the PUSCH scheduling scheme in Rel-16 can be reused, such as multiple TTIs scheduling and slot aggregation. In unlicensed band, the enhanced dynamic HARQ-ACK codebook and one-shot HARQ-ACK feedback introduced in Rel-16 NRU can still be supported. However, if useInterlace-PUCCH-PUSCH is enabled, then the interlace structure may need to re-design as larger bandwidth is supported for UE. Besides, if directional LBT is performed, how to improve the transmission opportunity for PUSCH/PUCCH should be considered.

In Rel-15/16 NR, the timeline related aspects are defined based on numerology (i.e. SCS), such as BWP switching times, UE processing, HARQ scheduling, UE processing, preparation and computation times for PDSCH, PUSCH/SRS and CSI, respectively. However, the timeline restriction cannot be simply scaled with numerology as the UE processing capability can not directly with the symbol length. For example, UE PDSCH processing time N1 based on µ of table 5.3-1 and table 5.3-2 in TS 38.214 [9] for UE processing capability 1 and 2 respectively does not scale with µ directly. Even so, considering that the symbol length becomes more shorter as the SCS becomes larger, the impacts on UE processing time and scheduling operation for new SCS should be studied.

Table 5.3-1: PDSCH processing time for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = pos0 in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB | dmrs-AdditionalPosition ≠ pos0 in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured |
| 0 | 8 | N1,0 |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |

Table 5.3-2: PDSCH processing time for PDSCH processing capability 2

|  | PDSCH decoding time N1 [symbols] |
| --- | --- |
|  | dmrs-AdditionalPosition = pos0 in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB |
| 0 | 3 |
| 1 | 4.5 |
| 2 | 9 for frequency range 1 |

Proposal 8: If introducing new numerology, the impacts on processing time and scheduling operation should be considered.

# Conclusion

In this contribution, we discuss NR above 52.6 GHz band operation and have the following observations and proposals.

Observation 1: Considering the requirement of OCB and the limitation of the maximum number of available RBs, it is difficult to use 480 kHz as the candidate SCS for 2.16 GHz bandwidth if the maximum FFT size 4096 is kept unchanged.

Observation 2: The selection of SCSs for Rel-17 NR above 52.6 GHz needs to consider the impacts of frequency band, bandwidth, phase noise, CP overhead and multi-path delay.

Observation 3: If SCSs larger than 240 kHz are supported, the short CP may be not enough to cover delay spread, beam switching time and possible timing errors.

Observation 4: Phase noise has limited impact on 16QAM modulation, and with PTRS PN compensation, different SCS (120kHz, 240kHz, 480kHz, 960kHz) shows similar performance.

Observation 5: Phase noise has significant impact on 64QAM modulation, and with PTRS PN compensation, larger SCS shows better performance.

Proposal 1: When determining supported bandwidths for NR above 52.6 GHz, RAN1 should take co-existence of IEEE 802.11ad/ay into account at least in unlicensed band. In licensed frequency band or in a controlled environment, it can be designed in a unified way with unlicensed band or independently.

Proposal 2: 400 MHz (and/or its integral multiple e.g. 800/1600 MHz) and 2.16 GHz can be served as candidates of supported bandwidths for Rel-17 NR above 52.6 GHz.

Proposal 3: Keep the same maximum FFT size 4096 as in Rel-15/16 NR to avoid higher PAPR and reduce implementation complexity.

Proposal 4: Rel-17 NR can maintain the maximum SCS with 240 kHz for SSB and/or 120 kHz for Type0-PDCCH, and reuse the initial access procedure in Rel-15/16 NR.

Proposal 5: Numerology (SCS as well as CP) of NR above 52.6 GHz can be scaled by an integral multiple of current numerology supported by Rel-15/16 NR, i.e. Δf = 2μ × 15 kHz (FFS: μ can be set as 4, 5, 6 or others).

Proposal 6: SSB patterns could be re-designed whether higher SCS is supported or not.

Proposal 7: Transmission opportunities, timing and QCI of Rel-17 SSB should be considered.

Proposal 8: If introducing new numerology, the impacts on processing time and scheduling operation should be considered.

# Reference

- RP-193259, New SID: Study on supporting NR from 52.6GHz to 71GHz. Intel, Sitges, Spain, RAN#86, December 9 – 12, 2019

- RP-193229, New WID on extending current NR operation to 71GHz. Qualcomm, Sitges, Spain, RAN#86, December 9 – 12, 2019

- Chairman notes. RAN1 #101-emeeting, June 2020

- R1-2005608, Discussion on the channel access mechanism for above 52.6 GHz, ZTE, RAN1 #102-e

- R1-2005609, Preliminary simulation results for above 52.6GHz, ZTE, RAN1 #102-e

- ETSI EN 301 893 V1.8.1, Harmonized European Standard, 5 GHz high performance RLAN. 2015-03

- ETSI EN 302 567 V2.1.1, Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard covering the essential requirements. 2017-07

- IEEE P802.11ayTM/D1.0, November 2017

- 3GPP TS 38.214 V16.2.0, Physical layer procedures for data, 2020-06

# Appendix: simulation assumptions

Table A1 LLS simulation assumption

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 60GHz |
| Channel Bandwidth | 400MHz |
| Subcarrier spacing | 120kHz/240kHz/480kHz/960kHz |
| Waveform | CP-OFDM |
| RB number | 256/128/64/32 |
| CP type | Normal CP |
| Channel Model | TDL-A 5ns CDL-B 20ns |
| PN model | 3GPP TR38.803 example 2 |
| DMRS Configuration | 2 DMRS symbols at (2,11) |
| PTRS Configuration | (K = 2, L = 1) |
| SLIV | (S=0, L=14) |
| Channel Estimation | Realistic |
| PN Estimation | Realistic |
| UE receiver | MMSE-IRC |
| BS antenna Array configuration | For TDL model: 1*2 For CDL model: (M, N, P, Mg, Ng) = (8, 16, 2, 1, 1), dH = dV = 0.5 λ |
| UE antenna Array configuration | For TDL model: 1*2 For CDL model: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH = dV = 0.5 λ |