3GPP TSG-RAN WG1 #102-e	R1- 2005663

# e-Meeting, August 17th – 28th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for PUCCH repetition transmission without corresponding PDCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For RRC configured UCI transmission (such as HARQ-ACK for SPS PDSCH, CSI and SR), it is possible that the PUCCH resource for the RRC configured UCI transmission collides with DL symbols or SSB symbols, due to unmatched periodicities. To solve this issue, the following agreement was made in RAN1#92bis meeting and was specified in Clause 11.1 in TS38.213. Agreements: For a UE-specific RRC configured UL transmission, if one OFDM symbol of the configured transmission falls on the semi-static DL symbol or a symbol SSB is transmitted as indicated, the UE shall cancel the transmission. For a UE-specific RRC configured DL reception, if one OFDM symbol of the configured reception falls on the semi-static UL symbol or a symbol PRACH is configured, the UE shall cancel the reception.. 11.1 Slot configuration […] For a set of symbols of a slot that are indicated to a UE as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not receive PDCCH, PDSCH, or CSI-RS when the PDCCH, PDSCH, or CSI-RS overlaps, even partially, with the set of symbols of the slot. For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with the set of symbols of the slot.. In current specification, UE always determines slots with enough available symbols for PUCCH resource starting from a first slot as determined for single-slot transmission and it is assumed that for PUCCH carrying HARQ-ACK dynamically scheduled by PDCCH, the PUCCH resource in the first slot does not collide with DL symbols or SSB symbols since gNB could ensure the first slot always has enough available symbols. However, for RRC configured UCI transmission, gNB cannot ensure that the PUCCH resource in the first slot does not collide with DL symbols or SSB symbols as for the single-slot transmission case. The UE behavior is not clear in this case, e.g. whether UE shall cancel the PUCCH transmission or PUCCH could be delayed to next slot satisfying the condition. |
| Summary of change: |  | Clarify RRC configured UCI transmission with PUCCH repetition is dropped from the first slot if the first slot does not have enough available symbols for PUCCH transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE behavior for the case of RRC configured UCI transmission with PUCCH repetition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has isolated impact on RRC configured UCI transmission with PUCCH repetition if the first slot determined by current PUCCH resource allocation method does not have enough available symbols. If the network is implemented according to the CR and the UE is not, or the UE is implemented according to this CR but the network is not, there may be different understandings between gNB and UE. However, we think the corrections in this CR are common understandings which may have no impact on both UE and gNB implementation, since it is aligned with RRC configured UCI with single-slot PUCCH transmission. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.2.6	PUCCH repetition procedure

For PUCCH formats 1, 3, or 4, a UE can be configured a number of slots, , for repetitions of a PUCCH transmission by respective nrofSlots.

For ,

-	the UE repeats the PUCCH transmission with the UCI over  slots

-	a PUCCH transmission in each of the  slots has a same number of consecutive symbols, as provided by nrofSymbols in PUCCH-format1, nrofSymbols in PUCCH-format3, or nrofSymbols in PUCCH-format4

-	a PUCCH transmission in each of the  slots has a same first symbol, as provided by startingSymbolIndex in PUCCH-format1, startingSymbolIndex in PUCCH-format3, or startingSymbolIndex in PUCCH-format4

-	the UE is configured by interslotFrequencyHopping whether or not to perform frequency hopping for PUCCH transmissions in different slots

-	if the UE is configured to perform frequency hopping for PUCCH transmissions across different slots

-	the UE performs frequency hopping per slot

-	the UE transmits the PUCCH starting from a first PRB, provided by startingPRB, in slots with even number and starting from the second PRB, provided by secondHopPRB, in slots with odd number. The slot indicated to the UE for the first PUCCH transmission has number 0 and each subsequent slot until the UE transmits the PUCCH in slots is counted regardless of whether or not the UE transmits the PUCCH in the slot

-	the UE does not expect to be configured to perform frequency hopping for a PUCCH transmission within a slot

-	If the UE is not configured to perform frequency hopping for PUCCH transmissions across different slots and if the UE is configured to perform frequency hopping for PUCCH transmissions within a slot, the frequency hopping pattern between the first PRB and the second PRB is same within each slot

If the UE determines that, for a PUCCH transmission in a slot, the number of symbols available for the PUCCH transmission is smaller than the value provided by nrofSymbols for the corresponding PUCCH format, the UE does not transmit the PUCCH in the slot.

A SS/PBCH block symbol is a symbol indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon.

For unpaired spectrum, the UE determines the  slots for a PUCCH transmission starting from a slot indicated to the UE as described in Clause 9.2.3 and having

-	an UL symbol, as described in Clause 11.1, or flexible symbol that is not SS/PBCH block symbol provided by startingSymbolIndex in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 as a first symbol, and

-	consecutive UL symbols, as described in Clause 11.1, or flexible symbols that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4

except that for repetitions of a PUCCH transmission which is not in response to a DCI format detection and a slot indicated to the UE as described in Clause 9.2.3 does not satisfy the above conditions, the UE does not transmit the PUCCH transmission.

For paired spectrum, the UE determines the  slots for a PUCCH transmission as the  consecutive slots starting from a slot indicated to the UE as described in Clause 9.2.3.

If a UE would transmit a PUCCH over a first number  of slots and the UE would transmit a PUSCH over a second number of slots, and the PUCCH transmission would overlap with the PUSCH transmission in one or more slots, and the conditions in Clause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied in the overlapping slots, the UE transmits the PUCCH and does not transmit the PUSCH in the overlapping slots.

A UE does not multiplex different UCI types in a PUCCH transmission with repetitions over  slots. If a UE would transmit a first PUCCH over more than one slot and at least a second PUCCH over one or more slots, and the transmissions of the first PUCCH and the second PUCCH would overlap in a number of slots then, for each slot of the number of slots and with UCI type priority of HARQ-ACK > SR > CSI with higher priority > CSI with lower priority

-	the UE does not expect the first PUCCH and any of the second PUCCHs to start at a same slot and include a UCI type with same priority

-	if the first PUCCH and any of the second PUCCHs include a UCI type with same priority, the UE transmits the PUCCH starting at an earlier slot and does not transmit the PUCCH starting at a later slot

-	if the first PUCCH and any of the second PUCCHs do not include a UCI type with same priority, the UE transmits the PUCCH that includes the UCI type with higher priority and does not transmit the PUCCH that include the UCI type with lower priority

A UE does not expect a PUCCH that is in response to a DCI format detection to overlap with any other PUCCH that does not satisfy the corresponding timing conditions in Clause 9.2.5.

If a UE would transmit a PUCCH over  slots and the UE does not transmit the PUCCH in a slot from the  slots due to overlapping with another PUCCH transmission in the slot, the UE counts the slot in the number of  slots.