3GPP TSG RAN WG1 #102-e					  	R1-2005757

e-Meeting, August 17th – 28th, 2020

Agenda item:	8.6.3

Source:	NEC

Title:              	Discussion on coverage recovery and capacity impact

Document for:	Discussion and Decision

- Introduction

One of the objective in the SID [1] of support of reduced capability NR devices is,

Study functionality that will enable the performance degradation of such complexity reduction to be mitigated or limited, including [RAN1]: - Coverage recovery to compensate for potential coverage reduction due to the device complexity reduction.

In this contribution, we provide our view on the coverage recovery and capacity impact.

- Discussion

  - Coverage impact due to reduced capability

The definition of coverage of coverage recovery in NR_RedCap and coverage enhancement in NR_CovEnh may be different. In coverage enhancement, the coverage is defined by the coverage to satisfy the date rate. In such definition, larger number of repetition has little benefit for coverage, since it will not increase the data rate than the case of less number of repetition. The coverage in reduced capability is more like coverage to satisfy the basic connection. After the operators deploy the network based on the normal UE and satisfy the basic connection for that normal UE after network optimization, the reduced capability UE should not lost the basic connection to the network that requires operator’s adjustment on deployed network.

In such definition, the reduced number of UE Rx/Tx antennas may have impact. For Rx antennas reduction, e.g. reducing 4 Rx into 2 Rx, from baseband point of view, about 3dB is lost at receiver. However, this Rx antennas means the Rx RF chains with e.g. LNA, ADC, and the receiving area of antennas may not have to reduce half. It depends on the implementation of devices since RF chain is the main cost of antennas port not antennas itself. Reducing antennas port doesn’t imply to reduce the antennas size of devices. In such structure, the real coverage less due to Rx antennas reduction may be less than 3dB.

For reduced bandwidth, due to the less frequency diversity gain, the transmission performance for downlink and uplink may have some impact. When the bandwidth of SSB and CORESET#0 is supported by reduced capability UE, reduced bandwidth will not cause to lose the basic connection.

Proposal 1: Coverage impact due to reduced capability should be studied. The coverage impact for basic connection and for date rate should be distinguished.

  - Potential coverage recovery functionality

Coverage enhancement is studied in Rel-17. The methods in coverage enhancement of Rel-17 and existing coverage improvement method could be the basic method for coverage recovery in reduced capability UE. Only reduced capability UE specific feature could be studied under NR_RedCap.

For example, basic connection enhancement is not much consider in NR_CovEnh. In NR_CovEnh, a minimum data rate is required which is not the case for NR_RedCap. Some potential enhancement, e.g. further increase the repetition number, could be considered in NR_RedCap. Another way to satisfy the basic connection but not increase the repetition number is to reduce the TBS. TBS scaling for small data can maintain the basic connection between reduced capability UE and network.

Another example is that some function may not be needed for reduced capability devices hence some field in DCI can be removed or reduced. The reduced DCI is reduced capability devices specific feature which could be studied under NR_RedCap.

Proposal 2: The methods in coverage enhancement of Rel-17 and existing coverage improvement method could be the basic method for coverage recovery in reduced capability UE. Only reduced capability UE specific coverage recovery could be studied under NR_RedCap.

- Conclusion

In this contribution, we provide our view on the coverage recovery and we propose that,

Proposal 1: Coverage impact due to reduced capability should be studied. The coverage impact for basic connection and for date rate should be distinguished.

Proposal 2: The methods in coverage enhancement of Rel-17 and existing coverage improvement method could be the basic method for coverage recovery in reduced capability UE. Only reduced capability UE specific coverage recovery could be studied under NR_RedCap.

- References

[1] RP-193238, New SID on support of reduced capability NR devices, TSG RAN Meeting #86, Ericsson.