3GPP TSG RAN WG1 Meeting #102-e                                       R1-2005780

e-Meeting, August 17th – August 28th, 2020

Source:         ZTE

Title:            Discussion on QCL for PRS

Agenda item:    7.2.8

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss issues in the current specification support about QCL source for DL PRS.

QCL for DL PRS

In UE feature agenda of RAN1#101-e meeting, FG 13-7 is agreed to be optional with UE capability signaling. If UE supports this FG, SSB from neighbor cell can be used to deliver QCL-TypeC for FR1, or QCL-TypeC and QCL-TypeD for FR2 to a PRS resource.

| 13. NR Positioning | 13-7 | Support of SSB from neighbor cell as QCL source of a DL PRS | - Support of SSB from neighbor cell as QCL source of a DL PRS - Support of reuse SSB measurement from RRM for receiving PRS Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2 |
| --- | --- | --- | --- |

Specifically, when SSB is configured as the QCL source of a DL PRS resource, it can only be the QCL source of TypeC in FR1, or the QCL source of TypeC and TypeD in FR2. In other words, when QCL-typeD is not applicable (i.e. FR1), SSB is only used to deliver QCL-TypeC, when QCL-typeD is applicable (i.e. FR2), SSB is used to deliver both QCL-typeC and QCL-typeD. Thus, there is no need to indicate QCL type. It it noted that due to the same reason, a similar description like 'QCL-TypeD' when applicable has existed in 38.214 [1] for non-positioning RS.

However, in the current 37.355 [2], one of three QCL types is indicated by rs-Type-r16, this causes unnecessary signaling overhead and misunderstanding. For example, it will cause undefined UE behavior if rs-Type-r16 indicates typeD in FR1 or typeC in FR2. So we suggest sending an LS to RAN2 to remove rs-Type-r16 or make it dummy, and update the corresponding description in 38.214. In addition, “-” should be deleted from 'QCL-Type-C' and 'QCL-Type-D' to align with description in other parts of 38.214.

DL-PRS-QCL-Info-r16 ::= CHOICE {

ssb-r16						SEQUENCE {

pci-r16							NR-PhysCellID-r16,

ssb-Index-r16					INTEGER (0..63),

rs-Type-r16						ENUMERATED {typeC, typeD, typeC-plus-typeD}

},

dl-PRS-r16					SEQUENCE {

qcl-dl-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16,

qcl-dl-PRS-ResourceSetID-r16	NR-DL-PRS-ResourceSetID-r16

}

}

Proposal 1: Send an LS to RAN2 to clarify rs-Type-r16 is not used in RAN1, and adopt the following TP for 38.214.

========================TS 38.214 clause 5.1.6.5 unchanged parts omitted=======================

-	dl-PRS-QCL-Info-r16 defines any quasi-colocation information of the DL PRS resource with other reference signals. The DL PRS may be configured to be 'QCL-TypeD' with a DL PRS from a serving cell or a non-serving cell. The DL PRS may be configured to be 'QCL-TypeC' and 'QCL-TypeD', when applicable, with a SS/PBCH Block from a serving or non-serving cell.

...

If the UE is configured with dl-PRS-QCL-Info-r16 and the QCL relation is between two DL PRS resources, then the UE assumes those DL PRS resources are from the same cell. If dl-PRS-QCL-Info-r16 is configured to the UE with 'QCL-TypeD' with a source DL-PRS-Resource then the nr-DL-PRS-ResourceSetId-r16 and the nr-DL-PRS-ResourceId-r16 of the source DL PRS resource are expected to be indicated to the UE.

===================================unchanged parts omitted==============================

Conclusion

In this contribution, we have discussed QCL source for DL PRS. We have following proposal,

Proposal 1: Send an LS to RAN2 to clarify rs-Type-r16 is not used in RAN1, and adopt the following TP for 38.214.

========================TS 38.214 clause 5.1.6.5 unchanged parts omitted=======================

-	dl-PRS-QCL-Info-r16 defines any quasi-colocation information of the DL PRS resource with other reference signals. The DL PRS may be configured to be 'QCL-TypeD' with a DL PRS from a serving cell or a non-serving cell. The DL PRS may be configured to be 'QCL-TypeC' and 'QCL-TypeD', when applicable, with a SS/PBCH Block from a serving or non-serving cell.

...

If the UE is configured with dl-PRS-QCL-Info-r16 and the QCL relation is between two DL PRS resources, then the UE assumes those DL PRS resources are from the same cell. If dl-PRS-QCL-Info-r16 is configured to the UE with 'QCL-TypeD' with a source DL-PRS-Resource then the nr-DL-PRS-ResourceSetId-r16 and the nr-DL-PRS-ResourceId-r16 of the source DL PRS resource are expected to be indicated to the UE.

===================================unchanged parts omitted==============================

References

- 3GPP TS 38.214 V16.2.0 (2020-06), Physical layer procedures for data (Release 16)

- 3GPP TS 37.355 V16.1.0(2020-07), LTE Positioning Protocol (LPP) (Release 16)