3GPP TSG RAN WG1 Meeting #102-e                                                                                  R1-2005810

E-meeting, August 17th – 28th, 2020

Agenda Item:	7.2.2.2.2

Source:	Huawei, HiSilicon

Title:	Maintenance on initial access procedures

Document for:	Discussion and Decision

Introduction

In this paper, we provide the text proposal on TS 38.212 reflecting the latest agreement in RAN2 on the existing of LSB of SFN in DCI format 1_0.

LSB of SFN in DCI format 1_0

In the last meeting, an LS [1] from RAN2 was received that LSB of SFN is not always in the DCI 1_0 when RAR/msgB window is smaller than 10ms. Since no confliction would occur under this case, the “LSBs of SFN” field is not necessary. The following TP should be captured in TS38.212.

------------------------------------ Start of TP#1 for TS38.212 ---------------------------------------------------------

7.3.1.2.1	Format 1_0

*** Unchanged text is omitted ***

-	LSBs of SFN – 2 bits for the DCI format 1_0 with CRC scrambled by MsgB-RNTI as defined in Clause 8.2A of [5, TS 38.213] if msgB-ResponseWindow-r16 is configured larger than 10ms; or 2 bits for the DCI format 1_0 with CRC scrambled by RA-RNTI as defined in Clause 8.2 of [5, TS 38.213] for operation in a cell with shared spectrum channel access if ra-ResponseWindow-r16 is configured larger than 10ms; 0 bit otherwise

*** Unchanged text is omitted ***

------------------------------------ End of TP#1 for TS38.212 ---------------------------------------------------------

Proposal 1: The agreement in RAN2 LS on LSB of SFN should be reflected in TS38.212 as TP#1.

Correspondingly in TS 38.213, the concern on the UE behavior to detect this field is clarified and wording “if included and applicable” for 4 step RACH in section 8.2 and “if applicable” for 2 step RACH in section 8.2 A should be fixed according to RAN2 agreement. The following TP should be captured in TS38.213.

------------------------------------ Start of TP#2 for TS38.213 ---------------------------------------------------------

8.2	Random access response - Type-1 random access procedure

*** Unchanged text is omitted ***

If the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI and LSBs of a SFN field in the DCI format 1_0, if included, are same as corresponding LSBs of the SFN where the UE transmitted PRACH, and the UE receives a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers parse the transport block for a random access preamble identity (RAPID) associated with the PRACH transmission. If the higher layers identify the RAPID in RAR message(s) of the transport block, the higher layers indicate an uplink grant to the physical layer. This is referred to as random access response (RAR) UL grant in the physical layer.

If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window and LSBs of a SFN field in the DCI format 1_0, if included, are not same as corresponding LSBs of the SFN where the UE transmitted PRACH, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit a PRACH. If requested by higher layers, the UE is expected to transmit a PRACH no later than  msec after the last symbol of the window, or the last symbol of the PDSCH reception, where  is a time duration of  symbols corresponding to a PDSCH processing time for UE processing capability 1 assuming  corresponds to the smallest SCS configuration among the SCS configurations for the PDCCH carrying the DCI format 1_0, the corresponding PDSCH when additional PDSCH DM-RS is configured, and the corresponding PRACH. For , the UE assumes  [6, TS 38.214]. For a PRACH transmission using 1.25 kHz or 5 kHz SCS, the UE determines  assuming SCS configuration .

If the UE detects a DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI and LSBs of a SFN field in the DCI format 1_0, if included, are same as corresponding LSBs of the SFN where the UE transmitted the PRACH, and the UE receives a transport block in a corresponding PDSCH, the UE may assume same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block or a CSI-RS resource the UE used for PRACH association, as described in Clause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI format 1_0.

*** Unchanged text is omitted ***

If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window and the LSBs of a SFN field in the DCI format 1_0, if included, are not same as corresponding LSBs of the SFN where the UE transmitted the PRACH, or the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321].

8.2A	Random access response - Type-2 random access procedure

*** Unchanged text is omitted ***

If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding MsgB-RNTI and LSBs of a SFN field in the DCI format 1_0, if included, are same as corresponding LSBs of the SFN where the UE transmitted PRACH, and the UE receives a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer

*** Unchanged text is omitted ***

The UE does not expect to be indicated to transmit the PUCCH with the HARQ-ACK information at a time that is prior to a time when the UE applies a TA command that is provided by the transport block. If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window and LSBs of a SFN field in the DCI format 1_0, if included, are not same as corresponding LSBs of the SFN where the UE transmitted the PRACH, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure [11, TS 38.321]. If requested by higher layers, the UE is expected to transmit a PRACH no later than NT,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].

Unless the UE is configured a SCS, the UE receives subsequent PDSCH using same SCS as for the PDSCH reception providing the RAR message.

If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window, or if the UE detects the 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window and LSBs of a SFN field in the DCI format 1_0, if included, are not same as corresponding LSBs of the SFN where the UE transmitted the PRACH, or the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321].

------------------------------------ End of TP#2 for TS38.213 ---------------------------------------------------------

Proposal 2: The agreement in RAN2 LS on LSB of SFN should be reflected in TS38.213 as TP#2.

Conclusions

In this paper, we provide text proposal corresponding to the LS from RAN2 on LSB of SFN in RACH procedure. We had following two proposals.

Proposal 1: The agreement in RAN2 LS on LSB of SFN should be reflected in TS38.212 as TP#1.

Proposal 2: The agreement in RAN2 LS on LSB of SFN should be reflected in TS38.213 as TP#2.

References

R2-2005865, “LS to RAN1 on signalling SFN bits for random access response”, Ericsson