3GPP TSG RAN WG1 Meeting #102-e                                                                              R1-2005813

E-meeting, August 17th – 28th, 2020

Agenda Item:	7.2.2.2.5

Source:	Huawei, HiSilicon

Title:	Maintenance on the wideband operation procedures

Document for:	Discussion and Decision

# Introduction

In the paper, we discuss the remaining issues on the wideband operation for shared spectrum operation after RAN1#101-e.

# PDCCH monitoring

When counting on the number of allocated PDCCH candidates and non-overlapped CCE, the UE subtracts the number of PDCCH candidates and non-overlapped CCEs per search space in ascending order of search space ID. In Rel-15, UE would drop the whole search space when the number of allocated PDCCH candidate or non-overlapped CCE exceed maximum number. If a search space has multiple monitoring locations, UE will subtract PDCCH candidate and non-overlapped CCE per monitoring location in ascending order of RB set where monitoring location is. UE drops the PDCCH blind detection in the rest of monitoring location(s) and search space(s).

Proposal 1: UE can drop PDCCH BD per monitoring location in a search space. TP#1 is used.

# Uplink resource mapping on the intra-cell guard band

For uplink resource allocation type 0, the granularity of resource allocation is one RBG. The size of RBG depends on number of PRB in a BWP, which is defined in the Table 6.1.2.2.1-1 in TS38.214. One RBG includes at most 16 PRB.  On the other side, the configuration of intra cell guard band is in the unit of RB, which usually does not align with the RBG boundaries. Based on NR Rel-15 mechanism, at most 15 PRB might be wasted in an RB set as illustrated in the Figure 1. It would be beneficial for UE to rate match RB indicated by FDRA while overlapping with intra cell guard band between two RB set where PUSCH is not scheduled on one of the RB set.

Figure 1 Resource allocation type 0 in RB set

Proposal 2: For resource allocation type 0, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated RBG(s) and union of RB set(s) overlapping with the indicated RBG(s) and intra cell guard bands between the adjacent RB sets overlapping with the indicated RBG(s), if any. The corresponding text proposal is in TP#2 in the appendix.

# Conclusions

Based on the discussion, we have following observations and conclusions.

Proposal 1: UE can drop PDCCH BD per monitoring location in a search space. TP#1 is used.

Proposal 2: For resource allocation type 0, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated RBG(s) and union of RB set(s) overlapping with the indicated RBG(s) and intra cell guard bands between the adjacent RB sets overlapping with the indicated RBG(s), if any. The corresponding text proposal is in TP#2 in the appendix.

# Appendix

## TP#1: TS38.213

10.1	UE procedure for determining physical downlink control channel assignment

*** Unchanged text is omitted ***

Denote by  the set of non-overlapping CCEs for search space set  and by  the cardinality of  where the non-overlapping CCEs for search space set  are determined considering the allocated PDCCH candidates for monitoring for the CSS sets and the allocated PDCCH candidates for monitoring for all search space sets , .

Set

Set

Set

while  AND

allocate  PDCCH candidates for monitoring to USS set

;

;

;

end while

if freqMonitorLocations-r16 is configured, there are KMLmonitoring locations in frequency domain in the search space set Sussj. Denote by VCCESussj the set of non-overlapping CCEs in each monitoring location of search space set Sussj and by CVCCESussj the cardinality of VCCESussj where the non-overlapping CCEs in each monitoring location of search space set Sussj are determined considering the allocated PDCCH candidates for monitoring for the CSS sets and the allocated PDCCH candidates for monitoring for all search space sets Sussj, 0≤k≤j .

Set  ;

Set ;

Set ;

While LMSussj L≤MPDCCHuss AND CVCCESussj≤CPDCCHuss

Set l=0;

While l+1∙LMSussj L≤MPDCCHuss  AND  l+1∙CVCCESussj≤CPDCCHuss AND l<KML

k=k+1;

end while

allocate k∙LMSussj L PDCCH candidates for monitoring to USS set

MPDCCHuss=MPDCCHuss-k∙LMSussj L;

CPDCCHuss=CPDCCHuss-k∙CVCCESussj;

j=j+1;

end while

*** Unchanged text is omitted ***

## TP#2: TS38.214

##### 6.1.2.2.1	Uplink resource allocation type 0

In uplink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Size configured in pusch-Config and the size of the bandwidth part as defined in Table 6.1.2.2.1-1. For operation with shared spectrum channel access mechanism, The UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated RBGs and the union of RB sets overlapping with the indicated RBGs and intra-cell guard bands defined in Clause 7 between the adjacent RB sets overlapping with the indicated RBGs, if any.

*** Unchanged text is omitted ***