3GPP TSG RAN WG1 Meeting #102-E					      R1-2005886

e-Meeting, August 17th – 28th, 2020

Source:	Intel Corporation

Title:	                       On PDCCH monitoring reduction techniques during active time

Agenda item:	8.7.1.3

Document for:	Discussion and Decision

Introduction

The RAN WG approved work item on Power Saving Enhancements [1]. The work item includes the following objective for power saving enhancements during active time.

- Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4] - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1] - NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized.

In this contribution, we express our views on DCI based power saving adaptation and PDCCH monitoring reduction schemes during active time.

# PDCCH monitoring reduction techniques during active time

PDCCH monitoring accounts for major power consumption during active time. In Rel-16, Scell dormancy was introduced which allows for sparse or no PDCCH monitoring activity in one or more activated Scells. However, in the scheduling or Pcell, no PDCCH monitoring reduction was considered. During Rel-16 SI, several PDCCH monitoring reduction techniques were evaluated and observations were captured in TR. We suggest to reuse relevant observations from TR 38.840 and avoid repeating similar evaluations again.

Proposal 1: Reuse observations made in TR 38.840 for the study on PDCCH monitoring reduction techniques and evaluation.

In this section, we evaluate two PDCCH monitoring reduction techniques:

- Go-to-sleep signal: When Go-to-sleep signal is received, UE may skip PDCCH monitoring for the indicated or configured duration. UE may optionally perform measurements during the sleep duration.

- Adaptive PDCCH monitoring periodicity: For the adaptive PDCCH monitoring periodicity scheme, UE monitors PDCCH in a sparse manner, and when grant is received, UE stays active for a certain duration, monitors PDCCH every slot, and if no further grant is received, UE returns to sparse monitoring after end of the duration.

## Performance comparison with respect to Rel-15/16 Schemes

During Rel-16, performance comparisons were shown with respect to baseline DRX configurations and it was shown that Go-to-sleep signal provides significant power saving gain with respect to C-DRX configuration [2]. In this contribution, we focus on additional power saving gain that can be achieved with respect to Rel-15 and Rel-16 schemes such as BWP adaptation and cross-slot scheduling, respectively.

We performed SLS evaluations assuming the following parameters in Table 1. Figures 1a, 1b show power consumption and throughput performances for baseline DRX configuration on top of NR Rel-16 power saving schemes. The results are presented for different types of UEs: 5% (cell edge), 50% and 95% (cell center) from SINR CDF statistics. IMT-2020 assumptions are listed in the appendix in Table 3. As can be seem from Figure 1, go-to-sleep signal provides significant additional power saving gain compared to Rel-15/16 schemes.

Table 1: Simulation parameters

| Simulation parameters | Value |
| --- | --- |
| DRX cycle | DRX cycle length = 160ms On Duration = 8ms Inactivity timer = 100ms |
| Traffic model | FTP model 3: 0.5 Mbyte packet size 200ms mean inter-arrival time |
| K0 (Cross-slot scheduling offset) | 1 |
| BWP adaptation | BWP for data – 100 MHz BWP for PDCCH monitoring – 20MHz |
| PDCCH reduction schemes | GTS: 3ms/10ms sleep duration Adaptive PDCCH monitoring scheme: 3ms sleep duration 8ms of mandatory active state duration after PDSCH reception |

(a)

(b)

Figure 1. Power consumption (a) and UPT performance (b) of different PDDCH monitoring reduction schemes

Observation 1: GTS provides significant additional power saving gain with respect to cross-slot scheduling with BWP adaptation enabled

- For cell center UE, i.e., 50% UE, GTS with up to 10ms sleep duration provides up to 30% additional power saving gain on top of power saving provided by Rel-16 schemes with less than 13% UPT loss.

In the appendix in Figure 3, we provide another set of results where it is assumed that UE would still perform measurements if an occasion falls within sleep duration. For the evaluation, it is assumed that UE performs measurement every 20ms. For this setup, go-to-sleep signal results in significant power saving gain with some UPT loss.

Next, we consider comparison between PDCCH monitoring reduction techniques and short DRX.

## Performance comparison with respect to short DRX

For comparison with respect to short DRX, following parameters are used as shown in Table 2.

Table 2: Simulation parameters

| Simulation parameters | Value |
| --- | --- |
| Long DRX cycle | DRX cycle length = 160ms On Duration = 8ms Inactivity timer = 100ms |
| Short DRX cycle | {Short DRX cycle length = 10ms, Short DRX timer = 10} {Short DRX cycle length = 20ms, Short DRX timer = 5} |
| Short DRX triggering | MAC CE with 10ms delay |
| Traffic model | FTP model 3: 0.5 Mbyte packet size 200ms mean inter-arrival time |
| Bandwidth | 100 MHz |
| GTS parameters | GTS: 3ms sleep duration 10ms sleep duration |

Figures 2 a, 2b shows comparison of power consumption and throughput performances for go-to-sleep signal and short DRX with baseline DRX configuration. As can be observed from results, go-to-sleep signal results in much higher power saving gain than short DRX, with respect to baseline DRX configuration. In our view, triggering of Go-to-sleep signaling is more flexible and faster than short DRX, and can be adapted to traffic better.

(a)

(b)

Figure 2. Power consumptions and UPT comparison of GTS and Short DRX

Observation 2: Go-to-sleep signal has better potential for power saving gain compared to Short DRX.

Observation 3: For cell center UE, Go-to-sleep signal with 10ms sleep duration provide 29% power saving gain with 12% UPT loss compared to short DRX with 10ms.

Based on these observations, we have the following proposal:

Proposal 1: NR supports DCI based go-to-sleep signals during active time for PDCCH monitoring reductions

- FFS: Extension of Rel-16 DCI based solutions for triggering go-to-sleep signal.

- FFS: Monitoring of measurement signals during sleep duration

Conclusions

In summary, we have following list of observations and proposals:

Observation 1: GTS provides significant additional power saving gain with respect to cross-slot scheduling with BWP adaptation enabled

- For cell center UE, i.e., 50% UE, GTS with up to 10ms sleep duration provides up to 30% additional power saving gain on top of power saving provided by Rel-16 schemes with less than 13% UPT loss.

Observation 2: Go-to-sleep signal has better potential for power saving gain compared to Short DRX.

Observation 3: For cell center UE, Go-to-sleep signal with 10ms sleep duration provide 29% power saving gain with 12% UPT loss compared to short DRX with 10ms.

Proposal 1: NR supports DCI based go-to-sleep signals during active time for PDCCH monitoring reductions

- FFS: Extension of Rel-16 DCI based solutions for triggering go-to-sleep signal.

- FFS: Monitoring of measurement signals during sleep duration

References

- RP-193239, “New WID: UE Power Saving Enhancements”, MediaTek.

- R1-1902507, “System level evaluation of UE power saving schemes”, RAN1 96.

Appendix

## Table 3: IMT-2020 Simulation assumptions

| Simulation Parameters | Scenario |
| --- | --- |
|  | Dense Urban Macro FR1 DL |
| Carrier Frequency | 4GHz |
| Simulation BW | 100MHz (TDD) |
| Sub-carrier Spacing | 30 kHz |
| Channel Model | IMT UMa A |
| Inter-Site Distance | 200m |
| BS Antenna Configuration | (1,1,4,1,2;2,8) 32 TXRU |
| BS Antenna Spacing | (0.8, 0.5) |
| UE Antenna Configuration | (1,1,1,2,2; 1,2) 4 TXRU (1,1,1,1,2; 1,2) 2 TXRU |
| UE Antenna Spacing | (0.5, 0.5) |
| Beam set at BS | Single beam at 102° electronic downtilt |
| Beam Set at UE | Single Beam Omni-Directional |
| UE Deployment | 80% Indoor; 20% Outdoor |
| Highest Modulation | 256 QAM |
| Channel Coding | LPDC |
| Transmission/Reception Scheme | Multi-user MIMO with 12 layers at BS |
| Scheduler | MU-PF rank adaptation and beam sweeping at BS |
| Feedback | Type II Codebook based CSI |

Figure 3. Power consumption (a) and UPT performance (b) of different PDDCH monitoring reduction schemes, when UE continues to perform measurements during sleep duration