3GPP TSG RAN WG1 #102-e                                                                                      R1-2005892

e-Meeting, August 17th – 28th, 2020

Source: 	Intel Corporation

Title:	Discussion on simulation assumptions for NR coverage enhancement

Agenda item:	8.8.3

Document for:	Discussion and Decision

# Introduction

At the RAN1#101-e meeting, the following agreements were made for NR coverage enhancement [1]:

Agreements:

- Adopt the following target data rates for eMBB performance evaluation for FR1.

  - Urban scenario: DL 10Mbps, UL 1Mbps

  - Rural scenario: DL 1Mbps, UL 100kbps

  - Rural with long distance scenario: DL 1Mbps, UL 100kbps, [30kbps] (optional)

Agreements:

- For VoIP performance evaluation based on link-level simulation for FR1.

  - A packet size of [320] bits with 20ms data arriving interval is adopted.

  - FFSTBD: TBS for SIP invite message. Payload of 1500 bytes can be a starting point.

Agreements:

- The basic evaluation methodology is based on link-level simulation for FR1.

  - Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements.

  - Step 2: Obtain the baseline performance based on required SINR and link budget template.

  - Note: aspects related to identifying target performance and coverage bottlenecks based on target performance metric is to be handled separately

- FFS: The evaluation methodology based on system-level simulation is optional for FR1.

  - Note: The simulation assumptions for SLS are up to companies’ reports.

Agreement:

- Down selection on the following options for the link budget template for FR1 in next meeting.

  - Option 1: Adopt single link budget template based on IMT-2020 self-evaluation with necessary revisions, including adding/removing/revising some parameters.

    - FFS: The template provided by FL in Tdoc R1-2005005.

  - Option 2: Adopt both templates, i.e. link budget template in IMT-2020 self-evaluation and link budget template in TR 36.824.

  - Option 3: Adopt single link budget template in TR 36.824 with necessary revisions, including adding/revising some parameters.

Agreement:

- Down selection on the following options for antenna array gain for LLS based methodology for FR1 in next meeting.

  - Option 1: Antenna array gain is included in the link budget template.

    - FFS: array gain = 10 * 1og10 (number of antenna elements/number of TxRUs)

    - FFS: For TDL channel model

    - FFS: Values reflective of realistic implementation and network operation.

  - Option 2: Antenna array gain is included in LLS.

    - FFS: For CDL channel model

Agreements:

- For link level simulation, adopt the following table for PUSCH and PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| Scenario and frequency | Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD) |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz Other frame structures can be reported by companies. |
| Pathloss model (select from LoS or NLoS) | Urban: NLoS Rural: NLoS and LoS |
| BWP | 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD 20MHz (optional for 10MHz) for 700MHz. (FDD) |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| Channel model for link-level simulation | TDL-C for NLOS, TDL-D for LOS. [CDL] |
| UE velocity | Urban: 3km/h for indoor Rural: 3km/h for indoor, 120km/h (optional 30km/h) for outdoor |
| Frequency hopping | w/ or w/o Intra-slot frequency hopping for PUSCH w/ frequency hopping for PUCCH is enabled. |

- FFS whether there are any additional simulation considerations for the extreme coverage scenarios (e.g., rural)

Agreement:

- For link level simulation, adopt the following table for PDSCH for FR1.

| Parameters | Values |
| --- | --- |
| Waveform | CP-OFDM |
| PRBs/MCS/TBS | Reported by companies. |
| PDSCH duration | 12 OS |
| Other parameters | FFS |

Agreements:

- For link level simulation, adopt following TBS for Msg3 for FR1

- 56 bits

Agreements

- For link level simulation, adopt the following table for PUSCH for eMBB data or VoIP for FR1.

| Parameters | Values |
| --- | --- |
| BLER for PUSCH | For eMBB, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains for PUSCH | 1，2 (optional) |
| DMRS configuration for PUSCH | For 120km/h, (Optional: 30km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type and DMRS position are reported by companies. Working assumption: For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |
| Waveform for PUSCH | DFT-s-OFDM, CP-OFDM (optional) |
| Repetitions for PUSCH | For eMBB, w/o repetition as baseline, w/ repetition (optional). For VoIP, w/ repetition. The actual number of repetitions is reported by companies. FFS: Repetition type B |
| HARQ configuration for PUSCH | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| Latency requirements for voice | 50ms/100ms |
| PUSCH duration | 14 OS |

Agreements

- For link level simulation, adopt the following table for PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| PUCCH format type | Format 1, 2bits UCI. Format 3, [4bits (3 bits A/N + 1 bit SR)]/11/22 bits UCI |
| BLER for PUCCH | For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. For PUCCH format 3: BLER for Ack/Nack, SR: 1% FFS: BLER for CSI (10% or 1%) |
| Number of PRBs for PUCCH | 1 PRB |
| Number of UE transmit chains for PUCCH | 1 |
| Number of repetitions for PUCCH | w/ repetition (optional), w/o repetition for PUCCH. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| DMRS configuration for PUCCH | FFS: number of DMRS symbols for PUCCH Format 3. |

Agreements:

- For link level simulation, adopt the following table for eMBB data or VoIP on PUSCH and for PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| Number of receive antenna elements for BS | Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of receive TxRUs for BS | gNB architectures to study for TDL: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz [gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB receive chains in LLS (as starting point). FFS: correlation - Option 2: Number of gNB receive chains = number of TXRUs in LLS. FFS: correlation.] [gNB architectures to study for CDL: - Urban: 64 receive chains for 2.6 and 4 GHz in LLS - Rural: 8 receive chains for 4GHz and 2.6GHz in LLS - 4 receive chains for 2GHz and 700MHz in LLS.] [gNB modeling in LLS for CDL: Number of gNB receive chains = number of TXRUs in LLS.] |
| Delay spread | Urban: 300ns Rural: 300ns Rural with long distance: 30ns |
| PRBs/TBS/MCS for eMBB for PUSCH | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use 30 PRBs for 1Mbps, 4 PRBs for 100kbps, 1 PRB for 30kbps as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP for PUSCH | [4 PRBs] for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK, pi/2 BPSK (optional) |

Note: For TDL models, companies report whether antenna array gain, obtained from mapping antenna elements to TXRU, is included in LLS or link budget template. Array gain calculation method and how channel estimation is accounted for is reported by companies

Agreements:

- For link level simulation, adopt the following table for Msg.3 for FR1.

| Parameters | Values |
| --- | --- |
| Number of PRBs | 2 |
| Waveform | DFT-s-OFDM |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| PUSCH duration | 14 OS |
| Other parameters | Reported by companies. |

Agreements:

- For link level simulation, adopt the following table for PDCCH for FR1.

| Parameters | Values |
| --- | --- |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Reported by companies |
| BLER for PDCCH | 1% BLER FFS: 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Agreements:

- For link level simulation, adopt the following table for SSB for FR1.

| Parameters | Values |
| --- | --- |
| Periodicity | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |
| Other parameters | Reported by companies. |

Agreements:

- For link level simulation, adopt the following table for PRACH for FR1.

| Parameters | Values |
| --- | --- |
| Format | Format 0, Format B4, or Format C2 |
| SCS | Reported by companies. |
| Performance metric | 1% missed detection at 0.1% false alarm probability FFS: 10% missed detection. |
| Other parameters | Reported by companies. |

Agreements:

- For link level simulation, for PDSCH of Msg.4 for FR1.

  - Reuse the following simulation assumption for PDSCH

    - Waveform, [PDSCH duration]

  - FFS: Payload size: [3000bits].

  - Other parameters: Reported by companies.

Agreements:

- For link level simulation, for SSB, PDCCH, PDSCH and PDCCH of Msg.2, PDSCH of Msg.4 and PDSCH for FR1.

  - Reuse following simulation assumptions agreed for PUSCH.

    - Scenario and frequency, frame structure, SCS, pathloss model, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS.

  - The number of UE receive chains: is 2.

    - 4 for 4GHz/2.6GHz

    - 2 or 4 for 2GHz

    - 2 for 700MHz

  - For PDSCH, reuse DM-RS configuration, BLER, HARQ, Latency requirements for voice agreed for PUSCH.

    - Reuse DM-RS configuration agreed for PUSCH except that 3 DMRS symbols is used for Msg2.

- For link level simulation, for PRACH and Msg.3 for FR1.

  - Reuse following simulation assumptions agreed for PUSCH

    - Scenario and frequency, frame structure, pathloss model, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS and Number of UE transmit chains.

  - For Msg.3, reuse SCS, HARQ configuration, frequency hopping agreed for PUSCH.

Agreements:

- For link level simulation, the packet size of VoIP for FR2 is the same as FR1.

Agreements:

- For link level simulation, TBS of Msg3 for FR2 is the same as FR1.

Agreements:

- The evaluation methodology for FR2 is the same as FR1.

Agreements:

- The link budget template for FR2 is the same as FR1.

Agreements:

- For link level simulation, adopt the following table for PUSCH and PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| Scenario and frequency | 28GHz |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) DDSU (S: 11D:3G:0U) Other frame structures can be reported by companies. |
| Subcarrier Space | 120kHz |
| UE velocity | Indoor scenario:3km/h Urban scenario: 3km/h for indoor, 30km/h for outdoor. Suburban scenario: 3km/h for indoor, 30km/h, (optional: 120km/h) for outdoor. |
| Occupied channel bandwidth for | 100MHz, [400MHz] |
| Frequency hopping for PUSCH | w/ or w/o frequency hopping |

Agreements:

- Adopt the following target data rates for eMBB performance evaluation for FR2.

- Indoor: DL: 25Mbps, UL:5Mbps

- Urban: DL: 25Mbps, UL: 5Mbps

- Suburban: FFS: (DL: 1Mbps, UL: 50kbps)

Agreements:

- For link level simulation, adopt the following table for eMBB data or VoIP on PUSCH and on PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| BLER | For eMBB, w/ HARQ, 10% iBLER, Optional: companies report rBLER. w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| DMRS configuration | For 30km/h (optional: 120km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping for PUSCH: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH/PDSCH mapping Type and DMRS position are reported by companies. Working assumption: For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |
| Waveform | DFT-s-OFDM for PUSCH, CP-OFDM for PDSCH FFS: CP-OFDM for PUSCH |
| Repetitions for PUSCH/PDSCH | For eMBB, w/o repetition as baseline, w/ repetition (optional). For VoIP, w/ repetition. The actual number of repetitions is reported by companies. FFS: Repetition type B for PUSCH. |
| HARQ configuration for PUSCH/PDSCH | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PUSCH/PDSCH duration | 14 OS for PUSCH, 12 OS for PDSCH |

Agreements:

- For link level simulation, adopt the following table for eMBB data or VoIP on PUSCH and on PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| Number of antenna elements for BS | Indoor scenario: 128 (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) Urban/suburban scenario: 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) Optional: 512, (M,N,P,Mg,Ng) = (8,8,2,2,2) |
| Number of TxRUs for BS | 2 Note: Analog beamforming is assumed. |
| Number of UE Tx/Rx chains | 1T2R, 2T2R |
| Channel model for link-level simulation | CDL- A, TDL-A, [urban/suburban: TDL-C] Note: company can provide simulation results based on either TDL channel or CDL model |
| Delay spread | Indoor scenario: 30ns Urban scenario: 100ns Suburban scenario: 100ns |
| Latency requirements for voice | 50ms/100ms |
| PRBs/TBS/MCS for eMBB for PUSCH/PDSCH | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use [30] PRBs for 5Mbps for PUSCH and full bandwidth for 25Mbps for PDSCH as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP for PUSCH/PDSCH | [4 PRBs] for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK for PDSCH/PUSCH Optional: pi/2 BPSK for PUSCH |

Agreements:

- For link level simulation, adopt the following simulation assumption for eMBB data or VoIP on PUSCH and on PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| Number of UE antenna elements | 8, one panel:(M, N, P) = (2,2,2), FFS: Two panels in link budget, one panel in LLS, 16 for each panel: (M, N, P) = (4,2,2) |

Agreements:

- For link level simulation, adopt the following table for PUCCH for FR2.

| Parameters | Values |
| --- | --- |
| Format | Format 1, 2bits UCI. Format 3, [4bits (3 bits A/N + 1 bit SR)]/11/22 bits UCI FFS: Format 0, 2 |
| BLER for PUCCH | The same as FR1 |
| Number of PRBs for PUCCH | The same as FR1 |
| Number of UE transmit chains for PUCCH | The same as FR1 |
| Number of repetitions for PUCCH | The same as FR1 |
| PUCCH duration | 14 OFDM symbols FFS: 4 OFDM symbols |
| DMRS configuration for PUCCH | FFS: [4] DMRS symbols for PUCCH Format 3. |

Agreements:

- For link level simulation, adopt the following table for PDCCH for FR2.

| Parameters | Values |
| --- | --- |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48PRBs |
| Tx Diversity | Reported by companies |
| BLER for PDCCH | 1% BLER. FFS: 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Agreements:

- For link level simulation, adopt the following table for PRACH for FR2.

| Parameters | Values |
| --- | --- |
| Format | Format B4, (Optional: Format C2) |
| SCS | Reported by companies. |
| Performance metric | 0.1% false alarm, 1% miss-detection FFS: 10% missed detection. |
| Number of SSB beams | Reported by companies |
| Other parameters | Reported by companies. |

Agreements:

- For link level simulation, for SSB, PDCCH, PDSCH and PDCCH of Msg.2, PDSCH of Msg.4 for FR2.

  - Reuse following simulation assumptions for PDSCH

    - Scenario and frequency, frame structure, SCS, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS, number of UE Tx/Rx chains and UE antenna elements.

- For link level simulation, for PUCCH, PRACH and Msg.3 for FR2.

  - Reuse following simulation assumptions for PUSCH

    - Scenario and frequency, frame structure, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS, number of UE antenna elements for PUSCH.

  - For PRACH and Msg.3, reuse number of UE Tx chains for PUSCH.

  - For PUCCH, reuse SCS for PUSCH.

  - For Msg.3, reuse SCS, HARQ configuration, frequency hopping for PUSCH.

In the contribution, we discuss link level simulation assumption for NR coverage enhancement in FR1 and FR2. Our views on baseline coverage performance for FR1 and FR2 are described in our companion contributions [2] and [3], respectively.

# Simulation assumptions for coverage enhancement in FR1

Table 1 summarizes link level simulation assumptions for NR coverage enhancement in FR1 for PDSCH and PUSCH, including TBS, MCS, number of PRBs and DMRS configurations in the simulations.

Table 1. Link level simulation assumptions for NR coverage enhancement in FR1

| Parameters | Values |
| --- | --- |
| Frame structure | DDDSU (S: 10D:2G:2U) at 4GHz |
| PDSCH related parameters | DMRS configuration: Type I, no multiplexing with data. eMBB - Maximum number of HARQ transmissions = 1 - Urban scenario, 4GHz - TBS = 9480, MCS = 4, 118 PRBs, 1 DMRS symbol - Rural scenario, 700MHz - 3km/h: TBS = 1128, 14 PRBs, MCS = 4, 1 DMRS symbol - 120km/h: TBS = 1128, 17 PRBs, MCS = 4, 3 DMRS symbols - Rural scenario, 4GHz - 3km/h: TBS = 928, 14 PRBs, MCS = 3, 1 DMRS symbol - 120km/h: TBS = 928, 14 PRBs, MCS = 4, 3 DMRS symbols - Rural long distance scenario - 3km/h: TBS = 1128, 17 PRBs, MCS = 3, 1 DMRS symbol - 120km/h: TBS = 1128, 17 PRBs, MCS = 4, 3 DMRS symbols VoIP - 4 repetitions and maximum number of HARQ transmissions = 2 - For all scenarios: - 3km/h: TBS = 320, MCS = 4, 4 PRBs, 1 DMRS symbol - 120km/h: TBS = 320, MCS = 5, 4 PRBs, 3 DMRS symbols Msg2 - For all scenarios - TBS = 72, MCS = 0, TBS scaling = 0.25, 11 PRBs, 3 DMRS symbols Msg4 - For all scenarios - TBS = 1064, MCS = 0, 37 PRBs, 2 DMRS symbols SIB1 - For all scenarios - TBS = 2976, MCS = 4, 40 PRBs, 2 DMRS symbols, 4 repetitions |
| PUSCH related parameters | DFT-s-OFDM waveform Intra-slot frequency hopping is enabled DMRS configuration: Type I, no multiplexing with data eMBB: - Maximum number of HARQ transmissions = 1 - Urban scenario, 4GHz - MCS = 5, TBS = 2856, 30 PRBs, 2 DMRS symbols - Rural scenario, 4GHz - 3km/h: MCS = 3, TBS = 288, 4 PRBs, 2 DMRS symbols - 120km/h: MCS = 4, TBS = 304, 4 PRBs, 4 DMRS symbols - Rural scenario, 700MHz - 3km/h: MCS = 0, TBS = 136, 4 PRBs, 2 DMRS symbols - 120km/h: MCS = 0, TBS = 120, 4 PRBs, 4 DMRS symbols - Rural with long distance, 700MHz - 3km/h: MCS = 0, TBS = 136, 4 PRBs, 2 DMRS symbols - 120km/h: MCS = 0, TBS = 120, 4 PRBs, 4 DMRS symbols VoIP: - 4 repetitions and maximum number of HARQ transmissions = 2 - For all scenarios: - MCS = 4, TBS = 320, 4 PRBs, 3 DMRS symbols Msg3: - For all scenarios: - MCS = 0, TBS = 56, 2 PRBs, 3 DMRS symbols |

# Simulation assumptions for coverage enhancement in FR2

Table 2 summarizes link level simulation assumptions for NR coverage enhancement in FR2 for PDSCH and PUSCH, including TBS, MCS, number of PRBs and DMRS configurations in the simulations.

Table 2. Link level simulation assumptions for NR coverage enhancement in FR2

| Parameters | Values |
| --- | --- |
| Frame structure | DDDSU (S: 10D:2G:2U) at 28GHz |
| PDSCH related parameters | DMRS configuration: Type I, no multiplexing with data. eMBB - Maximum number of HARQ transmissions = 1 - Indoor scenario - TBS = 5888, MCS = 5, 60 PRBs, 1 DMRS symbol - Urban scenario - 3km/h: TBS = 5888, MCS = 5, 60 PRBs, 1 DMRS symbol - 30km/h: TBS = 5888, MCS = 5, 66 PRBs, 2 DMRS symbols - Suburban scenario - 3km/h: TBS = 240, MCS = 4, 3 PRBs, 1 DMRS symbol - 30km/h: TBS = 304, MCS = 4, 4 PRBs, 2 DMRS symbols VoIP - 4 repetitions and maximum number of HARQ transmissions = 2 - For all scenarios: - 3km/h: TBS = 320, MCS = 4, 4 PRBs, 1 DMRS symbol - 30km/h: TBS = 320, MCS = 5, 4 PRBs, 3 DMRS symbols Msg2 - For all scenarios - TBS = 72, MCS = 0, TBS scaling = 0.25, 11 PRBs, 3 DMRS symbols Msg4 - For all scenarios - TBS = 1064, MCS = 3, 18 PRBs, 2 DMRS symbols SIB1 - For all scenarios - TBS = 2976, MCS = 4, 40 PRBs, 2 DMRS symbols, 4 repetitions |
| PUSCH related parameters | DFT-s-OFDM waveform Intra-slot frequency hopping is enabled DMRS configuration: Type I, no multiplexing with data eMBB: - Maximum number of HARQ transmissions = 1 - Urban and Indoor scenarios: - 3km/h: MCS = 6, TBS = 3824, 30 PRBs, 2 DMRS symbols - 30km/h: MCS = 7, TBS = 3752, 30 PRBs, 4 DMRS symbols - Suburban scenario: - 3km/h: MCS = 0, TBS = 40, 1 PRB, 2 DMRS symbols - 30km/h: MCS = 0, TBS = 40, 1 PRB, 4 DMRS symbols VoIP: - 4 repetitions and maximum number of HARQ transmissions = 2 - For all scenarios: - MCS = 4, TBS = 320, 4 PRBs, 3 DMRS symbols Msg3: - For all scenarios: - MCS = 0, TBS = 56, 2 PRBs, 3 DMRS symbols |

For NR coverage enhancement, it is more appropriate to align simulation assumptions for PDSCH and PUSCH, especially MCS, TBS, number of PRBs and DMRS configuration for various target deployment scenarios, so as to conduct meaningful link budget analysis. Further, baseline coverage performance and corresponding coverage enhancement target for each physical channel, especially for the physical channel with coverage bottleneck, can be derived accordingly.

Based on the discussion above, it is suggested to consider Table 1 and Table 2 as a starting point of discussion for link level simulation assumptions in FR1 and FR2 for NR coverage enhancement.

Proposal 1

- For link budget analysis, it is more desirable to align simulation assumptions for PDSCH and PUSCH, especially MCS, TBS, number of PRBs and DMRS configuration for various deployment scenarios.

  - Consider Table 1 and Table 2 as a starting point of discussion for link level simulation assumptions in FR1 and FR2 for NR coverage enhancement.

# Conclusions

In this contribution, we discussed the link level simulation assumptions for NR coverage enhancement in FR1 and FR2. Further, we summarize the proposal as follows:

Proposal 1

- For link budget analysis, it is more desirable to align simulation assumptions for PDSCH and PUSCH, especially MCS, TBS, number of PRBs and DMRS configuration for various deployment scenarios.

  - Consider Table 1 and Table 2 as a starting point of discussion for link level simulation assumptions in FR1 and FR2 for NR coverage enhancement.

# References

- Chairman’s notes, RAN1 #101 e-Meeting, May 2020

- R1-2005887, “Discussion on baseline coverage performance for FR1”, Intel Corporation, e-Meeting, August 2020

- R1-2005888, “Discussion on baseline coverage performance for FR2”, Intel Corporation, e-Meeting, August 2020