3GPP TSG RAN WG1 Meeting #102_e                                                                            R1-2005900

e-Meeting, August 17th – 28th, 2020

Agenda Item:		8.13.1

Source: 	Intel Corporation

Title:	On SCell scheduling PCell transmissions

Document for: 	Discussion/Decision

# Introduction

At RAN plenary meeting #86, a work item (WI) for the enhancement of dynamic spectrum sharing (DSS) was agreed [1], the following objective was identified.

- PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2]

  - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell

  - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI

    - The number of cells can be scheduled at once is limited to 2

    - The increase in DCI size should be minimized

- Note: The total PDCCH blind decoding budget should not be changed as a result of this work

- Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

In this contribution, we provide our views on the first objective, i.e. PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell.

# Discussions

Dynamic spectrum sharing (DSS) provides a very useful migration path from LTE to NR by allowing LTE and NR to share the same carrier. DSS was included already in Rel-15 and further enhanced in Rel-16. In the shared carrier of LTE and NR, the NR transmission needs to avoid the REs of LTE CRS and LTE PDCCH, which results in capacity limitation on NR PDCCH transmissions especially for the case when number of NR devices on PCell increases. From UL link performance point of view, it is beneficial that PCell is still configured as the cell with low frequency which coexists with a LTE cell. To solve the limited PDCCH capacity on the PCell, scheduling the PCell transmissions using a PDCCH on a scheduling SCell is motivated. In the following sections, we discuss the basic design issues for SCell scheduling PCell transmissions.

In the existing NR/LTE scheduling, the PDSCH/PUSCH of a cell is only transmitted on one cell. However, it is not the case if PCell transmission can be scheduled by PDCCH on a SCell. For the broadcast channels, e.g. SS/PBCH, RMSI, SIBx, paging and RACH RAR, it is straightforward that UE still needs to monitor such channels scheduled by a PDCCH on PCell, which can be viewed as self-scheduling. On the other hand, for unicast PDSCH/PUSCH, it is possible such transmissions could be scheduled by a PDCCH transmitted on a scheduling SCell, i.e. cross-carrier scheduling. One way to implement Proposal 1 is to differentiate CSS and USS for the UE. That is, only a USS for PCell transmission for the UE can be configured on the scheduling SCell.

Proposal 1: To support a combination of Self-scheduling and cross-carrier scheduling for the transmissions on PCell.

- For a broadcast channel on PCell, UE still monitor PDCCHs that are transmitted on PCell.

- For at least some unicast transmissions on PCell, UE may monitor PDCCHs that are transmitted on the scheduling SCell.

Proposal 2: Only a USS for PCell transmission for the UE can be configured on the scheduling SCell.

Multiple DCI formats are defined in NR including DCI format 0_0/1_0 which are for fallback operation, and DCI format 0_1/1_1 and 0_2/1_2 which are defined for efficient scheduling. Therefore, one issue to determine that which DCI formats could be transmitted on the scheduling SCell.

- Option 1: only DCI format 0_1/1_1 and 0_2/1_2 could be transmitted on the scheduling SCell. This Option is the most straightforward way since the existing cross-carrier scheduling only supports DCI format 0_1/1_1 and 0_2/1_2 too. However, it results in the split of USS, which may complicate UE detection on unicast PDCCH. That is, USS with fallback DCI format is still configured on PCell.

- Option 2: All DCI formats (0_0/1_0, 0_1/1_1 and 0_2/1_2) could be transmitted on the scheduling SCell. Option 2 avoid DCI format dependent PDCCH detections on different cells.

Proposal 3: Downselect a DCI format scheduling PCell transmissions that could be transmitted on the scheduling SCell:

- Option 1: Only DCI format 0_1/1_1 and 0_2/1_2.

- Option 2: All DCI formats (0_0/1_0, 0_1/1_1 and 0_2/1_2).

In NR/LTE cross-carrier scheduling, CIF=0 is fixedly assigned to the scheduling cell. Further, a value of CIF∈[1, 7] can be configured to a scheduled SCell. Since PCell must be scheduling cell if PCell is involved in cross-carrier scheduling, CIF for PCell, if applicable must be 0. Considering a PCell transmission can be cross-carrier scheduled by a scheduling SCell, one discussion point is how to allocate CIF=0.

- Option 1: CIF=0 is assigned to the scheduling SCell

Since the scheduling SCell may also schedule another SCell besides PCell, having CIF=0 for the scheduling SCell maintain same CIF allocation from the scheduled SCell point of view. In this case, a value CIF≠0 must be assigned to the PCell.

- Option 2: CIF=0 is assigned to the PCell

Option 2 maintain the commonality that PCell must use CIF=0. As a result, the scheduling SCell must be assigned a value CIF≠0.

In fact, both the two options could work since it just makes a rule how to assign CIF=0 and CIF≠0. We slightly prefer Option 1 since it sounds better aligned with existing cross-carrier scheduling.

Proposal 4:

- CIF=0 is assigned to the scheduling SCell.

- A value CIF≠0 is assigned to the PCell.

In NR system, the concept of DL and UL BWP is introduced. Consequently, many NR features are actually defined on a per-BWP basis. Therefore, one potential issue is whether SCell scheduling PCell transmission is defined per-BWP.

- Option 1: UE specific configuration.

This is the simplest option which achieves most/all benefit of SCell scheduling PCell transmissions. However, it is also most restrictive.

- Option 2: per-BWP configuration

This option enables larger flexibility on PDCCH transmission from gNB side. For example, in one BWP on PCell, if there are heavy LTE transmissions on the frequency of the BWP, PDCCH transmission for PCell could be offloaded to the scheduling SCell. On the other hand, for a BWP without LTE traffic or only limited LTE transmissions, PDCCH scheduling PCell transmission could be transmitted on the BWP on PCell.

Proposal 5: Further study on whether to allow a flexibility to configure SCell scheduling PCell transmission on a per-BWP basis.

According to RRC signalling in [2], the configuration of cross-carrier scheduling for a serving cell is done by IE CrossCarrierSchedulingConfig. For the scheduling SCell, cif-presence is set to true, CIF field exists in a DCI transmitted on the scheduling SCell. CIF=0 is assigned to the scheduling cell. On the other hand, for a scheduled cell including PCell, schedulingCellId indicates the serving cell index of the scheduling cell, and cif-InSchedulingCell indicates the CIF value used in the scheduling cell to indicate a DCI applicable for this scheduled cell or PCell.

#### –	CrossCarrierSchedulingConfig

The IE CrossCarrierSchedulingConfig is used to specify the configuration when the cross-carrier scheduling is used in a cell.

CrossCarrierSchedulingConfig information element

-- ASN1START

-- TAG-CrossCarrierSchedulingConfig-START

CrossCarrierSchedulingConfig ::=        SEQUENCE {

schedulingCellInfo                      CHOICE {

own                                     SEQUENCE {                  -- Cross carrier scheduling: scheduling cell

cif-Presence                            BOOLEAN

},

other                                   SEQUENCE {                  -- Cross carrier scheduling: scheduled cell

schedulingCellId                        ServCellIndex,

cif-InSchedulingCell                    INTEGER (1..7)

}

},

...

}

-- TAG-CrossCarrierSchedulingConfig-STOP

-- ASN1STOP

Proposal 6: The IE CrossCarrierSchedulingConfig is reused to in support of SCell scheduling PCell transmissions.

# Conclusion

In this contribution, we provide our views on SCell scheduling PCell transmissions. we make the following proposals.

Proposal 1: To support a combination of Self-scheduling and cross-carrier scheduling for the transmissions on PCell.

- For a broadcast channel on PCell, UE still monitor PDCCHs that are transmitted on PCell.

- For at least some unicast transmissions on PCell, UE may monitor PDCCHs that are transmitted on the scheduling SCell.

Proposal 2: Only a USS for PCell transmission for the UE can be configured on the scheduling SCell.

Proposal 3: Downselect a DCI format scheduling PCell transmissions that could be transmitted on the scheduling SCell:

- Option 1: Only DCI format 0_1/1_1 and 0_2/1_2.

- Option 2: All DCI formats (0_0/1_0, 0_1/1_1 and 0_2/1_2).

Proposal 4:

- CIF=0 is assigned to the scheduling SCell.

- A value CIF≠0 is assigned to the PCell.

Proposal 5: Further study on whether to allow a flexibility to configure SCell scheduling PCell transmission on a per-BWP basis.

Proposal 6: The IE CrossCarrierSchedulingConfig is reused to in support of SCell scheduling PCell transmissions.

# References

[1] RP-193260, New WID on NR Dynamic spectrum sharing (DSS), Ericsson

[2] 38.331-15.10.0, “Radio Resource Control (RRC) protocol specification”