3GPP TSG-RAN WG1 Meeting #102-e	R1-2005923

# e-Meeting, 17 – 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | xxxx | rev |  | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CFI related parameter names alignment between TS 36.331 and TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility, Lenovo, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_HRLLC-Core |  |  |  |  |  | Date: |  |  | 2020-07-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CFI related parameter names are misaligned between TS 36.331 and TS 36.213: TS 36.331, subclause 6.3.2, IE PhysicalConfigDedicated, IE PhysicalConfigDedicatedSCell-r10; TS 36.213, subclause 9.1.3. IE CFI-Config-r15:. - cfi-SubframeNonMBSFN-r15; semiStaticCFI-SubframeNonMBSFN. - cfi-SlotSubslotNonMBSFN-r15; semiStaticCFI-SlotSubslotNonMBSFN. - cfi-SubframeMBSFN-r15; semiStaticCFI-SubframeMBSFN. - cfi-SlotSubslotMBSFN-r15; semiStaticCFI-SlotSubslotMBSFN. IE CFI-PatternConfig-r15:. - cfi-PatternSubframe-r15; cfi-Radioframe-patternSubframe. - cfi-PatternSlotSubslot-r15; cfi-Radioframe-patternSlot. Furthermore, the wording of some conditions can be improved. |
| Summary of change: |  | The CFI related parameter names in TS 36.213 have been aligned with the ones specified in TS 36.331. Furthermore, the wording of some conditions has been improved to better align with the similar text in TS 36.213. Isolated impact analysis: This CR aligns CFI related parameter names between TS 36.331 and TS 36.213. There are no functionality changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment of CFI related parameter names between TS 36.331 and TS 36.213 remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 9.1.3	Control Format Indicator (CFI) assignment procedure

For a serving cell, if a UE is configured with higher layer parameter cfi-SlotSubslotNonMBSFN, the UE shall assume the CFI is equal to the value of the higher layer parameter cfi-SlotSubslotNonMBSFN for non-MBSFN subframes for receiving physical downlink shared channel with slot/subslot duration.

For a serving cell, if a UE is configured with higher layer parameter cfi-SlotSubslotMBSFN, the UE shall assume the CFI is equal to the value of the higher layer parameter cfi-SlotSubslotMBSFN for MBSFN subframes for receiving physical downlink shared channel with slot/subslot duration.

For a serving cell using frame structure 2, if a UE is configured with higher layer parameter cfi-PatternSlotSubslot, the UE shall assume the CFI is equal to the value of the higher layer parameter cfi-PatternSlotSubslot for the subframes for receiving physical downlink shared channel with slot duration.

For a serving cell, if a UE is configured with higher layer parameter cfi-SubframeNonMBSFN, the UE shall assume the CFI is equal to the value of the higher layer parameter cfi-SubframeNonMBSFN for non-MBSFN subframes for receiving physical downlink shared channel with subframe duration.

For a serving cell, if a UE is configured with higher layer parameter cfi-SubframeMBSFN, the UE shall assume the CFI is equal to the value of the higher layer parameter cfi-SubframeMBSFN for MBSFN subframes for receiving physical downlink shared channel with subframe duration.

For a serving cell using frame structure 2, if a UE is configured with higher layer parameter cfi-PatternSubframe, the UE shall assume the CFI is equal to the value of the higher layer parameter cfi-PatternSubframe for the subframes for receiving physical downlink shared channel with subframe duration.

For a serving cell, if a UE is configured with higher layer parameters cfi-SubframeNonMBSFN and cfi-SlotSubslotNonMBSFN, the UE is not expected to be configured with different values of cfi-SlotSubslotNonMBSFN and cfi-SubframeNonMBSFN.

For a serving cell, if a UE is configured with higher layer parameters cfi-SubframeMBSFN and cfi-SlotSubslotMBSFN, the UE is not expected to be configured with different values of cfi-SlotSubslotMBSFN and cfi-SubframeMBSFN.

For a serving cell using frame structure 2, if a UE is configured with higher layer parameters cfi-PatternSlotSubslot and cfi-PatternSubframe, the UE is not expected to be configured with different values of cfi-PatternSlotSubslot and cfi-PatternSubframe.

PHICH duration is signalled by higher layers according to Table 6.9.3-1 in [3]. The duration signalled puts a lower limit on the size of the control region determined from the control format indicator (CFI). When , if extended PHICH duration is indicated by higher layers then the UE shall assume that CFI is equal to PHICH duration.

In subframes indicated by higher layers to decode PMCH, when , a UE may assume that CFI is equal to the value of the higher layer parameter non-MBSFNregionLength [11].

End of changes