3GPP TSG-RAN WG1 Meeting #102-e	R1-2006001

e-Meeting, August 17 – 28, 2020

Agenda item:	5

Title:	Discussion on reply LS for maximum data rate calculation for NR sidelink

Source:	OPPO

Document for:	Discussion and Decision

1. Introduction

Towards the RAN1#101-e meeting, RAN1 received a LS from RAN2 in [1] requesting RAN1 to define the maximum data rate for NR sidelink. In this contribution, we provide discussion and calculation to define this max data rate in Rel-16.

# 2. Discussion

In order for RAN2 to derive the total layer-2 buffer size for sidelink in Rel-16, according to its current definition in Section 4.1.4 of TS38.306 [2], it is necessary to first calculate a maximum data rate from the physical layer. So far, a formula is defined in [2] for computing the supported max data rate for NR Uu as:

(1)

Description for each of the component in this formula can be found in Section 4.1.2 of [2]. Since the scope / supported number of SL carriers is 1, the above formula for NR Uu can be modified for NR sidelink in Rel-16 as:

data rate (in Mbps)=10-6⋅vLayers⋅Qm⋅Rmax⋅NPRBBW,μ⋅12Tsμ⋅1-OH	(2)

wherein

Rmax = 948/1024

vLayers is the maximum number of supported layers, which is 2.

Qm is the maximum supported modulation order given by higher layer parameter. For NR sidelink in Rel-16, it should be 8 (256QAM).

μ is the numerology (as defined in TS 38.211).

Tsμ is the average OFDM symbol duration in a subframe for numerology μ, i.e. Tsμ=10-314⋅2μ.

Note that normal cyclic prefix is assumed.

NPRBBW,μ is the maximum RB allocation in bandwidth BW with numerology μ, as defined in TS 38.101-1 and TS 38.101-2, where BW is the UE supported maximum bandwidth for a given band.

OH is the overhead and it has a value of 0.2203 for NR sidelink in FR1 and 0.238 in FR2.

For the SL overhead in computing the maximum data rate, i.e., PSCCH, PSFCH, DMRS, CSI-RS and PT-RS, we could assume the following:

- Minimum supported number of 10 PRBs and 2 symbols for PSCCH (10 x 12 x 2 = 240 REs)

- No PSFCH is configured in a resource pool

- PSSCH-DMRS occupies only 2 symbols / 12 REs per PRB

  - FR1: NRB is 273 RBs x 12 REs/RB = 3276 REs

  - FR2: NRB is 264 RBs x 12 REs/RB = 3168 REs

- For CSI-RS and PT-RS overhead:

  - In FR1, the sl-X-Overhead can be assumed to be absent, then the overhead is set to zero according to TS 38.331.

  - In FR2, PT-RS in time domain is 11 symbols and frequency domain is 11 REs per 4 PRBs. This translates to n=3 for the sl-X-Overhead parameter, which is 3 REs per PRB. Then the total REs for the PT-RS overhead is 264 RBs x 3 REs/RB = 792 REs

- Minimum overhead for the 2nd stage SCI is 36 REs (based on 38.212 rate matching calculation)

- Overhead for one symbol AGC and one gap symbol:

  - FR1: 2 symbols x 12 REs/RB x 273 RBs = 6552 REs

  - FR2: 2 symbols x 12 REs/RB x 264 RBs = 6336 REs

- Total number of REs in a SL slot:

  - FR1: 14 symbols x 12 REs/RB x 273 RBs = 45864 total REs

  - FR2: 14 symbols x 12 REs/RB x 264 RBs = 44352 total REs

Based on the above assumption, the total SL overhead would be 0.2203 in FR1 and 0.238 in FR2.

Proposal 1: Use the above Eq. (2) for computing the max data rate for SL in Rel-16.

4. Conclusion

In this contribution, we provided discussion and computation formula to define the maximum data rate for NR sidelink in Rel-16.

Proposal 1: Use the above Eq. (2) for computing the max data rate for SL in Rel-16.

Reference

[1] R1-2003258 “LS on UE capability”, RAN2