3GPP TSG-RAN WG1 Meeting #102-e	R1-2006002

e-Meeting, August 17 – 28, 2020

Agenda item:	6.2.5

Title:	Discussion on remaining issues of LTE feature list for 5G_V2X_NRSL

Source:	OPPO

Document for:	Discussion and Decision

1. Introduction

Based on the latest RAN1 email discussion outcomes/agreements for the LTE feature list on 5G_V2X_NRSL, [101-e-Post-NR-UE-Features-08], the followings are still the remaining open issues:

- Resolve squared brackets in FG 5-2, components (1), Notes column

- Resolve squared brackets in FG 5-3, components (1), Notes column

- Resolve FFS in FG 5-6, components (2) incl. consequence if the feature is not supported by the UE

- Resolve FFS in FG 5-8 for “Need for the eNB to know if the feature is supported”

- Resolve squared brackets in FG 5-14 (prerequisite feature groups)

- Resolve squared brackets in FG 5-15 (prerequisite feature groups)

In this contribution, we provide our views on these remaining open issues.

2. Discussions

FG 5-2

| 5-2 | Transmitting NR sidelink mode 1 configured by LTE Uu | 1) UE can transmit NR PSCCH/PSSCH using configured grant type 1 in NR sidelink mode 1 configured by LTE Uu. Up to 8 configured grants can be configured for a UE. Up to C=8 sidelink HARQ processes are supported including those for configured grants. 2) UE can transmit NR PSSCH according to the NR normal 64QAM MCS OFDM table. 3) UE supports PT-RS transmission in FR2. 4) UE can transmit using the subcarrier spacing and CP length it reports. 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. | 5-1 | Yes | N/A |  | Per band | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. [Note: the UE supports up max(B, C) as the total number of sidelink HARQ processes across both Mode 1 and Mode 2] … | Optional with capability signalling FFS: For UE supports LTE Uu configuring NR sidelink [in licensed spectrum], UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Looking/judging from WID history, the support for simultaneous configuration of Mode 1 and Mode 2 for a UE was removed from the WI objective in RAN#87e (March’20). Since mixed mode 1 and mode 2 operation is not supported, the related yellow highlighted note can be updated as “Note: the UE supports up to C number of sidelink HARQ processes for both Mode 1 and Mode 2”.

Proposal 1: For FG 5-2, in our understanding mixed mode 1 and mode 2 operation is not supported since it has been removed from the WID. The associated yellow highlighted note should be updated as “Note: the UE supports up to C number of sidelink HARQ processes for both Mode 1 and Mode 2”.

- The same handling should be done for the counterpart in the NR features list.

FG 5-3

Proposal 2: For FG 5-3, the same handling can be done as in Proposal 1 for FG-5-2.

FG 5-6, 5-8, 5-14 and 5-15

Proposal 3: For these FGs, they should align with their respective FGs once finalized in the NR features list.