3GPP TSG RAN WG1 #102-e  	                                                         	     R1-2006058

e-Meeting, August 17th – 28th, 2020

Source:	OPPO

Title:	HARQ-ACK enhancements for Rel-17 URLLC/IIoT

Agenda Item:	8.3.1.1

Document for:	Discussion and Decision

Introduction

In RAN #88e, the WID on enhanced IoT and URLLC support for NR was agreed with following RAN1 objectives:

- Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering

    - UE feedback enhancements for HARQ-ACK [RAN1]

    - CSI feedback enhancements to allow for more accurate MCS selection [RAN1]

Note: DMRS-based CSI feedback is not in scope of this WI

- Uplink enhancements for URLLC in unlicensed controlled environments [RAN1, RAN2]:

  - Specify support for UE-initiated COT for FBE with minimum specification effort

  - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

- Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]:

- Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH.

- Specify PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline

In this contribution, we show our views on the HARQ-ACK enhancements for Rel-17 IoT and URLLC.

Discussion

Sub-slot Based Type-1 HARQ-ACK Codebook

In Rel-15, two kinds of HARQ-ACK codebooks are supported, i.e. type-1 codebook and type-2 codebook. The payload of type-1 codebook is semi-statically determined based on RRC signaling and the payload of type-2 codebook is dynamically determined based on PDSCH scheduling. The advantages of type-1 codebook include lower DCI overhead and no ambiguity on the size of codebook caused by DL grant missing, and the advantage of type-2 codebook is lower UCI overhead.

In Rel-16 URLLC, sub-slot based HARQ-ACK feedback is supported to reduce the HARQ-ACK feedback delay. However, only type-2 codebook can be configured with sub-slot PUCCH. With lower UCI payload, higher reliability of PUCCH transmission can be expected for sub-slot based type-2 codebook. However, in case of we have more concern about the impact on reliability caused by PDCCH reception, the reliability of HARQ-ACK feedback transmitted by type-1 codebook may be higher than type-2 codebook. Furthermore, if sub-slot based type-1 codebook is supported, the payload of type-1 codebook can be optimized based on the proper configuration of K1 to guarantee the reliability of PUCCH transmission.

Observation 1: In case of we have more concern about the impact on reliability caused by PDCCH reception, the reliability of HARQ-ACK feedback transmitted by type-1 codebook may be higher than type-2 codebook.

Observation 2: The payload of type-1 codebook can be optimized based on the proper configuration of K1 to guarantee the reliability of PUCCH transmission.

Proposal 1: Sub-slot based type-1 HARQ-ACK codebook should be supported in Rel-17.

To support sub-slot based type-1 codebook, K1 should be defined following Rel-15 approach but in unit of sub-slot, then a slight modification for determining the PDSCH occasion is needed. In current pseudo-code for determining the set of occasions for candidate PDSCH receptions, the cardinality of TDRA rows are firstly excluded based on the TDD UL-DL configurations, then the overlapped rows are removed. One example is shown in Figure 1, five rows are configured in TDRA table, and a set of occasions for candidate PDSCH receptions includes PDSCH occasion 1 and 4 for slot-based type-1 codebook.

If sub-slot based type-1 codebook is supported, for a given sub-slot, the cardinality of TDRA rows should be further limited by the ending symbol. Specifically, if the last symbols of the PDSCH time resource derived by a TDRA row r is not in the sub-slot, row r is removed from the cardinality of TDRA rows. Take the example shown in Figure 1:

- For sub-slot 1, the set of occasions for candidate PDSCH receptions includes PDSCH occasion 1;

- For sub-slot 2, the set of occasions for candidate PDSCH receptions includes PDSCH occasion 3 and 4.

Proposal 2: If sub-slot based type-1 HARQ-ACK codebook is supported, to determine the occasions for candidate PDSCH receptions, the following limitation should be considered:

- For a given sub-slot, if the last symbols of the PDSCH time resource derived by a TDRA row r is not in the sub-slot, row r is removed from the cardinality of TDRA rows.

Figure 1. The sets of occasions for candidate PDSCH receptions

Retransmission of Low-priority HARQ-ACK

In Rel-16, 2-level priority is supported for PUCCH/PUSCH. To resolve intra-UE collision between UL transmissions, a UE performs the following:

  - Step 1: Resolve collision between UL transmissions with same priority based on Rel-15 multiplexing procedure.

  - Step 2: Resolve collision between UL transmissions with different priorities, i.e. the low-priority UL transmission is canceled.

Considering low-priority HARQ-ACK is usually configured for eMBB transmission, once the low-priority HARQ-ACK is canceled, significant system efficiency loss can be expected, since:

  - Retransmission of large amount of data will reduce the DL system efficiency; and

  - More collisions between UL transmissions with different priority may occur.

In order to reduce the impact caused by the cancellation of low-priority HARQ-ACK transmission, HARQ-ACK retransmission should be supported.

Proposal 3: HARQ-ACK retransmission should be supported for Rel-17 URLLC.

One-shot HARQ-ACK Transmission

For the following cases, PDSCH retransmission cannot satisfy the delay requirement of URLLC. HARQ-ACK feedback has no use for URLLC transmission, but causes the collision with other UL channels and leads to unnecessary dropping of eMBB transmission.

  - Case 1: One-shot PDSCH transmission.

  - Case 2: DL-heavy TDD carrier. One example is shown in Figure 2, the earliest HARQ timing for the PDSCH transmission in slot n is in slot n+4, which exceeds the delay requirement. Furthermore, if massive URLLC PDSCHs are transmitted, PUCCH blocking can be expected for this case.

Figure 2: Unnecessary HARQ-ACK feedback on DL-heavy TDD carrier

  - Case 3: The transmission is closed to the maximum time delay. As shown in Figure 3, assuming the maximum time delay is 1 ms, the gap between the data arriving and the end of the first retransmission is closed to 1 ms, and the earliest ACK/NACK transmission must exceed the time delay.

Figure 3: Unnecessary HARQ-ACK feedback for the transmission closed to the maximum time delay

To reduce the power consumption for transmitting the unnecessary HARQ-ACK and avoid the loss of eMBB transmission, one-shot HARQ-ACK transmission should be supported for Rel-17 URLLC.

Proposal 4: One-shot HARQ-ACK transmission should be supported for Rel-17 URLLC.

HARQ-ACK Enhancements for SPS PDSCH

In Rel-16, short DL SPS periodicity and multiple DL SPS configurations are supported. For cases where only HARQ-ACK feedback for SPS PDSCHs shall be reported (i.e. no dynamic PDSCH HARQ-ACK), more than one bit of HARQ-ACK feedback for SPS PDSCH without an associated grant can be reported in a PUCCH resource. However, one feedback timing K1 is still semi-statically configured for each DL SPS configuration, and once no available PUCCH resource is obtained, HARQ-ACK for SPS PDSCH will not be transmitted. One example is shown in Figure 4, in 5 ms DL-UL periodicity, 75% SPS PDSCHs have no HARQ-ACK feedback. This will significantly reduce the system efficiency and increase the latency.

Proposal 5: HARQ-ACK for all of available SPS PDSCHs should be reported for Rel-17 URLLC.

Figure 4: One example of Rel-16 HARQ-ACK transmission for SPS PDSCH

If HARQ-ACK report for all of available SPS PDSCHs is supported, the HARQ-ACK codebook for SPS PDSCH can be further enhanced to achieve lower UCI overhead. The following methods can be considered:

  - Option 1: The HARQ-ACK codebook for SPS PDSCHs is determined based on the HARQ processes associated with the multiple SPS PDSCH resources which are mapped to the same PUCCH.

  - Option 2: Multiple SPS PDSCH sources of multiple configurations configured for one service share one HARQ-ACK bit. One example is shown in Figure 5, SPS configuration 1~3 are configured for one service to solve jitter problem, 3 PDSCH resources during one periodicity share one HARQ-ACK bit in the codebook.

Figure 5: Multiple SPS PDSCH sources share one HARQ-ACK bit in SPS HARQ-ACK codebook

Proposal 6: If HARQ-ACK report for all of available SPS PDSCHs is supported, the HARQ-ACK codebook for SPS PDSCH can be further enhanced to achieve lower UCI overhead.

Conclusions

In this contribution, we show our views on the HARQ-ACK enhancements for Rel-17 IoT and URLLC with following observations and proposals:

Observation 1: In case of we have more concern about the impact on reliability caused by PDCCH reception, the reliability of HARQ-ACK feedback transmitted by type-1 codebook may be higher than type-2 codebook.

Observation 2: The payload of type-1 codebook can be optimized based on the proper configuration of K1 to guarantee the reliability of PUCCH transmission.

Proposal 1: Sub-slot based type-1 HARQ-ACK codebook should be supported in Rel-17.

Proposal 2: If sub-slot based type-1 HARQ-ACK codebook is supported, to determine the occasions for candidate PDSCH receptions, the following limitation should be considered:

- For a given sub-slot, if the last symbols of the PDSCH time resource derived by a TDRA row r is not in the sub-slot, row r is removed from the cardinality of TDRA rows.

Proposal 3: HARQ-ACK retransmission should be supported for Rel-17 URLLC.

Proposal 4: One-shot HARQ-ACK transmission should be supported for Rel-17 URLLC.

Proposal 5: HARQ-ACK for all of available SPS PDSCHs should be reported for Rel-17 URLLC.

Proposal 6: If HARQ-ACK report for all of available SPS PDSCHs is supported, the HARQ-ACK codebook for SPS PDSCH can be further enhanced to achieve lower UCI overhead.

References

- RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, Nokia, Nokia Shanghai Bell