3GPP TSG RAN WG1#102-e	R1-2006067

e-Meeting, August 17th – 28th, 2020

Agenda Item:	8.5.2

Source:	BUPT, ZTE, CAICT

Title:	Evaluation of achievable positioning accuracy and latency

Document for:	Discussion and decision

Introduction

In the RAN#86 meeting, we have the following conclusions on NR Positioning [1]:

- Study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases (incl. general commercial use cases and specifically (I)IoT use cases as exemplified in section 3 above (Justification)): - Define additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT). [RAN1] - Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1] - Identify and evaluate positioning techniques, DL/UL positioning reference signals, signaling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency. Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case. [RAN1, RAN2]

And in this contribution, we present our simulation results of positioning in indoor factory scenario.

Simulation results of positioning in indoor factory

Positioning accuracy evaluation for FR1

Table 1 Parameter of positioning accuracy evaluation for FR1 in InF-SH

| Parameter | [Source 1, InF-SH, FR1] |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | TS38.211 R16 PRS comb-12 pattern |
| Reference signal (type of sequence, number of ports, …) | TS38.211 R16 PRS |
| Number of sites | 18 |
| Number of symbols used per slot per positioning estimate | 12 symbols |
| Number of slots per positioning estimate | 8000 slots |
| Power-boosting level | 1 |
| Uplink power control (applied/not applied) | not applied |
| interference modelling (ideal muting, or other) | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Phase tracking |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Chan |
| Network synchronization assumptions | Perfect Synchronization |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Ideal alignment |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | nrof antenna elements used |
| Additional notes, if any |  |

Figure 1 CDF of positioning error for FR1 in InF-SH

Table 2 Positioning Error in different percentiles for source1

|  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Positioning Error | 0.161m | 0.273m | 0.387m | 0.617m |

Table 3 Parameter of positioning accuracy evaluation for FR1 in InF-DH

| Parameter | [Source 2, InF-DH, FR1] |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | TS38.211 R16 PRS comb-12 pattern |
| Reference signal (type of sequence, number of ports, …) | TS38.211 R16 PRS |
| Number of sites | 18 |
| Number of symbols used per slot per positioning estimate | 12 symbols |
| Number of slots per positioning estimate | 8000 slots |
| Power-boosting level | 1 |
| Uplink power control (applied/not applied) | not applied |
| interference modelling (ideal muting, or other) | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Phase tracking |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Chan |
| Network synchronization assumptions | Perfect Synchronization |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Ideal alignment |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | nrof antenna elements used |
| Additional notes, if any |  |

Figure 2 CDF of positioning error for FR1 in InF-DH

Table 4 Positioning Error in different percentiles for source2

|  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Positioning Error | 0.139m | 0.174m | 0.224m | 0.293m |

Positioning accuracy evaluation for FR2

Table 5 Parameter of positioning accuracy evaluation for FR2 in InF-SH

| Parameter | [Source 3, InF-SH, FR2] |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | TS38.211 R16 PRS comb-12 pattern |
| Reference signal (type of sequence, number of ports, …) | TS38.211 R16 PRS |
| Number of sites | 18 |
| Number of symbols used per slot per positioning estimate | 12 symbols |
| Number of slots per positioning estimate | 8000 slots |
| Power-boosting level | 1 |
| Uplink power control (applied/not applied) | not applied |
| interference modelling (ideal muting, or other) | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Phase tracking |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Chan |
| Network synchronization assumptions | Perfect Synchronization |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Ideal alignment |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | nrof antenna elements used |
| Additional notes, if any |  |

Figure 3 CDF of positioning error for FR2 in InF-SH

Table 6 Positioning Error in different percentiles for source3

|  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Positioning Error | 0.044m | 0.060m | 0.094m | 0.179m |

Table 7 Parameter of positioning accuracy evaluation for FR2 in InF-DH

| Parameter | [Source 4, InF-DH, FR2] |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | TS38.211 R16 PRS comb-12 pattern |
| Reference signal (type of sequence, number of ports, …) | TS38.211 R16 PRS |
| Number of sites | 18 |
| Number of symbols used per slot per positioning estimate | 12 symbols |
| Number of slots per positioning estimate | 8000 slots |
| Power-boosting level | 1 |
| Uplink power control (applied/not applied) | not applied |
| interference modelling (ideal muting, or other) | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Phase tracking |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Chan |
| Network synchronization assumptions | Perfect Synchronization |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Ideal alignment |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | nrof antenna elements used |
| Additional notes, if any |  |

Figure 4 CDF of positioning error for FR2 in InF-DH

Table 2 Positioning Error in different percentiles for source4

|  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Positioning Error | 0.028m | 0.044m | 0.073m | 0.116m |

Conclusions

In this contribution, we presented our simulation results of positioning in indoor factory based on R16 PRS.

- References

- 3GPP RP-193237, “New SID on NR Positioning Enhancements”, RAN#86, Sitges, Spain, December 9th – 12th, 2019.