3GPP TSG RAN WG1#102-e			R1-2006085

# e-Meeting, August 17th – 28th, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MSG3 power determination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2020-08-04 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In section 7.1 of TS 38.213, the transmission power determination for RAR scheduled PUSCH transmission and corresponding PUSCH re-transmission are using different “j” value. Specifically, the initial PUSCH transmission scheduled by RAR will use j=0 while the corresponding PUSCH re-transmission are using j=2. It cause the different power settings as following: j=0; j=2. P_o_UE_PUSCH(0)=0; Alpha(0)= msgA-Alpha or msg3-Alpha; or 1 (when not configured) P_o_NOMINAL(0)=P_o_PRE+delta_preamble_msg3; P_o_UE_PUSCH(2)= “the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets”; Alpha(2)= “the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets” P_o_NOMINAL(2)= p0-NominalWithGrant or P_o_NOMINAL(0) if not configured.. So that the actual power level of PUSCH re-transmission could be much lower than the power level of initial transmission. It will cause the frequent random access failure in the system. |
| Summary of change: |  | Modifiy the j value for PUSCH re-transmission to be the same as that for the corresponding RAR scheduled PUSCH transmission, i.e., j=0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This is wrongly configured power setting parameters will let the actual power level of PUSCH re-transmission could be much lower than the power level of initial transmission. It will cause the frequent random access failure in the system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Isolated Impact Analysis: No impact to gNB behavior is expected from this CR. UE will use j=0 for both RAR scheduled PUSCH transmission and corresponding re-transmission scheduled by DCI. |  |  |  |  |  |  |  |  |

### 7.1.1	UE behaviour

If a UE transmits a PUSCH on active UL BWP  of carrier  of serving cell  using parameter set configuration with index  and PUSCH power control adjustment state with index , the UE determines the PUSCH transmission power  in PUSCH transmission occasion  as

[dBm]

where,

-	is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier  of serving cell  in PUSCH transmission occasion .

-	 is a parameter composed of the sum of a component  and a component  where .

-	If a UE is not provided P0-PUSCH-AlphaSet or for a PUSCH (re-)transmission corresponding to a RAR UL grant as described in Clause 8.3, , , and , where the parameter preambleReceivedTargetPower [11, TS 38.321] (for ) and msg3-DeltaPreamble (for ) are provided by higher layers, or  dB if msg3-DeltaPreamble is not provided, for carrier  of serving cell

-	For a PUSCH (re)transmission configured by ConfiguredGrantConfig, ,  is provided by p0-NominalWithoutGrant, or  if p0-NominalWithoutGrant is not provided, and  is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP  of carrier  of serving cell

-	For , a  value, applicable for all , is provided by p0-NominalWithGrant, or  if p0-NominalWithGrant is not provided, for each carrier  of serving cell  and a set of values are provided by a set of p0 in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP  of carrier  of serving cell

-	If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId and if DCI format 0_1 includes a SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in DCI format 0_1 [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values. If the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, the UE determines the value of  from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value

-	If the PUSCH transmission (except the PUSCH re-transmission corresponding to a RAR UL grant) is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field, or if SRI-PUSCHPowerControl is not provided to the UE, , and the UE determines  from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

-	For

-	For ,  is a value of msg3-Alpha, when provided; otherwise,

-	For ,  is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP  of carrier  of serving cell

-	For , a set of  values are provided by a set of alpha in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP  of carrier  of serving cell

-	If the UE is provided SRI-PUSCH-PowerControl and more than one values of p0-PUSCH-AlphaSetId, and if DCI format 0_1 includes a SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in DCI format 0_1 [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values. If the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, the UE determines the values of  from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value

-	If the PUSCH transmission (except the PUSCH re-transmission corresponding to a RAR UL grant) is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, , and the UE determines  from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets