3GPP TSG RAN WG1 #102-e	R1-2006095

e-Meeting, August 17th – 28th, 2020

Agenda item:	7.2.2.2.1

Source:	Samsung

Title:	Channel Access Procedures for NR-U

Document for:	Discussion and Decision

# Introduction

This contribution discusses the following remaining issues of channel access for NR-U:

- Duty cycle of discovery burst

- CWS adjustment

- Applicability of CP extension for multiple UL transmissions scheduled by a DCI

# Duty cycle of discovery burst

In Rel-16, it was agreed that the configuration for Type0-PDCCH follows the same as Rel-15, which implies the value of parameter M in determining the Type0-PDCCH monitoring occasions can be one from 1/2, 1, or 2. When CORESET #0 and its associated PDSCH of RMSI are included in the discovery burst, the total duration of discovery burst may vary based on the starting point of the transmission subject to LBT, when M is 1 or 2.

An example of this observation is shown in Figure 1. In this example, Q=4, M=1, and O is chosen such that SS/PBCH blocks and its associated RMSI burst are located in the same half frame. The gNB intends to transmit SS/PBCH block with index 0 and 1, as well as their associated RMSI. If LBT succeeds before the SS/PBCH block candidate location with index 0, the discovery burst duration can be determined as 2 slots (1 ms), and Type 2 DL channel access procedure could be applicable; while if LBT succeeds before the SS/PBCH block candidate location with index 4, the discovery burst duration can be determined as 4 slots (2 ms), and Type 2 DL channel access procedure cannot be applicable. Hence, the duty cycle of discovery burst should be determined based on its actual transmission starting instance, and its associated periodicity should be determined regardless of the location where LBT succeeds. We have the following proposal to address this issue.

Figure 1 Illustration of a variable duration of discovery burst subject to LBT.

Proposal 1: Adopt the following TP for TS 37.213.

================================= Start of TP for TS 37.213 =================================

4.0	General

================================ Unchanged Texts Omitted =================================

-	A discovery burst refers to a DL transmission burst including a set of signal(s) and/or channel(s) confined within a window and associated with a duty cycle. The duty cycle is defined as DDB/PDB, wherein DDB is the duration of the discovery burst and determined according to the transmission starting instance of the discovery burst, and PDB is determined as the periodicity of the discovery burst transmission window, as described in subclause 4.1 of [7]. The discovery burst can be any of the following:

-	Transmission(s) initiated by an eNB that includes a primary synchronization signal (PSS), secondary synchronization signal (SSS) and cell-specific reference signal(s)(CRS) and may include non-zero power CSI reference signals (CSI-RS).

-	Transmission(s) initiated by a gNB that includes at least an SS/PBCH block consisting of a primary synchronization signal (PSS), secondary synchronization signal (SSS), physical broadcast channel (PBCH) with associated demodulation reference signal (DM-RS) and may also include CORESET for PDCCH scheduling PDSCH with SIB1, and PDSCH carrying SIB1 and/or non-zero power CSI reference signals (CSI-RS).

================================= End of TP for TS 37.213 =================================

# CWS adjustment

NR-U UL CWS adaptation is based on explicit or implicit HARQ-ACK feedbacks from gNB, i.e., by HARQ-ACK bitmap in CG-DFI, or by NDI in UL grant. For explicit HARQ-ACK in CG-DFI, RAN1 agreed that gNB configures minimum duration cg-minDFIDelay-r16. For single PUSCH and CG PUSCH repetition, HARQ-ACK in CG-DFI is valid only if a first symbol of the CG-DFI is after a last symbol of the PUSCH transmission, or of any repetition of the PUSCH transmission, by cg-minDFIDelay-r16. For schedule-based PUSCH repetition, valid ACK and NACK is defined separately. CWS should be adjusted according to valid HARQ-ACK, but the current description in TS 37.213 is not clear.

Proposal 2: Reuse the RRC configured minimum duration cg-minDFIDelay-r16 for the minimum latency between reference duration and the following UL grant or CG-DFI for CWS adjustment.

Proposal 3: Specify CWS adjustment based on valid HARQ-ACK defined in TS 38.213.

To resolve this, we have the following TP:

Proposal 4: Adopt the following TP for TS 37.213.

================================= Start of TP for TS 37.213 =================================

4.2.2.2	 Contention window adjustment procedures for UL transmissions scheduled/configured by gNB

================================ Unchanged Texts Omitted =================================

The HARQ-ACK feedback, reference duration and duration Tw  in the procedure above are defined as the following:

-	HARQ-ACK feedback for PUSCH(s) transmissions are expected to be provided to UE(s) explicitly or implicitly where implicit HARQ-ACK feedback for the purpose of contention window adjustment in this subclause, is determined based on the indication for a new transmission or retransmission in the DCI scheduling PUSCH(s) as follows:

-	If a new transmission is indicated, 'ACK' is assumed for the transport blocks or code block groups in the corresponding PUSCH(s) for the TB-based and CBG-based transmission, respectively.

-	If a retransmission is indicated for TB-based transmissions, 'NACK' is assumed for the transport blocks in the corresponding PUSCH(s).

-	If a retransmission is indicated for CBG-based transmissions, if a bit value in the code block group transmission information (CBGTI) field is '0' or '1' as described in subclause 5.1.7.2 in [8], 'ACK' or 'NACK' is assumed for the corresponding CBG in the corresponding PUSCH(s), respectively.

Explicit HARQ-ACK feedback for the purpose of contention window adjustment in this subclause is determined based on valid HARQ-ACK in the CG-DFI as described in subclause 10.5 in [7].

-	The reference duration corresponding to a channel occupancy initiated by the UE including transmission of PUSCH(s) is defined in this subclause as a duration starting from the beginning of the channel occupancy until the end of the first slot where at least one unicast PUSCH is transmitted over all the resources allocated for the PDSCH, or until the end of the first transmission burst by the gNB that contains unicast PUSCH(s) transmitted over all the resources allocated for the PDSCH, whichever occurs earlier, and the duration starting no later than a number of symbols provided by cg-minDFIDelay-r1 before an UL grant or a CG-DFI. If the channel occupancy includes a unicast PDSCH, but it does not include any unicast PDSCH transmitted over all the resources allocated for that PUSCH, and the duration starting no later than a number of symbols provided by cg-minDFIDelay-r1 before an UL grant or a CG-DFI, then, the duration of the first transmission burst by the UE within the channel occupancy that contains PUSCH(s) is the reference duration for CWS adjustment.

-	Tw=max⁡(TA,TB+1ms) where TB is the duraon of the transmission burst from start of the reference duration in ms and TA=5ms if the absence of any other technology sharing the channel cannot be guaranteed on a long-term basis (e.g. by level of regulation), and TA=10ms otherwise.

If a UE transmits transmissions using Type 1 channel access procedures associated with the channel access priority class p on a channel and the transmissions are not associated with explicit or implicit HARQ-ACK feedbacks as described above in this subclause, the UE adjusts CWp before step 1 in the procedures described in subclause 4.2.1.1, using the latest CWp used for any UL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. If the corresponding channel access priority class p has not been for any UL transmission on the channel, CWp=CWmin,p is used.

================================ Unchanged Texts Omitted =================================

================================= End of TP for TS 37.213 =================================

# Applicability of CP Extension for Multiple UL Transmissions Scheduled by a DCI

In the last meeting, CP extension has been agreed to be supported for SRS, together with PUSCH and PUCCH. However, regarding the details of TP for supporting this feature, we believe current specification is not sufficient yet.

A-SRS can be triggered by DCI 1_1/1_2, DCI 0_1/0_2 and DCI 2_3. DCI 0_1 and DCI 1_1 can include CP extension & channel access indication. gNB can schedule a PUSCH transmission and also trigger A-SRS by a single DCI 0_1. And also, gNB can schedule a PDSCH with corresponding PUCCH and also trigger A-SRS by a single DCI 1_1. In both cases, how to apply the indicated CP extension & channel access in the DCI should be decided, e.g. (1) apply CP extension to both A-SRS and PUCCH/PUSCH or (2) only apply CP extension to first UL signal/channel while no CP for second UL signal/channel. Considering CP extension is mainly for the scenario to occupy the channel before the first scheduled UL symbol and the previous transmission and LBT duration, which is a more typical scenario for a first UL signal/channel, applying the indicated CP extension and channel access only for first UL signal/channel is sufficient.

Based on above, we have the following TP:

Proposal 5: Adopt the following TP for TS 38.211.

================================= Start of TP for TS 38.211 =================================

5.3.1	OFDM baseband signal generation for all channels except PRACH and RIM-RS

================================ Unchanged Texts Omitted =================================

-	for dynamically scheduled PUSCH, SRS, and PUCCH transmissions

Text=minmaxText',0, Tsymb,(l-1)mod7∙2μμ

Text'=k=1CiTsymb,  l-kmod 7∙2μ μ-Δi

where Δi is given by Table 5.3.1-1 with C1=1 for μ∈0,1, C1=2 for μ=2, and C2 and C3 given by the higher-layer parameters cp-ExtensionC2-r16 and cp-ExtensionC3-r16, respectively, and TTA given by clause 4.3.1. For contention-based random access, or in absence of higher-layer configuration of C2 and C3, the value of Cishall be set to the largest integer fulfilling Text'<Tsymb,  (l-1)mod7∙2μ μ for each of the values of i∈2,3. Text is applied to the first UL transmission scheduled by the scheduling DCI.

================================ Unchanged Texts Omitted =================================

================================= End of TP for TS 38.211 =================================

# Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: Adopt the following TP for TS 37.213.

================================= Start of TP for TS 37.213 =================================

4.0	General

================================ Unchanged Texts Omitted =================================

-	A discovery burst refers to a DL transmission burst including a set of signal(s) and/or channel(s) confined within a window and associated with a duty cycle. The duty cycle is defined as DDB/PDB, wherein DDB is the duration of the discovery burst and determined according to the transmission starting instance of the discovery burst, and PDB is determined as the periodicity of the discovery burst transmission window, as described in subclause 4.1 of [7]. The discovery burst can be any of the following:

-	Transmission(s) initiated by an eNB that includes a primary synchronization signal (PSS), secondary synchronization signal (SSS) and cell-specific reference signal(s)(CRS) and may include non-zero power CSI reference signals (CSI-RS).

-	Transmission(s) initiated by a gNB that includes at least an SS/PBCH block consisting of a primary synchronization signal (PSS), secondary synchronization signal (SSS), physical broadcast channel (PBCH) with associated demodulation reference signal (DM-RS) and may also include CORESET for PDCCH scheduling PDSCH with SIB1, and PDSCH carrying SIB1 and/or non-zero power CSI reference signals (CSI-RS).

================================= End of TP for TS 37.213 =================================

Proposal 2: Reuse the RRC configured minimum duration cg-minDFIDelay-r16 for the minimum latency between reference duration and the following UL grant or CG-DFI for CWS adjustment.

Proposal 3: Specify CWS adjustment based on valid HARQ-ACK defined in TS 38.213.

Proposal 4: Adopt the following TP for TS 37.213.

================================= Start of TP for TS 37.213 =================================

4.2.2.2	 Contention window adjustment procedures for UL transmissions scheduled/configured by gNB

================================ Unchanged Texts Omitted =================================

The HARQ-ACK feedback, reference duration and duration Tw  in the procedure above are defined as the following:

-	HARQ-ACK feedback for PUSCH(s) transmissions are expected to be provided to UE(s) explicitly or implicitly where implicit HARQ-ACK feedback for the purpose of contention window adjustment in this subclause, is determined based on the indication for a new transmission or retransmission in the DCI scheduling PUSCH(s) as follows:

-	If a new transmission is indicated, 'ACK' is assumed for the transport blocks or code block groups in the corresponding PUSCH(s) for the TB-based and CBG-based transmission, respectively.

-	If a retransmission is indicated for TB-based transmissions, 'NACK' is assumed for the transport blocks in the corresponding PUSCH(s).

-	If a retransmission is indicated for CBG-based transmissions, if a bit value in the code block group transmission information (CBGTI) field is '0' or '1' as described in subclause 5.1.7.2 in [8], 'ACK' or 'NACK' is assumed for the corresponding CBG in the corresponding PUSCH(s), respectively.

Explicit HARQ-ACK feedback for the purpose of contention window adjustment in this subclause is determined based on valid HARQ-ACK in the CG-DFI as described in subclause 10.5 in [7].

-	The reference duration corresponding to a channel occupancy initiated by the UE including transmission of PUSCH(s) is defined in this subclause as a duration starting from the beginning of the channel occupancy until the end of the first slot where at least one unicast PUSCH is transmitted over all the resources allocated for the PDSCH, or until the end of the first transmission burst by the gNB that contains unicast PUSCH(s) transmitted over all the resources allocated for the PDSCH, whichever occurs earlier, and the duration starting no later than a number of symbols provided by cg-minDFIDelay-r1 before an UL grant or a CG-DFI. If the channel occupancy includes a unicast PDSCH, but it does not include any unicast PDSCH transmitted over all the resources allocated for that PUSCH, and the duration starting no later than a number of symbols provided by cg-minDFIDelay-r1 before an UL grant or a CG-DFI, then, the duration of the first transmission burst by the UE within the channel occupancy that contains PUSCH(s) is the reference duration for CWS adjustment.

-	Tw=max⁡(TA,TB+1ms) where TB is the duraon of the transmission burst from start of the reference duration in ms and TA=5ms if the absence of any other technology sharing the channel cannot be guaranteed on a long-term basis (e.g. by level of regulation), and TA=10ms otherwise.

If a UE transmits transmissions using Type 1 channel access procedures associated with the channel access priority class p on a channel and the transmissions are not associated with explicit or implicit HARQ-ACK feedbacks as described above in this subclause, the UE adjusts CWp before step 1 in the procedures described in subclause 4.2.1.1, using the latest CWp used for any UL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. If the corresponding channel access priority class p has not been for any UL transmission on the channel, CWp=CWmin,p is used.

================================ Unchanged Texts Omitted =================================

================================= End of TP for TS 37.213 =================================

Proposal 5: Adopt the following TP for TS 38.211.

================================= Start of TP for TS 38.211 =================================

5.3.1	OFDM baseband signal generation for all channels except PRACH and RIM-RS

================================ Unchanged Texts Omitted =================================

-	for dynamically scheduled PUSCH, SRS, and PUCCH transmissions

Text=minmaxText',0, Tsymb,(l-1)mod7∙2μμ

Text'=k=1CiTsymb,  l-kmod 7∙2μ μ-Δi

where Δi is given by Table 5.3.1-1 with C1=1 for μ∈0,1, C1=2 for μ=2, and C2 and C3 given by the higher-layer parameters cp-ExtensionC2-r16 and cp-ExtensionC3-r16, respectively, and TTA given by clause 4.3.1. For contention-based random access, or in absence of higher-layer configuration of C2 and C3, the value of Cishall be set to the largest integer fulfilling Text'<Tsymb,  (l-1)mod7∙2μ μ for each of the values of i∈2,3. Text is applied to the first UL transmission scheduled by the scheduling DCI.

================================ Unchanged Texts Omitted =================================

================================= End of TP for TS 38.211 =================================