3GPP TSG RAN WG1 #102-e	R1-2006119

# e-Meeting, August 17th – 28th, 2020

Agenda Item:		7.2.7

Source:				Samsung

Title:					On maintenance of UE power saving

Document for:	Discussion/Decision

Introduction

This contribution considers remaining issues on UE power saving.

Remaining issues on UE power saving

2.1 Clarification on long DRX cycle

The PDCCH monitoring indication is applicable for long DRX cycle. In the current spec, there are some missed wording of “long DRX cycle”. In addition, the UE should not monitor PDCCH for detecting DCI format 2_6 when short DRX cycle is running. These should be corrected together.

Proposed TP for TS 38.213 in Section 10.3

## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331] […] - an offset by ps-Offset indicating a time, where the UE starts monitoring PDCCH for detection of DCI format 2_6 according to the number of search space sets, prior to a slot where the drx-onDuarationTimer for long DRX cycle would start on the PCell or on the SpCell [11, TS 38.321] - for each search space set, the PDCCH monitoring occasions are the ones in the first Ts slots indicated by duration, or Ts=1 slot if duration is not provided, starting from the first slot of the first Ts slots and ending prior to the start of drx-onDurationTimer for long DRX cycle. On PDCCH monitoring occasions associated with a same long DRX Cycle, a UE does not expect to detect more than one DCI format 2_6 with different values of the Wake-up indication bit for the UE or with different values of the bitmap for the UE. The UE does not monitor PDCCH for detecting DCI format 2_6 during Active Time and short DRX cycle [11, TS 38.321]. If a UE reports for an active DL BWP a requirement of X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer for long DRX cycle, the UE is not required to monitor PDCCH for detection of DCI format 2_6 during the X slots, where X corresponds to the requirement of the SCS of the active DL BWP in Table 10.3-1.

2.2 Correction to Type3-PDCCH CSS for PS-RNTI

DCI format 2_6 scrambled by PS-RNTI can be monitored PCell only. However, in the paragraph related to Type3-CSS in 38.213 Section 10.1, it is ambiguous whether the PS-RNTI can be monitored in PCell only or not since PS-RNTI is located wrongly. This should be fixed.

Proposed TP for TS 38.213 in Section 10.1

10.1 UE procedure for determining physical downlink control channel assignment A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets - a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG - a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG - a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell - a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG - a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, or CI-RNTI, or PS-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, or CS-RNTI(s), or PS-RNTI and - a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), SL-RNTI, SL-CS-RNTI, or SL-L-CS-RNTI.

Conclusion

This contribution discussed remaining issues on PDCCH-based power saving signal/channel and corresponding TPs are provided in Section 2.

- References

[1] 3GPP TS 38.213, “NR; Physical layer procedures for control (Release 16)” V16.2.0 (2020-06)