3GPP TSG RAN WG1 #102-e			R1-2006143

e-Meeting, August 17th – 28th, 2020

Agenda Item:		8.3.4

Source:		Samsung

Title:		Discussion for propagation delay compensation enhancements

Document for:		Discussion and decision

# Introduction

In Rel-16, RAN 1 had some study on time synchronization for TSN, considering the potential impact on propagation delay. However, in Rel-16 IIoT work item, there is no compensation on propagation delay on time synchronization for TSN. In WID of Rel-17 IIoT and URLLC [1], enhancements for support of time synchronization were approved with propagation delay compensation enhancements, which requires some work in RAN1.

- Enhancements for support of time synchronization: - RAN impacts of SA2 work on uplink time synchronization for TSN, if any. [RAN2] - Propagation delay compensation enhancements (including mobility issues, if any). [RAN2, RAN1, RAN3, RAN4]

This contribution discusses propagation delay compensation from RAN 1 perspective.

# Discussion

In Rel-16 IIoT study item, RAN 1 did some evaluation on the accuracy of time synchronization for TSN with and without propagation delay.  Some observations were made and captured in TR 38.825 [2] (details can be found in Appendix). For larger service area, e,g, 20 km2, the distance from gNB to UE is about 80m, which brings 267ns propagation delay. In order to achieve the requirement, propagation compensation is needed.

Observation #1: For large service area, propagation compensation is needed.

Based on the study in Rel-16, the estimation errors are related to subcarrier spacing and mainly consist of the following parts:

- gNB transmit error (Tgte): Based on the definition in TS 38.104, the typical value of BS timing error caused by Tgte can be seen as within ± 65ns.

- UE transmit timing error (Te): As defined in TS 38.133 Table 7.1.1-2, this error Te is about as ±12*64*Tc , ±8*64*Tc and ± 7*64*Tc for 15KHz, 30KHz and 60KHz respectively. This includes the error on DL timing estimation.

- gNB estimation error (Tge): This error is introduced when gNB estimates UL channel/signals to obtain propagation delay. PRACH may not able to provide enough accuracy of time estimation. SRS/UL DMRS or PUSCH can be used. The estimation accuracy depends on occupied BW. The error about ~100ns can be assumed with 20MHz bandwidth.

- TA (Propagation delay) granularity (Tpd): If using current TA granularity, i.e., 16∙64∙Tc/2μ, and the error is  ±8∙64∙Tc/2μ.

- Timing Advance adjustment accuracy (Tadj): Based on the requirement in TS 38.133, the accuracy of TA adjustment are ±256 Tc/ ±256 Tc/ ±128 Tc / ±32 Tc respectively for SCS 15/30/60/120 kHz.

Since TA is two times of propagation delay, therefore, the error for propagation delay estimation is half of the sum of the above Te , Tge , Tpd  and Tpd . And when UE compensate the propagation delay, the total time synchronization need to add gNB transmit error (Tgte). Table 1 provide a summary of the above errors. In order to reduce the total error, one easy way is to introduce finer granularity to reduce the error caused by quantization, especial for smaller subcarrier space case, e.g., 15kHz and 30kHz. Other errors are either limited by hardware, or depend on UE or gNB detection performance.

Observation #2: Some other UL channel/signals, e.g., SRS, DMRS, PUSCH occupied larger bandwidth can be used for gNB to estimate propagation delay to increase the accuracy.

Observation #3: Finer granularity for propagation delay compensation can help to improve the performance and it is the easiest way to reduce estimation error.

Proposal #1: Introduce a finer granularity for propagation delay compensation.

Table 1

|  | 15kHz | 30kHz | 60kHz | 120kHz |
| --- | --- | --- | --- | --- |
| gNB transmit error (Tgte) | 65ns | 65ns | 65ns | 65ns |
| UE transmit timing error (Te) | 391ns | 260ns | 228ns | 114ns |
| gNB estimation error (Tge) | 100ns | 100ns | 100ns | 100ns |
| Propagation delay granularity (Tpd) | 260ns | 130ns | 65ns | 32.5ns |
| Timing Advance adjustment accuracy (Tadj) | 130ns | 130ns | 65ns | 16.25ns |
| Total (Tgte+1/2*( Te + Tge +Tpd +Tpd) | 505.5 ns | 375ns | 294ns | 196.4ns |

# Conclusions

This contribution discusses propagation delay compensation from RAN 1 perspective.  The following observations are made:

Observation #1: For large service area, propagation compensation is needed.

Observation #2: Some other UL channel/signals, e.g., SRS, DMRS, PUSCH occupied larger bandwidth can be used for gNB to estimate propagation delay to increase the accuracy.

Observation #3: Finer granularity for propagation delay compensation can help to improve the performance and it is the easiest way to reduce estimation error.

Based on the observations, the following proposals are made:

Proposal #1: Introduce a finer granularity for propagation delay compensation.

# References

- RP- 201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”

- TR 38.825 Study on NR Industrial Internet of Things (IoT)

# Appendix TR 38.825 Study on NR Industrial Internet of Things (IoT)

6.3.2.4	Synchronization accuracy

Based on a request by SA2 in S2-189051 [19] and further clarification by RAN2 in R2-1816043 [28], RAN1 has performed analysis on the achievable time synchronization accuracy over Uu interface between a gNB and a single UE. The effects of the granularity and accuracy of the absolute timing indication information provided by the gNB are not considered. A range of service areas are considered.

The evaluation results on maximum timing synchronization error for different inter-site distances without UE propagation delay compensation are summarized in Table 6.3.2.4-1, while the results on maximum timing synchronization error with UE propagation delay compensation are shown in Table 6.3.2.4-2.

Table 6.3.2.4-1: Summary of maximum timing synchronization error results for 
different inter-site distances without UE propagation delay compensation.

|  | 15kHz SCS | 30kHz SCS | 60kHz SCS | 120kHz SCS |
| --- | --- | --- | --- | --- |
| Source R1-1900156 [21] | [-278ns,376ns] | [-147ns,245ns] |  | [-82ns,180ns] |
| Source R1-1900903 [30] | 355ns (114m ISD) |  |  |  |
| Source R1-1900935 [23] | 215ns (20m ISD) 315ns (60m ISD) |  |  |  |
| Source (1) R1-1901072 [27] | 133ns (10m ISD) |  |  |  |
| Source R1-1901252 [24] | 506ns (20m ISD) | 441ns (20m ISD) | 343ns (20m ISD) |  |
| Source R1-1901353 [31] | 315ns (10m ISD) 350ns (20m ISD) 1080ns (250m ISD) |  |  |  |
| Note (1): Half of the reported values of R1-1901072 [27] are included in this table, to align the results with the other sources and to only account for gNB-to-UE and not UE-to-UE synchronization accuracy. |  |  |  |  |

Table 6.3.2.4-2: Summary of maximum timing synchronization error results 
with UE propagation delay compensation.

|  | 15kHz SCS | 30kHz SCS | 60kHz SCS |
| --- | --- | --- | --- |
| Source R1-1900156 [21] | 488ns | 357.5ns | 276.5ns |
| Source R1-1901334 [22] | 505ns | 371ns | 287.5ns |
| Source R1-1900935 [23] | 472.5ns | 338.5ns |  |
| Source R1-1901252 [24] | 536ns | 438ns | 357ns |

Based on the evaluation results, the following has been observed:

-	If a UE were not to apply propagation delay compensation, a gNB-to-UE timing synchronization accuracy of

-	130 to 376ns for an ISD of 10m (3 sources)

-	215 to 506ns for an ISD of 20m (3 sources)

-	315 ns for an ISD of 60m (1 source)

-	355ns for an ISD of 114m (1 source)

-	1080ns for an ISD of 250m (1 source)

Based on the RAN1 agreed evaluation assumptions can be achieved for Rel-15 NR with 15kHz SCS. The achievable accuracy without propagation delay compensation becomes worse as the ISD increases. 2 out of 6 sources note that a better synchronization accuracy can be achieved for higher SCS (i.e. the higher the SCS, the better the accuracy).

-	If a UE was to apply propagation delay compensation, a gNB-to-UE synchronization accuracy of 470ns to 540ns (from a total of 4 sources) for 15kHz SCS can be achieved independently of the ISD. The synchronization accuracy with propagation delay compensation improves for higher SCS (i.e. the higher the SCS, the better the accuracy).

-	For small service areas with dense small cell deployments a propagation delay compensation by the UE is not required. The propagation delay compensation needs to be applied by the TSN UEs for larger service areas with more sparse cell deployments (inter-site distances >200m to achieve a synchronization accuracy better than 1us).