3GPP TSG RAN WG1 #102-e	    			             R1-2006168

e-Meeting, August 17th – 28th, 2020

Agenda item:	8.11.1

Source: 	Samsung

Title: 	On Sidelink Evaluation Methodology Updates for Power Saving

Document for:	Discussion and Decision

# Introduction

In RAN#86, a work item on “NR Sidelink enhancement” was approved [1]. This work item was further updated in RAN#88-e [2] to account for the delay in starting the work item due to travel restrictions.

One of the objectives of the work item is to update the sidelink evaluation methodology for power saving up reusing the LTE device-to-device TR (TR 36.843) [3] and the NR power savings TR (TR 38.840) [4].

1. Sidelink evaluation methodology update: Define evaluation assumption and performance metric for power saving by reusing TR 36.843 and/or TR 38.840 (to be completed by RAN#89) [RAN1]

- Note: TR 37.885 is reused for the other evaluation assumption and performance metric. Vehicle dropping model B and antenna option 2 shall be a more realistic baseline for highway and urban grid scenarios.

In this contribution, we present Samsung’s views on evaluation methodology updates for simulating the power saving sidelink enhancements.

# Overview

## Simulation Scenarios

The evaluation assumptions and scenarios defined in TR 37.885 [5] are a starting point for the evaluation of the proposed resource allocation enhancements. The following scenarios are considered for evaluation:

- Urban grid (Figure A-1 of [5]) and highway (Figure A-2 of [5]) road configurations.

- Focus is on FR1 with the evaluation scenario as described in Table 6.1.1-1 of [5].

- Dropping model:

  - Vehicular UEs: Option A for highway scenario [5]. Option A for urban grid scenario [5].

  - Pedestrian UEs: For urban grid only as described in section A.1.2 of [6].

Proposal 1: Reuse the simulation scenarios and simulation scenario parameters from TR 37.885 [5].

Proposal 2: Simulations are performed in FR1.

## UE Power Consumption Model

One of the focus areas of resource allocation enhancements in release 17 is power savings, hence it is important to agree on a UE power consumption model and performance metrics related to power savings. We consider a relative power model covering the following operations; PSCCH/PSSCH Tx, PSCCH Rx, PSSCH Rx, PSFCH Tx, PSFCH Rx, SSB Tx and SSB Rx. The objective of this model is to find the power ratio between these operations for a given resource allocation, and how to scale the power as a function of the number of allocated symbols Nsym, number of allocated sub-channels NsubCh, number of PRBs per sub-channel NPRBsubCh, number of received PSFCH PRBs NRx_PRBPSFCH and transmit power. The model developed for NR power saving in [4] is used as a guiding principle to develop the SL UE power consumption model. SL receive channels are analogous to DL channels that are received by a UE. SL transmit channels are analogous to UL channels that are transmitted by a UE.

According to [4], the downlink power scales with the BWP bandwidth as 0.4+0.6100Xr-2080=0.25+0.75Xr, where Xr is the relative BW of the BWP. Using a similar principle for the reception of sidelink channels, we can derive the a formula for the relative power consumption to receive NsubChwith NPRBsubCh PRBs per sub-channel

Relative SL Receivee Power Consumption=0.25+ NPRBsubCh∙NsubCh360

For PSSCH reception, let NsubChi be the number of sub-channels in symbol i. The number of symbols in a slot including the duplicated symbol (for AGC) is Nsym. For the duplicated symbol, we assume a relative receive power consumption of 0.25, for the remaining symbols, the receive power consumption is calculated according to the above formula. As a reference, the power is normalized to the maximum power in a SL slot (i.e. when there are 12 SL symbols, excluding the AGC symbol):

Relative PSSCH Receive Power Consumption=0.25∙Nsym12.25+ NPRBsubChi=1Nsym-1NsubChi360∙12.25

For PSCCH reception, let α be the number of PSCCH decodes per sub-channel, where due to the orthogonal cover code on PSCCH α can be 1, 2 or 3. NPSSCHRx,subCh is the number of PSCCH sub-channels received in a slot including sub-channels received due to sensing, and NPSCCH,sym is the number of PSCCH symbols, excluding the symbol duplicated for AGC. The relative PSCCH receive power consumption is

Relative PSCCH Receive Power Consumption=0.25∙NPSCCH,sym+112.25+ α∙NPRBsubCh∙NPSSCHRx,subCh∙NPSCCH,sym360∙12.25

In case of PSCCH/PSSCH reception in the same slot, duplication of the fixed power is avoided. For example, in case of reception of PSCCH/PSSCH with no sensing, the relative PSCCH/PSSCH receive power consumption is

Relative PSCCH/PSSCH Receive Power Consumption=0.25∙Nsym12.25+ NPRBsubCh∙NsubCh∙Nsym-1360∙12.25

For S-SSB reception, the S-SSB slot has 13 symbols, excluding the gap symbol. The relative S-SSB receive power consumption is 0.3.

For PSFCH reception, a PSFCH occupies a cyclic shift in a PRB. As a first order approximation, we only consider the total number of PRBs on which a PSFCH is received, i.e. NRx_PRBPSFCH. The structure of PSFCH has one symbol that is duplicated for AGC purposes. For the duplicated symbol, we assume a relative receive power consumption of 0.25. The relative PSFCH receive power consumption is:

Relative PSFCH Receive Power Consumption=0.25∙212.25+ NRx_PRBPSFCH360∙12.25

Now we consider the SL transmit channels. When transmitting at a power level PTx, the power doesn’t scale with the number of sub-channels or PRBs. But the power scales linear with the number of symbols. We now want to find the ratio between the receive power consumption and the transmit power consumption. According to Table 20 of TR 38.840 [4], when the UL transmit power is 0 dBm (1 mW), the relative ratio of uplink to downlink processing is 250 to 300. When the UL transmit power 23 dBm (200 mW), the relative ratio of uplink to downlink processing is 700 to 300. In general, for a transmit power P (in mW), the ratio of uplink to downlink processing is assumed to be Pr to 300. Where, Pr scales linearly with PTx, i.e.   Pr=450199PTx-1+250.

For PSCCH/PSSCH transmission with power P over Nsym symbols, the relative power (energy per slot) can be expressed as:

Pr300∙Nsym12.25

For S-SSB transmission with power P, the relative power can be expressed as:

Pr300∙1312.25

For PSFCH transmission with power P, the relative power can be expressed as:

Pr300∙212.25

Table 1 summarizes the relative power and dependencies for sidelink operations.

Table 1: Sidelink Power Consumption Model.

| Operation | Relative power including dependencies |
| --- | --- |
| PSSCH Rx | 0.25∙Nsym12.25+ NPRBsubChi=1Nsym-1NsubChi360∙12.25 |
| PSCCH Rx | 0.25∙NPSCCH,sym+112.25+ α∙NPRBsubCh∙NPSSCHRx,subCh∙NPSCCH,sym360∙12.25 |
| S-SSB Rx | 0.3 |
| PSFCH Rx | 0.25∙212.25+ NRx_PRBPSFCH360∙12.25 |
| PSCCH/PSSCH Tx | Pr300∙Nsym12.25 |
| S-SSB Tx | Pr300∙1312.25 |
| PSFCH Tx | Pr300∙212.25 |
| Nsym: Number of symbols for PSSCH including duplicate symbol for AGC. NPRBsubCh: Number of PRBs per sub-channel NsubChi: Number of sub-channels in symbol i for PSCCH NPSCCH,sym: Number of PSCCH symbols α: Number of PSCCH decode attempts due to OCC per sub-channel NPSSCHRx,subCh: Number of PSCCH receive sub-channels. NRx_PRBPSFCH: Number of PRBs on which PSFCH is received. Pr=450199PTx-1+250, where PTx is the transmit power in mW. |  |

Proposal 3: For the evaluation of power savings of the enhanced resource allocation schemes, adopt the sidelink UE power consumption model of Table 1.

## Performance Metrics

For sidelink performance, packet reception ratio (PRR) and packet-inter-reception (PIR) are used as performance metrics.

For sidelink power saving performance, relative power reduction ratio (RPRR) can be used as performance metric. Where the relative power reduction ration is defined as:

Relative Power Reduction Ratio (RPRR)= Power of baseline scheme-Power of proposed schemePower of baseline scheme

The baseline scheme is based on release 16 NR sidelink resource allocation mechanisms.

One of the focus areas of the NR sidelink enhancement work item is to study resource allocation mechanism to enhance reliability and reduce latency. The reliability aspect can be determined based on PRR and PIR. For latency, we can measure the time between packet reception and successful packet arrival at the receiving UE. The performance metric can include CDF of latency and the 5% latency.

Proposal 4: Adopt the following performance metrics for the evaluation of the enhanced resource allocation mechanisms:

- PRR and PIR

- Relative Power Reduction Ratio (RPRR) – for power savings resource allocation schemes.

- CDF of latency and 5% latency for schemes targeting improved reliability and low latency.

Proposal 5:

For the evaluation of the power saving resource allocation schemes, relative power reduction ratio is adopted as a performance metric. The relative power reduction ratio (RPRR) is defined as:

Relative Power Reduction Ratio (RPRR)= Power of baseline scheme-Power of proposed schemePower of baseline scheme

The baseline scheme is based on release 16 NR sidelink resource allocation mechanisms.

# Conclusions

In this contribution we presented our views regarding the simulation scenarios, UE power consumption model and performance metrics for evaluating the enhanced resource allocation schemes. The following proposals have been made:

Proposal 1: Reuse the simulation scenarios and simulation scenario parameters from TR 37.885 [5].

Proposal 2: Simulations are performed in FR1.

Proposal 3: For the evaluation of power savings of the enhanced resource allocation schemes, adopt the sidelink UE power consumption model of Table 1.

Proposal 4: Adopt the following performance metrics for the evaluation of the enhanced resource allocation mechanisms:

- PRR and PIR

- Relative Power Reduction Ratio (RPRR) – for power savings resource allocation schemes.

- CDF of latency and 5% latency for schemes targeting improved reliability and low latency.

Proposal 5:

For the evaluation of the power saving resource allocation schemes, relative power reduction ratio is adopted as a performance metric. The relative power reduction ratio (RPRR) is defined as:

Relative Power Reduction Ratio (RPRR)= Power of baseline scheme-Power of proposed schemePower of baseline scheme

The baseline scheme is based on release 16 NR sidelink resource allocation mechanisms.

# References

- RP‑193231, “NR Sidelink enhancement,” RAN#86, Stiges, Spain, December, 2019.

- RP‑201385, “NR Sidelink enhancement,” RAN#88-e, e-meeting, June, 2020.

- TR 36.843, “Study on LTE Device to Device Proximity Services,” 3GPP, March 2014.

- TR 38.840, “Study on User Equipment (UE) power saving in NR,” 3GPP, June 2019.

- TR 37.885, “Study on evaluation methodology of new Vehicle-toEverything (V2X) use cases for LTE and NR,” 3GPP, June 2019.

- TR 36.885, “Study on LTE-based V2X Services,” 3GPP, June 2016.