3GPP TSG RAN WG1 #102-e Meeting	    			             R1-2006174

e-Meeting, August 17th – 28th, 2020

Agenda item:	8.12.2

Source: 	Samsung

Title: 		On Mechanisms to improve reliability for RRC_CONNECTED UEs

Document for:	Discussion and Decision

Introduction

In order to support multicast transmission in NR, the work item was approved for Rel-17 [1]. There would be many applications of multicast data transmission, which was not supported by NR in Rel-15 and Rel-16, where the use cases include mission critical, V2X applications, multicast delivery, IPTV, software delivery over wireless, group communications and IoT applications.

In this contribution, how to improve reliability for RRC_conneted UEs in supporting multicast services is discussed.

Discussion

One of the critical points to improve reliability is introducing UL feedback. For RRC_connected UEs, the gNB can configure the resources for PUCCH to deliver HARQ-ACK feedback for multicast. Therefore, those PUCCH resource can be utilized for HARQ-ACK feedback corresponding to multicast PDSCH.

Proposal 1. HARQ-ACK feedback corresponding to PDSCH is supported for multicast.

For RRC_connected UEs, unicast and multicast PDSCH’s can be scheduled slot-by-slot. It means, dynamic scheduling of unicast and multicast services could be possible. In this regards, a UE can be required to transmit HARQ-ACK feedback for unicast and multicast PDSCH at the same time. In this case, it can be discussed whether to multiplex the HARQ-ACK feedback bits for unicast and multicast for a given UEs. In order to have flexibility and have high reliability for many use cases, multiplexing and prioritization may be needed.

Proposal 2. Both multiplexing and prioritization of HARQ-ACK feedback between unicast and multicast transmission are supported.

In Rel-15 NR, three MCS tables are defined: 1) 64-QAM MCS table, 2) 256-QAM MCS table, and 3) low-coding rate MCS table. Depending on use cases, all of three MCS tables could be an optimal one.

Proposal 3. All MCS tables defined in Rel-15 NR are supported for multicast.

Conclusions

This contribution considered improvement of reliability to support multicast transmission in NR and provided the following proposals.

Proposal 1. HARQ-ACK feedback corresponding to PDSCH is supported for multicast.

Proposal 2. Both multiplexing and prioritization of HARQ-ACK of unicast and multicast transmission are supported.

Proposal 3. All MCS tables defined in Rel-15 NR are supported for multicast.

- References:

[1] RP‑201038, “Revised Work Item on NR Multicast and Broadcast Services,” June 2020.