3GPP TSG RAN WG1 #102-e	R1-2006176

# e-Meeting, August 17th – 28th, 2020

Agenda item:	8.13.1

Source:	Samsung

Title:	Cross-carrier scheduling from SCell to PCell

Document for:	Discussion and decision

- Introduction

Rel-17 Dynamic spectrum sharing (DSS) consists of two main objectives as following [1]:

This work item is limited to FR1, and includes the following objectives for NR Dynamic Spectrum Sharing (DSS): - PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2] - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI - The number of cells can be scheduled at once is limited to 2 - The increase in DCI size should be minimized - Note: The total PDCCH blind decoding budget should not be changed as a result of this work - Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

This contribution discusses the aspects for cross-carrier scheduling from SCell to PCell including search space sets, PDCCH monitoring, PUCCH, and so on.

- Search space sets

Table 1 summarizes Rel-15/16 search space, corresponding RNTI/DCI format, and search space location.

Table 1. Search space, corresponding RNTI/DCI format, and location

| Search space | RNTI | DCI format | Rel-15/16 search space location (PCell/SCell) |
| --- | --- | --- | --- |
| Type0 CSS | SI-RNTI | 1_0 | PCell of the MCG |
| Type0A CSS | SI-RNTI | 1_0 | PCell of the MCG |
| Type1 CSS | RA-RNTI, MsgB-RNTI | 1_0 | PCell (PSCell) |
|  | TC-RNTI | 1_0, 0_0 | PCell (PSCell) |
| Type2 CSS | P-RNTI | 1_0 | PCell of the MCG |
| Type3 CSS | INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, CI-RNTI | 2_0, 2_1, 2_2, 2_3, 2_4 | PCell/SCell |
|  | PS-RNTI | 2_6 | PCell (PSCell) |
|  | C-RNTI, MCS-C-RNTI, CS-RNTI(s) | 0_0, 0_1, 1_0, 1_1 | PCell (PSCell) |
| USS | C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), SL-RNTI, SL-CS-RNTI, SL-L-CS-RNTI | 0_0, 0_1, 0_2, 1_0, 1_1, 1_2 | PCell/SCell |

A few observations with respect to cross-carrier scheduling from SCell to PCell are as follows:

For USS

- A UE can be configured with CIF for DCI formats 0_1/0_2/1_1/1_2 but not for DCI formats 0_0/1_0.

For Type0/0A/1/2 CSS

- Applicable DCI formats are DCI formats 1_0/0_0 where no CIF exists.

- Regardless of whether or not cross-carrier scheduling from SCell to PCell is supported, Type0/0A/1/2 CSS need to be on the PCell of MCG to support UEs in single cell operation and legacy UEs.

- PDCCH for Type0/0A/1/2 CSS is not frequently transmitted, it is UE-common, and therefore does not materially contribute to PDCCH overhead on the PCell – can be accommodated as in Rel-15/16.

- Therefore, it is not essential or of particular importance to support cross-carrier scheduling from SCell to PCell for Type0/0A/1/2 CSS.

For Type3 CSS

- PDCCHs with DCI formats 2_0/2_1/2_2/2_3/2_4 are transmitted on either PCell or SCell and are able to indicate each serving cell by higher layer configuration (e.g., servingCellId, positionInDCI).

- PDCCH with DCI format 2_6 is transmitted on PCell (PSCell). “SCell dormancy indication” field indicates one of the SCell group(s) configured by higher layer.

  - As DCI format 2_6 needs to be detected within a small window to improve UE power savings and as DCI format 2_6 is at most group-cast over few UEs (especially when it also indicates SCell dormancy/non-dormancy), it can be considered to also support on the SCell as for all other DCI formats with PDCCH monitored according to Type3 CSS.

Observation 1: For Type0/0A/1/2 CSS, there is no need or material benefit to support cross-carrier scheduling from SCell to PCell (no spec change is required).

Observation 2: There is benefit to support DCI format 2_6 from SCell and align support from SCell for all DCI formats with PDCCH monitored according to Type3 CSS.

- PDCCH monitoring

A UE capability for PDCCH monitoring is currently partitioned as follows (Clause 10.1 of [2]).

*** Unchanged text is omitted *** If a UE - is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is not provided monitoringCapabilityConfig-r16 or is provided monitoringCapabilityConfig-r16 = r15monitoringcapability, and - with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cell(s) using SCS configuration μ, where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap, and - a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,slot,μ=Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j PDCCH candidates or more than CPDCCHtotal,slot,μ=Ncellscap⋅CPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the Ncells,0DL+Ncells,1DL downlink cells. ## *** Unchanged text is omitted ***

When the PCell can be scheduled by either the PCell or from an SCell, the PCell needs to be counted as 2 scheduled cells. For example, if the UE has active DL BWPs using 15 kHz SCS on the PCell and 30 kHz SCS on the SCell, it is clear that the PCell is a scheduled cell for both the PCell scheduling cell and the SCell scheduling cell (NcellsDL,μ exist for both μ=0 and μ=1), corresponding search space sets need to exist on both scheduling cells, and require corresponding allocation of PDCCH candidates and non-overlapping CCEs.

A next issue is whether M-TRP operation needs to be same on both PCell and SCell (there is no such link among cells in Rel-16). There is no need to have such restriction in the specifications. Also, from an operational perspective it would be detrimental (e.g. PCell can be a macro-cell with single TRP, SCell can be a small cell with two TRPs for improved mobility support and “cell edge” data rates).

Observation 3: For DSS operation, multi/single-TRP operation should remain independent on the PCell and the SCell.

Proposal 1: For determining a total number of PDCCH candidates and non-overlapping CCEs for scheduling cells in DSS operation, the PCell is counted as a scheduled cell for both the PCell and the SCell scheduling cell.

A UE can be configured with more than one DL BWPs, or can be provided multiple search space set groups. Then, the UE can switch an active DL BWP to a default DL BWP, or can switch between search space set groups, based on expiration of corresponding timers that run independently on the PCell and the SCell. For DSS, one possible issue is the impact the timers on the SCell have for scheduling on the PCell. For example, the UE may not be scheduled for some time on the SCell, the bwp-InactivityTimer expires, and the UE switches to the default BWP on the SCell that typically has small bandwidth and sparse PDCCH monitoring. The gNB can control this by not configuring a timer/default BWP for the UE on the SCell. An alternative is for the UE to not switch to the default BWP on the SCell if the active DL BWP is not the default BWP on the PCell (e.g. the UE can have reduced PDCCH monitoring for scheduling on the PCell when the active DL BWP is the default BWP on both the PCell and the SCell). A similar issue is the potential dormancy behavior of the SCell. Even though a network can choose to not configure the SCell in the groups of SCells for which a dormancy indication is applicable, it needs to be determined whether the UE is expected to continue monitoring PDCCH for scheduling on the PCell if the UE did not receive a WUS for the PCell for a corresponding DRX cycle and the SCell remains in the non-dormant state.

Observation 4: The UE behavior for PDCCH monitoring on the SCell for scheduling on the PCell needs to be defined with respect to timers causing a change in active DL BWP or search space set group and to PCell (non-)WUS indication.

For search space set dropping, given that Rel-16 supports PDCCH monitoring according to Type-3 CSS on an SCell and also supports the SCell being the scheduling cell for more than one cell, there are no new requirements introduced by DSS operation.

Observation 5: Search space set dropping should remain as in Rel-16 (applicable only for the PCell).

- PUCCH

This section considers potential impacts on PUCCH/HARQ-ACK reporting due to DSS operation.

A set of PUCCH resources is configured per UL BWP and therefore DSS operation does not have impact on PUCCH resource selection. PUCCH power control is determined by UL-specific parameters, pathloss from DL PCell, and TPC commands in DCI format 1_X and DCI format 2_2. Pathloss from DL PCell is a long-term measurement and is always available regardless of DSS operation. Regarding TPC commands in DCI format 1_X and DCI format 2_2, as discussed in section 2, there is no impact from DSS operation as Type-3 CSS can be on the SCell. Therefore, existing PUCCH power control mechanism can work properly with DSS operation.

A PUCCH may include multiple UCI types (HARQ-ACK, SR and CSI report). PUCCH configuration for SR and CSI report is independent of DSS operation. For HARQ-ACK report, there are three types of HARQ-ACK codebook determination, i.e. type-1 HARQ-ACK codebook (semi-static codebook), type-2 HARQ-ACK codebook (dynamic codebook, including the enhanced one), and type-3 HARQ-ACK codebook (one-shot codebook). In general, the type-1 and type-3 codebooks are based on PDSCH parameters and PUCCH parameters, e.g. TDRA table for PDSCH receptions, UL/DL configurations for serving cells of PDSCH, number of HARQ processes for PDSCH and K1 for PUCCH, and are all independent of the scheduling cells. In case of multiple TRPs, although type-1 codebook is also based on CORESET configuration, this is not relevant to DSS operation. For the type-2 codebook, the HARQ-ACK codeword is based on the counter and total DAIs that are accumulated in 3 dimensions, i.e. in order of the PDSCH reception starting time for the same {PDSCH reception cell, PDCCH monitoring occasion} pair, then in order of PDSCH reception cell index, and then in order of PDCCH MO index m. Clearly, the first two dimensions are irrelevant to PDCCH reception. Although the PDCCH MO is relevant to search space sets, the index of the PDCCH MO is determined only by the start time of search space sets and therefore DSS operation has no impact.

Therefore, DSS operation has no impact on Rel-16 PUCCH operation or on HARQ-ACK codebook generation. There is also no impact from DSS operation on PRACH, SRS, or PUSCH scheduling/transmissions.

Observation 6: DSS operation has no impact on the UL.

- Other

When a gNB enables cross-carrier scheduling from SCell to PCell for a UE (e.g., in the process of SCell addition), the association between PCell and SCell needs to be provided similar to CrossCarrierSchedulingConfig in Rel-16. Further, the CIF value used in the scheduling cell to indicate the scheduled cell can be configured for the DCI formats 0_1/0_2/1_1/1_2 as in Rel-16.

SCell activation/deactivation functionality can be maintained. Specifically, upon SCell activation, UE monitors PDCCH on the SCell for scheduling on the PCell and/or SCell(s) (the UE monitors PDCCH for Type0/0A/1/2 CSS on the PCell).

- Conclusion

Based on above discussion for cross-carrier-scheduling from SCell to PCell, we provide the following observations and proposal:

Proposal 1: For determining a total number of PDCCH candidates and non-overlapping CCEs for scheduling cells in DSS operation, the PCell is counted as a scheduled cell for both the PCell and the SCell scheduling cell.

Observation 1: For Type0/0A/1/2 CSS, there is no need or material benefit to support cross-carrier scheduling from SCell to PCell (no spec change is required).

Observation 2: There is benefit to support DCI format 2_6 from SCell and align support from SCell for all DCI formats with PDCCH monitored according to Type3 CSS.

Observation 3: For DSS operation, multi/single-TRP operation should remain independent on the PCell and the SCell.

Observation 4: The UE behavior for PDCCH monitoring on the SCell for scheduling on the PCell needs to be defined with respect to timers causing a change in active DL BWP or search space set group and to PCell (non-)WUS indication.

Observation 5: Search space set dropping should remain as in Rel-16 (applicable only for the PCell).

Observation 6: DSS operation has no impact on the UL.

References

[1] RP-193260, New WID on NR Dynamic spectrum sharing (DSS), Ericsson

- [2] TS 38.213 v16.2.0, “NR; Physical layer procedures for control”