3GPP TSG RAN WG1 #102-e			 R1-2006183

e-Meeting, August 17th – August 28th, 2020

Agenda Item:	8.11.2

Source:	InterDigital, Inc.

Title:	Sidelink Resource allocation for Power saving

Document for:	Discussion and Decision

# Introduction

The Rel17 WID has the following objective [1]:

2. Resource allocation enhancement: - Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

In this contribution, we discuss the aspects to enhance resource allocation for power saving

# Resource allocation enhancement

The resources for sidelink transmission are configured with a resource pool. The same applies for PUE cases and the following two options can be considered:

- Option 1: A dedicated resource pool is used for discovery transmission.

- Option 2: A shared resource pool is used for both PUE and VUE.

Each option seems to have pros and cons. For example, the dedicated resource pool for PUE may degrade system performance due to the resource fragmentation. On the other hand, the shared resource pool between PUE and VUE could impact legacy UE (e.g. VUE) performance if the same sensing scheme is not used for the PUE to lower the power consumption.

In LTE V2X, a shared resource pool between PUE and VUE is a supported configuration and no such a restriction not to allow the resource sharing between those UEs. For example, resource pool configuration can support both options (i.e. shared resource pool between PUE and VUE or a dedicated resource pool for PUE) [2]. Considering that NR sidelink should have at least the same level resource pool configuration flexibility, both options should also be supported in NR.

Proposal 1: Resource pool configuration flexibility for VUE and PUE should be at least the same as LTE as the following:

- Shared resource pool between VUE and PUE

- Dedicated resource pool for PUE

In LTE V2X, P2V scenario was only optimized in terms of power saving by allowing the PUE to perform random resource selection and/or partial sensing-based resource selection. On the other hand, V2P scenario was not considered for power consumption reduction since the PUE shall decode all subchannels in the Rx resource pools to determine the transmission to itself for V2P communication,. In NR, it is also expected that the PUE needs to receive the message from other PUE and VUE; therefore, V2P should be optimized. Specifically, at least a mechanism to reduce SCI decoding of PUE should be supported.

Proposal 2: Support power saving mechanism for sensing and resource selection of PUE

In R16 NR V2X, VUE is required to perform contiguous sensing. However, it should not be required for PUE since contiguous sensing is not a power efficient feature. In LTE V2X, slot-based partial sensing, in which the PUE performs sensing in a subset of slots is supported. This feature can help to improve the power efficiency of the sensing procedure and can be supported in NR.

Proposal 3: Support slot-based partial sensing

NR and LTE V2X support two types of resource reservation, which are periodic and aperiodic reservations. Long-term partial sensing, in which the UE selectively performs sensing in a (pre-)configured set of slots, can be used to detect periodic reservation. In long-term partial sensing, potential collision with periodic reservations can be detected by sensing in the slots having a (pre-)configured time gap with the potential transmission slots. The (pre-)configured time gap can be any of the allowed reservation periods in the resource pool.

Short-term partial sensing, in which the UE performs sensing to detect potential collision before a potential transmission resource, can be used to detect aperiodic reservation. The short-term partial sensing window should be smaller than the maximum dynamic resource reservation of SCI (i.e. 32 slots).

Both long-term and short-term partial sensing are necessary to reduce the potential collision of the selected resources. Therefore, both types of sensing should be supported.

Proposal 4: Slot-based partial sensing should support detection of periodic and aperiodic reservations.

In NR V2X, VUE can support sensing and resource selection pool of up to 27 subchannels [3]. For PUE, performing sensing and resource selection in such a large resource pool may not power efficient since the PUE needs to perform blind detection of SCI in each subchannel. Furthermore, two types of SCI shall be decoded with different associated DM-RS, which requires higher computational power at the receiver as compared with the LTE. One possibility to reduce the number of blind detections is to allow the PUE to perform sensing in a subset of the frequency resources.

Proposal 5: Support partial sensing and resource selection based on subset(s) of frequency resources.

In R16 NR V2X, re-evaluation and pre-emption are supported in the sensing and resource selection procedure. Re-evaluation can be used to determine potential collision of the selected resource (e.g. the resource that has not been reserved by SCI) and pre-emption can be used to detect potential collision of the reserved resources.

Re-evaluation and pre-emption are two essential features to reduce collision and should be supported for PUE. Since partial sensing is used for PUE, RAN1 needs to further study how to perform re-evaluation and pre-emption in a power efficient manner (e.g. partial sensing-based resource re-evaluation and pre-emption).

Proposal 6: Support partial sensing-based resource re-evaluation and pre-emption

# Conclusion

In this contribution, the following conclusions were made for resource allocation for power saving:

Proposal 1: Resource pool configuration flexibility for VUE and PUE should be at least the same as LTE as the following:

- Shared resource pool between VUE and PUE

- Dedicated resource pool for PUE

Proposal 2: Support power saving mechanism for sensing and resource selection of PUE

Proposal 3: Support slot-based partial sensing

Proposal 4: Slot-based partial sensing should support detection of periodic and aperiodic reservations.

Proposal 5: Support partial sensing and resource selection based on subset(s) of frequency resources.

Proposal 6: Support partial sensing-based resource re-evaluation and pre-emption

# References

- RP-201385, “WID revision: NR sidelink enhancement”, LG Electronics

- TS 36.331, “LTE RRC Specification”, v.15.5.0

- TS 38.331, “NR RRC Specification”, v.16.1.0.