3GPP TSG RAN WG1 #102-e									    R1-2006184

e-Meeting, August 17th – August 28th, 2020

Agenda Item:	8.11.2.1

Source:	InterDigital, Inc.

Title:	NR SL Mode 2 enhancement for reliability improvement

Document for:	Discussion and Decision

# Introduction

In RAN #86, the following [1] study of enhancement of Mode 2 resource allocation was agreed to improve the reliability and latency performance of R17 SL operation.

- Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2]

  - Inter-UE coordination with the following until RAN#90.

    - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission.

  - Note: The study scope after RAN#90 is to be decided in RAN#90.

  - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios.

In this contribution, we discuss concepts and design aspects of inter-UE coordination based on the description provided in R17 SL Enhancement WID [1] for a candidate solution for enhancement of Mode 2 resource allocation.

# Discussion

During the R16 V2X SI discussions, a resource allocation mechanism, namely Mode 2(b), in which a UE assists SL resource selection for other UE(s) [2] was proposed and discussed.  It was concluded at the end of the R16 V2X SI phase to further study of the UE assistance mechanism as a part of the Mode 2 functionality [2].  The concept of inter-UE coordination as described in R17 SL Enhancement WID [1] is closely related to that of Mode 2(b).  Therefore, it is important to re-visit the design aspects for Mode 2(b) below [2] summarized in RAN1 #94 meeting and continue the discussions under the context of R17 inter-UE coordination based the recently finalized R16 SL design.

Agreements:

- The following aspects about assistance information are studied for Mode 2(b)

  - Which assistance information is used and how it is acquired

  - Which UE sends assistance information

  - How to deliver assistance information, including physical channel and UE behavior

  - How assistance information is taken into account in determination of sidelink resource for transmission

It was discussed during SI phase that assistance information is more applicable for resource selection for feedback signaling, e.g. HARQ feedback (PSFCH) and CSI reporting.  However, in R16 V2X, PSFCH resource is implicitly based on association with the corresponding PSSCH transmission and CSI reporting carried in a MAC CE using resources based on sensing.  Thus, the inter-UE coordination in R17 should be focused on the resource selection for PSSCH transmission carrying a user data transport block (TB).  In addition, the assistance information can be primarily a set of resources as indicated in WID [1].

The R16 Mode 2 resource selection for data transmission is based on sensing performed by the transmitting UE.  The main purpose of the inter-UE coordination is to enhance the reliability and/or reduce the latency of the Mode 2 SL transmission, e.g. by mitigating consecutive packet loss due to persistent collisions.  It is therefore expected in certain SL operation scenarios that the resources sent by UE A to UE B result in better transmission performance than the resources selected by UE B based on its own sensing.  Examples of such SL operation scenarios include:

- A unicast link between UE A and UE B and UE A provides a resource set specific to the unicast link for UE B’s transmissions.  In this scenario, UE B’s transmissions are effectively based on receiver sensing, which can address “hidden node” issues.

- A groupcast link between UE A and UE B and UE A provides a resource set including resources designated exclusively to the group transmission.  The resources in this group-specific resource set are less subject to collisions with other SL transmissions.

It is not apparent that the reliability and/or latency of a broadcast transmission by UE B can also be improved by using a resource set received from UE A.  However, the inter-UE coordination could lead to power saving benefit, e.g. when UE B is a VRU device with battery constraint.  In this case, UE B could perform SL transmissions without sensing to conserve battery and use resources received from UE A.

- Proposal 1: Support at least unicast and groupcast PSSCH transmissions using resources based on a set of resources indicated by a peer UE.

There are several options regarding what resources to be included in the resource set provided by UE A.  For example, the set of resources may be based on

- Sensing-based resources

  - Sensing result (Set A) using transmission parameters provided by UE B

  - Intermediate sensing result (decoded L1 priority and RSRP)

  - Resource selection result based on sensing result using transmission parameters provided by UE B

- Designated resources

  - A set of resources designate for UE B’s use

It is important that the sensing-based resources are obtained in accordance with the QoS requirements of UE B’s SL transmission, e.g. priority and delay packet budget.  When a UE performs its own sensing, these parameters are included in the sensing and resource selection configuration.  Thus, it is desired that the sensing configuration of UE A are based on UE B transmission parameters as much as possible in the inter-UE coordination.  One option is that UE B provides the necessary transmission parameters to UE A prior to the UE A sensing for the resources to be sent to UE B.  The details of such mechanism including e.g. steps of UE B transmission of sensing configuration, UE A sensing using the received parameters and UE A transmission of resource set based on the sensing should be further studied.

Another option is that UE A provides raw sensing data including decoded L1 priority and measured RSRP of each resource in the resource set to allow further processing by UE B.  Both options can result in transmissions of large amount of resource set data and thus using PSSCH transmission for the resource set information should be considered.  To further reduce the payload of the resource set information, UE A can perform both sensing and resource selection using the transmission parameters provided by UE B and provide the resource selection result to UE B.

- Proposal 2: A UE provides a set of sidelink resources to the peer UE based on sensing in a PSSCH transmission for a unicast transmission.

It is important to improve the reliability of the SL transmission carrying the resource set in Mode 2 operation, i.e. UE A obtains resources for such transmissions based on sensing.  For example, a high priority can be assigned or dedicated resources can be applied, e.g. a resource pool or a set frequency resources configured exclusively for SL transmissions carrying the selected resource set information.

- Proposal 3: Support Mode 2 mechanisms to improve reliability of the unicast transmissions carrying the selected resource set information to the peer UE.

The provided set of resources as discussed above can be used for resource selection for a SL transmission by UE B. To further enhance reliability, UE B can perform sensing for the SL transmission and consider combining both its own sensing result and the received set of resources for the resource selection, e.g. by applying only the resources in both sets.

- Proposal 4: Support UE combining of resources determined in its own sensing and received from another UE.

As discussed above, the inter-UE coordination of a unicast link can require a series of exchanges between the UEs and are more suitable for periodic traffic and aperiodic traffic with relaxed latency requirement.  Considering the reliability enhancement is mostly targeted at consecutive packet loss due to semi-persistent reservations, it is reasonable to start with discussions on inter-UE coordination for periodic traffic.

- Proposal 5: Support inter-UE coordination for at least periodic traffic.

A set of designated resources for exclusive use of a group transmission can become beneficial, e.g. when the resources are assigned by the network and used by group members operating SL transmissions in Mode 2.  These member UEs can be out-of-coverage or alternatively, they can be in-coverage and configured for Mode 2 for signalling efficiency, e.g. in a platoon application.  In this scenario, UE A can be a UE in coverage and assigned by higher layer to receive a set of resources from network for the group transmission and send them further a member UE, i.e. UE B.

Proposal 6: A member UE in a group receive a set of resources from the network for use of the group.

As UE A receives the set of resources for group use from the network, it is further helpful to receive a resource grant for the transmission of the received resource set to UE B.  This can ensure the reliability of the resource set transmission.

Proposal 7: A member UE in a group transmits the resource set using resources granted by the network.

The set of resources can be a resource pool or resources designated to the group.  Since the resources are granted by the network, they will lead to less collision compared to resources based on UE B’s own sensing.  UE B can further perform sensing within the resource set provided by UE A for resource selection.

- Proposal 8: A member UE in a group performs sensing and/or resource selection on the resources provided by another member UE.

# Conclusion

In this contribution, we discussed concepts and design aspects of inter-UE coordination based on the description provided in R17 SL Enhancement WID [1] for a candidate solution for enhancement of Mode 2 resource allocation.  We propose the following:

- Proposal 1: Support at least unicast and groupcast PSSCH transmissions using resources based on a set of resources indicated by a peer UE.

- Proposal 2: A UE provides a set of sidelink resources to the peer UE based on sensing in a PSSCH transmission for a unicast transmission.

- Proposal 3: Support Mode 2 mechanisms to improve reliability of the unicast transmissions carrying the selected resource set information to the peer UE.

- Proposal 4: Support UE combining of resources determined in its own sensing and received from another UE.

- Proposal 5: Support inter-UE coordination for at least periodic traffic.

Proposal 6: A member UE in a group receive a set of resources from the network for use of the group.

Proposal 7: A member UE in a group transmits the resource set using resources granted by the network.

- Proposal 8: A member UE in a group performs sensing and/or resource selection on the resources provided by another member UE.

# References

- RP-201385, “WID revision: NR sidelink enhancement”, LG Electronics

- R1-1809867, “Offline Summary for NR-V2X Agenda Item - 7.2.4.1.4 Resource Allocation Mechanism”, Intel